3GPP TSG-CT Meeting #90e	CP-203020

# E-Meeting, 06th – 08th December 2020

(revision of CP-yyxxxx)

# Source:	3GPP TSG CT WG4

# Title:	New WID on Restoration of PDN connections in PGW-C/SMF Set

# Document for:	Approval

Agenda Item:	17.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Restoration of PDN Connections in PGW-C/SMF Set

## Acronym: RPCPSET

## Unique identifier: 	900002

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 750025 | CT aspects of 5G System - Phase 1 | Specifies procedures to support 5GS PDU sessions |
| 820045 | Enhancements to the Service-Based 5G System Architecture | Specifies procedures to support NF set |

## 3	Justification

SMF scalability and resiliency can be supported for PDU sessions in 5GS using the SMF set concept, i.e. a set of SMF instances that are functionally equivalent and inter-changeable and that share the same contexts. This enables e.g. the AMF, PCF or UPF to reselect a different SMF in the same SMF set when the SMF serving a PDU session fails, restarts or is removed from the SMF set (scale-in operation), without interrupting the services and the PDN connectivity of the PDU session.

CT4 also defined procedures in Rel-16 enabling a UP function to reselect a PGW-C/SMF in the same PGW-C/SMF set, for a PFCP session, when the PGW-C/SMF earlier controlling the PFCP session is no longer reachable or available.

Inter-system mobility between 5GS and EPS relies on combo PGW-C/SMF. Combo PGW-C/SMF can be deployed in PGW-C/SMF set. However, no PDN connection restoration procedure has been defined in EPC to restore PDN connections supported by a PGW-C/SMF, when a PGW-C/SMF in a PGW-C/SMF set fails, restarts or is removed from the PGW-C/SMF set, which defeats the benefits of deploying PGW-C/SMF set since:

- It is not possible to scale-in a PGW-C/SMF set without tearing down and re-establishing all the PDN connections of the PGW-C/SMF that is removed from the set;

- An MME cannot reselect a different PGW-C/SMF in the PGW-C/SMF set for an on-going PDN connection if one PGW-C/SMF fails or restarts. All PDN connections are lost and need to be re-established;

- When a PCF or PGW-U/UPF reselects a different PGW-C/SMF from the PGW-C/SMF set, no procedure exists enabling to update the SGW and MME with new PGW-C/SMF information.

Tearing down and re-establishing PDN connections cause excessive signalling traffic towards the RAN and over the radio interface, and result in end user services disruption.

## 4	Objective

The objective of this work is to define a PDN connection restoration procedure that enables to restore PDN connections in EPC after a PGW-C/SMF failure, restart or scale-in operation, by allowing to move PDN connections to a different PGW-C/SMF of the PGW-C/SMF set.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.007 | New PDN connection restoration procedure that supports moving a PDN connection to a different PGW-C/SMF of a PGW-C/SMF set. | TSG#90 |  |
| 29.274 | GTPv2 enhancements to support PGW-C/SMF change | TSG#90 |  |
| 29.303 | MME procedure to discover a PGW-C/SMF in PGW-C/SMF set | TSG#90 |  |
| 23.003 | Definition of PGW Set FQDN | TSG#90 |  |

## 6	Work item Rapporteur(s)

LANDAIS, Bruno, Nokia, bruno.landais@nokia.com

## 7	Work item leadership

CT4

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Ericsson

Orange

NTT DOCOMO