3GPP TSG CT#90-e	CP-203069

Online – 7th – 9th December 2020	was C4-205710

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0417 | rev | 4 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCP Domain Routing Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2020-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a SCP belongs to multiple SCP Domains, it can bridge message between SCP domains it belongs to, i.e. the SCP Domains are considered "inter-connected" via the SCP. If a NFc needs to send a message to an NFp and NFc and NFp belong to different SCP domains that are not interconnected, intermediate SCP needs to relay the message to the target SCP domain. In order to find the routing path, the SCPc needs to hop by hop finding the interconnected SCP Domain and the serving SCPs from NRF, which leads to exponentially increasing complexity per extra hop. As SCPs registered to NRF with SCP domains information, the NRF can derive the SCP domain routing information and allow the SCP to fetch the complete routing information as well as subscribe to routing information change, thus to perform the path finding locally and significantly reduce the complexity of path finding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Introduce new service operation on Nnrf_NFDiscovery service to allow SCP to fetch the routing information, and further subscribe to routing information change, in 2/ Define resource for the new service operation In 6.2.3 3/ Specify new Notification in 6.2.5. 4/ Specify new data types for the new service operation in 6.2.6. 5/ Specify new Optional feature "SCPDRI" in 6.2.9. 6/ Update OpenAPI accordingly in A.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Significant complexity for path finding when NFc and NFp located in SCP domains that are not interconnected, which heavily impact system performance with indirect communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1, 5.3.2.a(New), 5.3.2.b(New), 5.3.2.c(New), 5.3.2.d(New), 6.2.3.1, 6.2.3.x(New), 6.2.3.y(New), 6.2.3.z(New), 6.2.6.1, 6.2.6.2.a(New) , 6.2.6.2.b(New) , 6.2.6.2.c(New) , 6.2.6.2.d(New), 6.2.5, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features into OpenAPI file of Nnrf_NFDiscovery API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: 1/ Optional feature "SCP Domain Routing Information (SCPDRI)" specified. 2/ Definition of Notification for SCP Domain Routing Information Change specified in 6.2.5. 3/ Editor's Note added for FFS on multiple NRF scenarios. . Rev2: Update release to Rel-17 Rev4: 1/ Add schema for {subscriptionId} in OpenAPI 2/ Fixed a typo in 5.2.2.d-1 figure title. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.3.2.1	Introduction

The service operations defined for the Nnrf_NFDiscovery service are as follows:

-	NFDiscover: It provides to the NF service consumer or SCP the profile (including IP address(es) or FQDN) of the NF Instance(s) or NF Service(s) matching certain input criteria. It also provides to the SCP the profile (including IP address(es) or FQDN) of the SCP Instance(s) matching certain input criteria.

The NFDiscover operation can be invoked by an NF Service Consumer (i.e., "source NF") or SCP requesting to discover NF instances (i.e., "target NFs") located in the same PLMN, or in a different PLMN. It can also be invoked by an SCP requesting to discover SCP instances located in the same PLMN.

In the description of these operations in clause 5.3.2.2, when the NF instances are located in the same PLMN, both source NF and target NFs are said to be located in the "Serving PLMN" but, in the general case, the functionality is not restricted to the PLMN that is serving a given UE, and it shall be applicable as well to any scenario in which source NF and target NFs belong to the same PLMN.

When source NF and target NFs are located in different PLMNs, the source NF is said to be in the "Serving PLMN", and the target NFs (and the NRF where they are registered) are said to be in the "Home PLMN", similarly to the scenarios described in 3GPP TS 23.502 [3], but the functionality shall be equally applicable to any scenario between any pair of PLMNs (e.g. with the source NF in the Home PLMN and the target NF in the Serving PLMN).

The SCP is treated by the Nnrf_NFDiscovery service in the same way as NFs. Specifically, the SCP is designated with a specific NF type and NF Instance ID. However, the SCP does not support services. Accordingly, references to "NF" or "NF Profile" in the description of the service operations in the following clauses also apply to an SCP.

-	SCPDomainRoutingInfoGet: It allows a service consumer (i.e. SCP) to fetch the SCP domain routing information (list of all SCP Domains registered by SCPs in the NRF and the interconnected SCP domains per SCP domain), if both the SCP and the NRF supports the "SCPDRI" feature.

NOTE:	Two SCP domains are considered interconnected when at least one SCP belongs to both SCP domains, i.e. at least one SCP can bridge messages between these two SCP domains.

-	SCPDomainRoutingInfoSubscribe: It allows a service consumer (i.e. SCP) to create a subscription for changes of the SCP domain routing information, if both the SCP and the NRF supports the "SCPDRI" feature.

-	SCPDomainRoutingInfoNotify: It allows the NRF to send notification(s) to a service consumer (i.e. SCP) previously subscribed to the changes of the SCP domain routing information, if both the SCP and the NRF supports the "SCPDRI" feature.

-	SCPDomainRoutingInfoUnsubscribe: It allows a service consumer (i.e. SCP) to delete a previously created subscription for changes of the SCP domain routing information, if both the SCP and the NRF supports the "SCPDRI" feature.

Editor's Note:	How the SCP Domain Routing Information will be aggregated when multiple NRFs are deployed in the network, i.e. SCPs register to different NRFs, is FFS.

* * * Next Change * * * *

#### 5.3.2.a	SCPDomainRoutingInfoGet

This service operation retrieves the SCP domain routing information, by sending a HTTP GET request to the resource URI representing the "SCP Domain Routing Information" resource.

Figure 5.3.2.a-1: SCP Domain Routing Information Get

1.	The Service Consumer (i.e. SCP) shall send an HTTP GET request to the resource URI "scp-domain-routing-info" document resource.

2a.	On success, "200 OK" shall be returned with SCP Domain Routing Information in response body. SCP Domain Routing Information with empty map indicates that no SCP domain is registered in the NRF.

2b.	On failure, the NRF shall return "4xx/5xx" response and the response body may contain a ProblemDetails object describing the detailed information of the failure.

* * * Next Change * * * *

#### 5.3.2.b	SCPDomainRoutingInfoSubscribe

This service operation is used to create a subscription to get notification when SCP Domain Routing Information is changed, e.g. due to a SCP has registered or update in the NRF or deregistered. The operation is invoked by issuing a POST request to the resource URI representing the "SCP Domain Routing Info Subscriptions" collection resource.

Figure 5.3.2.b-1: Subscription to SCP Domain Routing Information change

1.	The Service Consumer (i.e. SCP) shall send a POST request to the URI representing the "SCP Domain Routing Info Subscriptions" collection resource. The request body shall contain the callback URI on the Service Consumer to receive the notifications.

2a.	On success, "201 Created" shall be returned with "Location" header containing the resource URI to the newly created subscription resource. The response shall contain the data related to the created subscription, including the validity time, as determined by the NRF, after which the subscription becomes invalid. Once the subscription expires, if the Service Consumer wants to keep receiving notifications, it shall create a new subscription in the NRF.

2b.	On failure, the NRF shall return "4xx/5xx" response and the response body may contain a ProblemDetails object describing the detailed information of the failure.

* * * Next Change * * * *

#### 5.3.2.c	SCPDomainRoutingInfoNotify

This service operation notifies each subscriber for SCP Domain Routing Information change. The notification is sent to a callback URI that Service Consumer provided during the subscription (see SCPDomainRoutingInfoSubscribe operation in 5.3.2.b). The operation is invoked by sending a POST request to the callback URI.

Figure 5.3.2.c-1: Notification of SCP Domain Routing Info Change

1.	The NRF shall send a POST request to the callback URI. The request body shall contain the updated SCP Domain Routing Information. SCP Domain Routing Information with empty map indicates that no SCP domain is registered in the NRF after the change.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure, the NRF shall return "4xx/5xx" response and the response body may contain a ProblemDetails object describing the detailed information of the failure.

* * * Next Change * * * *

#### 5.3.2.d	SCPDomainRoutingInfoUnSubscribe

This service operation removes an existing subscription to SCP Domain Information Change. The operation is invoked by issuing a DELETE request on the resource URI representing the "Individual SCP Domain Routing Info Subscription", which was received in the Location header field of the "201 Created" response received during a successful subscription (see clause 5.3.2.b).

Figure 5.3.2.d-1: Unsubscribe to SCP Domain Routing Information Change

1.	The Service Consumer (i.e. SCP) shall send a DELETE request to the resource URI representing the individual subscription. The request body shall be empty.

2a.	On success, "204 No Content" shall be returned. The response body shall be empty.

2b.	On failure, the NRF shall return "4xx/5xx" response and the response body may contain a ProblemDetails object describing the detailed information of the failure.

* * * Next Change * * * *

#### 6.2.3.1	Overview

The structure of the Resource URIs of the NFDiscovery service is shown in figure 6.2.3.1-1.

Figure 6.2.3.1-1: Resource URI structure of the NFDiscovery API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| nf-instances (Store) | /nf-instances | GET | Retrieve a collection of NF Instances according to certain filter criteria. |
| Stored Search (Document) | /searches/{searchId} | GET | Retrieve a collection of NF Instances, previously stored by NRF as a consequence of a prior search result. |
| Complete Stored Search (Document) | /searches/{searchId}/complete | GET | Retrieve a collection of NF Instances, previously stored by NRF as a consequence of a prior search result, without applying any client restriction on the number of instances (e.g. "limit" or "max-payload-size" query parameters). |
| SCP Domain Routing Information (Document) | /scp-domain-routing-info | GET | Retrieve the SCP Domain Routing Information. |
| SCP Domain Routing Info Subscriptions (Collection) | /scp-domain-routing-info-subs | POST | Subscribe to SCP Domain Routing Information change. |
| Individual SCP Domain Routing Info Subscription (Document) | /scp-domain-routing-info-subs/{subscriptionID} | DELETE | Unsubscribe to SCP Domain Routing Information change. |

* * * Next Change * * * *

#### 6.2.3.x	Resource: SCP Domain Routing Informaiton (Document)

##### 6.2.3.x.1	Description

This resource represents SCP Domain Routing Information, calculated by NRF based on SCPs registered in the NRF.

This resource is modelled as the Document resource archetype (see clause C.3 of 3GPP TS 29.501 [5]).

##### 6.2.3.x.2	Resource Definition

Resource URI: {apiRoot}/nnrf-disc/v1/scp-domain-routing-info

This resource shall support the resource URI variables defined in table 6.2.3.x.2-1.

Table 6.2.3.x.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

###### 6.2.3.x.2.1	GET

This method retrieves the SCP Domain Routing Information.

This method shall support the URI query parameters specified in table 6.2.3.x.2.1-1.

Table 6.2.3.3.2.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.x.2.1-2 and the response data structure and response codes specified in table 6.2.3.x.2.1-3.

Table 6.2.3.3.2.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.2.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ScpDomainRoutingInfo | M | 1 | 200 OK | The response body contains SCP Domain Routing Information. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

* * * Next Change * * * *

#### 6.2.3.y	Resource: SCP Domain Routing Information Subscriptions (Collection)

##### 6.2.3.y.1	Description

This resource represents a collection of subscriptions of SCP Domain Routing Information.

##### 6.2.3.y.2	Resource Definition

Resource URI: {apiRoot}/nnrf-disc/v1/scp-domain-routing-info-subs

This resource shall support the resource URI variables defined in table 6.2.3.y.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.2.3.y.3	Resource Standard Methods

###### 6.2.3.y.3.1	POST

This method creates a new subscription. This method shall support the URI query parameters specified in table 6.2.3.y.3.1-1.

Table 6.2.3.y.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.y.3.1-2 and the response data structure and response codes specified in table 6.2.3.y.3.1-3.

Table 6.2.3.y.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ScpDomainRoutingInfoSubscription | M | 1 | The request body contains the input parameters for the subscription. |

Table 6.2.3.y.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ScpDomainRoutingInfoSubscription | M | 1 | 201 Created | This case represents the successful creation of a subscription. Upon success, the HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.4.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnrf-disc/v1/scp-domain-routing-info-subs/{subscriptionId} |

#### 6.2.3.z	Resource: Individual SCP Domain Routing Information Subscription (Document)

##### 6.1.3.z.1	Description

This resource represents an individual subscription of a given NF Instance to newly registered NF Instances.

##### 6.2.3.z.2	Resource Definition

Resource URI: {apiRoot}/nnrf-disc/v1/scp-domain-routing-info-subs/{subscriptionID}

This resource shall support the resource URI variables defined in table 6.2.3.z.2-1.

Table 6.2.3.z.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| subscriptionID | string | Represents a specific subscription |

##### 6.2.3.z.3	Resource Standard Methods

###### 6.2.3.z.3.1	DELETE

This method terminates an existing subscription. This method shall support the URI query parameters specified in table 6.2.3.z.3.1-1.

Table 6.2.3.z.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.z.3.1-2 and the response data structure and response codes specified in table 6.2.3.z.3.1-3.

Table 6.2.3.z.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.z.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

* * * Next Change * * * *

### 6.2.5	Notifications

#### 6.2.5.1	General

This clause specifies the notifications provided by the Nnrf_NFDiscovery service.

The delivery of notifications shall be supported as specified in clause 6.2 of 3GPP TS 29.500 [4] for Server-initiated communication.

Table 6.2.5.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| SCP Domain Routing Information Change Notification | {callbackUri} (NF Service Consumer provided callback reference) | POST | Notify about change of SCP Domain Routing Information |

#### 6.2.5.2	SCP Domain Routing Information Change Notification

##### 6.2.5.2.1	Description

The NF Service Consumer provides a callback URI for getting notified about change of SCP Domain Routing Information, the NRF shall notify the NF Service Consumer, when the SCP Domain Routing Information is updated.

##### 6.2.5.2.2	Notification Definition

The POST method shall be used for SCP Domain Routing Information Change Notification and the URI shall be the callback reference provided by the NF Service Consumer during the subscription to this notification.

Resource URI: {callbackUri}

Support of URI query parameters are specified in table 6.2.5.2.2-1.

Table 6.2.5.2.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.2.5.2.2-2, and support of response data structures and response codes is specified in table 6.2.5.2-3.

Table 6.2.5.2.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ScpDomainRoutingInfoNotification | M | 1 | Representation of the SCP Domain Routing Information Change Notification. |

Table 6.2.5.2.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | This case represents a successful notification of SCP Domain Routing Information Change. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

* * * Next Change * * * *

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nnrf service based interface protocol.

Table 6.2.6.1-1: Nnrf_NFDiscovery specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SearchResult | 6.2.6.2.2 | Contains the list of NF Profiles returned in a Discovery response. |
| NFProfile | 6.2.6.2.3 | Information of an NF Instance discovered by the NRF. |
| NFService | 6.2.6.2.4 | Information of a given NF Service Instance; it is part of the NFProfile of an NF Instance discovered by the NRF. |
| StoredSearchResult | 6.2.6.2.5 | Contains a complete search result (i.e. a number of discovered NF Instances), stored by NRF as a consequence of a prior search result. |
| PreferredSearch | 6.2.6.2.6 | Contains information on whether the returned NFProfiles match the preferred query parameters. |
| ScpDomainRoutingInfo | 6.2.6.2.a | SCP Domain Routing Information |
| ScpDomainConnectivity | 6.2.6.2.b | SCP Domain Routing Information |
| ScpDomainRoutingInfoSubscription | 6.2.6.2.c | SCP Domain Routing Information Subscription |
| ScpDomainRoutingInfoNotification | 6.2.6.2.d | Notification for SCP Domain Routing Information Update |

Table 6.2.6.1-2 specifies data types re-used by the Nnrf_NFDiscovery service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnrf_NFDiscovery service-based interface.

Table 6.2.6.1-2: Nnrf_NFDiscovery re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Snssai | 3GPP TS 29.571 [7] |  |
| PlmnId | 3GPP TS 29.571 [7] |  |
| Dnn | 3GPP TS 29.571 [7] |  |
| Tai | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] |  |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| Uri | 3GPP TS 29.571 [7] |  |
| Gpsi | 3GPP TS 29.571 [7] |  |
| GroupId | 3GPP TS 29.571 [7] |  |
| Guami | 3GPP TS 29.571 [7] |  |
| IPv4Addr | 3GPP TS 29.571 [7] |  |
| IPv6Addr | 3GPP TS 29.571 [7] |  |
| UriScheme | 3GPP TS 29.571 [7] |  |
| Dnai | 3GPP TS 29.571 [7] |  |
| NfGroupId | 3GPP TS 29.571 [7] | Identifier of a NF Group |
| PduSessionType | 3GPP TS 29.571 [7] |  |
| AtsssCapability | 3GPP TS 29.571 [7] |  |
| PlmnIdNid | 3GPP TS 29.571 [7] |  |
| NfSetId | 3GPP TS 29.571 [7] |  |
| NfServiceSetId | 3GPP TS 29.571 [7] |  |
| ExtSnssai | 3GPP TS 29.571 [7] |  |
| EventId | 3GPP TS 29.520 [32] | Defined in Nnwdaf_AnalyticsInfo API. |
| NwdafEvent | 3GPP TS 29.520 [32] | Defined in Nnwdaf_EventsSubscription API. |
| ExtGroupId | 3GPP TS 29.503 [36] |  |
| ExternalClientType | 3GPP TS 29.572 [33] |  |
| DefaultNotificationSubscription | 3GPP TS 29.510 | See clause 6.1.6.2.4 |
| IPEndPoint | 3GPP TS 29.510 | See clause 6.1.6.2.5 |
| NFType | 3GPP TS 29.510 | See clause 6.1.6.3.3 |
| UdrInfo | 3GPP TS 29.510 | See clause 6.1.6.2.6 |
| UdmInfo | 3GPP TS 29.510 | See clause 6.1.6.2.7 |
| AusfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.8 |
| SupiRange | 3GPP TS 29.510 | See clause 6.1.6.2.9 |
| AmfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.11 |
| SmfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.12 |
| UpfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.13 |
| PcfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.20 |
| BsfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.21 |
| ChfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.32 |
| ChfServiceInfo | 3GPP TS 29.510 | See clause 6.1.6.2.33 |
| NFServiceVersion | 3GPP TS 29.510 | See clause 6.1.6.2.19 |
| PlmnSnssai | 3GPP TS 29.510 | See clause 6.1.6.2.44 |
| NwdafInfo | 3GPP TS 29.510 | See clause 6.1.6.2.45 |
| NFStatus | 3GPP TS 29.510 | See clause 6.1.6.3.7 |
| DataSetId | 3GPP TS 29.510 | See clause 6.1.6.3.8 |
| ServiceName | 3GPP TS 29.510 | See clause 6.1.6.3.11 |
| NFServiceStatus | 3GPP TS 29.510 | See clause 6.1.6.3.12 |
| LmfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.46 |
| GmlcInfo | 3GPP TS 29.510 | See clause 6.1.6.2.47 |
| NefInfo | 3GPP TS 29.510 | See clause 6.1.6.2.48 |
| PfdData | 3GPP TS 29.510 | See clause 6.1.6.2.49 |
| AfEventExposureData | 3GPP TS 29.510 | See clause 6.1.6.2.50 |
| PcscfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.53 |
| HssInfo | 3GPP TS 29.510 | See clause 6.1.6.2.57 |
| ImsiRange | 3GPP TS 29.510 | See clause 6.1.6.2.58 |
| VendorSpecificFeature | 3GPP TS 29.510 | See clause 6.1.6.2.62 |
| ScpInfo | 3GPP TS 29.510 | See clause 6.1.6.2.65 |
| NefId | 3GPP TS 29.510 | See clause 6.1.6.3 |
| VendorId | 3GPP TS 29.510 | See clause 6.1.6.3 |
| AnNodeType | 3GPP TS 29.510 | See clause 6.1.6.3.13 |

* * * Next Change * * * *

##### 6.2.6.2.a	Type: ScpDomainRoutingInformation

Table 6.1.6.2.a-1: Definition of type ScpDomainRoutingInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| scpDomainList | map(ScpDomainConnectivity) | M | 0..N | This IE shall contain map of SCP domain interconnection information, where the key of the map is a SCP domain. The value of each entry shall be the interconnectivity information of the SCP domain indicated by the key. An empty map indicates that there is no SCP domain currently registered in the NRF. |

EXAMPLE:	The SCP Domain Routing Information is derived from the SCP domains registered by SCPs, e.g. if SCP x, SCP y and SCP z have registered in NRF with following SCP domains:

SCP x Profile includes: {"scpDomains":["SCP_Domain_1","SCP_Domain_2"];}

SCP y Profile includes: {"scpDomains":["SCP_Domain_2","SCP_Domain_3"];}

SCP z Profile includes: {"scpDomains":["SCP_Domain_4"];}

then the SCP Domain Routing Information should be as following:

{

"SCP_Domain_1":{"connectedScpDomainList":["SCP_Domain_2"];};

"SCP_Domain_2":{"connectedScpDomainList":["SCP_Domain_1","SCP_Domain_3"];};

"SCP_Domain_3":{"connectedScpDomainList":["SCP_Domain_2"];};

"SCP_Domain_4":{"connectedScpDomainList":[];};

}

* * * Next Change * * * *

##### 6.2.6.2.b	Type: ScpDomainConnectivity

Table 6.1.6.2.b-1: Definition of type ScpDomainConnectivity

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| connectedScpDomainList | array(string) | M | 0..N | This IE shall contain the list of interconnected SCP domains. An empty array indicates there is no SCP Domain currently interconnected. |

* * * Next Change * * * *

##### 6.2.6.2.c	Type: ScpDomainRoutingInfoSubscription

Table 6.1.6.2.c-1: Definition of type ScpDomainRoutingInfoSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| callbackUri | Uri | M | 1 | Callback URI where the Service Consumer will receive the notifications from NRF. |
| validityTime | DateTime | C | 0..1 | Time instant after which the subscription becomes invalid. This parameter may be sent by the client, as a hint to the server, but it shall be always sent back by the server (regardless of the presence of the attribute in the request) in the response to the subscription creation request. |
| reqInstanceId | NfInstanceId | O | 0..1 | If present, this IE shall contain the NF instance id of the Service consumer. |

* * * Next Change * * * *

##### 6.2.6.2.d	Type: ScpDomainRoutingInfoNotification

Table 6.1.6.2.d-1: Definition of type ScpDomainRoutingInfoNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| routingInfo | ScpDomainRoutingInformation | M | 1 | This IE shall contain the SCP Domain Routing Information after the change. |

* * * Next Change * * * *

### 6.2.9	Features supported by the NFDiscovery service

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [7].

The following features are defined for the Nnrf_NFDiscovery service.

Table 6.2.9-1: Features of supportedFeatures attribute used by Nnrf_NFDiscovery service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | Complex-Query | O | Support of Complex Query expression (see clause 6.2.3.2.3.1) |
| 2 | Query-Params-Ext1 | O | Support of the following query parameters: - limit - max-payload-size - required-features - pdu-session-types |
| 3 | Query-Param-Analytics | O | Support of the query parameters for Analytics identifier: - event-id-list - nwdaf-event-list |
| 4 | MAPDU | O | This feature indicates whether the NRF supports selection of UPF with ATSSS capability. |
| 5 | Query-Params-Ext2 | O | Support of the following query parameters: - requester-nf-instance-id - upf-ue-ip-addr-ind - pfd-data - target-snpn - af-ee-data - w-agf-info - tngf-info - twif-info - target-nf-set-id - target-nf-service-set-id - preferred-tai - nef-id - preferred-nf-instances - notification-type - serving-scope - internal-group-identity - preferred-api-versions - v2x-support-ind - redundant-gtpu - redundant-transport - lmf-id - an-node-type - rat-type - ipups - scp-domain-list - address-domain - ipv4-addr - ipv6-prefix - served-nf-set-id - remote-plmn-id - data-forwarding - preferred-full-plmn - requester-snpn-list - max-payload-size-ext |
| 6 | Service-Map | M | This feature indicates whether it is supported to identify the list of NF Service Instances as a map (i.e. the "nfServiceList" attribute of NFProfile is supported). |
| 7 | Query-Params-Ext3 | O | Support of the following query parameters: - ims-private-identity - ims-public-identity - msisdn - requester-plmn-specific-snssai-list - n1-msg-class - n2-info-class |
| x | SCPDRI | O | SCP Domain Routing Information An NRF supporting this feature shall allow a service consumer (i.e. a SCP) to get the SCP Domain Routing Information and subscribe/unsubscribe to the change of SCP Domain Routing Information with following service operations: - SCPDomainRoutingInfoGet (see clause 5.3.2.a) - SCPDomainRoutingInfoSubscribe (see clause 5.3.2.b) - SCPDomainRoutingInfoUnsubscribe (see clause 5.3.2.d) A service consumer (i.e. a SCP) supporting this feature shall be able to handle SCPDomainRoutingInfoNotify as specified in clause 5.3.2.c, if subscribed to the change of SCP Domain Routing Information in the NRF. |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

* * * Next Change * * * *

## A.3	Nnrf_NFDiscovery API

openapi: 3.0.0

info:

version: '1.1.1'

title: 'NRF NFDiscovery Service'

description: |

NRF NFDiscovery Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.510 V16.5.0; 5G System; Network Function Repository Services; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.510/'

servers:

- url: '{apiRoot}/nnrf-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnrf-disc

*************** Text Skipped for Clarify ***********************

/searches/{searchId}:

get:

operationId: RetrieveStoredSearch

tags:

- Stored Search (Document)

parameters:

- $ref: '#/components/parameters/searchId'

- name: Accept-Encoding

in: header

description: Accept-Encoding, described in IETF RFC 7231

schema:

type: string

responses:

'200':

$ref: '#/components/responses/200'

/searches/{searchId}/complete:

get:

operationId: RetrieveCompleteSearch

tags:

- Complete Stored Search (Document)

parameters:

- $ref: '#/components/parameters/searchId'

- name: Accept-Encoding

in: header

description: Accept-Encoding, described in IETF RFC 7231

schema:

type: string

responses:

'200':

$ref: '#/components/responses/200'

/scp-domain-routing-info:

get:

operationId: SCPDomainRoutingInfoGet

tags:

- SCP Domain Routing Information (Document)

parameters:

- name: Accept-Encoding

in: header

description: Accept-Encoding, described in IETF RFC 7231

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScpDomainRoutingInformation'

headers:

Content-Encoding:

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

'307':

description: Temporary Redirect

headers:

Location:

description: 'The URI pointing to the resource located on the redirect target NRF'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/scp-domain-routing-info-subs:

post:

summary: Create a new subscription

operationId: ScpDomainRoutingInfoSubscribe

tags:

- SCP Domain Routing Information Subscriptions (Collection)

parameters:

- name: Content-Encoding

in: header

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

- name: Accept-Encoding

in: header

description: Accept-Encoding, described in IETF RFC 7231

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ScpDomainRoutingInfoSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScpDomainRoutingInfoSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnrf-disc/v1/scp-domain-routing-info-subs/{subscriptionID}'

required: true

schema:

type: string

Accept-Encoding:

description: Accept-Encoding, described in IETF RFC 7694

schema:

type: string

Content-Encoding:

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

onScpDomainRoutingInformaitonChange:

'{$request.body#/callbackUri}':

post:

parameters:

- name: Content-Encoding

in: header

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

requestBody:

description: Notification Payload

content:

application/json:

schema:

$ref: '#/components/schemas/ScpDomainRoutingInfoNotification'

responses:

'204':

description: Expected response to a successful callback processing

headers:

Accept-Encoding:

description: Accept-Encoding, described in IETF RFC 7694

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/scp-domain-routing-info-subs/{subscriptionID}:

delete:

summary: Deletes a subscription

operationId: ScpDomainRoutingInfoUnsubscribe

tags:

- Individual SCP Domain Routing Information Subscription (Document)

parameters:

- name: subscriptionID

in: path

required: true

description: Unique ID of the subscription to remove

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

*************** Text Skipped for Clarify ***********************

PreferredSearch:

description: Contains information on whether the returned NFProfiles match the preferred query parameters

type: object

properties:

preferredTaiMatchInd:

type: boolean

default: false

preferredFullPlmnMatchInd:

type: boolean

default: false

ScpDomainRoutingInformation:

description: SCP Domain Routing Information

type: object

required:

- scpDomainList

properties:

scpDomainList:

description: |

This IE shall contain map of SCP domain interconnection information, where

the key of the map is a SCP domain. The value of each entry shall be the

interconnectivity information of the the SCP domain indicated by the key.

An empty map indicates that there is no SCP domain currently registered in

the NRF.

type: object

additionalProperties:

$ref: '#/components/schemas/ScpDomainConnectivity'

ScpDomainConnectivity:

description: SCP Domain Connectivity Information

type: object

required:

- connectedScpDomainList

properties:

connectedScpDomainList:

type: array

items:

type: string

ScpDomainRoutingInfoSubscription:

description: SCP Domain Routing Information Subscription

type: object

required:

- callbackUri

properties:

callbackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

reqInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ScpDomainRoutingInfoNotification:

description: SCP Domain Routing Information Notification

type: object

required:

- routingInfo

properties:

routingInfo:

$ref: '#/components/schemas/ScpDomainRoutingInformation'

* * * End of Changes * * * *