3GPP TSG-CT6 Meeting #103-e	C6-200716

# Online, 17th Nov 2020 - 20th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0568 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TC 27.22.13.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Comprion GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Alignment of the ENVELOPE (CALL CONTROL) defininitions in the test cases with the envelope structure and usage defined in 3GPP TS 31.111 and the command header definitions specified in 3GPP TS 31.101. Editorial corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the NG-RAN Cell Identifier. Deletion of capability configuration parameters not used for PDU session establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The envelopes used in the sequences are not aligned with 3GPP TS 31.111 and 3GPP TS 31.101. Correctly implemented MEs might unfairly fail the tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 27.22.13.1.4.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

…

### 27.22.13	CALL CONTROL on PDU Session Establishment for NG-RAN

…

**** editorial changes ****

##### 27.22.13.1.2	Conformance requirement

The ME shall support the CALL CONTROL facility as defined in:

-	3GPP TS 31.111 [15] clause 7.3.1.6, clause 7.3.1.10, clause 8.143

-	3GPP TS 24.501 [40], clause 6.4.1, 8.3.1, 9.11.4.6 and 9.11.4.15.

…

**** editorial change ****

##### 27.22.13.1.3	Test purpose

To verify that when the service 128 "Call Control on PDU Session by USIM" is available in the USIM Service Table, then for all PDU Session Establishment Requests (including those resulting from an OPEN CHANNEL proactive command where NG-RAN is selected), the ME shall first pass the corresponding PDU Session Establishment Request message to the UICC, using the ENVELOPE (CALL CONTROL) command. The ME shall also pass to the UICC in the ENVELOPE (CALL CONTROL) command the current serving cell.

To verify the PDU Session Establishment Request parameters are used in the ENVELOPE (CALL CONTROL), as defined in 3GPP TS 24.501 [40].

…

**** editorial changes ****

27.22.13.1.4.2	Procedure

Expected Sequence 1.1 (CALL CONTROL on PDU Session for NG-RAN, always-on PDU Session Establishment, allowed without modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure DNN as "TestGp.rs" in the terminal configuration for Always-on if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For Always-on PDU Session establishment |
| 2 | UICC ME | CALL CONTROL RESULT 1.1.1 | [Call control result: "Allowed", no modification] |
| 3 | ME NG-SS | The PDU Session Establishment is successfull without modification | Same PDU Session Establishment parameters used by the ME within the ENVELOPE CALL CONTROL are used to establish the PDU Session |

…

**** first change **** **** editorial changes ****

ENVELOPE CALL CONTROL 1.1.1

Logically:

Device identities:

Source device:							ME

Destination device:							UICC

PDU Session Establishment parameters:

DNN:							TestGp.rs

Extended Protocol Discriminator:			5GS Session Management messages (2E)

PDU Session identity:							No PDU Session identity assigned (01)

Procedure Transaction Identity:				1

5GS Session message identity:				PDU session establishment request (C1)

PDU Session Type:							IPv4v6

Extended Protocol configuration options:

Extended Protocol config. optional contents:		content not checked

Location Information	:

Mobile Country Codes (MCC):				001

Mobile Network Codes (MNC):			01

Tracking Area Code (TAC):					000001

NG-RAN Cell Identifier (NCI):				0001 (36 bits)

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 0C | Note 2 | 0A | 06 | 54 | 65 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 2E | 01 | 01 | C1 | Note 3 |
|  | Note 3 | 93 | Note 4 | 13 | 0B | 00 | F1 | 10 | 00 | 00 | 01 | 00 |
|  | 00 | 00 | 00 | 1F |

Note 1: The length of the BER-TLV is present here.

Note 2: Length of PDU session establishment parameters, dependent of optional fields.

Note 3: Integrity protection maximum data rate

Note 4:

Optional fields / extended protocol configuration options. (byte 27 to 27+n if available)

…

**** editorial change ****

CALL CONTROL RESULT 1.1.1

Logically:

Call control result:	'00' = Allowed, no modification

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

…

**** editorial changes ****

Expected Sequence 1.2 (CALL CONTROL on PDU Session for NG-RAN, always-on PDU Session Establishment, Not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure DNN as "TestGp.rs" in the terminal configuration for Always-on if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For Always-on PDU Session establishment |
| 2 | UICC ME | CALL CONTROL RESULT 1.2.1 | [Call control result: "Not Allowed"] |
| 3 | ME NG-SS | ME shall not send PDU Session Establishment Request | No Always-on PDU Session is established |

…

**** editorial change ****

CALL CONTROL RESULT 1.2.1

Logically:

Call control result:	'01' = Not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

…

**** editorial changes ****

Expected Sequence 1.3 (CALL CONTROL on PDU Session for NG-RAN, PDU Session Establishment triggered by user, UICC sends 90 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure DNN as "TestGp.rs" in the terminal configuration for Always-on if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For Always-on PDU Session establishment |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure DNN as "Test12.rs" in the terminal configuration for a new PDU Session if required and trigger ME to establish the PDU Session. | [PDU Session triggered by the user] |
| 4 | ME  UICC | ENVELOPE CALL CONTROL 1.3.1 |  |
| 5 | UICC  ME | 90 00 |  |
| 6 | ME NG-SS | PDU SESSION ESTABLISHMENT REQUEST | Same PDU Session Establishment parameters within the CALL CONTROL ENVELOPE in step 4 are used to establish the PDU Session |
| 7 | NG-SS ME | PDU SESSION ESTABLISHMENT ACCEPT |  |
| 8 | ME NG-SS | The PDU Session is established successfully without modification |  |

…

**** next change **** **** editorial changes ****

ENVELOPE CALL CONTROL 1.3.1

Logically:

Device identities:

Source device:							ME

Destination device:							UICC

PDU Session Establishment parameters:

DNN:							Test12.rs

Extended Protocol Discriminator:			5GS Session Management messages (2E)

PDU Session identity:								No PDU Session identity assigned (01)

Procedure Transaction Identity:				1

5GS Session message identity:				PDU session establishment request (C1)

PDU Session Type:							IPv4v6

Extended Protocol configuration options:

Extended Protocol config. optional contents:		content not checked

Location Information	:

Mobile Country Codes (MCC):					001

Mobile Network Codes (MNC):			01

Tracking Area Code (TAC):					000001

NG-RAN Cell Identifier (NCI):					0001 (36 bits)

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 0C | Note 2 | 0A | 06 | 54 | 65 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 73 | 74 | 31 | 32 | 02 | 72 | 73 | 2E | 01 | 01 | C1 | Note 3 |
|  | Note 3 | 93 | Note 4 | 0B | 00 | F1 | 10 | 00 | 00 | 01 | 00 | 00 |
|  | 00 | 00 | 1F |

Note 1: The length of the BER-TLV is present here.

Note 2: Length of PDU session establishment parameters, dependent of optional fields.

Note 3: Integrity protection maximum data rate

Note 4:

Optional fields / extended protocol configuration options. (byte 27 to 27+n if available).

…

**** editorial changes ****

Expected Sequence 1.4 (CALL CONTROL on PDU Session for NG-RAN, PDU Session Establishment triggered by user, UICC sends 93 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure DNN as "TestGp.rs" in the terminal configuration for Always-on if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For Always-on PDU Session establishment |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure DNN as "Test12.rs" in the terminal configuration for a new PDU Session if required and trigger ME to establish the PDU Session | [PDU Session triggered by the user] |
| 4 | ME  UICC | ENVELOPE CALL CONTROL 1.3.1 |  |
| 5 | UICC  ME | 93 00 |  |
| 6 | ME NG-SS | ME shall not sent PDU SESSION ESTABLISHMENT REQUEST | The ME may retry to send the command. |

…

**** editorial changes ****

Expected Sequence 1.5 (CALL CONTROL on PDU Session for NG-RAN, PDU Session Establishment triggered by user, allowed with modification of SM PDU DN request container)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure DNN as "TestGp.rs" in the terminal configuration for Always-on if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | [For Always-on PDU Session establishment] |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure DNN as "Test12.rs" and DN-Specific Identity as 0123456789@Test.org in the terminal configuration for a new PDU Session if required and trigger ME to establish the PDU Session | [PDU Session triggered by the user] |
| 4 | ME  UICC | ENVELOPE CALL CONTROL 1.5.1 |  |
| 5 | UICC  ME | CALL_CONTROL_RESULT 1.5.1 | [Call control result: "Allowed with modifications",DN-Specific Identity is modified to 0123456789@Test3gpp.org] NOTE: DNN is also modified to Test34.rs |
| 6 | ME NG-SS | PDU SESSION ESTABLISHMENT REQUEST | [The modified SM PDU DN request container and rest of the same PDU Session Establishment parameters within the CALL CONTROL ENVELOPE in step 4 are used to establish the PDU Session.] |
| 7 | NG-SS ME | PDU SESSION ESTABLISHMENT ACCEPT |  |
| 8 | ME NG-SS | The PDU Session is established successfully with modified SM PDU DN request container (DN-Specific Identity). | NOTE: DNN should be same as the DNN in CALL CONTROL ENVELOPE, Test12.rs. |

…

**** next change **** **** editorial changes ****

ENVELOPE CALL CONTROL 1.5.1

Logically:

Device identities:

Source device:							ME

Destination device:							UICC

PDU Session Establishment parameters:

DNN:							Test12.rs

Extended Protocol Discriminator:			5GS Session Management messages (2E)

PDU Session identity:		No PDU Session identity assigned (01)

Procedure Transaction Identity:				1

5GS Session message identity:				PDU session establishment request (C1)

PDU Session Type:							IPv4v6

SM PDU DN request container							0123456789@Test.org

Extended Protocol config:

Extended Protocol configuration options:

Extended Protocol config. optional contents:		content not checked

Location Information	:

Mobile Country Codes (MCC):				001

Mobile Network Codes (MNC):			01

Tracking Area Code (TAC):					000001

NG-RAN Cell Identifier (NCI):				0001 (36 bits)

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 0C | Note 2 | 0A | 06 | 54 | 65 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 73 | 74 | 31 | 32 | 02 | 72 | 73 | 2E | 01 | 01 | C1 | Note 3 |
|  | Note 3 | 93 | Note 4 | 39 | 13 | 30 | 31 | 32 | 33 | 34 | 35 | 36 |
|  | 37 | 38 | 39 | 40 | 54 | 65 | 73 | 74 | 2E | 6F | 72 | 67 |
|  | Note 4 | 13 | 0B | 00 | F1 | 10 | 00 | 00 | 01 | 00 | 00 | 00 |
|  | 00 | 1F |

Note 1: The length of the BER-TLV is present here.

Note 2: Length of PDU session establishment parameters, dependent of optional fields.

Note 3: Integrity protection maximum data rate

Note 4: Optional fields / extended protocol configuration options. (byte 27 to 27+n if available, second Note 4 is handled accordingly).

…

**** editorial changes ****

CALL CONTROL RESULT 1.5.1

Logically:

Call control result:								'02' = Allowed, with modification

PDU Session Establishment parameters:

DNN:										Test34.rs

Extended Protocol Discriminator:				5GS Session Management messages (2E)

PDU Session identity:					No PDU Session identity assigned (01)

Procedure Transaction Identity:				1

5GS Session message identity:			PDU session establishment request (C1)

PDU Session Type:	IPv4v6

SM PDU DN request container:			0123456789@Test3gpp.org

Extended Protocol configuration options:

Extended Protocol config. optional contents:		content not checked

Coding:

| BER-TLV: | 02 | Note 1 | 0C | Note 2 | 0A | 06 | 54 | 65 | 73 | 74 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | 72 | 73 | 2E | 01 | 01 | C1 | Note 3 | 93 | Note 4 | 39 | 17 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 54 |
|  | 65 | 73 | 74 | 33 | 67 | 70 | 70 | 2E | 6F | 72 | 67 | Note 5 |

Note 1: The length of the BER-TLV is present here.

Note 2: Length of PDU session establishment parameters, dependent of optional fields.

Note 3: Integrity protection maximum data rate (same as received in ENVELOPE)

Note 4: Optional fields / extended protocol configuration options (same as received in ENVELOPE)

Note 5:

Optional fields (if available in ENVELOPE)

…

**** editorial change ****

Expected Sequence 1.6 (CALL CONTROL on PDU Session for NG-RAN, PDU Session Establishment triggered by user, allowed with modification of ePCO)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure DNN as "TestGp.rs" in the terminal configuration for Always-on if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | [For Always-on PDU Session establishment] |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure DNN as "Test12.rs" and the terminal is configured to include any ePCO containers in PDU Session Establishment if required and trigger ME to establish the PDU Session. | [PDU Session triggered by the user] |
| 4 | ME  UICC | ENVELOPE CALL CONTROL 1.6.1 |  |
| 5 | UICC  ME | CALL_CONTROL_RESULT 1.6.1 | [Call control result: "Allowed with modifications", include rfc1877_primary_dns_server IPv4 Address as 192.168.3.3 in IPCP container] NOTE: DNN is also modified to Test34.rs |
| 6 | ME NG-SS | PDU SESSION ESTABLISHMENT REQUEST | [The modified SM PDU DN request container and rest of the same PDU Session Establishment parameters within the CALL CONTROL ENVELOP in step 4 are used to establish the PDU Session.] |
| 7 | NG-SS ME | PDU SESSION ESTABLISHMENT ACCEPT |  |
| 8 | ME NG-SS | The PDU Session is established successfully with modified ePCO as included in the CALL_CONTROL_RESULT 1.6.1). | NOTE: DNN should be same as the DNN in CALL CONTROL ENVELOP, Test12.rs. |

…

**** editorial change ****

ENVELOPE CALL CONTROL 1.6.1

Logically:

Device identities

Source device:						ME

Destination device:						UICC

PDU Session Establishment parameters

DNN:										Test12.rs

Extended Protocol Discriminator:			5GS Session Management messages (2E)

PDU Session identity:						No PDU Session identity assigned (01)

Procedure Transaction Identity:			1

5GS Session message identity:			PDU session establishment request (C1)

PDU Session Type:						IPv4v6

Extended Protocol configuration options:

Extended Protocol config. optional contents: 		content not checked

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information:

Mobile Country Codes (MCC):			001

Mobile Network Codes (MNC):			01

Tracking Area Code (TAC):				000001

NG-RAN Cell Identifier (NCI):			0001 (36 bits)

Capability configuration parameters 2:

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 0C | Note 2 | 0A | 06 | 54 | 65 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 73 | 74 | 31 | 32 | 02 | 72 | 73 | 2E | 01 | 01 | C1 | Note 3 |
|  | Note 3 | 93 | Note 4 | 7B | Note 5 | Note 5 | Note 6 | Note 4 | 13 | 0B | 00 | F1 |
|  | 10 | 00 | 00 | 01 | 00 | 00 | 00 | 00 | 1F |

NOTE 1: The length of the BER-TLV is present here

NOTE 2: Length of PDU session establishment parameters, dependent on optional fields

NOTE 3: Integrity protection maximum data rate

NOTE 4: Optional fields / extended protocol configuration options. (byte 27 to 27+n if available, second Note 4 is handled accordingly)

NOTE 5: Length of the ePCO (2 bytes)

NOTE 6: Data of extended protocol configuration fields

…

**** end of textual changes ****

**** editorial change ****

CALL CONTROL RESULT 1.6.1

Logically:

Call control result:									'02' = Allowed, with modification

PDU Session Establishment parameters:

DNN:												Test34.rs

Extended Protocol Discriminator:				5GS Session Management messages (2E)

PDU Session identity:						No PDU Session identity assigned (01)

Procedure Transaction Identity:						1

5GS Session message identity:						PDU session establishment request (C1)

PDU Session Type:						IPv4v6

Extended Protocol configuration options:

Protocol or Container:						IPCP

rfc1332_conf_req

rfc1877_primary_dns_server_IPv4_address:	192.168.3.3

rfc1877_sec_dns_server_IPv4_address:		0.0.0.0

Protocol or Container:						DNS Server IPv4 Address Request

Protocol or Container:						DNS Server IPv6 Address Request

Protocol or Container:						P-CSCF IPv6 Address Request

Protocol or Container:						P-CSCF IPv4 Address Request

Coding:

| BER-TLV: | 02 | Note 1 | 0C | Note 2 | 0A | 06 | 54 | 65 | 73 | 74 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | 72 | 73 | 2E | 01 | 01 | C1 | Note 3 | Note 3 | 93 | Note 4 | 7B |
|  | Note 5 | Note 5 | 80 | 80 | 21 | Note 6 | 01 | 00 | Note 7 | Note 7 | 81 | 06 |
|  | C0 | A8 | 03 | 03 | Note 8 |

NOTE 1: The length of the BER-TLV is present here

NOTE 2: Length of PDU session establishment parameters, dependent of optional fields

NOTE 3: Integrity protection maximum data rate (same as received in ENVELOPE)

NOTE 4: Optional fields / extended protocol configuration options (same as received in ENVELOPE)

NOTE 5: Length of the ePCO (2 bytes, same as received in ENVELOPE)

NOTE 6: Length of the IPCP content

NOTE 7: Length of rfc1332_conf_req

NOTE 8: Additional protocol and container data as provided in ENVELOPE

…

**** editorial change ****

Expected Sequence 1.7 (CALL CONTROL on PDU Session for NG-RAN, PDU Session Establishment triggered by OPEN CHANNEL, UICC sends 90 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure DNN as "TestGp.rs" in the terminal configuration for Always-on if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For Always-on PDU Session establishment |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure DNN as "Test12.rs" in the terminal configuration for a new PDU Session by the OPEN CHANNEL if required. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.7.1 | [PDU Session triggered by the proactive command OPEN CHENNEL with Bearer Type 0x0C] |
| 5 | ME UICC | FETCH |  |
| 6 | ME USER | The ME may display channel opening information |  |
| 7 | ME  UICC | ENVELOPE CALL CONTROL 1.3.1 |  |
| 8 | UICC  ME | 90 00 |  |
| 9 | ME NG-SS | PDU SESSION ESTABLISHMENT REQUEST | Same PDU Session Establishment parameters within the CALL CONTROL ENVELOPE in step 4 are used to establish the PDU Session. |
| 10 | NG-SS ME | PDU SESSION ESTABLISHMENT ACCEPT |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.7.1 | [Command performed successfully] |
| 12 | ME NG-SS | The PDU Session is established successfully without modification |  |

…

**** editorial change ****

PROACTIVE COMMAND: OPEN CHANNEL 1.7.1

Logically:

Command details:

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities:

Source device:	UICC

Destination device:	ME

Bearer description:

Bearer type:	NG-RAN

Bearer parameter:

PDU Session Type:	IPv4v6

Buffer:

Buffer size:				1400

Network access name:			TestGp.rs

Text String:						"UserLog" (User login)

Text String:						"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number: 	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 3D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | 0C | 93 | 39 | 02 | 05 | 78 | 47 | 0A | 06 | 54 | 65 |
|  | 73 | 74 | 31 | 32 | 02 | 72 | 73 | 0D | 08 | F4 | 55 | 73 |
|  | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 | 55 | 73 | 65 | 72 |
|  | 50 | 77 | 64 | 3C | 03 | 02 | AD | 9C | 3E | 05 | 21 | 01 |
|  | 01 | 01 | 01 |

…

**** editorial change ****

TERMINAL RESPONSE: OPEN CHANNEL 1.7.1

Logically:

Command details:

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities:

Source device:	ME

Destination device:	UICC

Result:

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established, or PDP context activated or PDU Session established

Bearer description:

Bearer type:	NG-RAN

Bearer parameter:

PDU Session Type:	IPv4v6

Buffer:

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 02 | 0C | 93 | 39 | 02 | 05 | 78 |