3GPP TSG-CT6 Meeting #103-e	C6-200726

# Online, 17th Nov 2020 - 20th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0570 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TC 27.22.7.12 Seq 1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Comprion GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If no Access technology is available respectively not access technology change is detected no such ENVELOPE can be send. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the unneeded step 6b from Expected sequence 1.4, change step 6a to step 6. Minor editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test procedure in Expected sequence 1.4 is incorrect and might cause MEs implemented in accordance to Rel-13 and later to fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 27.22.7.12.1.2, 27.22.7.12.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

…

#### 27.22.7.12	Access Technology Change event

…

**** editorial change ****

27.22.7.12.1.2	Conformance requirement

The ME shall support the EVENT: Access Technology Change event as defined in:

-	3GPP TS 31.111 [15] clause 4.7, 4.12, 7.5.12 and clause 8.61.

…

**** editorial change ****

27.22.7.12.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD – Access Technology Change, single access technology)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4a | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 4b | ME UICC | ENVELOPE: EVENT DOWNLOAD - Access technology change Event | This step applies only if A.1/171 |
| 5 | E-USS | ME detects a change in its current access technology | E-UTRA cell is enabled and UTRA cell is disabled |
| 6 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.1 | Access Technology = E-UTRAN |
| 7 | E-USS | ME detects a change in its current access technology | E-UTRA cell is disabled and UTRA cell is enabled |
| 8 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.2 | Access Technology = UTRAN |

**** editorial change ****

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details:

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities:

Source device:	UICC

Destination device:	ME

Event list:

Event 1:	Access Technology Change (single access technology)

…

**** editorial change ****

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details:

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities:

Source device:	ME

Destination device:	UICC

Result:

General Result:	Command performed successfully

…

**** editorial change ****

ENVELOPE: EVENT DOWNLOAD – Access Technology Change 1.1.1

Logically:

Event list:	Access Technology Change (single access technology)

Device identities:

Source device:	ME

Destination device:	UICC

Access Technology:	E-UTRAN

…

**** editorial change ****

ENVELOPE: EVENT DOWNLOAD – Access Technology Change 1.1.2

Logically:

Event list:	Access Technology Change (single access technology)

Device identities:

Source device:	ME

Destination device:	UICC

Access Technology:	UTRAN

…

**** editorial change ****

Expected Sequence 1.3 (EVENT DOWNLOAD – Access Technology Change, single access technology – WB-S1 (Cat M1)/NB-S1)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Access technology change Event 1.1.3 |  |
| 6 | E-USS/NB-SS | ME detects a change in its current access technology | E-UTRA/NB-IoT cell is enabled and GSM cell is disabled |
| 7 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.1 | Access Technology = E-UTRAN |
| 8 | E-USS/NB-SS | ME detects a change in its current access technology | E-UTRA/NB-IoT cell is disabled and GSM cell is enabled |
| 9 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.3 | Access Technology = GSM |

**** editorial change ****

ENVELOPE: EVENT DOWNLOAD – Access Technology Change 1.1.3

Logically:

Event list:	Access Technology Change (single access technology)

Device identities:

Source device:	ME

Destination device:	UICC

Access Technology:	GSM

…

**** first change **** **** editorial change ****

Expected Sequence 1.4 (EVENT DOWNLOAD – Access Technology Change, single access technology, NG-RAN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | NG-SS | No NG-RAN cell initially available |  |
| 2 | USER ME | Switch on the ME |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 6 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 7 | NG-SS | The NG-RAN cell is switched on |  |
| 8 | ME NG-SS | The ME registers to the NG-RAN cell |  |
| 9 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.4.1 | Access Technology = 3GPP NR |

**** editorial change ****

ENVELOPE: EVENT DOWNLOAD – Access Technology Change 1.4.1

Logically:

Event list:	Access Technology Change (single access technology)

Device identities:

Source device:	ME

Destination device:	UICC

Access Technology:	3GPP NR

**** end of changes ****