3GPP TSG-CT6 Meeting #103-e	C6-200736

# Online, 17th Nov 2020 - 20th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.121 | CR | 0397 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 3GPP TS 31.121 to Rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Comprion GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As a valid Rel-16 version of this specification is available already, the Rel-15 version shall no longer be maintained |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference to Rel-16 of the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All but Rel-16 specific changes have to be provided and implemented in two versions of the same specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

| 3GPP TS 31.121 V15.9.0 (2020-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; UICC-terminal interface; Universal Subscriber Identity Module (USIM) application test specification (Release 15) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	27

Introduction	28

1	Scope	29

2	References	29

3	Definitions, symbols, abbreviations and coding	31

3.1	Definitions	31

3.2	Symbols	32

3.3	Abbreviations	33

3.4	Coding Conventions	34

3.5	Generic procedures for E-UTRAN/UTRAN/GERAN/IMS/NB-IoT/5G-NR	35

3.6	Applicability	35

3.6.1	Applicability of the present document	35

3.6.2	Applicability to terminal equipment	35

3.6.3	Applicability of the individual tests	35

3.7	Table of optional features	36

3.8	Applicability table	39

4	Default Values	63

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document defines the application test specification.

The aim of the present document is to ensure interoperability between an UICC and a terminal independently of the respective manufacturer, card issuer or operator. The present document does not define any aspects related to the administrative management phase of the UICC. Any internal technical realisation of either the UICC or the Terminal is only specified where these are reflected over the interface.

Application specific details for applications residing on an UICC are specified in the respective application specific documents. The logical and physical Characteristics of the UICC Terminal interface is specified in document ETSI TS 102 221 [5]. The Universal Subscriber Identity Module (USIM)-application for 3G telecommunication networks is specified in document TS 31.102 [4].

# 1	Scope

The present document provides the UICC–Terminal Interface Conformance Test Specification between a terminal and the USIM (Universal Subscriber Identity Module) as an application on the UICC and the Terminal for a 3GPP network operation:

-	the default setting of the USIM;

-	the applicability of each test case;

-	the test configurations;

-	the conformance requirement and reference to the core specifications;

-	the test purposes; and

-	a brief description of the test procedure and the specific acceptance criteria.

For the avoidance of doubt, references to clauses of ETSI TS 102 221 [5] include all the clauses of that clause, unless specifically mentioned.

ETSI TS 102 221 [5] contains material that is outside of the scope of 3GPP requirements. A 3GPP ME may support functionality that is not required by 3GPP, but the requirements to do so are outside of the scope of 3GPP. Thus, the present document does not contain tests for features defined in ETSI TS 102 221 [5] which are out of scope of 3GPP.

In the present document, unless explicitly stated otherwise, for Rel-13 onwards the term E-UTRAN implicitly refers to E-UTRAN in WB-S1 mode. E-UTRAN in NB-S1 mode is always explicitly referred to as NB-IoT.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version in same release as the implementation release of the terminal under test applies.

[1] to [4]	Void

[5]	If the device under test is a

-	R99 ME:	ETSI TS 102 221 v3.18.0: "UICC-Terminal interface; Physical and logical characteristics",

-	Rel-4 ME:	ETSI TS 102 221 v4.16.0: "UICC-Terminal interface; Physical and logical characteristics",

-	Rel-5 ME:	ETSI TS 102 221 v5.10.0: "UICC-Terminal interface; Physical and logical characteristics",

-	Rel-6 ME:	ETSI TS 102 221 v6.15.0: "UICC-Terminal interface; Physical and logical characteristics",

-	Rel-7 ME:	ETSI TS 102 221 v7.17.0: "UICC-Terminal interface; Physical and logical characteristics",

-	Rel-8 ME:	ETSI TS 102 221 v8.5.0: "UICC-Terminal interface; Physical and logical characteristics",

-	Rel-9 ME:	ETSI TS 102 221 v9.2.0: "UICC-Terminal interface; Physical and logical characteristics"",

-	Rel-10 ME:	ETSI TS 102 221 v10.0.0: "UICC-Terminal interface; Physical and logical characteristics",

-	Rel-11 ME:	ETSI TS 102 221 v11.1.0: "UICC-Terminal interface; Physical and logical characteristics",

-	Rel-12 ME:	ETSI TS 102 221 v12.1.0: "UICC-Terminal interface; Physical and logical characteristics".

-	Rel-13 ME:	ETSI TS 102 221 v13.2.0: "UICC-Terminal interface; Physical and logical characteristics".

-	Rel-14 ME:	ETSI TS 102 221 v14.1.0: "UICC-Terminal interface; Physical and logical characteristics".

-	Rel-15 ME:	ETSI TS 102 221 v15.0.0: "UICC-Terminal interface; Physical and logical characteristics".

[6] to [47]	void

-

[48]	3GPP TS 31.121: Rel-16 "UICC-terminal interface; Universal Subscriber Identity Module (USIM) application test specification"

# 3	Definitions, symbols, abbreviations and coding

# Void

-

-

| - |  |  |
| --- | --- | --- |

# 4	Default values, requirements and test cases

# The default values, requirements and test cases of the present document are provided in 3GPP TS 31.121 [39].

--

-

-

-

-

-

--

-

-

-

--

-

-

-

  - -

-

-

-

  - -

-

-

-

-

-

-

-

-

-

-

-

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| CT-28 | CT-28 | CP-050139 | 068 | - | B | Introduction of BCD number/ SSC content extension tests | 6.0.0 |
| CT-28 | CT-28 | CP-050139 | 072 | - | B | Introduction of ACL tests | 6.0.0 |
| CT-28 | CT-28 | CP-050139 | 073 | - | B | Introduction of SDN tests | 6.0.0 |
| CT-28 | CT-28 | CP-050139 | 074 | - | B | Introduction of phonebook selection/ local phonebook handling test | 6.0.0 |
| CT-29 | CT-29 | CP-050325 | 080 | - | A | Incorrect PUK2 value used in TC 6.1.6 | 6.1.0 |
| CT-29 | CT-29 | CP-050325 | 088 | - | A | Incorrect value used for EF SMS in TC 8.2.2 | 6.1.0 |
| CT-29 | CT-29 | CP-050331 | 076 | - | A | Incorrect MCC value used in TC 7.4.1 | 6.1.0 |
| CT-29 | CT-29 | CP-050336 | 083 | - | F | Clarification of BCD number/ SSC content extension tests | 6.1.0 |
| CT-29 | CT-29 | CP-050336 | 081 | - | F | Numbering and minor corrections | 6.1.0 |
| CT-30 | CT-30 | CP-050494 | 090 | - | F | CR to create combined R99 – Rel-6 version | 6.2.0 |
| CT-31 | CT-31 | CP-060020 | 095 | - | F | Essential correction of TC 7.2.1 | 6.3.0 |
| CT-31 | CT-31 | - | - | - | - | Update of Change History by TB Officer | 6.3.1 |
| CT-32 | CT-32 | CP-060285 | 0096 | - | F | Essential correction of acceptance criteria in 6.1.3.5 and 6.1.4.5 | 6.4.0 |
| CT-32 | CT-32 | CP-060286 | 0097 | - | F | Essential corrections on ACL TC 9.1.1 | 6.4.0 |
| CT-33 | CT-33 | CP-060507 | 0098 | 1 | F | Essential corrections on Applicability table | 6.5.0 |
| CT-34 | CT-34 | CP-060543 | 0099 | 1 | F | Essential corrections to 8.1.2 | 6.6.0 |
| CT-34 | CT-34 | CP-060543 | 0100 | 1 | F | Essential correction of EF PLMNwACT coding | 6.6.0 |
| CT-35 | CT-35 | CP-070064 | 0101 | 1 | F | Essential correction of UICC presence detection test | 6.7.0 |
| CT-36 | CT-36 | - | - | - | - | Update to Rel-7 version (MCC) | 7.0.0 |
| CT-37 | CT-37 | CP-070617 | 0103 | - | F | Essential correction of Location Area Information parameter in test 7.3 | 7.1.0 |
| CT-37 | CT-37 | CP-070617 | 0104 | - | F | Essential correction of the applicability of tests 6.4.2, 6.4.3, 6.4.4 | 7.1.0 |
| CT-38 | CT-38 | CP-070846 | 0105 | - | F | Essential correction on test case 7.1.2 | 7.2.0 |
| CT-38 | CT-38 | CP-070846 | 0106 | 1 | C | Introduction of Rel-7 test case applicability | 7.2.0 |
| CT-38 | CT-38 | CP-070846 | 0107 | 1 | F | Essential correction of Location Updating Accept/ Attach Accept parameters in 7.1.1 | 7.2.0 |
| CT-38 | CT-38 | CP-070846 | 0108 | 1 | F | Essential correction of network parameters and acceptance criteria in 7.4.1 | 7.2.0 |
| CT-38 | CT-38 | CP-070846 | 0109 | 1 | F | Correction of reference to 3GPP TS 23.140 | 7.2.0 |
| CT-39 | CT-39 | CP-080171 | 0110 | - | F | Sequence indication error in 7.1.2.4.2 fixed after wrong implementation of CR 0105 | 7.3.0 |
| CT-39 | CT-39 | CP-080171 | 0111 | 1 | F | Essential correction to Access Class Barred IE coding in tables 5-1a and 5-1b | 7.3.0 |
| CT-40 | CT-40 | CP-080387 | 0112 | - | F | Essential correction to network simulator configuration in test 5.2.1 | 7.4.0 |
| CT-40 | CT-40 | CP-080387 | 0113 | - | F | Essential correction to test 7.3.1 | 7.4.0 |
| CT-40 | CT-40 | CP-080387 | 0114 | 1 | F | Essential correction of the test case applicability for terminals not supporting speech calls | 7.4.0 |
| CT-41 | CT-41 | CP-080587 | 0115 | - | F | Essential correction of TC 8.4 applicability | 7.5.0 |
| CT-42 | CT-42 | ------ | - | - | - | Upgrade to copyright, keywords and logo for LTE | 8.0.0 |
| CT-43 | CT-43 | CP-090458 | 0116 | 1 | B | Definition of LTE™ tests | 8.1.0 |
| CT-43 | CT-43 | CP-090458 | 0118 | - | B | Definition of clause 7.2, 7.3 and 7.4 LTE™ tests | 8.1.0 |
| CT-43 | CT-43 | CP-090458 | 0119 | 1 | F | Clarification of class 2 SMS test | 8.1.0 |
| CT-43 | CT-43 | CP-090458 | 0120 | 2 | F | Clarification of PIN MMI string test | 8.1.0 |
| CT-45 | CT-45 | CP-090717 | 0121 | 1 | B | Definition of CSG related test cases for E-UTRA | 8.2.0 |
| CT-45 | CT-45 | CP-090717 | 0122 | 1 | B | Definition of EPS NAS Security Context cases for E-UTRA/EPC | 8.2.0 |
| CT-45 | CT-45 | CP-090717 | 0123 |  | F | Essential correction of test 6.1.12 | 8.2.0 |
| CT-45 | CT-45 | CP-090717 | 0124 | 1 | F | Correction of tests 7.4.2 and 7.4.4 | 8.2.0 |
| CT-45 | CT-45 | CP-090717 | 0126 | 1 | B | Definition of CSG related test cases for E-UTRA, Part 2 | 8.2.0 |
| CT-45 | CT-45 | CP-090717 | 0127 |  | F | Essential corrections on LTE related test cases | 8.2.0 |
| CT-46 | CT-46 | CP-090997 | 0130 |  | F | Correction of EF UST in clause 4.4.1 | 8.3.0 |
| CT-46 | CT-46 | CP-090998 | 0133 | 1 | F | Editorial correction of the naming of RRCConnectionRequest | 8.3.0 |
| CT-46 | CT-46 | CP-090998 | 0131 | 3 | F | Definition of the UICC presence detection test when connected to E-UTRAN/EPC | 8.3.0 |
| CT-46 | CT-46 | CP-090998 | 0132 | 3 | F | Definition of ACL related test cases for E-UTRA | 8.3.0 |
| CT-46 | CT-46 | CP-090998 | 0135 |  | F | Update of CGS list tests | 8.3.0 |
| CT-46 | CT-46 | - | - | - | - | Upgrade of the specification to Rel-9 | 9.0.0 |
| CT-47 | CT-47 | CP-100195 | 0139 | 1 | D | Editorial modifications about missing clause numbers | 9.1.0 |
| CT-47 | CT-47 | CP-100193 | 0137 | - | F | Introduction of Rel-9 applicability and applicability table corrections | 9.1.0 |
| CT-48 | CT-48 | CP-100394 | 0141 |  | F | Essential correction of Unblock Universal PIN value description in 4.1.1.21 | 9.2.0 |
| CT-48 | CT-48 | CP-100394 | 0140 | 1 | F | Essential correction to applicability table | 9.2.0 |
| CT-48 | CT-48 | CP-100394 | 0143 | 1 | F | Essential corrections on the initial conditions of E-UTRAN related test cases | 9.2.0 |
| CT-48 | CT-48 | CP-100394 | 0144 |  | F | Correction of inconsistent terminology for E-UTRAN-related elements | 9.2.0 |
| CT-49 | CT-49 | CP-100589 | 0147 | 1 | F | Addition of CSG Type and Home NodeB Name display test | 9.3.0 |
| CT-49 | CT-49 | CP-100590 | 0146 | 1 | F | Essential corrections on E-UTRAN / EPC related test cases | 9.3.0 |
| CT-49 | CT-49 | CP-100592 | 0145 | 1 | F | Update of references | 9.3.0 |
| CT-50 | CT-50 | CP-100831 | 0150 | 1 | F | Correction of incorrect Expected IMSI value in TC 5.1.8 acceptance criteria | 9.4.0 |
| CT-50 | CT-50 | CP-100831 | 0151 | 1 | F | Addition of LTE-related services to EF_UST in LTE test cases | 9.4.0 |
| CT-50 | CT-50 | CP-100830 | 0148 | 1 | B | Definition of E-UTRAN/EPC ISIM-UICC for ISIM related testing | 9.4.0 |
| CT-51 | CT-51 | CP-110264 | 0153 | 3 | C | Split of test 8.2.1 for independent verification of correct 'Class 2 SM storage on the USIM' and 'SM memory capacity exceeded handling' support | 9.5.0 |
| CT-51 | CT-51 | CP-110232 | 0154 | 7 | B | Introduction of ISIM related SMS tests | 9.5.0 |
| CT-51 | CT-51 | CP-110232 | 0155 |  | B | Introduction of ISIM and USIM related SMS reading test | 9.5.0 |
| CT-51 | CT-51 | CP-110264 | 0156 | 1 | F | Essential correction on TC 5.1.10 | 9.5.0 |
| SP-51 | SP-51 |  |  |  |  | Automatic upgrade from previous version 9.5.0 | 10.0.0 |
| CT-52 | CT-52 | CP-110502 | 0158 | 1 | F | Removal of tel URI in EF_IMPU of the default configuration of a E-UTRAN/EPC ISIM UICC | 10.1.0 |
| CT-52 | CT-52 | CP-110502 | 0159 |  | F | Essential correction of test 11.1 regarding EF EPSNSC updates | 10.1.0 |
| CT-53 | CT-53 | CP-110718 | 0161 | 2 | F | Essential correction of tests 11.2 and 11.3 | 10.2.0 |
| CT-53 | CT-53 | CP-110718 | 0162 | 1 | F | Essential correction of test 5.2.2 | 10.2.0 |
|  |  |  |  |  |  | Correction of formatting of table 5-2 | 10.2.1 |
| CT-54 | CT-54 | CP-110903 | 0163 | 1 | F | Essential correction of the phonebook and MMS tests applicability | 10.3.0 |
| CT-54 | CT-54 | CP-110903 | 0164 | 1 | F | Deletion of test 9.1.6 (Access Point Name Control List handling for terminals not supporting ACL connected to E-UTRAN/EPC) | 10.3.0 |
| CT-55 | CT-55 | CP-120150 | 0165 | 1 | F | Essential correction of 5.2.2 | 10.4.0 |
| CT-56 | CT-56 | CP-120390 | 0166 |  | F | Correction of test 6.1.16 | 10.5.0 |
| CT-56 | CT-56 | CP-120390 | 0168 | 1 | F | Test case applicability modification of test 8.2.3 for terminals with large SMS storage capabilities | 10.5.0 |
| CT-56 | CT-56 | CP-120391 | 0167 |  | F | Changes in test sequences for ACL in E-UTRAN | 10.5.0 |
| SP-57 | SP-57 |  |  |  |  | Automatic upgrade to Rel-11 | 11.0.0 |
| CT-58 | CT-58 | CP-120874 | 0172 | 1 | F | Essential correction on TC 11.1 | 11.1.0 |
| CT-58 | CT-58 | CP-120874 | 0173 | 1 | F | Essential correction on TC 11.2 | 11.1.0 |
| CT-58 | CT-58 | CP-120874 | 0171 | 1 | F | Essential correction on the applicability for TC 5.2.2 | 11.1.0 |
| CT-58 | CT-58 | CP-120874 | 0170 | 3 | F | Change of ACL test sequence | 11.1.0 |
| CT-60 | CT-60 | CP-130368 | 0174 | 7 | B | Introduction of Operator CSG lists and CSG list display control related TCs for E-UTRA and UTRA (Rel-9) | 11.2.0 |
| CT-60 | CT-60 | CP-130368 | 0176 | 2 | B | Introduction of Allowed CSG Lists related TCs for UTRA (Rel-8) | 11.2.0 |
| CT-62 | CT-62 | CP-130798 | 0179 |  | F | Removal of the unused applicability condition C030 | 11.3.0 |
| CT-62 | CT-62 | CP-130789 | 0180 | 3 | B | ntroduction of EPS NAS Security Context Storage test case | 11.3.0 |
| CT-62 | CT-62 | CP-130798 | 0181 | 1 | F | Essential correction for Applicability table related to new CSG tests | 11.3.0 |
| CT-63 | CT-63 | CP-140170 | 0182 |  | D | Clarification of call types to be used inside test case 8.4. | 11.4.0 |
| CT-63 | CT-63 | CP-140170 | 0183 | 1 | F | Corrections to test case 10.1.2, 10.1.4 and 10.1.5 | 11.4.0 |
| CT-63 | CT-63 | CP-140170 | 0186 | 1 | F | Corrections to the applicabilities of CSG test cases | 11.4.0 |
| CT-63 | CT-63 | CP-140170 | 0184 | 1 | F | Corrections to test case 10.1.8 and 10.2.2 | 11.4.0 |
| CT-63 | CT-63 | CP-140170 | 0185 | 1 | F | Corrections to test case 10.2.1, 10.2.2 and 10.2.3 | 11.4.0 |
| CT-65 | CT-65 | CP-140706 | 0187 |  | F | Correction to the text description of default value in EFOPLMNwAcT | 11.5.0 |
| CT-65 | CT-65 | CP-140706 | 0190 |  | F | Corrections to the test sequence of TC 6.4.3 and 6.4.4 | 11.5.0 |
| SP-65 | SP-65 |  |  |  |  | Automatic upgrade to Rel-11 | 12.0.0 |
| CT-66 | CT-66 | CP-140962 | 0192 |  | F | Default GUTI correction | 12.1.0 |
| CT-66 | CT-66 | CP-140962 | 0191 | 1 | F | Removal of TC 9.1.5 | 12.1.0 |
| CT-66 | CT-66 | CP-140962 | 0188 | 2 | F | Update of references to ETSI TS 102 221 and addition of Rel-12 applicability | 12.1.0 |
| CT-67 | CT-67 | CP-150161 | 0194 |  | F | Corrections to test case 7.1.1 | 12.2.0 |
| CT-67 | CT-67 | CP-150160 | 0196 |  | F | Corrections to test case 7.1.6 and 10.2.3 | 12.2.0 |
| CT-67 | CT-67 | CP-150161 | 0197 | 1 | F | Corrections to test case 8.2.4 | 12.2.0 |
| CT-67 | CT-67 | CP-150161 | 0195 | 1 | F | Corrections to test case 7.1.3, 7.2.2, 7.2.3, 7.3.1, 7.3.2, 7.4.1 and 7.4.2 | 12.2.0 |
| CT-67 | CT-67 | CP-150161 | 0198 | 2 | F | Essential correction on EFEPSNSC (EPS NAS Security Context) defined for E-UTRAN/EPC UICC | 12.2.0 |
| CT-68 | CT-68 | CP-150161 | 0194 |  | F | Corrections to test case 7.1.1 | 12.3.0 |
| CT-69 | CT-69 | CP-150564 | 0200 | 2 | C | Relaxing the mandatory clause and making features optional | 13.0.0 |
| CT-70 | CT-70 | CP-150828 | 0206 |  | C | Essential correction of initial conditions in test case 10.1.5 and 10.1.6. | 13.1.0 |
| CT-70 | CT-70 | CP-150828 | 0202 | 1 | B | Addition of Rel-13 column to applicability table | 13.1.0 |
| CT-70 | CT-70 | CP-150828 | 0207 | 1 | F | Essential corrections on test cases 5.1.1 and 5.1.2 | 13.1.0 |
| CT-70 | CT-70 | CP-150828 | 0208 | 1 | F | Essential corrections on test case 7.2.3 | 13.1.0 |
| CT-70 | CT-70 | CP-150828 | 0205 | 2 | F | Essential corrections to test case 7.1.1 | 13.1.0 |
| CT-70 | CT-70 | CP-150828 | 0203 | 1 | D | Editorial corrections in CSG related test cases | 13.1.0 |
| CT-70 | CT-70 | CP-150828 | 0204 | 1 | C | Additional check on UE after manual CSG attempt rejected | 13.1.0 |
| CT-70 | CT-70 | CP-150828 | 0210 |  | C | Clarification on acceptance criteria on EF-ACSGL in test case 10.1.3. | 13.1.0 |
| CT-71 | CT-71 | CP-160144 | 0212 | 1 | F | Essential correction on applicability table for test case execution in GERAN | 13.2.0 |
| CT-71 | CT-71 | CP-160144 | 0213 | 1 | F | Essential correction to test case for NAS security context parameter handling | 13.2.0 |
| CT-71 | CT-71 | CP-160144 | 0211 | 1 | B | Introduction of new test cases on Non-Access Stratum (NAS) configuration | 13.2.0 |
| CT-71 | CT-71 | CP-160145 | 0214 | 1 | C | Making Local Phone Book Optional | 13.2.0 |
| CT-72 | CT-72 | CP-160346 | 0216 |  | F | Modification of TAC value in the Acceptance criteria of test case 10.1.2 | 13.3.0 |
| CT-72 | CT-72 | CP-160346 | 0217 | 1 | D | Editorial corrections to test case 12.2 | 13.3.0 |
| CT-72 | CT-72 | CP-160346 | 0218 | 1 | B | Introduction of new test cases on Non-Access Stratum (NAS) configuration | 13.3.0 |
| CT-72 | CT-72 | CP-160346 | 0221 | 1 | F | Correction to test cases updating the EF-ACSGL | 13.3.0 |
| CT-72 | CT-72 | CP-160349 | 0219 | 1 | C | Making phonebook support optional for MEs with reduced feature capabilities | 13.3.0 |
| CT-73 | CT-73 | CP-160547 | 0222 |  | B | Introduction of new test cases on Non-Access Stratum (NAS) configuration | 13.4.0 |
| CT-73 | CT-73 | CP-160547 | 0223 |  | F | Correction of Acceptance criteria of test case 12.4 | 13.4.0 |
| CT-74 | CT-74 | CP-160791 | 225 |  | F | Essential correction to test case for E-UTRA Disabling Allowed for EMM cause #15 | 13.5.0 |
| CT-74 | CT-74 | CP-160791 | 232 |  | F | Condition for test case for EFNASCONFIG - Override NAS signalling low priority | 13.5.0 |
| CT-74 | CT-74 | CP-160791 | 234 |  | F | Applicability of test case for EFNASCONFIG - SM_RetryWaitTime | 13.5.0 |
| CT-74 | CT-74 | CP-160791 | 235 |  | F | Condition for Verifying Timer T3245 Behaviour test case | 13.5.0 |
| CT-74 | CT-74 | CP-160791 | 237 |  | F | Correction of Applicabilty for test case 9.1.4 | 13.5.0 |
| CT-74 | CT-74 | CP-160791 | 231 | 1 | F | Condition for test case for EFNASCONFIG - Override Extended Access Barring | 13.5.0 |
| CT-75 | CT-75 | CP-170231 | 0236 | 4 | B | Introduction of new test executions for NB-IoT UE implementation | 13.6.0 |
| CT-75 | CT-75 | CP-170231 | 0239 | 1 | B | Modification of some E-UTRAN test cases under cl. 7.1/3/4 to cover NB-IoT | 13.6.0 |
| CT75 | CT75 | CP-170231 | 0240 | 1 | B | Modification of test cases under cl. 11 and 8.5 to cover NB-IoT | 13.6.0 |
| CT75 | CT75 | CP-170231 | 0238 | 3 | B | Introduction of NB-IoT Applicability to some LTE test cases and addition of a new test case to cl. 8.2 | 13.6.0 |
| CT-75 | CT-75 | CP-170166 | 0226 | 2 | F | Removal of test case for EFNASCONFIG – Extended Access Barring | 13.6.0 |
| CT-75 | CT-75 | CP-170166 | 0230 | 3 | F | Removal of test case for EFNASCONFIG – NMO I Network Mode of Operation I handling | 13.6.0 |
| 2017-03 | 2017-03 | SA-75 | - | - | - | Update to Rel-14 version (MCC) | 14.0.0 |
| CT-76 | CT-76 | CP-171160 | 0243 | - | B | Addition of new test case 5.2.X | 14.1.0 |
| CT-76 | CT-76 | CP-171166 | 0228 | 3 | F | Removal of test case for EFNASCONFIG - Minimum Periodic Search Timer | 14.1.0 |
| CT-76 | CT-76 | CP-171166 | 0229 | 3 | F | Removal of test case for EFNASCONFIG - NAS Signalling Priority | 14.1.0 |
| CT-76 | CT-76 | CP-171166 | 0233 | 3 | F | Removal of test case for EFNASCONFIG - SM_RetryAtRATChange | 14.1.0 |
| CT-76 | CT-76 | CP-171166 | 0224 | 3 | F | Removal of test case for Attach with IMSI | 14.1.0 |
| CT-76 | CT-76 | CP-171166 | 0244 | - | F | Correction to test case 11.4 for EPS NAS security context Storage | 14.1.0 |
| CT-76 | CT-76 | CP-171166 | 0242 | - | F | Added applicability for Rel.14 terminals | 14.1.0 |
| CT-76 | CT-76 | CP-171166 | 0224 | 4 | F | Removal of test case for Attach with IMSI | 14.1.0 |
| CT-76 | CT-76 | CP-171166 | 0245 | - | D | Editorial corrections to clause 12.5 | 14.1.0 |
| 2017-09 | CT-77 | CP-172065 | 0250 | 3 | F | Correction of handling NAS reject messages | 14.2.0 |
| 2017-09 | CT-77 | CP-172064 | 0252 | 1 | F | Correction of wrong implementation of CRs in TS 31.121 | 14.2.0 |
| 2017-09 | CT-77 | CP-172065 | 0253 | 1 | B | Addition of new test case on handling non-integrity protected NAS reject messages | 14.2.0 |
| 2017-12 | CT-78 | CP-173150 | 0254 | - | F | Fixed typos in table B.1 | 14.3.0 |
| 2017-12 | CT-78 | CP-173150 | 0255 | 2 | F | Fixed conditions for test cases for terminals that supports only E-UTRA | 14.3.0 |
| 2017-12 | CT-78 | CP-173150 | 0256 | 2 | B | Introduction of test cases for UICC interface during PSM and eDRX | 14.3.0 |
| 2018-03 | CT79 | CP-180129 | 0257 | 1 | F | Remove references to hardcoded timers for testcases for UICC interface during PSM and eDRX | 14.4.0 |
| 2018-03 | CT79 | CP-180129 | 0258 | 1 | F | Correction of implementation error in applicability table | 14.4.0 |
| 2018-03 | CT79 | CP-180129 | 0259 | 1 | F | Minor corrections to testcases for UICC interface during PSM and eDRX | 14.4.0 |
| 2018-03 | CT79 | CP-180129 | 0260 | - | F | Move PSM/eDRX procedure selection decision to the network | 14.4.0 |
| 2018-03 | CT79 | CP-180129 | 0261 | - | F | Correction to test case 11.4 for EPS NAS security context Storage | 14.4.0 |
| 2018-06 | CT80 | CP-181157 | 0262 | 1 | B | Introduction of new test cases for user and operator controlled PLMN selector handling | 14.5.0 |
| 2018-06 | CT80 | CP-181151 | 0263 | - | F | Applicability modification of TC 7.2.6, 7.2.7 and 12.10 | 14.5.0 |
| 2018-06 | CT80 | CP-181151 | 0264 | - | F | Addition of logical description of EFUMPC | 14.5.0 |
| 2018-06 | SA80 |  |  |  |  | Automatic updgrade to Rel-15 | 15.0.0 |
| 2018-09 | CT#81 | CP-182188 | 0265 | 2 | D | Editorial corrections to TC 13.2 and cl. 3.8 | 15.1.0 |
| 2018-09 | CT#81 | CP-182188 | 0266 | - | F | Modification of test procedures under cl. 13, and cl. 14 | 15.1.0 |
| 2018-09 | CT#81 | CP-182188 | 0267 | 1 | F | Correction of wrong implementation of CR#0262 | 15.1.0 |
| 2018-09 | CT#81 | CP-182190 | 0268 | 1 | F | Correction to the applicability table | 15.1.0 |
| 2018-09 | CT#81 | CP-182190 | 0269 | 1 | F | TS 31.121: Adding applicability for Rel.15 terminals | 15.1.0 |
| 2018-12 | CT#82 | CP-183142 | 0271 | - | F | Correction of typos in 7.1.8 | 15.2.0 |
| 2018-12 | CT#82 | CP-183139 | 0272 | - | F | Extend the scope of 31.121 to cover 5G aspects | 15.2.0 |
| 2018-12 | CT#82 | CP-183142 | 0273 | 1 | F | Modification of test requirements for UICC re-activation/re-initialisation during PSM and eDRX | 15.2.0 |
| 2019-03 | CT#83 | CP-190046 | 0274 | - | F | Applicability update of TC 7.2.7 | 15.3.0 |
| 2019-06 | CT#84 | CP-191014 | 0276 | - | F | Correction to test case 10.1.6 | 15.4.0 |
| 2019-06 | CT#84 | CP-191014 | 0275 | 1 | F | Correction to test case 11.4 | 15.4.0 |
| 2019-06 | CT#84 | CP-191017 | 0277 | 1 | B | Introduction of Handling subscription identifier privacy for 5G related test cases | 15.4.0 |
| 2019-09 | CT#85 | CP-192018 | 0283 | 2 | B | Introducing 5G-NG TCs 5.3.1 and 5.3.2 for UE identification by SUCI during initial registration | 15.5.0 |
| 2019-09 | CT#85 | CP-192017 | 0278 | 1 | F | Corrections to 5G-NG TC 5.3.4 | 15.5.0 |
| 2019-09 | CT#85 | CP-192017 | 0279 | 2 | F | Corrections to 5G-NG TC 5.3.5 | 15.5.0 |
| 2019-09 | CT#85 | CP-192017 | 0280 | 2 | F | Corrections to 5G-NG TC 5.3.6 | 15.5.0 |
| 2019-09 | CT#85 | CP-192017 | 0281 | 1 | F | Update of 5G-NG TC 5.3.8 | 15.5.0 |
| 2019-09 | CT#85 | CP-192017 | 0282 | 1 | F | Corrections to 5G-NG TC 5.3.9 | 15.5.0 |
| 2019-12 | CT#86 | CP-193073 | 0294 | 1 | F | Improvement of definitions | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0285 | 2 | B | Introducing SUCI calculation by USIM testcase 5.3.3 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0286 | 2 | B | Introducing SUCI calculation by USIM testcase 5.3.7 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0287 | 2 | B | Introducing Universal Access Control testcase 5.4.1 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0288 | 2 | B | Introducing Universal Access Control test case 5.4.2 | 15.6.0 |
| 2019-12 | CT#86 | CP-193073 | 0300 | - | D | Correction of subclause numbering in TC 5.4.2 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0289 | 1 | B | Enhance SUCI calculation testcase 5.3.5 to ensure UE uses correct identity amongst the multiple available | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0290 | 1 | F | Correction of EFIMSI for the default 5G-NR UICC | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0291 | 1 | F | Correction of TC 5.3.1 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0292 | 1 | F | Correction of TC 5.3.2 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0293 | - | F | Update of case applicability for TC 5.3.x | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0295 | 2 | F | Align wording of TC 5.3.1 with 3GPP TS 31.102 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0296 | 3 | F | Align wording of TC 5.3.2 with 3GPP TS 31.102 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0297 | 3 | F | Align wording of TC 5.3.7 with 3GPP TS 31.102 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0298 | 2 | F | Adding 5GS_Ph1 test cases to Applicability Table where missed in the initial CR | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0299 | 1 | D | Addition of 5G specific abbreviations | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0302 | 1 | B | Introducing Universal Access Control test case 5.4.5 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0303 | 1 | B | Introducing Universal Access Control test case 5.4.8 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0304 | 1 | B | Introducing Universal Access Control test case 5.4.9 | 15.6.0 |
| 2019-12 | CT#86 | CP-193074 | 0305 | 1 | B | Introducing test case 5.3.11, SUCI calculation by ME using Profile A | 15.6.0 |
| 2020-03 | CT#87e | CP-200084 | 0306 | 2 | B | Introduction of Authentication procedure and NAS security context handling for 5G test cases | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0307 | 2 | B | Introducing Universal Access Control test case 5.4.3 | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0308 | 1 | F | Title update of test case 5.3.10 | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0309 | 2 | F | Update of default content of EFSUCI_Calc_Info | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0310 | 1 | F | Order update of Protection Scheme Identifier in TC 5.3.11 | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0314 | - | F | Correct the length of 'A1' for EFSUCI_Calc_Info in 4.9.4 | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0315 | - | B | Introduction test case – 5.3.x, UE identification by SUCI during initial registration, SUCI calculation by USIM using profile A | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0316 | - | F | Correct the length of 'A1' for EFSUCI_Calc_Info in 5.3.1 | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0317 | 1 | B | Introducing a test case, SUCI calculation by ME using null scheme – no Protection Scheme Identifier provisioned in the USIM | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0318 | 1 | B | Introducing a test case, SUCI calculation by ME using null scheme –no Home Network Public Key for supported protection scheme provisioned in the USIM | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0319 | - | B | Introducing a test case, SUCI calculation by ME using null scheme with the E-UTRAN/EPC UICC | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0320 | - | B | Introducing Universal Access Control test case 5.4.6 | 15.7.0 |
| 2020-03 | CT#87e | CP-200084 | 0321 | 2 | B | Introducing test case, SUCI calculation by ME using the lower priority protection scheme when the higher priority protection scheme is not supported by the ME | 15.7.0 |
| 2020-06 | CT#88e | CP-201143 | 0311 | 3 | C | Correction of UICC interface during eDRX tests | 15.8.0 |
| 2020-06 | CT#88e | CP-201143 | 0343 | 2 | F | Correction of IMSI in the default 5G-NR UICC | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0312 | 4 | D | Correction of incorrectly coded EF_ACC bytes in TC 5.4.9 | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0313 | 2 | D | Correction of incorrect numbering in TC 5.4.1 | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0322 | 1 | F | Adding a default UICC profile to the initial conditions of 5.4.x test cases | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0323 | 1 | B | Introduction test case – 5.5.1 – Display of 5G Operator PLMN name from USIM | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0324 | 1 | B | Introduction test case – 5.5.2 – Display of 5G Operator PLMN name from ME | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0325 | 1 | C | Correction of Initial conditions of TC 5.4.3 and 5.4.6 | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0326 | 2 | B | Introduction of test 5.4.4 - UAC - Access Identity 1 – no MPS indication by USIM and SUPI is changed | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0327 | 2 | B | Introduction of test 5.4.7 - UAC - Access Identity 2 – no MCS indication by USIM and SUPI is changed | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0328 | 3 | B | Introduction of tests 15.1.1 to 15.1.4 - Authentication procedure for EAP-AKA' | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0330 | 3 | F | Clarification to test cases 5.3.5 | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0337 | 1 | F | Correction test case – 5.3.16 – SUCI calculation by ME using the lower priority protection scheme when the higher priority protection scheme is not supported by the ME | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0338 | 1 | F | Correction of EF_SUCI_Calc_Info in the 5G-NR UICC | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0339 | - | F | Correction of EF_SUCI_Calc_Info in test case 5.3.1 | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0340 | - | F | Correction of the Home Network Public Key used in test case 5.3.3 | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0341 | - | F | Correction of the Home Network Public Key 2 used in test case 5.3.11 | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0342 | - | B | Introduction new test case 5.3.x - SUCI calculation by by ME using Profile B with compressed Home Network Public Key | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0344 | 2 | F | Clarification to several test cases | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0345 | - | F | Correction to Test Case 5.3.4 | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0347 | 1 | F | Correction test case – 5.3.9 UE identification by 5G-GUTI – Last Registered TAI stored by ME | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0348 | 1 | F | Correction test case – 5.3.10 UE identification after SUPI is changed | 15.8.0 |
| 2020-06 | CT#88e | CP-201151 | 0349 | 1 | F | Correction to testcase 5.3.8: UE identification by 5G-GUTI – Last Registered TAI stored on USIM | 15.8.0 |
| 2020-06 | CT#88e | CP-201145 | 0329 | 2 | F | Update the scope of 31.121 to cover 5G aspects | 15.8.0 |
| 2020-06 | CT#88e | CP-201145 | 0346 | 1 | F | Correction to handling of 5G timers in 5G test cases | 15.8.0 |
| 2020-06 | CT#88e | CP-201333 | 0332 | 6 | C | Correction of minor issues in 15.2.x test cases | 15.8.0 |
| 2020-09 | CT#89 | CP-202134 | 0351 | 1 | F | Corrections to authentication procedure for EAP-AKA' test cases | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0352 | 1 | F | Corrections to authentication procedure for 5G AKA test cases | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0360 | 1 | F | Modify Test Case 5.3.7 | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0361 | 1 | F | Update of test case 5.3.5 | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0362 | 1 | F | Update of test case 5.3.6 | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0363 | 2 | F | Update of test case 5.3.9 | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0364 | 1 | F | Update of test case 5.4.x | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0367 | 1 | F | Update of test case 5.5.x | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0369 | 1 | F | Correction Test case – 5.3.15 – SUCI calculation by ME using null scheme with the E-UTRAN/EPC UICC | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0372 | 2 | B | Introducing Universal Access Control test case 5.4.11 for Operator-defined Access categories | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0374 | 2 | B | Introducing Universal Access Control test case 5.4.12 for Operator-defined Access categories | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0376 | - | F | Correction to coding of AC 14 in the Universal Access Control test case 5.4.9 | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0378 | 1 | F | Clarification to test case 5.3.1 SUCI calculation by ME using null scheme | 15.9.0 |
| 2020-09 | CT#89 | CP-202134 | 0380 | 1 | B | Introducing Universal Access Control test case 5.4.10 for Operator-defined Access categories | 15.9.0 |
|  |  |  | 0397 |  |  | Reference to Rel-16 version | 15.10.0 |