3GPP TSG-CT WG3 Meeting #111e	C3-205307

E-meeting, 19th – 28th August 2020											(Revision of C3-20xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0069 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to IPv6 Prefix Delegation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NOTE description " The delegated IPv6 prefix shall be present if the user was delegated an IPv6 prefix from a local pool." in Change-of-Authorization Request message sent from DN-AAA server to SMF, does not align with the description " When PLMN based parameter configuration is used, the SMF provides the requested Ipv6 prefix from a locally provisioned pool ", and not aligned with the referred specs in clause 10.2.5. Delegated-Ipv6-Prefix in this release is not consistent with Delegated-IPv6-Prefix in the reference RFC 4818. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates the NOTE description to be "The delegated IPv6 prefix shall be present if IPv6 prefix delegation is required from AAA server. " Correct Delegated-Ipv6-Prefix as Delegated-IPv6-Prefix to be aligned with RFC 4818. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting the external DN-AAA server delegating IPv6 prefix correctly, wrong external IPv6 prefix delegated to PLMN assigned UE IP address. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2, 12.6.2, 12.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

### 11.3.2	Change-of-Authorization Request (optionally sent from DN-AAA server to SMF)

Table 11.3.2-1 describes the attributes of the Change-of-Authorization Request message. Other RADIUS attributes may be used as defined in IETF RFC 5176 [27].

Table 11.3.2-1: The attributes of the Change-of-Authorization Request message

| Attr # | Attribute Name | Description | Content | Presence Requirement |
| --- | --- | --- | --- | --- |
| 1 | User-Name | Username provided by the user (extracted from the PCO field received during PDN connection establishment). If no username is available a generic username, configurable on a per DNN basis, shall be present. If the User-Name has been sent in the Access-Accept message, this user-name shall be used in preference to the above | String | Optional |
| 6 | Service-Type | Indicates the type of service for this user. | 17 (Authorize Only) | Optional |
| 8 | Framed-IP-Address | User IPv4 address | Ipv4 | Conditional NOTE 2 |
| 30 | Called-Station-Id | Identifier for the target network | DNN (UTF-8 encoded characters) | Optional |
| 31 | Calling-Station-Id | This attribute is the identifier for the UE, and it shall be configurable on a per DNN basis. | MSISDN in international format according to 3GPP TS 23.003 [28], UTF-8 encoded decimal character. (NOTE 5) | Optional |
| 96 | Framed-Interface-Id | User IPv6 Interface Identifier | Ipv6 | Conditional NOTE 1 NOTE 2 |
| 44 | Acct-Session-Id | User session identifier. | SMF IP address (IPv4 or IPv6) and Charging-ID concatenated in a UTF-8 encoded hexadecimal characters. | Mandatory |
| 79 | EAP-Message | This attribute encapsulates EAP message (as defined in IETF RFC 3748 [6]) exchanged between the SMF and DN-AAA, see IETF RFC 3579 [7] for details. | String | Conditional NOTE 3 |
| 80 | Message-Authenticator | This attribute includes the message authenticator, see IETF RFC 3579 [7] for details. | String | Conditional NOTE 3 |
| 97 | Framed-Ipv6-Prefix | User IPv6 prefix | Ipv6 | Conditional NOTE 2 |
| 123 | Delegated-IPv6-Prefix | Delegated IPv6 prefix to the user. | Ipv6 | Conditional NOTE 4 |
| 26/10415 | 3GPP Vendor-Specific | Sub-attributes according clause 11.3, the encoding of this attribute is specified in 3GPP TS 29.061 [5]. | See clause 11.3 | Optional |
| NOTE 1: Included if the prefix alone is not unique for the user. This may be the case, for example, if a static IPv6 address is assigned. NOTE 2: Either IPv4 or IPv6 address/prefix attribute shall be present. The IP protocol version for end-user and network may be different. NOTE 3: Shall be present if EAP is used. NOTE 4: The delegated IPv6 prefix shall be present if IPv6 prefix delegation is required from the external DN-AAA server. NOTE 5: There are no leading characters in front of the country code. |  |  |  |  |

# *** 2nd Change ***

### 12.6.2	DER Command

The DER command, defined in IETF RFC 4072 [25], is indicated by the Command-Code field set to 268 and the 'R' bit set in the Command Flags field. It is sent by the SMF to the DN-AAA server upon reception of an initial access request (e.g. Nsmf_PDUSession_CreateSMContext) message for a given DNN to request user authentication and authorization.

The relevant AVPs that are of use for the N6 interface are detailed in the ABNF description below. Other valid AVPs for this command are not used for N6 purposes and should be ignored by the receiver or processed according to the relevant specifications.

The bold marked AVPs in the message format indicate new optional AVPs for N6, or modified existing AVPs.

Message Format:

<Diameter-EAP-Request> ::= < Diameter Header: 268, REQ, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Auth-Request-Type }

[ Destination-Host ]

[ NAS-Port ]

[ NAS-Port-Id ]

[ NAS-Port-Type ]

[ Origin-State-Id ]

[ Port-Limit ]

[ User-Name ]

{ EAP-Payload }

[ EAP-Key-Name ]

[ Service-Type ]

[ Authorization-Lifetime ]

[ Auth-Grace-Period ]

[ Auth-Session-State ]

[ Callback-Number ]

[ Called-Station-Id ]

[ Calling-Station-Id ]

[ Originating-Line-Info ]

[ Connect-Info ]

*	[ Framed-Compression ]

[ Framed-Interface-Id ]

[ Framed-IP-Address ]

*	[ Framed-Ipv6-Prefix ]

*	[ Delegated-IPv6-Prefix ]

[ Framed-IP-Netmask ]

[ Framed-MTU ]

[ Framed-Protocol ]

*	[ Tunneling ]

*	[ Proxy-Info ]

*	[ Route-Record ]

[ External-Identifier ]

[ 3GPP-IMSI ]

[ 3GPP-NAI ]

*	[ 3GPP-UE-MAC-Address ]

[ 3GPP-Charging-ID ]

[ 3GPP-PDP-Type ]

[ 3GPP-CG-Address ]

[ 3GPP-GPRS-Negotiated-QoS-Profile ]

[ 3GPP-SGSN-Address ]

[ 3GPP-GGSN-Address ]

[ 3GPP-IMSI-MCC-MNC ]

[ 3GPP-GGSN-MCC-MNC ]

[ 3GPP-NSAPI ]

[ 3GPP-Selection-Mode ]

[ 3GPP-Charging-Characteristics ]

[ 3GPP-CG-Ipv6-Address ]

[ 3GPP-SGSN-Ipv6-Address ]

[ 3GPP-GGSN-Ipv6-Address ]

[ 3GPP-SGSN-MCC-MNC ]

[ 3GPP-User-Location-Info ]

[ 3GPP-RAT-Type ]

[ 3GPP-Negotiated-DSCP ]

[ 3GPP-Allocate-IP-Type ]

[ TWAN-Identifier ]

*	[ 3GPP-IP-Address-Pool-Info]

*	[ Supported-Features ]

*	[ AVP ]

# *** 3rd Change ***

### 12.6.3	DEA Command

The DEA command, defined in IETF RFC 4072 [25], is indicated by the Command-Code field set to 268 and the 'R' bit cleared in the Command Flags field. It is sent by the DN-AAA server to the SMF in response to the DER command.

The relevant AVPs that are of use for the N6 interface are detailed in the ABNF description below. Other valid AVPs for this command are not used for N6 purposes and should be ignored by the receiver or processed according to the relevant specifications.

The bold marked AVPs in the message format indicate new optional AVPs for N6, or modified existing AVPs.

Message Format:

<Diameter-EAP-Answer> ::= < Diameter Header: 268, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Auth-Request-Type }

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ EAP-Payload ]

[ EAP-Reissued-Payload ]

[ EAP-Master-Session-Key ]

[ EAP-Key-Name ]

[ Multi-Round-Time-Out ]

[ Accounting-EAP-Auth-Method ]

[ Service-Type ]

*	[ Class ]

[ Acct-Interim-Interval ]

[ Error-Message ]

[ Error-Reporting-Host ]

[ Failed-AVP ]

[ Idle-Timeout ]

[ Authorization-Lifetime ]

[ Auth-Grace-Period ]

[ Auth-Session-State ]

[ Re-Auth-Request-Type ]

[ Session-Timeout ]

*	[ Reply-Message ]

[ Origin-State-Id ]

*	[ Filter-Id ]

[ Port-Limit ]

[ Callback-Id ]

[ Callback-Number ]

*	[ Framed-Compression ]

[ Framed-Interface-Id ]

[ Framed-IP-Address ]

*	[ Framed-Ipv6-Prefix ]

[ Framed-Ipv6-Pool ]

*	[ Framed-Ipv6-Route ]

*	[ Delegated-IPv6-Prefix ]

[ Framed-IP-Netmask ]

*	[ Framed-Route ]

[ Framed-Pool ]

[ Framed-IPX-Network ]

[ Framed-MTU ]

[ Framed-Protocol ]

[ Framed-Routing ]

*	[ NAS-Filter-Rule ]

*	[ QoS-Filter-Rule ]

*	[ Tunneling ]

*	[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*	[ Proxy-Info ]

*	[ External-Identifier ]

[ 3GPP-Ipv6-DNS-Servers ]

[ 3GPP-Notification ]

0*16 [ 3GPP-UE-MAC-Address ]

[ 3GPP-Authorization-Reference ]

[ 3GPP-Policy-Reference ]

[ 3GPP-Session-AMBR ]

[ 3GPP-Session-AMBR-v2 ]

0*2 [ 3GPP-IP-Address-Pool-Info]

*	[ Supported-Features ]

*	[ AVP ]

*** End of Changes ***