3GPP TSG-CT Meeting #90-e	CP-203162

# E-Meeting, 07th – 09th December 2020													   was C4-205124

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.531 | CR | 0082 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HTTP 3xx redirection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 29.500 specifies for the support of stateless NF service producers: 6.5.3.3 Stateless NF as service producer 3. An NF service consumer may become aware of a NF service producer change, by receiving an updated binding information, or via an Error response from the old or a selected new NF, via link level failures (e.g. no response from the NF), or via a notification from the NRF that the NF has deregistered. The HTTP error response may be a 3xx redirect response pointing to a new NF. Example use case: An NSSF instance deployed in an NSSF set may want to redirect the request to another NSSF instance within the same NSSF set during graceful shutdown (e.g. NSSF set scale-in) or for load re-balancing (e.g. if the NSSF instance experiences an overload) as described in clause 6.4.2.4 of TS 29.500. TS 29.500 also specifies in clause 6.10.9 that an HTTP request sent using indirect communication may also be redirected to a different target NF service instance (from a same NF service set or NF set). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The status codes 307 Temporary Redirect and 308 Permanent Redirect are supported by all service operations of the NSSF API. A new feature (ES3XX) is defined to let the NSSF know whether the NF service consumer (e.g. AMF) supports 307/308 redirection for all service operations of the Nnssf_NSSelection and Nnssf_NSSAIAvailability services. The identity of the new NSSF is signalled in a new 3gpp-Sbi-Target-Nf-Id header in the 307/308 response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications causing interoperability issues and preventing support of NSSF set and indirect communications with an NSSF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 5.2.2.2.3, 5.2.2.2.4, 5.3.2.2, 5.3.2.3.1, 5.3.2.3.2, 5.3.2.4, 5.3.2.5, 5.3.2.6, 5.3.2.7, 6.1.3.2.3, 6.1.8, 6.1.x (New), 6.2.3.2.3.1, 6.2.3.2.3.2, 6.2.3.2.3.3, 6.2.3.3.3, 6.2.3.4.3.1, 6.2.3.4.3.2, 6.2.3.5.3.1, 6.2.5.2.3, 6.2.8, 6.2.x (New), A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR introduces backward compatible corrections to the OpenAPI specification files of the Nnssf_NSSelection and Nnssf_NSSAIAvailability service APIs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Corrections in A.3 Nnssf_NSSAIAvailability API to: - Replace "TAB" at the beginning of lines with "spaces" - corrected position of "header" which was wrong in one place of the API - Table 6.2.3.5.3.1-4 of clause 6.2.3.5.3.1 was missing. It is readded. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.2.2	Get service operation of Nnssf_NSSelection service

In this procedure, the NF Service Consumer (e.g. AMF) retrieves the Allowed NSSAI, Configured NSSAI, target AMF Set or the list of candidate AMF(s) and other optional information.

Figure 5.2.2.2.2-1: Retrieve the network slice information during the registration procedure

1	The AMF shall send a GET request to the NSSF. One or more of the following parameters shall be included as query parameters: Requested NSSAI, Subscribed S-NSSAI(s) with the indication if marked as default S-NSSAI, PLMN ID of the SUPI, TAI, NF type of the NF service consumer, Requester ID.

2a	On success, "200 OK" shall be returned when the NSSF is able to find authorized network slice information for the requested network slice selection information, the response body shall include a payload body containing at least the Allowed NSSAI, target AMF Set or the list of candidate AMF(s); the payload body may additionally contain a target AMF Service Set.

2b	If no slice instances can be found for the requested slice selection information, then the NSSF shall return a 403 Forbidden response with the "ProblemDetails" IE containing the Application Error "SNSSAI_NOT_SUPPORTED" (cf. Table 6.2.7.3-1).

On failure or redirection, the NSSF shall return one of the HTTP status codes together with the response body listed in Table 6.1.3.2.3.1-3.

* * * Next Change * * * *

##### 5.2.2.2.3	Get service operation of Nnssf_NSSelection service during the PDU session establishment

In this procedure, the NF Service Consumer (e.g. AMF) retrieves the NRF and the optionally the NSI ID of the network slice instance:

Figure 5.2.2.2.3-1: Retrieve the network slice information during the PDU session establishment procedure

1	The NF Service consumer (e.g. AMF or NSSF in the different PLMN) shall send a GET request to the NSSF. The request shall include query parameters, contain at least S-NSSAI, S-NSSAI from the HPLMN that maps to the S-NSSAI from the Allowed NSSAI of the Serving PLMN, the NF type of the NF service consumer and Requester ID. For the procedure invoked in the Serving PLMN, the query parameters shall also contain non-roaming/LBO roaming/HR roaming indication, PLMN ID of the SUPI and TAI.

2a	On success, "200 OK" shall be returned when the NSSF is able to find network slice instance information for the requested network slice selection information, the response body shall include a payload body containing at least the NRF to be used to select NFs/services within the selected Network Slice instance;

2b	If no slice instances can be found for the requested slice selection information, then the NSSF shall return a 403 Forbidden response with the "ProblemDetails" IE containing the Application Error "SNSSAI_NOT_SUPPORTED" (cf. Table 6.2.7.3-1).

On failure or redirection, the NSSF shall return one of the HTTP status codes together with the response body listed in Table 6.1.3.2.3.1-3.

* * * Next Change * * * *

##### 5.2.2.2.4	Get service operation of Nnssf_NSSelection service during UE configuration update procedure

In this procedure, the NF Service Consumer (e.g. AMF) retrieves network slice configuration information (e.g. the Allowed NSSAI and the Configured NSSAI) during the UE configuration update procedure.

Figure 5.2.2.2.4-1: Retrieve the network slice information during UE configuration update procedure

1	The NF Service consumer (e.g. AMF) shall send a GET request to the NSSF. The request shall include query parameters: Subscribed S-NSSAI(s) with the indication if the S-NSSAI is marked as default S-NSSAI, PLMN ID of the SUPI, TAI, NF type of the NF service consumer and the NF instance ID of the requester NF.

2a	On success, "200 OK" shall be returned when the NSSF is able to find authorized network slice information for the requested network slice selection information, the response body shall include a payload body containing at least the Allowed NSSAI, Configured NSSAI;

2b	If no slice instances can be found for the requested slice selection information, then the NSSF shall return a 403 Forbidden response with the "ProblemDetails" IE containing the Application Error "SNSSAI_NOT_SUPPORTED" (cf. Table 6.2.7.3-1).

On failure or redirection, the NSSF shall return one of the HTTP status codes together with the response body listed in Table 6.1.3.2.3.1-3.

* * * Next Change * * * *

#### 5.3.2.2	Update Service Operation

##### 5.3.2.2.1	General

The Update operation shall be used by an NF Service Consumer (e.g. AMF) to update the NSSF with the S-NSSAIs the NF service consumer (e.g. AMF) supports per TA, and get the availability of the S-NSSAIs per TA for the S-NSSAIs the NF service consumer (e.g. AMF) supports.

Figure 5.3.2.2.1-1: Update the S-NSSAIs the AMF supports per TA

1.	The NF service consumer (e.g. AMF) shall send a PUT request to the resource representing the NSSAI Availability information of the individual NF, identified by the {nfId}, to replace or create the NSSAI Availability information of the NF. The payload of the body shall contain the NssaiAvailabilityInfo which contains one or more representations of the individual supportedSnssai information to be replaced.

The NF service consumer (e.g. AMF) shall send a PATCH request to the resource representing the NSSAI Availability information of the individual NF, identified by the {nfId}, to update the NSSAI Availability information of the NF. The payload of the body shall contain the PatchDocument which contains one or more PatchItem instructions for updating the individual supportedSnssai resources.

2.	On success, "204 No content" shall be returned if Authorized NSSAI Availability is empty after the update; otherwise, "200 OK" shall be returned, the payload body of the PUT/PATCH response shall contain the representation describing the status of the request and the complete AuthorizedNssaiAvailabilityData information representing the current state of the AuthorizedNssaiAvailabilityInfo. If there is no supported S-NSSAIs authorized by the NSSF for the TA, the NSSF shall not return the AuthorizedNssaiAvailabilityData for the corresponding TA in the response.

On failure or redirection, the NSSF shall return one of the HTTP status code together with the response body listed in Table 6.2.3.2.3.1-2 / Table 6.2.3.2.3.2-2.

* * * Next Change * * * *

##### 5.3.2.3.1	Creation of a subscription

The Subscribe Operation is used by a NF Service Consumer (e.g. AMF) to subscribe to a notification of any changes in status of the NSSAI availability information (e.g. S-NSSAIs available per TA and the restricted S-NSSAI(s) per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF.

Figure 5.3.2.3.1-1 Create a subscription

1.	The NF Service Consumer shall send a POST request to create a subscription resource in the NSSF. The payload body of the POST request shall contain a representation of the individual event subscription resource to be created in the NssfEventSubscriptionCreateData. The request may contain an expiry time, suggested by the NF Service Consumer as a hint, representing the time upto during which the subscription is desired to be kept active and describes the maximum duration after which the subscribed event shall stop generating report. The request may also indicate a specific AMF Set to restrict the subscriptions to notifications applicable to the AMF Set (i.e. notifications related to S-NSSAIs supported by the AMF Set).

2.	On success, "201 Created" shall be returned, and the payload body of the POST response shall contain the representation describing the status of the created subscription in NssfEventSubscriptionCreatedData that may contain the AuthorizedNssaiAvailabilityData information, if available. If there is no supported S-NSSAIs authorized by the NSSF for the TA, the NSSF shall not return the AuthorizedNssaiAvailabilityData for the corresponding TA in the response. The Location header shall contain the location (URI) of the created subscription resource.

The response, based on operator policy and taking into account the expiry time included in the request, may contain the expiry time, as determined by the NSSF, after which the subscription becomes invalid. Once the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the NSSF. The NSSF shall not provide the same expiry time for many subscriptions in order to avoid all of them expiring and recreating the subscription at the same time. If the expiry time is not included in the response, the NF Service Consumer shall consider the subscription to be valid without an expiry time.

On failure or redirection, the NSSF shall return one of the HTTP status code together with the response body listed in Table 6.2.3.3.3.1-2.

* * * Next Change * * * *

##### 5.3.2.3.2	Modification of a subscription

The Subscribe Operation may be used by a NF Service Consumer (e.g. AMF) towards an NSSF supporting the SUMOD feature, when it needs to modify an existing subscription previously created by itself at the AMF.

The NF Service Consumer shall modify the subscription by using HTTP method PATCH with the URI of the individual subscription resource to be modified.

Figure 5.3.2.3.2-1 Modify a subscription

1.	The NF Service Consumer (e.g. AMF) shall send a PATCH request to the resource URI identifying the individual subscription resource. The payload of the body shall contain the PatchDocument which contains one or more PatchItem instructions for updating the subscription data. The NF Service Consumer shall not change the event IE included in the NssfEventSubscriptionCreateData by invoking the PATCH request message. The taiList IE may only be set to an empty array in PATCH request if the NF service consumer and NSSF support the ONSSAI feature.

2.	On success, "200 OK" shall be returned, the payload body of the PATCH response shall contain the representation describing the updated subscription in NssfEventSubscriptionCreatedData. If there is no supported S-NSSAIs authorized by the NSSF for the TA, the NSSF shall not return the AuthorizedNssaiAvailabilityData for the corresponding TA in the response.

On failure or redirection, the NSSF shall return one of the HTTP status code together with the response body listed in Table 6.2.3.4.3.2-2.

* * * Next Change * * * *

#### 5.3.2.4	Unsubscribe Service Operation

##### 5.3.2.4.1	General

The Unsubscribe Operation is used by a NF Service Consumer (e.g. AMF) to unsubscribe to a notification of any previously subscribed changes to the NSSAI availability information.

Figure 5.3.2.4.1-1 Unsubscribe a subscription

1.	The NF Service Consumer shall send a DELETE request to delete an existing subscription resource in the NSSF.

2.	If the request is accepted, the NSSF shall respond with the status code 204 indicating the resource identified by subscription ID is successfully deleted.

On failure or redirection, the NSSF shall return one of the HTTP status code together with the response body listed in Table 6.2.3.4.3.1-2.

* * * Next Change * * * *

#### 5.3.2.5	Notify Service Operation

##### 5.3.2.5.1	General

The Notify Service operation shall be used by the NSSF to update the NF Service Consumer (e.g. AMF) with any change in status, on a per TA basis, of the S-NSSAIs available per TA (unrestricted) and the S-NSSAIs restricted per PLMN in that TA in the serving PLMN of the UE.

Figure 5.3.2.5.1-1: Update the AMF with any S-NSSAIs restricted per TA

1.	The NSSF shall send a POST request to the resource representing the NSSF availability resource in the NF service consumer (e.g. AMF). The payload body of the POST request shall contain the one representations of the individual NssfEventNotification resource. If there is no supported S-NSSAIs authorized by the NSSF for the TA, the NSSF shall not return the AuthorizedNssaiAvailabilityData for the corresponding TA in the notification. If there is no supported S-NSSAIs authorized by the NSSF for all TAs and the NF Service Consumer has indicated support of "EANAN" feature, the NSSF shall set authorizedNssaiAvailabilityData attribute to an empty array.

2.	On success, "204 No Content" shall be returned and the payload body of the POST response shall be empty.

On failure or redirection, the NF service consumer shall return one of the HTTP status code together with the response body listed in Table 6.2.5.2.3.1-2.

* * * Next Change * * * *

#### 5.3.2.6	Delete Service Operation

##### 5.3.2.6.1	General

The Delete Service operation shall be used by the NF service consumer (e.g. AMF) to delete the NSSAI availability information stored for the NF service consumer in the NSSF.

Figure 5.3.2.6.1-1: Delete the NSSAI Availability Information at NSSF

1.	The NF service consumer (e.g. AMF) shall send a DELETE request to remove the NSSAI availability information for the NF service consumer represented by the {nfId} (e.g. AMF ID).

2.	The NSSF shall delete the NSSAI Availability information for the individual AMF and shall return the 204 No Content status code.

On failure or redirection, the NSSF shall return one of the HTTP status code together with the response body listed in Table 6.2.3.2.3.3-2.

* * * Next Change * * * *

#### 5.3.2.7	Options Service Operation

##### 5.3.2.7.1	General

The Options service operation is used by a NF Service Consumer (e.g. AMF) to discover the communication options supported by the NSSF for the ressource.

Figure 5.3.2.7.1-1: Procedure for the discovery of communication options supported by the NSSF

1.	The NF service consumer (e.g. AMF) shall send an OPTIONS request to discover the communication options supported by the NSSF for the ressource.

2.	If the request is accepted, the NSSF shall respond with the status code 200 OK and include an Accept-Encoding header (as described in IETF RFC 7694).

On failure or redirection, the NSSF shall return one of the HTTP status code listed in Table 6.2.3.5.3.1-3.

* * * Next Change * * * *

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	GET

This method retrieves the information related to the selected slice based on the input query parameters provided by the NF service consumer specified in table 6.1.3.2.3.1-1.

This method shall support input query parameters specified in table 6.1.3.2.3.1-1 and the response data structure and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nf-type | NFType | M | 1 | This IE shall contain the NF type of the NF service consumer. |
| nf-id | NfInstanceId | M | 1 | This IE shall contain the NF identifier of the NF service consumer. |
| slice-info-request-for-registration | SliceInfoForRegistration | C | 0..1 | This IE shall be present when the network slice information is requested during the Registration procedure or during EPS to 5GS handover procedure using N26 interface towards an NSSF in the serving PLMN. |
| slice-info-request-for-pdu-session | SliceInfoForPDUSession | C | 0..1 | This IE shall be present when the network slice information is requested during the PDU session establishment procedure. |
| slice-info-request-for-ue-cu | SliceInfoForUEConfigurationUpdate | C | 0..1 | This IE shall be present when the network slice information is requested during UE configuration update procedure. |
| home-plmn-id | PlmnId | C | 0..1 | This IE shall be present in the request towards an NSSF in the serving PLMN if the subscriber is a roamer to the serving PLMN. When present, this IE shall contain the home PLMN Id of the UE. |
| tai | Tai | C | 0..1 | This IE shall be present in the request towards an NSSF in the serving PLMN. When present, this IE shall contain the TAI the UE is currently located. |
| supported-features | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthorizedNetworkSliceInfo | M | 1 | 200 OK | This case represents a successful return of the authorized network slice information selected for the corresponding request. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This represents the case, where the NF service consumer is not authorized to retrieve the slice selection information or the all of the SNSSAIs included in the requested slice selection information is not supported in the PLMN. The application specific error information shall be provided in the "cause" attribute. The "cause" attribute shall be set to: - SNSSAI_NOT_SUPPORTED, if the SNSSAI included in the requested slice selection information is not allowed in the PLMN and there is no default NSSAI value provided in the request. - NOT_AUTHORIZED, if the NF service consumer identified by the NF Id is not authorized to retrieve the slice selection information. See table 6.1.7.3-1 for the description of this error. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

### 6.1.8	Feature negotiation

The feature negotiation mechanism specified in clause 6.6 of 3GPP TS 29.500 [4] shall be used to negotiate the optional features applicable between the NSSF and the NF Service Consumer, for the Nnssf_NSSelection service, if any.

The NF Service Consumer shall indicate the optional features it supports for the Nnssf_NSSelection service, if any, by including the supportedFeatures attribute in the HTTP GET request when requesting the NSSF to provide the allowed NSSAI information.

The NSSF shall determine the supported features for the requested network slice information resource as specified in clause 6.6 of 3GPP TS 29.500 [4] and shall indicate the supported features by including the supportedFeatures attribute in the allowed NSSAI information it returns in the HTTP response.

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [7].

The following features are defined for the Nnssf_NSSelection service.

Table 6.1.8-1: Features of supportedFeatures attribute used by Nnssf_NSSelection service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| x | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. AMF) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the Nnssf_NSSelection service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

* * * Next Change * * * *

### 6.1.x	HTTP redirection

An HTTP request may be redirected to a different NSSF service instance, within the same NSSF or a different NSSF of an NSSF set, e.g. when an NSSF service instance is part of an NSSF (service) set or when using indirect communications (see 3GPP TS 29.500 [4]). See the ES3XX feature in clause 6.1.8.

An SCP that reselects a different NSSF producer instance will return the NF Instance ID of the new NSSF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an NSSF within an NSSF set redirects a service request to a different NSSF of the set using a 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new NSSF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

* * * Next Change * * * *

###### 6.2.3.2.3.1	PUT

This method shall support the request data structures specified in table 6.2.3.2.3.1-1 and the response data structures and response codes specified in table 6.2.3.2.3.1-2.

Table 6.2.3.2.3.1-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NssaiAvailabilityInfo | M | 1 | This IE contains the information regarding the NssaiAvailabilityData for the NF Service Consumer (e.g AMF). |

Table 6.2.3.2.3.1-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthorizedNssaiAvailabilityInfo | M | 1 | 200 OK | This case represents a successful update of the NSSF with the S-NSSAIs the AMF supports per TA. The authorized NSSAI availability (i.e. S-NSSAIs available per TA (unrestricted) and any S-NSSAIs restricted per PLMN in that TA in the serving PLMN of the UE) information shall be returned in the response payload body. |
| N/A |  |  | 204 No Content | This case represents a successful update of the NSSF with the S-NSSAIs the AMF supports per TA, and the authorized NSSAI availability is empty after the update. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 403 Forbidden | When the NF service consumer is not authorized to update the NSSAI availability information or the TAI/S-NSSAI information provided is not supported in the PLMN, the "cause" attribute shall be set to: - SNSSAI_NOT_SUPPORTED, if the S-NSSAI provided is not supported in the PLMN. - NOT_AUTHORIZED, if the NF service consumer identified by the NF Id is not authorized to update the NSSAI availability information. See table 6.2.7.3-1 for the description of this error. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.2.3.1-3: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Content-Encoding | string | O | 0..1 | Content-Encoding, described in IETF RFC 7231 [18] |

Table 6.2.3.2.3.1-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| # Accept-Encoding | # string | # O | # 0..1 | # Accept-Encoding, described in IETF RFC 7694 [19] |

Table 6.2.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

###### 6.2.3.2.3.2	PATCH

This method shall support the request data structures specified in table 6.2.3.2.3.2-1 and the response data structures and response codes specified in table 6.2.3.2.3.2-2.

Table 6.2.3.2.3.2-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PatchDocument | M | 1 | This IE contains the information regarding the JSON patch instructions for updating the AuthorizedNssaiAvailabilityInfo. |

Table 6.2.3.2.3.2-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthorizedNssaiAvailabilityInfo | M | 1 | 200 OK | This case represents a successful update of the NSSF with the S-NSSAIs the AMF supports per TA. If the authorized NSSAI availability (i.e. S-NSSAIs available per TA (unrestricted) and any S-NSSAIs restricted per PLMN in that TA in the serving PLMN of the UE) is changed, the NSSF shall return a data structure of type "AuthorizedNssaiAvailabilityInfo" in the response payload body. |
| N/A |  |  | 204 | This case represents a successful update of the NSSF with the S-NSSAIs the AMF supports per TA, and the authorized NSSAI availability is empty after the update. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 403 Forbidden | When the NF service consumer is not authorized to update the NSSAI availability information or the S-NSSAI information provided is not supported in the PLMN, the "cause" attribute shall be set to: - SNSSAI_NOT_SUPPORTED, if the S-NSSAI provided is not supported in the PLMN. - NOT_AUTHORIZED, if the NF service consumer identified by the NF Id is not authorized to update the NSSAI availability information. See table 6.2.7.3-1 for the description of this error. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application error: - RESOURCE_NOT_FOUND See table 6.2.7.3-1 for the description of this error. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.2.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.3.2.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

###### 6.2.3.2.3.3	DELETE

This method shall support the request data structures specified in table 6.2.3.2.3.3-1 and the response data structures and response codes specified in table 6.2.3.2.3.3-2.

Table 6.2.3.2.3.3-1: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.2.3.3-2: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application error: - RESOURCE_NOT_FOUND See table 6.2.7.3-1 for the description of this error. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.2.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.3.2.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

##### 6.2.3.3.3	Resource Standard Methods

###### 6.2.3.3.3.1	POST

This method shall support the request data structures specified in table 6.2.3.3.3.1-1 and the response data structures and response codes specified in table 6.2.3.3.3.1-2.

Table 6.2.3.3.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NssfEventSubscriptionCreateData | M | 1 | This IE contains the information regarding the SubscriptionData for the AMF to notify any changes to the NSSAI availability information. |

Table 6.2.3.3.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NssfEventSubscriptionCreatedData | M | 1 | 201 Created | This case represents a successful creation of subscription to the change of NSSAI availability information. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application error: - NOT_AUTHORIZED See table 6.2.7.3-1 for the description of these errors. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.3.3.1-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnssf-nssaiavailability/v1/nssai-availability/subscriptions/{subscriptionId} |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

###### 6.2.3.4.3.1	DELETE

This method shall support the request data structures specified in table 6.2.3.4.3.1-1 and the response data structures and response codes specified in table 6.2.3.4.3.1-2.

Table 6.2.3.4.3.1-1: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 6.2.3.4.3.1-2: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 NO Content | This case represents a successful deletion of the subscription. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 404 Not Found | This represents the case when the subscription resource is unavailable. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.4.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.3.3.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

###### 6.2.3.4.3.2	PATCH

This method shall support the request data structures specified in table 6.2.3.4.3.2-1 and the response data structures and response codes specified in table 6.2.3.4.3.2-2.

Table 6.2.3.4.3.2-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PatchDocument | M | 1 | This IE contains the information regarding the JSON patch instructions for updating the NssfEventSubscriptionCreateData. |

Table 6.2.3.4.3.2-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NssfEventSubscriptionCreatedData | M | 1 | 200 OK | This case represents a successful update of the subscription. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the modification of subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - SUBSCRIPTION_NOT_FOUND. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.4.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.3.4.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

###### 6.2.3.5.3.1	OPTIONS

This method queries the communication options supported by the NSSF (see clause 6.9 of 3GPP TS 29.500 [4]). This method shall support the URI query parameters specified in table 6.1.3.5.3.1-1.

Table 6.2.3.5.3.1-1: URI query parameters supported by the OPTIONS method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.5.3.1-2 and the response data structures and response codes specified in table 6.2.3.5.3.2-3.

Table 6.2.3.5.3.1-2: Data structures supported by the OPTIONS Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.5.3.1-3: Data structures supported by the OPTIONS Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 200 OK |  |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set. |
| ProblemDetails | O | 0..1 | 405 Method Not Allowed |  |
| ProblemDetails | O | 0..1 | 501 Not Implemented |  |
| NOTE: The mandatory HTTP error status codes for the OPTIONS method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.5.3.1-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Accept-Encoding | string | O | 0..1 | Accept-Encoding, described in IETF RFC 7694 [19] |

Table 6.2.3.5.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.3.5.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

##### 6.2.5.2.3	Notification Standard Methods

###### 6.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.2.5.2.3.1-1 and the response data structures and response codes specified in table 6.2.5.2.3.1-2.

Table 6.2.5.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NssfEventNotification | M | 1 | Representation of the data to be sent to the NF service consumer (e.g. AMF) to update NSSAI availability information, authorized by the NSSF in the serving PLMN. |

Table 6.2.5.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents a successful update of the NF service consumer (e.g. AMF) with NSSAI availability information. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The NF service consumer shall generate a Location header field containing a URI pointing to the endpoint of another NF service consumer to which the notification should be sent. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The NF service consumer shall generate a Location header field containing a URI pointing to the endpoint of another NF service consumer to which the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location |  | string |  | M |  | 1 |  | A URI pointing to the endpoint of another NF service consumer to which the notification should be sent |  |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of another NF service consumer to which the notification should be sent |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

### 6.2.8	Feature negotiation

The feature negotiation mechanism specified in clause 6.6 of 3GPP TS 29.500 [4] shall be used to negotiate the optional features applicable between the NSSF and the NF Service Consumer, for the Nnssf_NSSAIAvailability service, if any.

The NF Service Consumer shall indicate the optional features it supports for the Nnssf_NSSAIAvailability service, if any, by including the supportedFeatures attribute in the HTTP PUT request when requesting the NSSF to update the NSSAI Availability information.

The NSSF shall determine the supported features for the updated NSSAI Availability information resource as specified in clause 6.6 of 3GPP TS 29.500 [4] and shall indicate the supported features by including the supportedFeatures attribute in the authorized NSSAI availability information it returns in the HTTP response.

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [7].

The following features are defined for the Nnssf_NSSAIAvailability service.

Table 6.2.8-1: Features of supportedFeatures attribute used by Nnssf_NSSAIAvailability service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | ONSSAI | O | Optimized NSSAI Availability Data encoding When this feature is supported: - NSSAI Availability data may be signalled per list or range(s) of TAIs (see clauses 6.2.6.2.3 and 6.2.6.2.4); and - RestrictedSnssai may encode a list of Home PLMN IDs or may be applicable to all of the Home PLMN IDs (see clause 6.2.6.2.5). - NSSF event subscription may encode a list of TAI ranges (see clause 6.2.6.2.8). |
| 2 | SUMOD | O | Subscription Modification in Subscribe Service Operation When this feature is supported, the subscription of NSSAI availability information is supported to be modified (see clause 5.3.2.3.2). |
| 3 | EANAN | O | Empty Authorized NSSAI Availability Notification A NSSF supporting this feature shall send a notification to NF consumer (as subscriber) with empty array of Authorized NSSAI Availability Data, when no supported NSSAI Authorized by the NSSF for all TAs after latest update and the NF consumer indicated support of this feature. A NF Consumer support this feature shall accept empty array of Authorized NSSAI Availability Data in a notification from NSSF and delete locally stored Authorized NSSAI Availability Data previously received. |
| x | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. AMF) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the Nnssf_NSSAIAvailability service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

* * * Next Change * * * *

### 6.2.x	HTTP redirection

An HTTP request may be redirected to a different NSSF service instance, within the same NSSF or a different NSSF of an NSSF set, e.g. when an NSSF service instance is part of an NSSF (service) set or when using indirect communications (see 3GPP TS 29.500 [4]). See the ES3XX feature in clause 6.2.8.

An SCP that reselects a different NSSF producer instance will return the NF Instance ID of the new NSSF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an NSSF within an NSSF set redirects a service request to a different NSSF of the set using a 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new NSSF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

* * * Next Change * * * *

## A.2	Nnssf_NSSelection API

openapi: 3.0.0

info:

version: '2.1.0'

title: 'NSSF NS Selection'

description: |

NSSF Network Slice Selection Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- nnssf-nsselection

servers:

- url: '{apiRoot}/nnssf-nsselection/v2'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

externalDocs:

description: 3GPP TS 29.531 V16.3.0; 5G System; Network Slice Selection Services; Stage 3

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.531/

paths:

/network-slice-information:

get:

summary:  Retrieve the Network Slice Selection Information

tags:

- Network Slice Information (Document)

operationId: NSSelectionGet

parameters:

- name: nf-type

in: query

description: NF type of the NF service consumer

required: true

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- name: nf-id

in: query

description: NF Instance ID of the NF service consumer

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

- name: slice-info-request-for-registration

in: query

description: Requested network slice information during Registration procedure

content:

application/json:

schema:

$ref: '#/components/schemas/SliceInfoForRegistration'

- name: slice-info-request-for-pdu-session

in: query

description: Requested network slice information during PDU session establishment procedure

content:

application/json:

schema:

$ref: '#/components/schemas/SliceInfoForPDUSession'

- name: slice-info-request-for-ue-cu

in: query

description: Requested network slice information during UE confuguration update procedure

content:

application/json:

schema:

$ref: '#/components/schemas/SliceInfoForUEConfigurationUpdate'

- name: home-plmn-id

in: query

description: PLMN ID of the HPLMN

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: tai

in: query

description: TAI of the UE

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

- name: supported-features

in: query

description: Features required to be supported by the NFs in the target slice instance

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful Network Slice Selection)

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizedNetworkSliceInfo'

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnssf-nsselection: Access to the Nnssf_NSSelection API

schemas:

AuthorizedNetworkSliceInfo:

type: object

properties:

allowedNssaiList:

type: array

items:

$ref: '#/components/schemas/AllowedNssai'

minItems: 1

configuredNssai:

type: array

items:

$ref: '#/components/schemas/ConfiguredSnssai'

minItems: 1

targetAmfSet:

type: string

pattern: '^[0-9]{3}-[0-9]{2-3}-[A-Fa-f0-9]{2}-[0-3][A-Fa-f0-9]{2}$'

candidateAmfList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

rejectedNssaiInPlmn:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

rejectedNssaiInTa:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

nsiInformation:

$ref: '#/components/schemas/NsiInformation'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

nrfAmfSet:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfAmfSetNfMgtUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfAmfSetAccessTokenUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

targetAmfServiceSet:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

SubscribedSnssai:

type: object

required:

- subscribedSnssai

properties:

subscribedSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

defaultIndication:

type: boolean

AllowedSnssai:

type: object

required:

- allowedSnssai

properties:

allowedSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nsiInformationList:

type: array

items:

$ref: '#/components/schemas/NsiInformation'

minItems: 1

mappedHomeSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

AllowedNssai:

type: object

required:

- allowedSnssaiList

- accessType

properties:

allowedSnssaiList:

type: array

items:

$ref: '#/components/schemas/AllowedSnssai'

minItems: 1

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

NsiInformation:

type: object

required:

- nrfId

properties:

nrfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nsiId:

$ref: '#/components/schemas/NsiId'

nrfNfMgtUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfAccessTokenUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

MappingOfSnssai:

type: object

required:

- servingSnssai

- homeSnssai

properties:

servingSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

homeSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

SliceInfoForRegistration:

type: object

properties:

subscribedNssai:

type: array

items:

$ref: '#/components/schemas/SubscribedSnssai'

minItems: 1

allowedNssaiCurrentAccess:

$ref: '#/components/schemas/AllowedNssai'

allowedNssaiOtherAccess:

$ref: '#/components/schemas/AllowedNssai'

sNssaiForMapping:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

requestedNssai:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

defaultConfiguredSnssaiInd:

type: boolean

mappingOfNssai:

type: array

items:

$ref: '#/components/schemas/MappingOfSnssai'

minItems: 1

requestMapping:

type: boolean

SliceInfoForPDUSession:

type: object

required:

- sNssai

- roamingIndication

properties:

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

roamingIndication:

$ref: '#/components/schemas/RoamingIndication'

homeSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

SliceInfoForUEConfigurationUpdate:

type: object

properties:

subscribedNssai:

type: array

items:

$ref: '#/components/schemas/SubscribedSnssai'

minItems: 1

allowedNssaiCurrentAccess:

$ref: '#/components/schemas/AllowedNssai'

allowedNssaiOtherAccess:

$ref: '#/components/schemas/AllowedNssai'

defaultConfiguredSnssaiInd:

type: boolean

requestedNssai:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

mappingOfNssai:

type: array

items:

$ref: '#/components/schemas/MappingOfSnssai'

minItems: 1

ConfiguredSnssai:

type: object

required:

- configuredSnssai

properties:

configuredSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

mappedHomeSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

RoamingIndication:

anyOf:

- type: string

enum:

- NON_ROAMING

- LOCAL_BREAKOUT

- HOME_ROUTED_ROAMING

- type: string

NsiId:

type: string

* * * Next Change * * * *

## A.3	Nnssf_NSSAIAvailability API

openapi: 3.0.0

info:

version: '1.1.1'

title: 'NSSF NSSAI Availability'

description: |

NSSF NSSAI Availability Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- nnssf-nssaiavailability

servers:

- url: '{apiRoot}/nnssf-nssaiavailability/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

externalDocs:

description: 3GPP TS 29.531 V16.4.0; 5G System; Network Slice Selection Services; Stage 3

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.531/

paths:

/nssai-availability/{nfId}:

put:

summary: Updates/replaces the NSSF with the S-NSSAIs the NF service consumer (e.g AMF)supports per TA

tags:

- NF Instance ID (Document)

operationId: NSSAIAvailabilityPut

parameters:

- name: nfId

in: path

description: Identifier of the NF service consumer instance

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

- name: Content-Encoding

in: header

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

- name: Accept-Encoding

in: header

description: Accept-Encoding, described in IETF RFC 7231

schema:

type: string

requestBody:

description: Parameters to update/replace at the NSSF, the S-NSSAIs supported per TA

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NssaiAvailabilityInfo'

responses:

'200':

description: OK (Successful update of SNSSAI information per TA)

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizedNssaiAvailabilityInfo'

headers:

Accept-Encoding:

description: Accept-Encoding, described in IETF RFC 7694

schema:

type: string

Content-Encoding:

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

'204':

description: No Content (No supported slices after Successful update)

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: Updates an already existing S-NSSAIs per TA provided by the NF service consumer (e.g AMF)

tags:

- NF Instance ID (Document)

operationId: NSSAIAvailabilityPatch

parameters:

- name: nfId

in: path

description: Identifier of the NF service consumer instance

required: true

schema:

type: string

- name: Content-Encoding

in: header

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

- name: Accept-Encoding

in: header

description: Accept-Encoding, described in IETF RFC 7231

schema:

type: string

requestBody:

description: JSON Patch instructions to update at the NSSF, the S-NSSAIs supported per TA

required: true

content:

application/json-patch+json::

schema:

$ref: '#/components/schemas/PatchDocument'

responses:

'200':

description: OK (Successful update of SNSSAI information per TA)

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizedNssaiAvailabilityInfo'

headers:

Accept-Encoding:

description: Accept-Encoding, described in IETF RFC 7694

schema:

type: string

Content-Encoding:

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

'204':

description: No Content (No supported slices after Successful update)

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

delete:

summary: Deletes an already existing S-NSSAIs per TA provided by the NF service consumer (e.g AMF)

tags:

- NF Instance ID (Document)

operationId: NSSAIAvailabilityDelete

parameters:

- name: nfId

in: path

description: Identifier of the NF service consumer instance

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of SNSSAI information per TA)

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/nssai-availability/subscriptions:

post:

summary: Creates subscriptions for notification about updates to NSSAI availability information

tags:

- Subscriptions (Collection)

operationId: NSSAIAvailabilityPost

parameters:

- name: Content-Encoding

in: header

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

requestBody:

description: Subscription for notification about updates to NSSAI availability information

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NssfEventSubscriptionCreateData'

callbacks:

nssaiAvailabilityNotification:

'{request.body#/nfNssaiAvailabilityUri}':

post:

parameters:

- name: Content-Encoding

in: header

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NssfEventNotification'

responses:

'204':

description: No Content (successful notification)

headers:

Accept-Encoding:

description: Accept-Encoding, described in IETF RFC 7694

schema:

type: string

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the NF service consumer to which the notification should be sent'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the NF service consumer to which the notification should be sent '

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

responses:

'201':

description: Created (Successful creation of subscription for notification)

content:

application/json:

schema:

$ref: '#/components/schemas/NssfEventSubscriptionCreatedData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnssf-nssaiavailability/v1/nssai-availability/subscriptions/{subscriptionId}'

required: true

schema:

type: string

Content-Encoding:

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/nssai-availability/subscriptions/{subscriptionId}:

delete:

summary: Deletes an already existing NSSAI availability notification subscription

tags:

- Subscription ID (Document)

operationId: NSSAIAvailabilityUnsubscribe

parameters:

- name: subscriptionId

in: path

description: Identifier of the subscription for notification

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of subscription for NSSAI Availability notification)

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: updates an already existing NSSAI availability notification subscription

tags:

- Subscription ID (Document)

operationId: NSSAIAvailabilitySubModifyPatch

parameters:

- name: subscriptionId

in: path

description: Identifier of the subscription for notification

required: true

schema:

type: string

- name: Content-Encoding

in: header

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

requestBody:

description: JSON Patch instructions to update at the NSSF, the NSSAI availability notification subscription

required: true

content:

application/json-patch+json::

schema:

$ref: '#/components/schemas/PatchDocument'

responses:

'200':

description: OK (Successful update of NSSAI availability notification subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/NssfEventSubscriptionCreatedData'

headers:

Content-Encoding:

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/nssai-availability:

options:

summary: Discover communication options supported by NSSF for NSSAI Availability

operationId: NSSAIAvailabilityOptions

tags:

- NSSAI Availability Store

responses:

'200':

description: OK

headers:

Accept-Encoding:

description: Accept-Encoding, described in IETF RFC 7694

schema:

type: string

'307':

description: temporary redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same NSSF or NSSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnssf-nssaiavailability: Access to the Nnssf_NSSAIAvailability API

schemas:

NssaiAvailabilityInfo:

type: object

required:

- supportedNssaiAvailabilityData

properties:

supportedNssaiAvailabilityData:

type: array

items:

$ref: '#/components/schemas/SupportedNssaiAvailabilityData'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

amfSetId:

type: string

pattern: '^[0-9]{3}-[0-9]{2-3}-[A-Fa-f0-9]{2}-[0-3][A-Fa-f0-9]{2}$'

SupportedNssaiAvailabilityData:

type: object

required:

- tai

- supportedSnssaiList

properties:

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

supportedSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExtSnssai'

minItems: 1

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

taiRangeList:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TaiRange'

minItems: 1

AuthorizedNssaiAvailabilityData:

type: object

required:

- tai

- supportedSnssaiList

properties:

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

supportedSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExtSnssai'

minItems: 1

restrictedSnssaiList:

type: array

items:

$ref: '#/components/schemas/RestrictedSnssai'

minItems: 1

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

taiRangeList:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TaiRange'

minItems: 1

RestrictedSnssai:

type: object

required:

- homePlmnId

- sNssaiList

properties:

homePlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

sNssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExtSnssai'

minItems: 1

homePlmnIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

roamingRestriction:

type: boolean

default: false

AuthorizedNssaiAvailabilityInfo:

type: object

required:

- authorizedNssaiAvailabilityData

properties:

authorizedNssaiAvailabilityData:

type: array

items:

$ref: '#/components/schemas/AuthorizedNssaiAvailabilityData'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

NssfEventSubscriptionCreateData:

type: object

required:

- nfNssaiAvailabilityUri

- taiList

- event

properties:

nfNssaiAvailabilityUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

event:

$ref: '#/components/schemas/NssfEventType'

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

amfSetId:

type: string

pattern: '^[0-9]{3}-[0-9]{2-3}-[A-Fa-f0-9]{2}-[0-3][A-Fa-f0-9]{2}$'

taiRangeList:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TaiRange'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

NssfEventSubscriptionCreatedData:

type: object

required:

- subscriptionId

properties:

subscriptionId:

type: string

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

authorizedNssaiAvailabilityData:

type: array

items:

$ref: '#/components/schemas/AuthorizedNssaiAvailabilityData'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

NssfEventNotification:

type: object

required:

- subscriptionId

- authorizedNssaiAvailabilityData

properties:

subscriptionId:

type: string

authorizedNssaiAvailabilityData:

type: array

items:

$ref: '#/components/schemas/AuthorizedNssaiAvailabilityData'

NssfEventType:

anyOf:

- type: string

enum:

- SNSSAI_STATUS_CHANGE_REPORT

- type: string

PatchDocument:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

* * * End of Changes * * * *