3GPP TSG-CT WG1 Meeting #126-e	C1-206678

# Electronic meeting, 15-23 October 2020												was C1-206424

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.380 | CR | 0230 | rev | 8 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authentication of the MIKEY-SAKKE I_Message validation in pre-established session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17 |  |  |  |  |  | Date: |  |  | 2020-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No procedure is defined to return the authentication failure if MIKEY-SAKKE I_Message validation fails in case of the private call or ambient call handling based on pre-established session for auto commencement mode in 3GPP TS 24.379 and 3GPP TS 24.380. There is no mechanisam to convey the call request rejection or call gets terminated or participant is removed from the ongoing call or any other error cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The new Cause Code field has been added to the ‘Disconnect’ message of Pre-established session call control messages to indicate about the authentication failure or any other failure reason and existing Reason code is enhanced to provide the additional error codes if the terminating client rejects it.. Client can send the appropriate error code if the call is rejected and server can pass the error code back to the originator. For any other case either participant is removed or call gets terminated then the reason cause shall be sent to the participant with appropriate information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Call continues even if the user plane data decoding is not possible and there is no information about call disconnect reason. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.1, 8.3.3.8, 8.3.3.11(NEW), 8.3.5, 9.2.2.3.2 and 9.3.2.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 8: 9.3.2.4.7: Updated the wording in step 2) a) iii) 8.3.3.1: Reordered the reason cause field entry. Placed before PCK I_MESSAGE 8.3.3.11: Corrected text for ‘8’, replaced session with call for ‘5’ and appended expired word to a text for ‘7’. Rev 7: As per the 125e meeting discussion, the connect ack will use the existing field to convey error response with the additional error codes instead of new field. Rev 6: Corrected 9.2.2.3.2 and 9.3.2.4.7 as per review feedback. Rewording and ‘and’ is updated properly. Rev 5: Re-uploaded without encrypted file Rev 4: Handling added in participating server to copy the acknowledgement content to disconnect. Added additional info in 9.2.2.3.2. Rev 3: In table 8.3.3.1-1, addition of new element is re-ordered to expect to see the new element line to be after “Reason Code” to keep the Field ID in ascending numerical order. Table 8.3.5-1, Corrected the addition of new element as ‘Reason cause’ instead of ‘Reason code’ as it was typo error. |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

#### 8.3.3.1	Introduction

This subclause describes the pre-established session call control specific data fields.

The pre-established session call control specific data fields are contained in the application-dependent data of the pre-established session call control message. The pre-established session call control specific data fields follow the syntax specified in subclause 8.1.3.

Table 8.3.3.1-1: Void

Table 8.3.3.1-2 lists the available data fields including the assigned field ID.

Table 8.3.3.1-2: Pre-established session call control data fields

| Field name | Field ID |  | Reference |
| --- | --- | --- | --- |
|  | Decimal | Binary |  |
| Media Streams | 000 | 00000000 | Subclause 8.3.3.2 |
| MCPTT Session Identity | 001 | 00000001 | Subclause 8.3.3.3 |
| Warning Text | 002 | 00000010 | Subclause 8.3.3.4 |
| MCPTT Group Identity | 003 | 00000011 | Subclause 8.3.3.5 |
| Answer State | 004 | 00000100 | Subclause 8.3.3.6 |
| Inviting MCPTT User Identity | 005 | 00000101 | Subclause 8.3.3.7 |
| Reason Code | 006 | 00000110 | Subclause 8.3.3.8 |
| Reason Cause | 007 | 00000111 | Subclause 8.3.3.11 |
| PCK I_MESSAGE | 192 | 11000000 | Subclause 8.3.3.10 |

The following subclauses describe the coding of each data field.

* * * * * * * NEXT CHANGE * * * * * * *

#### 8.3.3.8	Reason Code field

The Reason Code field contains the answer to a pre-established call control message.

Table 8.3.3.8-1 describes the coding of the Reason Code field.

Table 8.3.3.8-1: Reason Code field coding

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|Reason Code    |Reason Code    |Reason Code  value             |

|field ID value |length value   |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The <Reason Code field ID> value is a binary value and shall be set according to table 8.3.3.1-2.

The <Reason Code length> value is a binary value and shall have the value '2' indicating the total length in octets of the <Reason Code> value item.

The <Reason Code> value is a 16-bit binary value with the following values:

'0'	Accepted

'1'	Busy

'2'	Not Accepted

'3'	Authentication of the MIKEY-SAKKE I_MESSAGE failed

'4'	Integrity protection check failed

'5'	Decrypting XML content failed

All other values are reserved for future use.

* * * * * * * NEXT CHANGE * * * * * * *

#### 8.3.3.11	Reason Cause field

The Reason Cause field contains the disconnect or reject reason for the pre-established call. The values can be subset from the Reason Code field contains the answer to a pre-established call control message other than 'Accepted' Reason code value as described in subclause 8.3.3.8 or any other values.

Table 8.3.3.11-1 describes the coding of the Reason Cause field.

Table 8.3.3.11-1: Reason Cause field coding

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|Reason Cause   |Reason Cause   |Reason Cause value             |

|field ID value |length value   |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The <Reason Cause field ID> value is a binary value and shall be set according to table 8.3.3.1-2.

The <Reason Cause length> value is a binary value and shall have the value '2' indicating the total length in octets of the <Reason Cause> value item.

The <Reason Cause> value is a 16-bit binary value with the following values:

'0'	Busy

'1'	authentication of the MIKEY-SAKKE I_MESSAGE failed

'2'	integrity protection check failed

'3'	unable to decrypt XML content

'4'	inactivity timer expired

'5'	there are only one or no participants in the MCPTT call

'6'	the minimum number of affiliated MCPTT group members is not present

'7'	group call timer expired

'8'	the MCPTT session has lasted longer than the maximum duration of a private call

All other values are reserved for future use.

* * * * * * * NEXT CHANGE * * * * * * *

### 8.3.5	Disconnect message

Table 8.3.5-1 shows the content of the Connect message.

Table 8.3.5-1: Disconnect message

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P| Subtype |   PT=APP=204  |          Length               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|               SSRC of participating MCPTT function            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                          name=MCPC                            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   MCPTT Session Identity field                |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      Reason cause field                        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

With the exception of the three first 32-bit words the order of the fields are irrelevant.

Subtype:

The subtype shall be coded according to table 8.3.2-1.

Length:

The length shall be coded as specified in subclause 8.1.2.

SSRC:

The SSRC field shall carry the SSRC of the participating MCPTT function.

The SSRC field shall be coded as specified in IETF RFC 3550 [3].

MCPTT Session Identity:

The MCPTT Session Identity field is coded as described in subclause 8.3.3.3.

Reason Cause:

The Reason Cause field is coded as described in subclause 8.3.3.11.

* * * * * * * NEXT CHANGE * * * * * * *

##### 9.2.2.3.2	Receive Connect message (R: Connect)

Upon reception of a Connect message:

1.	if the MCPTT client accepts the incoming call the MCPTT client:

a.	shall send the Acknowledgement message with Reason Code field set to 'Accepted';

b.	shall use only the media streams of the pre-established session which are indicated as used in the associated call session Media Streams field, if the Connect contains a Media Streams field;

c.	shall create an instance of the 'Floor participant state transition diagram for basic operation' as specified in subclause 6.2.4; and

d. shall enter the 'U: Pre-established session in use' state; or

2.	Otherwise the MCPTT client:

a.	shall send the Acknowledgement message with the Reason Code field set to an appropriate <Reason Code> value; and

b. 	shall remain in 'U: Pre-established session not in use' state.

* * * * * * * NEXT CHANGE * * * * * * *

##### 9.3.2.4.7	Receive Acknowledge message ((R: successful Ack) or (R: failure Ack))

Upon receiving an Acknowledge message from the MCPTT client, the participating MCPTT function:

1.	shall stop timer T55 (Connect);

2.	if the Reason Code is not 'Accepted' the participating MCPTT function:

a.	shall send Disconnect message to the MCPTT client;

i.	shall include the MCPTT Session Identity field with the same content as sent in the Connect message when the MCPTT call was established;

ii.	should set the first bit in the subtype of the Disconnect message to '1' (Acknowledgment is required) as described in subclause 8.3.2; and

iii.	if a Reason Code field was received in the Acknowledge message with other than an 'Accepted' reason code value, shall include the Reason Cause field with a value corresponding to the received Reason Code value; and

b.	shall start timer T56 (Disconnect);

c.	shall initialise counter C56 (Disconnect) to 1;

d.	shall send a call release indication to the controlling MCPTT function (see 3GPP TS 24.379 [2]); and

e.	shall enter the 'G: Call releasing' state; and

3.	otherwise the participating MCPTT function:

a.	shall send a SIP 200 (OK) response to the SIP INVITE request as specified in 3GPP TS 24.379 [2] if the SIP 200 (OK) response to the SIP INVITE request is not already sent; and

b.	shall remain in the 'G: Pre-established session in use' state.

* * * * * * * END CHANGES * * * * * * *