3GPP TSG-CT WG1 Meeting #90e	CP-203224

Electronic meeting, 06th – 08th December 2020	Revision of C1-207276

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.502 | CR | 0176 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Setting TCP source port number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc17-non3GPP |  |  |  |  |  | Date: |  |  | 2020-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In the current description of TCP packet encapsulation, UE shall set the source port number for inner IPv6 datagram as specified in clause 8.2.2 of TS 24.502 A) if the UE constructs the inner IPv6 datagram: - the source address field shall be set to the IPv6 address in the INTERNAL_IP6_ADDRESS attribute; - the source port number shall be set to the UE's TCP port number; However there is no such description for inner IPv4 datagram. There is no reason to have different UE implementations for IPv6 type and IPv4 type. It is proposed to keep the same text in the specification for IPv4 type as for IPv6 type, i.e., UE sets the source port number for inner IPv4 datagram. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE shall set the source port number when it constructs the inner IPv4 datagram |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | incoherent specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev of C1-207276 There is no technical change but to update the Reason for change and Consequences if not approved |  |  |  |  |  |  |  |  |

***** start of change*****

8.2.2	TCP packet encapsulation

NOTE 1: 	This subclause is used for encapsulating of TCP packets when establishing TCP connection as described in subclause 8.2.3, when re-establishing TCP connection as described in subclause 8.2.3A, when transporting NAS messages over TCP connection as described in subclause 8.2.4, and when releasing TCP connection as described in subclause 8.2.5.

If a TCP packet is transported between the UE and the N3IWF for untrusted non-3GPP access or the TNGF for trusted non-3GPP access, and:

a)	if the IKE_AUTH response message contained the INTERNAL_IP4_ADDRESS attribute and the NAS_IP4_ADDRESS notify payload, an inner IPv4 datagram shall be constructed where:

1)	the TCP packet shall be encapsulated in the inner IPv4 datagram with IPv4 header where:

A)	if the UE constructs the inner IPv4 datagram:

-	the source address field shall be set to the IPv4 address in the INTERNAL_IP4_ADDRESS attribute;

-	the source port number shall be set to the UE's TCP port number;

-	the destination address field shall be set to the IPv4 address in the NAS_IP4_ADDRESS notify payload; and

-	the destination port number shall be set to the NAS_TCP_PORT notify payload;

B)	if the N3IWF for untrusted non-3GPP access or the TNGF for trusted non-3GPP access constructs the inner IPv4 datagram:

-	the source address field shall be set to the IPv4 address in the NAS_IP4_ADDRESS notify payload;

-	the source port number shall be set to the NAS_TCP_PORT notify payload;

-	the destination address field shall be set to the IPv4 address in the INTERNAL_IP4_ADDRESS attribute; and

-	the destination port number shall be set to the UE's TCP port number; and

NOTE 2:	Since the UE always initiates the NAS message exchange with the N3IWF for untrusted non-3GPP access and the TNGF for trusted non-3GPP access, the N3IWF for untrusted non-3GPP access and the TNGF for trusted non-3GPP access receive the UE's TCP port number in the TCP SYN packet exchange and use it when sending NAS messages towards the UE or when re-establishing the TCP connection upon failure.

C)	the protocol field shall be set to 06H;

2)	the inner IPv4 datagram shall be protected employing the ESP protocol in tunnel mode as specified in IETF RFC 4303 [11] where:

A)	the SPI field in the ESP packet shall be set to the SPI of the signalling IPsec SA; and

B)	the next header field in the ESP packet shall be set to 04H; and

3)	the IP packet encapsulating the ESP protected inner IPv4 datagram shall be sent to the peer for the SPI of the signalling IPsec SA; or

b)	if the IKE_AUTH response message contained the INTERNAL_IP6_ADDRESS attribute and the NAS_IP6_ADDRESS notify payload, an inner IPv6 datagram shall be constructed where:

1)	the TCP packet shall be encapsulated in the inner IPv6 datagram with IPv6 header where:

A)	if the UE constructs the inner IPv6 datagram:

-	the source address field shall be set to the IPv6 address in the INTERNAL_IP6_ADDRESS attribute;

-	the source port number shall be set to the UE's TCP port number;

-	the destination address field shall be set to the IPv6 address in the NAS_IP6_ADDRESS notify payload; and

-	the destination port number shall be set to the NAS_TCP_PORT notify payload;

B)	if the N3IWF for untrusted non-3GPP access or the TNGF for trusted non-3GPP access constructs the inner IPv6 datagram:

-	the source address field shall be set to the IPv6 address in the NAS_IP6_ADDRESS notify payload;

-	the source port number shall be set to the NAS_TCP_PORT notify payload;

-	the destination address field shall be set to the IPv6 address in the INTERNAL_IP6_ADDRESS attribute; and

-	the destination port number shall be set to the UE's TCP port number; and

NOTE 3:	Since the UE always initiates the NAS message exchange with the N3IWF for untrusted non-3GPP access and the TNGF for trusted non-3GPP access, the N3IWF for untrusted non-3GPP access and the TNGF for trusted non-3GPP access receive the UE's TCP port number in the TCP SYN packet exchange and use it when sending NAS messages towards the UE or when re-establishing the TCP connection upon failure.

C)	the next header field shall be set to 06H;

2)	the inner IPv6 datagram shall be protected employing the ESP protocol in tunnel mode as specified in IETF RFC 4303 [11] where:

A)	the SPI field in the ESP packet shall be set to the SPI of the signalling IPsec SA; and

B)	the next header field in the ESP packet shall be set to 29H, and

3)	the IP packet encapsulating the ESP protected inner IPv6 datagram shall be sent to the peer for the SPI of the signalling IPsec SA.

If the UE receives an IKE_AUTH response message containing both NAS_IP4_ADDRESS and NAS_IP6_ADDRESS notify payload, the UE:

a)	shall select and use either NAS_IP4_ADDRESS or NAS_IP6_ADDRESS;

b)	shall not switch between NAS_IP4_ADDRESS and NAS_IP6_ADDRESS for TCP packet transport during the lifetime of the IKE SA; and

c)	shall not switch between NAS_IP4_ADDRESS and NAS_IP6_ADDRESS when rekeying any child SA or IKE SA.

The ESP packet format is shown in figure 8.2.2-1 and figure 8.2.2-2:

| Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octets |
| Security Parameters Index (SPI) |  |  |  |  |  |  |  | 1-4 |
| Sequence Number |  |  |  |  |  |  |  | 5-8 |
| Payload data (inner IP packet containing TCP packet) |  |  |  |  |  |  |  | 9-m |
| Padding |  |  |  |  |  |  |  | (m+1) - n |
| Padding length |  |  |  |  |  |  |  | n+1 |
| Next header |  |  |  |  |  |  |  | n+2 |
| Integrity Check Value (ICV) |  |  |  |  |  |  |  | (n+3) - x |

Figure 8.2.2-1: ESP packet format for TCP packet (re-)establishing or releasing TCP connection

| Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octets |
| Security Parameters Index (SPI) |  |  |  |  |  |  |  | 1-4 |
| Sequence Number |  |  |  |  |  |  |  | 5-8 |
| Payload data (inner IP packet containing TCP packet encapsulating NAS message or partial NAS message) |  |  |  |  |  |  |  | 9-m |
| Padding |  |  |  |  |  |  |  | (m+1) - n |
| Padding length |  |  |  |  |  |  |  | n+1 |
| Next header |  |  |  |  |  |  |  | n+2 |
| Integrity Check Value (ICV) |  |  |  |  |  |  |  | (n+3) - x |

Figure 8.2.2-2: ESP packet format for TCP packet encapsulating NAS message or partial NAS message

*****end of  change*****