3GPP TSG-CT Meeting #91e	CP-210076

# E-Meeting, 18th – 24th March 2021

(revision of CP-aabbcc)

# Source:	3GPP TSG CT WG4

# Title:	New WID on Start of Pause of Charging via User Plane path

# Document for:	Approval

Agenda Item:	17.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Start of Pause of Charging via User Plane

## Acronym: SPOCUP

## Unique identifier: 	910007

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

PGW or SMF Pause of Charging is specified for EPS and 5GS in 3GPP TS 23.401 and 3GPP TS 23.502 respectively.

When CUPS is deployed in the network, as specified in 3GPP TS 29.244, when the UP Function (SGW-U or I/V-UPF) detects the Dropped DL Traffic Threshold is reached, the UP function sends a report to the CP function (i.e. SGW-C or I/V-SMF) which leads the CP function to send a "Start of Pause of Charging" request via control plane signalling to the peer CP function (PGW, or (h)SMF) and in turn the peer CP function then sends an Sxb/N4 message to instruct the peer UP function (PGW-U, PSA UPF) to pause charging measurements.

Such start of pause of charging takes time, i.e. transferring time of 2 PFCP messages plus transferring time between CP functions plus additional processing time (to handle PFCP, or GTPv2 for EPS or HTTP message for 5GC), more DL data packets will be charged in the PGW-U or PSA UPF while the packets will be discarded in the SGW-U and I/V-UPF. With the very high throughputs that are supported in EPS and 5GS today, this can still result in significant charging discrepancies, especially considering eMBB services.

A solution is desired to speed up the triggering of charging pause in the PGW-U or PSA UPF.

## 4	Objective

The objective of this work is to define a User Plane (GTP-U) based solution to start pause of charging for a PDN connection or a PDU session to reduce the Charging Data Record discrepancies between SGW or I/V-SMF and PGW or (h)SMF caused by the control plane signalling latency in existing solution.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.244 | Define an indication over Sxa/N4 to request UP function to send new GTP-U message to start of pause of charging. Define an indication over Sxb/N4 to indicate to the UP function to stop the usage measurement for URR(s) at receiving the new GTP-U message to start of pause of charging. | TSG#92 | CT4 responsibility |
| 29.060 | Reserve the new message code and IE type for the new GTP-U message and IE(s) included. | TSG#92 | CT4 responsibility |
| 29.281 | Definition of new GTP-U message to support triggering pause of charging | TSG#92 | CT4 responsibility |
| 29.274 | Addition of a new flag in Create Session Response to indicate whether the PGW supports the new feature. | TSG#92 | CT4 responsibility |
| 29.502 | Addition of a new flag in Create Response to indicate whether the SMF supports the new feature. | TSG#92 | CT4 responsibility |

## 6	Work item Rapporteur(s)

YONG, YANG, Ericsson, frank.yong.yang@ericsson.com

## 7	Work item leadership

CT4

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Ericsson

Nokia

Nokia Shanghai Bell

Verizon

China Mobile

NTTDOCOMO