3GPP TSG-CT Meeting #92e	CP-211094

# E-Meeting, 14th – 16th June 2021

(revision of CP-210071)

# Source:	3GPP TSG CT WG4

# Title:	Revised WID on CT aspects on Same PCF Selection For AMF and SMF

# Document for:	Approval

Agenda Item:	17.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: New WID on CT aspects on Same PCF Selection For AMF and SMF

## Acronym: TEI17_SPSFAS

## Unique identifier: 910003

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI17_SPSFAS | SA2 | 880016 | Same PCF Selection For AMF and SMF |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification: None

## 3	Justification

In Rel 15&16, the same PCF selection for the AMF and the SMF is achieved by the mechanism of AMF optionally forwarding the selected PCF to SMF instance(s) during the PDU Session Establishment procedure(s), and the SMF may select the received PCF for SM Policy Control based on local policies.

For pure 5GS, there might be multiple DNNs/S-NSSAIs subscribed, and consequently there might be multiple SMFs and multiple PCFs selected for these DNNs/S-NSSAIs, and only one of the PCFs can be the same as the one for the AMF; Besides, for EPS to 5GS mobility, the PCF selection for AMF is performed after the PCF selection for SMF in 5GS, e.g. UE initially attaching in EPS and establish PDN connection and then handover to 5GS, because the AMF has not mechanism to acquire the PCF the SMF selected before.

Considering the benefits of binding AM and SM policies (e.g. change RFSP values for a UE when the quota for the DNN for eMBB reaches the limit, or simplify the network deployment), stage 2 has proposed to specify a complete mechanism for same PCF selection to cover both the EPS to 5GS mobility scenario and the initially accessing via 5GS scenario. And the WID on Same PCF Selection For AMF and SMF in SP-200447 was approved, the normative work will start soon.

- Stage 3 work item supports:

-	the same PCF selection for AMF and SMF in the EPS to 5GS mobility scenarios;

## 4	Objective

The objective of this work item is to specify the following stage 3 procedures:

CT3:

-	Protocol enhancement for description of PCF selection

CT4:

-	Protocol enhancement for Nsmf_PDUSession service to indicate SMF of same PCF selection.

-	Protocol enhancement for Nudm_UEContextManagement service to support AMF selects the same PCF for AMF and SMF.

-	Protocol enhancement for Nudr_DataRepository service to support AMF selects the same PCF for AMF and SMF.

NOTE:	The impacts in the UDR API may be reflected as changes in the UDM API (3GPP TS 29.503).

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.513 | Enhancement of the description of PCF selection. | CT#93 (September 2021) | CT3 responsibility |
| 29.502 | Nsmf_PDUSession service to indicate SMF of same PCF selection | CT#93 ( September 2021) | CT4 responsibility |
| 29.503 | Enhancement of Nudm_UEContextManagement service to support AMF selects the same PCF for AMF and SMF | CT#93 ( September 2021) | CT4 responsibility |

## 6	Work item Rapporteur(s)

Yue Sun, China Telecom, suny20@chinatelecom.cn

## 7	Work item leadership

CT4

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

China Telecom

Huawei

ZTE

CATT

China Mobile