3GPP TSG-CT TSG Meeting #92-e	CP-211103

# E-Meeting, 14th – 16th June 2021	is revision of C4-213212

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0278 | rev | 1- | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additions of description in OpenAPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the open API descriptions are missing for some definitions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Descriptions are added in Annex A.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some definitions of Data types do not have descriptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces bacward compatible changes to - TS29571_CommonData. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: correction of typo's and wrong identations |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

## A.2	Data related to Common Data Types

openapi: 3.0.0

info:

version: '1.3.0-alpha.1'

title: 'Common Data Types'

description: |

Common Data Types for Service Based Interfaces.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.571 Common Data Types for Service Based Interfaces, version 17.1.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.571/'

paths: {}

components:

schemas:

#

# Common Data Types for  Generic usage definitiones as defined in clause 5.2

#

#

# COMMON SIMPLE DATA TYPES

#

Binary:

format: binary

type: string

description: string with format "binary" as defined in OpenAPI

BinaryRm:

format: binary

type: string

nullable: true

description: string with format "binary" as defined in OpenAPI OpenAPI with "nullable=true" property.

Bytes:

format: byte

type: string

description: string with format "bytes" as defined in OpenAPI

BytesRm:

format: byte

type: string

nullable: true

description: string with format "bytes" as defined in OpenAPI OpenAPI with "nullable=true" property.

Date:

format: date

type: string

description: string with format "date" as defined in OpenAPI

DateRm:

format: date

type: string

nullable: true

description: string with format "date" as defined in OpenAPI OpenAPI with "nullable=true" property.

DateTime:

format: date-time

type: string

description: string with format "date-time" as defined in OpenAPI.

DateTimeRm:

format: date-time

type: string

nullable: true

description: string with format "date-time" as defined in OpenAPI with "nullable=true" property.

DiameterIdentity:

type: string

pattern: '^([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}$'

description: string containing an FQDN or realm as defined in RFC 6733.

DiameterIdentityRm:

type: string

pattern: '^([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}$'

nullable: true

description: string containing an FQDN or realm as defined in RFC 6733 with "nullable=true" property.

Double:

format: double

type: number

description: string with format "double" as defined in OpenAPI

DoubleRm:

format: double

type: number

nullable: true

description: string with format "double" as defined in OpenAPI with "nullable=true" property.

DurationSec:

type: integer

description: indicating a time in seconds.

DurationSecRm:

type: integer

nullable: true

description: indicating a time in seconds with OpenAPI defined "nullable=true" property.

Float:

format: float

type: number

description: string with format "float" as defined in OpenAPI.

FloatRm:

format: float

type: number

nullable: true

description: string with format "float" as defined in OpenAPI with the OpenAPI defined "nullable=true" property.

Int32:

format: int32

type: integer

description: string with format "int32" as defined in OpenAPI.

Int32Rm:

format: int32

type: integer

nullable: true

description: string with format "int32" as defined in OpenAPI with the OpenAPI defined "nullable=true" property.

Int64:

type: integer

format: int64

description: string with format "int64" as defined in OpenAPI.

Int64Rm:

format: int64

type: integer

nullable: true

description: string with format "int64" as defined in OpenAPI with the OpenAPI defined "nullable=true" property.

Ipv4Addr:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

description: String identifying a IPv4 address formatted in the "dotted decimal" notation as defined in RFC 1166.

example: '198.51.100.1'

Ipv4AddrRm:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

nullable: true

description: String identifying a IPv4 address formatted in the "dotted decimal" notation as defined in RFC 1166 with the OpenAPI defined "nullable=true" property.

Ipv4AddrMask:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])(\/([0-9]|[1-2][0-9]|3[0-2]))$'

description: String identifying a IPv4 address mask formatted in the "dotted decimal" notation as defined in RFC 1166.

example: '198.51.0.0/16'

Ipv4AddrMaskRm:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])(\/([0-9]|[1-2][0-9]|3[0-2]))$'

example: '198.51.0.0/16'

nullable: true

description: String identifying a IPv4 address mask formatted in the "dotted decimal" notation as defined in RFC 1166 with the OpenAPI defined "nullable=true" property.

Ipv6Addr:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

description: String identifying an IPv6 address formatted according to clause 4 of RFC5952. The mixed IPv4 IPv6 notation according to clause 5 of RFC5952 shall not be used

Ipv6AddrRm:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

nullable: true

description: String identifying an IPv6 address formatted according to clause 4 of RFC5952 with the OpenAPI "nullable= true" property. The mixed IPv4 IPv6 notation according to clause 5 of RFC5952 shall not be used.

Ipv6Prefix:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

description: String identifying an IPv6 address prefix formatted according to clause 4 of RFC 5952. IPv6Prefix data type may contain an individual /128 IPv6 address.

example: '2001:db8:abcd:12::0/64'

Ipv6PrefixRm:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

nullable: true

description: String identifying an IPv6 address prefix formatted according to clause 4 of RFC 5952 with the OpenAPI "nullable= true" property. IPv6Prefix data type may contain an individual /128 IPv6 address.

MacAddr48:

type: string

pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'

description: String identifying a MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of RFC 7042

MacAddr48Rm:

type: string

pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'

nullable: true

description: String identifying a MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of RFC 7042 with the OpenAPI "nullable= true" property.

SupportedFeatures:

type: string

pattern: '^[A-Fa-f0-9]*$'

description: A string used to indicate the features supported by an API that is used as defined in clause 6.6 in 3GPP TS 29.500. The string shall contain a bitmask indicating supported features in hexadecimal representation Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent the support of 4 features as described in table 5.2.2-3. The most significant character representing the highest-numbered features shall appear first in the string, and the character representing features 1 to 4 shall appear last in the string. The list of features and their numbering (starting with 1) are defined separately for each API. If the string contains a lower number of characters than there are defined features for an API, all features that would be represented by characters that are not present in the string are not supported

Uinteger:

type: integer

minimum: 0

description: Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

UintegerRm:

type: integer

minimum: 0

nullable: true

description: Unsigned Integer, i.e. only value 0 and integers above 0 are permissible with the OpenAPI "nullable= true" property.

Uint16:

type: integer

minimum: 0

maximum: 65535

description: Integer where the allowed values correspond to the value range of an unsigned 16-bit integer.

Uint16Rm:

type: integer

minimum: 0

maximum: 65535

nullable: true

description: Integer where the allowed values correspond to the value range of an unsigned 16-bit integer with the OpenAPI "nullable= true" property.

Uint32:

type: integer

minimum: 0

maximum: 4294967295 #(2^32)-1

description: Integer where the allowed values correspond to the value range of an unsigned 32-bit integer.

Uint32Rm:

format: int32

type: integer

minimum: 0

maximum: 4294967295 #(2^32)-1

nullable: true

description: Integer where the allowed values correspond to the value range of an unsigned 32-bit integer with the OpenAPI "nullable= true" property.

Uint64:

type: integer

minimum: 0

maximum: 18446744073709551615 #(2^64)-1

description: Integer where the allowed values correspond to the value range of an unsigned 64-bit integer.

Uint64Rm:

type: integer

minimum: 0

maximum: 18446744073709551615 #(2^64)-1

nullable: true

description: Integer where the allowed values correspond to the value range of an unsigned 64-bit integer with the OpenAPI "nullable= true" property.

Uri:

type: string

description: String providing an URI formatted according to RFC 3986

UriRm:

type: string

nullable: true

description: String providing an URI formatted according to RFC 3986 with the OpenAPI "nullable= true" property.

VarUeId:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|msisdn-[0-9]{5,15}|extid-[^@]+@[^@]+|gci-.+|gli-.+|.+)$'

description: String represents the SUPI or GPSI

VarUeIdRm:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|msisdn-[0-9]{5,15}|extid-[^@]+@[^@]+|gci-.+|gli-.+|.+)$'

nullable: true

description: String represents the SUPI or GPSI with the OpenAPI "nullable= true" property.

TimeZone:

type: string

example: '-08:00+1'

description: String with format "<time-numoffset>" optionally appended by "<daylightSavingTime>", where -  <time-numoffset> shall represent the time zone adjusted for daylight saving time and be encoded as time-numoffset as defined in clause 5.6 of IETF RFC 3339; - <daylightSavingTime> shall represent the adjustment that has been made and shall be encoded as "+1" or "+2" for a +1 or +2 hours adjustment. The example is for 8 hours behind UTC, +1 hour adjustment for Daylight Saving Time.

TimeZoneRm:

type: string

nullable: true

description: String with format "<time-numoffset>" optionally appended by "<daylightSavingTime>", where -  <time-numoffset> shall represent the time zone adjusted for daylight saving time and be encoded as time-numoffset as defined in clause 5.6 of IETF RFC 3339; - <daylightSavingTime> shall represent the adjustment that has been made and shall be encoded as "+1" or "+2" for a +1 or +2 hours adjustment. But with the OpenAPI "nullable= true" property.

StnSr:

type: string

description: String representing the STN-SR as defined in clause 18.6 of 3GPP TS 23.003.

StnSrRm:

type: string

nullable: true

description: String representing the STN-SR as defined in clause 18.6 of 3GPP TS 23.003 with the OpenAPI "nullable= true" property.

CMsisdn:

type: string

pattern: '^[0-9]{5,15}$'

description: String representing the C-MSISDN as defined in clause 18.7 of 3GPP TS 23.003.

CMsisdnRm:

type: string

pattern: '^[0-9]{5,15}$'

nullable: true

description: String representing the C-MSISDN as defined in clause 18.7 of 3GPP TS 23.003 with the OpenAPI "nullable= true" property.

DayOfWeek:

type: integer

minimum: 1

maximum: 7

description: integer between and including 1 and 7 denoting a weekday. 1 shall indicate Monday, and the subsequent weekdays shall be indicated with the next higher numbers. 7 shall indicate Sunday.

TimeOfDay:

type: string

description: String with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339. Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC).

#

#   COMMON ENUMERATED DATA TYPES

#

PatchOperation:

anyOf:

- type: string

enum:

- add

- copy

- move

- remove

- replace

- test

- type: string

description: Operations as defined in IETF RFC 6902.

UriScheme:

anyOf:

- type: string

enum:

- http

- https

- type: string

description: HTTP and HTTPS URI scheme.

ChangeType:

anyOf:

- type: string

enum:

- ADD

- MOVE

- REMOVE

- REPLACE

- type: string

description: Indicates the type of change to be performed.

HttpMethod:

anyOf:

- type: string

enum:

- GET

- POST

- PUT

- DELETE

- PATCH

- OPTIONS

- HEAD

- CONNECT

- TRACE

- type: string

description: HTTP methodes.

NullValue:

enum:

- null

description: JSON's null value.

#

#  COMMON STRUCTURED DATA TYPES

#

ProblemDetails:

type: object

properties:

type:

$ref: '#/components/schemas/Uri'

description: A URI reference according to IETF RFC 3986 that identifies the problem type.

title:

type: string

description: A short, human-readable summary of the problem type.

status:

type: integer

description: The HTTP status code for this occurrence of the problem.

detail:

type: string

description: A human-readable explanation specific to this occurrence of the problem.

instance:

$ref: '#/components/schemas/Uri'

description: A URI reference that identifies the specific occurrence of the problem.

cause:

type: string

description: A machine-readable application error cause specific to this occurrence of the problem. This IE should be present and provide application-related error information, if available.

invalidParams:

type: array

items:

$ref: '#/components/schemas/InvalidParam'

minItems: 1

description: Description of invalid parameters, for a request rejected due to invalid parameters.

supportedFeatures:

$ref: '#/components/schemas/SupportedFeatures'

description: Features supported by the NF Service Producer.

targetScp:

$ref: '#/components/schemas/Uri'

accessTokenError:

$ref: 'TS29510_Nnrf_AccessToken.yaml#/components/schemas/AccessTokenErr'

description: It contain the Access Token Error payload received from the NRF.

accessTokenRequest:

$ref: 'TS29510_Nnrf_AccessToken.yaml#/components/schemas/AccessTokenReq'

description: It contain the Access Token Request that was sent by the SCP.

nrfId:

type: string

description: It contain the Identity (i.e. FQDN) of the NRF that rejected the access token request.

Link:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

description: It contains the URI of the linked resource.

LinkRm:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

nullable: true

description: It contains the URI of the linked resource with the OpenAPI "nullable= true" property.

PatchItem:

type: object

properties:

op:

$ref: '#/components/schemas/PatchOperation'

description: indicates the patch operation as defined in IETF RFC 6902 to be performed on the resource.

path:

type: string

description: contains a JSON pointer value (as defined in IETF RFC 6901) that references a location of a resource on which the patch operation shall be performed.

from:

type: string

description: indicates the path of the source JSON element (according to JSON Pointer syntax) being moved or copied to the location indicated by the "path" attribute.

value: {}

required:

- op

- path

LinksValueSchema:

oneOf:

- type: array

items:

$ref: '#/components/schemas/Link'

minItems: 1

description: Array of links

- $ref: '#/components/schemas/Link'

description: contains a link

description: A list of mutually exclusive alternatives of 1 or more links

SelfLink:

type: object

properties:

self:

$ref: '#/components/schemas/Link'

required:

- self

description: It contains the URI of the linked resource

InvalidParam:

type: object

properties:

param:

type: string

description: If the invalid parameter is an attribute in a JSON body, this IE shall contain the attribute's name and shall be encoded as a JSON Pointer. If the invalid parameter is an HTTP header, this IE shall be formatted as the concatenation of the string "header " plus the name of such header. If the invalid parameter is a query parameter, this IE shall be formatted as the concatenation of the string "query " plus the name of such query parameter. If the invalid parameter is a variable part in the path of a resource URI, this IE shall contain the name of the variable, including the symbols "{" and "}" used in OpenAPI specification as the notation to represent variable path segments.

reason:

type: string

description: A human-readable reason, e.g. "must be a positive integer". In cases involving failed operations in a PATCH request, the reason string should identify the operation that failed using the operation's array index to assist in correlation of the invalid parameter with the failed operation, e.g." Replacement value invalid for attribute (failed operation index= 4)"

required:

- param

ChangeItem:

type: object

properties:

op:

$ref: '#/components/schemas/ChangeType'

description: Indicates the operation to be performed on the resource

path:

type: string

description: contains a JSON pointer value (as defined in IETF RFC 6901) that references a target location within the resource on which the change has been applied.

from:

type: string

description: indicates the path of the source JSON element (according to JSON Pointer syntax) being moved or copied to the location indicated by the "path" attribute. It shall be present if the "op" attribute is of value "MOVE".

origValue: {}

newValue: {}

required:

- op

- path

NotifyItem:

type: object

required:

- resourceId

- changes

properties:

resourceId:

$ref: '#/components/schemas/Uri'

description: Contains the URI of the resource which has been changed.

changes:

type: array

items:

$ref: '#/components/schemas/ChangeItem'

minItems: 1

description: Contains the changes which have been applied on the resource identified by the resourceId attribute.

description: Indicates changes on a resource.

ComplexQuery:

oneOf:

- $ref: '#/components/schemas/Cnf'

- $ref: '#/components/schemas/Dnf'

description: The ComplexQuery data type is either a conjunctive normal form or a disjunctive normal form. The attribute names "cnfUnits" and "dnfUnits" (see clause 5.2.4.11 and clause 5.2.4.12) serve as discriminator.

Cnf:

type: object

required:

- cnfUnits

properties:

cnfUnits:

type: array

items:

$ref: '#/components/schemas/CnfUnit'

minItems: 1

description: A conjunctive normal form

Dnf:

type: object

required:

- dnfUnits

properties:

dnfUnits:

type: array

items:

$ref: '#/components/schemas/DnfUnit'

minItems: 1

description: A disjunctive normal form

CnfUnit:

type: object

required:

- cnfUnit

properties:

cnfUnit:

type: array

items:

$ref: '#/components/schemas/Atom'

minItems: 1

description: During the processing of cnfUnits attribute, all the members in the array shall be interpreted as logically concatenated with logical "AND".

DnfUnit:

type: object

required:

- dnfUnit

properties:

dnfUnit:

type: array

items:

$ref: '#/components/schemas/Atom'

minItems: 1

description: During the processing of dnfUnits attribute, all the members in the array shall be interpreted as logically concatenated with logical "OR".

Atom:

type: object

required:

- attr

- value

properties:

attr:

type: string

description: contains the name of a defined query parameter.

value: {}

negative:

type: boolean

description: indicates whether the negative condition applies for the query condition

PatchResult:

type: object

required:

- report

properties:

report:

type: array

items:

$ref: '#/components/schemas/ReportItem'

minItems: 1

description: The execution report contains an array of report items. Each report item indicates one failed modification.

description: The execution report result on failed modification.

ReportItem:

type: object

required:

- path

properties:

path:

type: string

description: Contains a JSON pointer value (as defined in IETF RFC 6901) that references a location of a resource to which the modification is subject.

reason:

type: string

description: A human-readable reason providing details on the reported modification failure. The reason string should identify the operation that failed using the operation's array index to assist in correlation of the invalid parameter with the failed operation, e.g. "Replacement value invalid for attribute (failed operation index= 4)".

description: indicates performed modivications.

HalTemplate:

type: object

required:

- method

properties:

title:

type: string

description: A human-readable string that can be used to identify this template

method:

$ref: '#/components/schemas/HttpMethod'

description: The HTTP method that should be applied for the corresponding link. If the value is not understood, the value shall be treated as an HTTP GET

contentType:

type: string

description: The media type that should be used for the corresponding request. If the attribute is missing, or contains an unrecognized value, the client should act as if the contentType is set to "application/json".

properties:

type: array

items:

$ref: '#/components/schemas/Property'

minItems: 1

description: The properties that should be included in the body of the corresponding request. If the contentType attribute is set to "application/json", then this attribute describes the attributes of the JSON object of the body.

Property:

type: object

required:

- name

properties:

name:

type: string

description: The name of the property

required:

type: boolean

description: Indicates whether the property is required – true= required - false(default)= not required

regex:

type: string

description: A regular expression string to be applied to the value of the property.

value:

type: string

description: The property value. When present, it shall be a valid JSON string.

#

# Data Types related to Subscription, Identification and Numbering as defined in clause 5.3

#

#

# SIMPLE DATA TYPES

#

Dnn:

type: string

description: String representing a Data Network as defined in clause 9A of 3GPP TS 23.003; it shall contain either a DNN Network Identifier, or a full DNN with both the Network Identifier and Operator Identifier, as specified in 3GPP TS 23.003 clause 9.1.1 and 9.1.2. It shall be coded as string in which the labels are separated by dots (e.g. "Label1.Label2.Label3").

DnnRm:

type: string

nullable: true

description: String representing a Data Network as defined in clause 9A of 3GPP TS 23.003; it shall contain either a DNN Network Identifier, or a full DNN with both the Network Identifier and Operator Identifier, as specified in 3GPP TS 23.003 clause 9.1.1 and 9.1.2. It shall be coded as string in which the labels are separated by dots (e.g. "Label1.Label2.Label3") with the OpenAPI "nullable= true" property.

WildcardDnn:

type: string

pattern: '^[*]$'

description: String representing the Wildcard DNN. It shall contain the string "*".

WildcardDnnRm:

type: string

pattern: '^[*]$'

nullable: true

description: String representing the Wildcard DNN. It shall contain the string "*" but with the OpenAPI "nullable= true" property.

Gpsi:

type: string

pattern: '^(msisdn-[0-9]{5,15}|extid-[^@]+@[^@]+|.+)$'

description: String identifying a Gpsi shall contain either an External Id or an MSISDN. It shall be formatted as follows -External Identifier= "extid-<extid>, where <extid> shall be formatted according to clause 19.7.2 of 3GPP TS 23.003 that describes an External Identifier.

GpsiRm:

type: string

pattern: '^(msisdn-[0-9]{5,15}|extid-[^@]+@[^@]+|.+)$'

nullable: true

description: String identifying a Gpsi shall contain either an External Id or an MSISDN. It shall be formatted as follows -External Identifier= "extid-<extid>, where <extid> shall be formatted according to clause 19.7.2 of 3GPP TS 23.003 that describes an External Identifier with the OpenAPI "nullable= true" property.

GroupId:

type: string

pattern: '^[A-Fa-f0-9]{8}-[0-9]{3}-[0-9]{2,3}-([A-Fa-f0-9][A-Fa-f0-9]){1,10}$'

description: String identifying a group of devices network internal globally unique ID which identifies a set of IMSIs, as specified in clause 19.9 of 3GPP TS 23.003.

GroupIdRm:

type: string

pattern: '^[A-Fa-f0-9]{8}-[0-9]{3}-[0-9]{2,3}-([A-Fa-f0-9][A-Fa-f0-9]){1,10}$'

nullable: true

description: String identifying a group of devices network internal globally unique ID which identifies a set of IMSIs, as specified in clause 19.9 of 3GPP TS 23.003 with the OpenAPI "nullable= true" property.

ExternalGroupId:

type: string

pattern: '^extgroupid-[^@]+@[^@]+$'

description: String identifying External Group Identifier that identifies a group made up of one or more subscriptions associated to a group of IMSIs, as specified in clause 19.7.3 of 3GPP TS 23.003.

ExternalGroupIdRm:

type: string

pattern: '^extgroupid-[^@]+@[^@]+$'

nullable: true

description: String identifying External Group Identifier that identifies a group made up of one or more subscriptions associated to a group of IMSIs, as specified in clause 19.7.3 of 3GPP TS 23.003 with the OpenAPI "nullable= true" property.

Pei:

type: string

pattern: '^(imei-[0-9]{15}|imeisv-[0-9]{16}|mac((-[0-9a-fA-F]{2}){6})(-untrusted)?|eui((-[0-9a-fA-F]{2}){8})|.+)$'

description: String representing a Permanent Equipment Identifier that may contain -	an IMEI or IMEISV, as specified in clause 6.2 of 3GPP TS 23.003; a MAC address for a 5G-RG or FN-RG via wireline access, with an indication that this address cannot be trusted for regulatory purpose if this address cannot be used as an Equipment Identifier of the FN-RG, as specified in clause 4.7.7 of 3GPP TS23.316. Examples are imei-012345678901234 or imeisv-0123456789012345.

PeiRm:

type: string

pattern: '^(imei-[0-9]{15}|imeisv-[0-9]{16}|mac((-[0-9a-fA-F]{2}){6})(-untrusted)?|eui((-[0-9a-fA-F]{2}){8})|.+)$'

nullable: true

description: This data type is defined in the same way as the "Pei" data type but with the OpenAPI "nullable= true" property.

Supi:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|gci-.+|gli-.+|.+)$'

description: String identifying a Supi that shall contain either an IMSI, a network specific identifier, a Global Cable Identifier (GCI) or a Global Line Identifier (GLI) as specified in clause 2.2A of 3GPP TS 23.003. It shall be formatted as follows-	for an IMSI "imsi-<imsi>", where <imsi> shall be formatted according to clause 2.2 of 3GPP TS 23.003 that describes an IMSI.-	for a network specific identifier "nai-<nai>, where <nai> shall be formatted according to clause 28.7.2 of 3GPP TS 23.003 that describes an NAI.-	for a GCI "gci-<gci>", where <gci> shall be formatted according to clause 28.15.2 of 3GPP TS 23.003.-	for a GLI "gli-<gli>", where <gli> shall be formatted according to clause 28.16.2 of 3GPP TS 23.003.To enable that the value is used as part of an URI, the string shall only contain characters allowed according to the "lower-with-hyphen" naming convention defined in 3GPP TS 29.501.

SupiRm:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|gci-.+|gli-.+|.+)$'

nullable: true

description: This data type is defined in the same way as the "Supi" data type, but with the OpenAPI "nullable= true" property.

NfInstanceId:

type: string

format: uuid

description: String uniquely identifying a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122.

AmfId:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: String identifying the AMF ID composed of AMF Region ID (8 bits), AMF Set ID (10 bits) and AMF Pointer (6 bits) as specified in clause 2.10.1 of 3GPP TS 23.003. It is encoded as a string of 6 hexadecimal characters (i.e., 24 bits)

AmfRegionId:

type: string

pattern: '^[A-Fa-f0-9]{2}$'

description: String identifying the AMF Set ID (10 bits) as specified in clause2.10.1 of 3GPP TS 23.003.It is encoded as a string of 3 hexadecimal characters where the first character is limited to values 0 to 3 (i.e. 10 bits)..

AmfSetId:

type: string

pattern: '^[0-3][A-Fa-f0-9]{2}$'

description: String identifying the AMF Set ID (10 bits) as specified in clause 2.10.1 of 3GPP TS 23.003. It is encoded as a string of 3 hexadecimal characters where the first character is limited to values 0 to 3 (i.e. 10 bits)

RfspIndex:

type: integer

minimum: 1

maximum: 256

description: Unsigned integer representing the "Subscriber Profile ID for RAT/Frequency Priority" as specified in 3GPP TS36.413.

RfspIndexRm:

type: integer

minimum: 1

maximum: 256

nullable: true

description: Unsigned integer representing the "Subscriber Profile ID for RAT/Frequency Priority" as specified in 3GPP TS 36.413 with the OpenAPI "nullable= true" property.

NfGroupId:

type: string

description: Identifier of a group of NFs.

MtcProviderInformation:

type: string

description: String uniquely identifying MTC provider information.

CagId:

type: string

pattern: '^[A-Fa-f0-9]{8}$'

description: String containing a Closed Access Group Identifier.

SupiOrSuci:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|gli-.+|gci-.+|suci-(0-[0-9]{3}-[0-9]{2,3}|[1-7]-.+)-[0-9]{1,4}-(0-0-.+|[a-fA-F1-9]-([1-9]|[1-9][0-9]|1[0-9]{2}|2[0-4][0-9]|25[0-5])-[a-fA-F0-9]+)|.+)$'

description: String identifying a SUPI or a SUCI.

#

# STRUCTURED DATA TYPES

#

Guami:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnIdNid'

description: PLMN Identity and Network Identity

amfId:

$ref: '#/components/schemas/AmfId'

description: AMF Identity

required:

- plmnId

- amfId

description: Globally Unique AMF Identifier constructed out of PLMN, Network and AMF identity.

GuamiRm:

anyOf:

- $ref: '#/components/schemas/Guami'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "Guami" data type, but with the OpenAPI "nullable= true" property.

NetworkId:

type: object

properties:

mnc:

$ref: '#/components/schemas/Mnc'

mcc:

$ref: '#/components/schemas/Mcc'

description: contains PLMN and Network identity.

#

# Data Types related to 5G Network as defined in clause 5.4

#

#

# SIMPLE DATA TYPES

#

ApplicationId:

type: string

description: String providing an application identifier.

ApplicationIdRm:

type: string

nullable: true

description: String providing an application identifier with the OpenAPI "nullable= true" property.

PduSessionId:

type: integer

minimum: 0

maximum: 255

description: Unsigned integer identifying a PDU session, within the range 0 to 255, as specified in clause 11.2.3.1b, bits 1 to 8, of 3GPP TS 24.007. If the PDU Session ID is allocated by the Core Network for UEs not supporting N1 mode, reserved range 64 to 95 is used. PDU Session ID within the reserved range is only visible in the Core Network.

Mcc:

type: string

pattern: '^\d{3}$'

description: Mobile Country Code part of the PLMN, comprising 3 digits, as defined in clause 9.3.3.5 of 3GPP TS 38.413 .

MccRm:

type: string

pattern: '^\d{3}$'

nullable: true

description: Mobile Country Code part of the PLMN, comprising 3 digits, as defined in clause 9.3.3.5 of 3GPP TS 38.413 with the OpenAPI "nullable= true" property.

Mnc:

type: string

pattern: '^\d{2,3}$'

description: Mobile Network Code part of the PLMN, comprising 2 or 3 digits, as defined in clause 9.3.3.5 of 3GPP TS 38.413.

MncRm:

type: string

pattern: '^\d{2,3}$'

nullable: true

description: Mobile Network Code part of the PLMN, comprising 2 or 3 digits, as defined in clause 9.3.3.5 of 3GPP TS 38.413 with the OpenAPI "nullable= true" property.

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

description: 2 or 3-octet string identifying a tracking area code as specified in clause 9.3.3.10 of 3GPP TS 38.413, in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TAC shall appear first in the string, and the character representing the 4 least significant bit of the TAC shall appear last in the string.

TacRm:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

nullable: true

description: This data type is defined in the same way as the "Tac" data type, but with the OpenAPI "nullable= true" property..

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

description: 28-bit string identifying an E-UTRA Cell Id as specified in clause 9.3.1.9 of 3GPP TS 38.413, in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the Cell Id shall appear first in the string, and the character representing the 4 least significant bit of the Cell Id shall appear last in the string.

EutraCellIdRm:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

nullable: true

description: This data type is defined in the same way as the "EutraCellId" data type, but with the OpenAPI "nullable= true" property.

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

description: 36-bit string identifying an NR Cell Id as specified in clause 9.3.1.7 of 3GPP TS 38.413, in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the Cell Id shall appear first in the string, and the character representing the 4 least significant bit of the Cell Id shall appear last in the string.

NrCellIdRm:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

nullable: true

description: This data type is defined in the same way as the "NrCellId" data type, but with the OpenAPI "nullable= true" property.

Dnai:

type: string

description: DNAI (Data network access identifier), see clause 5.6.7 of 3GPP TS 23.501.

DnaiRm:

type: string

nullable: true

description: This data type is defined in the same way as the "Dnai" data type, but with the OpenAPI "nullable= true" property.

5GMmCause:

$ref: '#/components/schemas/Uinteger'

description: This represents the 5GMM cause code values as specified in 3GPP TS 24.501.

AmfName:

type: string

AreaCode:

type: string

description: Values are operator specific.

AreaCodeRm:

type: string

nullable: true

description: This data type is defined in the same way as the "AreaCode" data type, but with the OpenAPI "nullable= true" property.

N3IwfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: This represents the identifier of the N3IWF ID as specified in clause 9.3.1.57 of 3GPP TS 38.413 in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the N3IWF ID shall appear first in the string, and the character representing the 4 least significant bit of the N3IWF ID shall appear last in the string.

WAgfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: This represents the identifier of the W-AGF ID as specified in clause 9.3.1.162 of 3GPP TS 38.413 in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the W-AGF ID shall appear first in the string, and the character representing the 4 least significant bit of the W-AGF ID shall appear last in the string.

TngfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: This represents the identifier of the TNGF ID as specified in clause 9.3.1.161 of 3GPP TS 38.413 in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TNGF ID shall appear first in the string, and the character representing the 4 least significant bit of the TNGF ID shall appear last in the string.

NgeNbId:

type: string

pattern: '^(MacroNGeNB-[A-Fa-f0-9]{5}|LMacroNGeNB-[A-Fa-f0-9]{6}|SMacroNGeNB-[A-Fa-f0-9]{5})$'

description: This represents the identifier of the ng-eNB ID as specified in clause 9.3.1.8 of 3GPP TS 38.413. The value of the ng-eNB ID shall be encoded in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The padding 0 shall be added to make multiple nibbles, so the most significant character representing the padding 0 if required together with the 4 most significant bits of the ng-eNB ID shall appear first in the string, and the character representing the 4 least significant bit of the ng-eNB ID (to form a nibble) shall appear last in the string.

example: SMacroNGeNB-34B89

Nid:

type: string

pattern: '^[A-Fa-f0-9]{11}$'

description: This represents the Network Identifier, which together with a PLMN ID is used to identify an SNPN (see 3GPP TS 23.003 and 3GPP TS 23.501 clause 5.30.2.1).

NidRm:

type: string

pattern: '^[A-Fa-f0-9]{11}$'

nullable: true

description: This data type is defined in the same way as the "Nid" data type, but with the OpenAPI "nullable= true" property.

NfSetId:

type: string

description: NF Set Identifier (see clause 28.12 of 3GPP TS 23.003), formatted as the following string " set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC>", or "set<SetID>. <NFType>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with <MCC> encoded as defined in clause 5.4.2 ("Mcc" data type definition) <MNC> encoded as defined in clause 5.4.2 ("Mnc" data type definition) <NFType> encoded as a value defined in Table 6.1.6.3.3-1 of 3GPP TS 29.510 but with lower case characters <Set ID> encoded as a string of characters consisting of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit.

NfServiceSetId:

type: string

description: NF Service Set Identifier (see clause 28.12 of 3GPP TS 23.003) formatted as the following string  " set<Set ID>.sn<Service Name>.nfi<NF Instance ID>.5gc.mnc<MNC>.mcc<MCC>">", or "set<SetID>.sn<ServiceName>.nfi<NFInstanceID>.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with <MCC> encoded as defined in clause 5.4.2 ("Mcc" data type definition)  <MNC> encoded as defined in clause 5.4.2 ("Mnc" data type definition)  <NID> encoded as defined in clause 5.4.2 ("Nid" data type definition) <NFInstanceId> encoded as defined in clause 5.3.2 <ServiceName> encoded as defined in 3GPP TS 29.510 <Set ID> encoded as a string of characters consisting of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit.

PlmnAssiUeRadioCapId:

$ref: '#/components/schemas/Bytes'

description: String with format "byte" as defined in OpenAPI Specification, i.e. base64-encoded characters, encoding the "UE radio capability ID" IE  as specified in clause 9.11.3.68 of 3GPP TS 24.501 (starting from octet 1).

ManAssiUeRadioCapId:

$ref: '#/components/schemas/Bytes'

description: String with format "byte" as defined in OpenAPI Specification, i.e. base64-encoded characters, encoding the "UE radio capability ID" IE  as specified in clause 9.11.3.68 of 3GPP TS 24.501 (starting from octet 1).

TypeAllocationCode:

type: string

pattern: '^[0-9]{8}$'

description: Type Allocation Code (TAC) of the UE, comprising the initial eight-digit portion of the 15-digit IMEI and 16-digit IMEISV codes. See clause 6.2 of 3GPP TS 23.003.

HfcNId:

type: string

maxLength: 6

description: This IE represents the identifier of the HFC node Id as specified in CableLabs WR-TR-5WWC-ARCH. It is provisioned by the wireline operator as part of wireline operations and may contain up to six characters.

HfcNIdRm:

type: string

maxLength: 6

nullable: true

description: This data type is defined in the same way as the "HfcNId" data type, but with the OpenAPI "nullable= true" property..

ENbId:

type: string

pattern: '^(MacroeNB-[A-Fa-f0-9]{5}|LMacroeNB-[A-Fa-f0-9]{6}|SMacroeNB-[A-Fa-f0-9]{5}|HomeeNB-[A-Fa-f0-9]{7})$'

description: This represents the identifier of the eNB ID as specified in clause 9.2.1.37 of 3GPP TS 36.413. The string shall be formatted with the following pattern '^('MacroeNB-[A-Fa-f0-9]{5}|LMacroeNB-[A-Fa-f0-9]{6}|SMacroeNB-[A-Fa-f0-9]{5}|HomeeNB-[A-Fa-f0-9]{7})$' The value of the eNB ID shall be encoded in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The padding 0 shall be added to make multiple nibbles, so the most significant character representing the padding 0 if required together with the 4 most significant bits of the eNB ID shall appear first in the string, and the character representing the 4 least significant bit of the eNB ID (to form a nibble) shall appear last in the string.

Gli:

$ref: '#/components/schemas/Bytes'

description: Global Line Identifier uniquely identifying the line connecting the 5G-BRG or FN-BRG to the 5GS. See clause 28.16.3 of 3GPP TS 23.003. This shall be encoded as a string with format "byte" as defined in OpenAPI Specification, i.e. base64-encoded characters, representing the GLI value (up to 150 bytes) encoded as specified in BBF WT-470

Gci:

type: string

description: Global Cable Identifier uniquely identifying the connection between the 5G-CRG or FN-CRG to the 5GS. See clause 28.15.4 of 3GPP TS 23.003. This shall be encoded as a string per clause 28.15.4 of 3GPP TS 23.003, and compliant with the syntax specified in clause 2.2 of IETF RFC 7542 for the username part of a NAI. The GCI value is specified in CableLabs WR-TR-5WWC-ARCH.

#

# ENUMERATED DATA TYPES

#

AccessType:

type: string

enum:

- 3GPP_ACCESS

- NON_3GPP_ACCESS

description: Indicates wether the access is  via 3GPP or via non-3GPP.

AccessTypeRm:

anyOf:

- $ref: '#/components/schemas/AccessType'

- $ref: '#/components/schemas/NullValue'

description: Indicates wether the access is via 3GPP or via non-3GPP but with the OpenAPI "nullable= true" property.

RatType:

anyOf:

- type: string

enum:

- NR

- EUTRA

- WLAN

- VIRTUAL

- NBIOT

- WIRELINE

- WIRELINE_CABLE

- WIRELINE_BBF

- LTE-M

- NR_U

- EUTRA_U

- TRUSTED_N3GA

- TRUSTED_WLAN

- UTRA

- GERA

- type: string

description: Indicates the radio access used.

RatTypeRm:

anyOf:

- $ref: '#/components/schemas/RatType'

- $ref: '#/components/schemas/NullValue'

description: Provides infromatiuon about the radio access but with the OpenAPI "nullable= true" property.

PduSessionType:

anyOf:

- type: string

enum:

- IPV4

- IPV6

- IPV4V6

- UNSTRUCTURED

- ETHERNET

- type: string

description: PduSessionType indicates the type of a PDU session. It shall comply with the provisions defined in table 5.4.3.3-1.

PduSessionTypeRm:

anyOf:

- $ref: '#/components/schemas/PduSessionType'

- $ref: '#/components/schemas/NullValue'

description: PduSessionType indicates the type of a PDU session. It shall comply with the provisions defined in table 5.4.3.3-1 but with the OpenAPI "nullable= true" property.

UpIntegrity:

anyOf:

- type: string

enum:

- REQUIRED

- PREFERRED

- NOT_NEEDED

- type: string

description: indicates whether UP integrity protection is required, preferred or not needed for all the traffic on the PDU Session. It shall comply with the provisions defined in table 5.4.3.4-1.

UpIntegrityRm:

anyOf:

- $ref: '#/components/schemas/UpIntegrity'

- $ref: '#/components/schemas/NullValue'

description: indicates whether UP integrity protection is required, preferred or not needed for all the traffic on the PDU Session. It shall comply with the provisions defined in table 5.4.3.4-1.but with the OpenAPI "nullable= true" property.

UpConfidentiality:

anyOf:

- type: string

enum:

- REQUIRED

- PREFERRED

- NOT_NEEDED

- type: string

description: indicates whether UP confidentiality protection is required, preferred or not needed for all the traffic on the PDU Session. It shall comply with the provisions defined in table 5.4.3.5-1.

UpConfidentialityRm:

anyOf:

- $ref: '#/components/schemas/UpConfidentiality'

- $ref: '#/components/schemas/NullValue'

description: indicates whether UP confidentiality protection is required, preferred or not needed for all the traffic on the PDU Session. It shall comply with the provisions defined in table 5.4.3.5-1 but with the OpenAPI "nullable= true" property.

SscMode:

anyOf:

- type: string

enum:

- SSC_MODE_1

- SSC_MODE_2

- SSC_MODE_3

- type: string

description: represents the service and session continuity mode It shall comply with the provisions defined in table 5.4.3.6-1.

SscModeRm:

anyOf:

- $ref: '#/components/schemas/SscMode'

- $ref: '#/components/schemas/NullValue'

description: represents the service and session continuity mode It shall comply with the provisions defined in table 5.4.3.6-1 but with the OpenAPI "nullable= true" property.

DnaiChangeType:

anyOf:

- type: string

enum:

- EARLY

- EARLY_LATE

- LATE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- EARLY: Early notification of UP path reconfiguration.

- EARLY_LATE: Early and late notification of UP path reconfiguration. This value shall only be present in the subscription to the DNAI change event.

- LATE: Late notification of UP path reconfiguration.

DnaiChangeTypeRm:

anyOf:

- $ref: '#/components/schemas/DnaiChangeType'

- $ref: '#/components/schemas/NullValue'

description: It can take the values  as specified for DnaiChangeType but with the OpenAPI "nullable= true" property.

RestrictionType:

anyOf:

- type: string

enum:

- ALLOWED_AREAS

- NOT_ALLOWED_AREAS

- type: string

description: It contains the restriction type ALLOWED_AREAS or NOT_ALLOWED_AREAS.

RestrictionTypeRm:

anyOf:

- $ref: '#/components/schemas/RestrictionType'

- $ref: '#/components/schemas/NullValue'

description: It contains the restriction type ALLOWED_AREAS or NOT_ALLOWED_AREAS but with the OpenAPI "nullable= true" property.

CoreNetworkType:

anyOf:

- type: string

enum:

- 5GC

- EPC

- type: string

description: It contains the Core Network type 5GC or EPC.

CoreNetworkTypeRm:

anyOf:

- $ref: '#/components/schemas/CoreNetworkType'

- $ref: '#/components/schemas/NullValue'

description: It contains the Core Network type 5GC or EPC but with the OpenAPI "nullable= true" property.

PresenceState:

anyOf:

- type: string

enum:

- IN_AREA

- OUT_OF_AREA

- UNKNOWN

- INACTIVE

- type: string

description: >

Possible values are:

-IN_AREA: Indicates that the UE is inside or enters the presence reporting area.

-OUT_OF_AREA: Indicates that the UE is outside or leaves the presence reporting area

-UNKNOW: Indicates it is unknown whether the UE is in the presence reporting area or not

-INACTIVE: Indicates that the presence reporting area is inactive in the serving node..

StationaryIndication:

anyOf:

- type: string

enum:

- STATIONARY

- MOBILE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- STATIONARY: Identifies the UE is stationary

- MOBILE: Identifies the UE is mobile

StationaryIndicationRm:

anyOf:

- $ref: '#/components/schemas/StationaryIndication'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "StationaryIndication" enumeration, but with the OpenAPI "nullable= true" property.

ScheduledCommunicationType:

anyOf:

- type: string

enum:

- DOWNLINK_ONLY

- UPLINK_ONLY

- BIDIRECTIONAL

- type: string

description: >

Possible values are:

-DOWNLINK_ONLY: Downlink only

-UPLINK_ONLY: Uplink only

-BIDIRECTIONA: Bi-directional

ScheduledCommunicationTypeRm:

anyOf:

- $ref: '#/components/schemas/ScheduledCommunicationType'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "ScheduledCommunicationTypen" enumeration, but with the OpenAPI "nullable= true" property.

TrafficProfile:

anyOf:

- type: string

enum:

- SINGLE_TRANS_UL

- SINGLE_TRANS_DL

- DUAL_TRANS_UL_FIRST

- DUAL_TRANS_DL_FIRST

- MULTI_TRANS

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- SINGLE_TRANS_UL: Uplink single packet transmission.

- SINGLE_TRANS_DL: Downlink single packet transmission.

- DUAL_TRANS_UL_FIRST: Dual packet transmission, firstly uplink packet transmission with subsequent downlink packet transmission.

- DUAL_TRANS_DL_FIRST: Dual packet transmission, firstly downlink packet transmission with subsequent uplink packet transmission.

TrafficProfileRm:

anyOf:

- $ref: '#/components/schemas/TrafficProfile'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "TrafficProfile" enumeration, but with the OpenAPI "nullable= true" property..

LcsServiceAuth:

anyOf:

- type: string

enum:

- "LOCATION_ALLOWED_WITH_NOTIFICATION"

- "LOCATION_ALLOWED_WITHOUT_NOTIFICATION"

- "LOCATION_ALLOWED_WITHOUT_RESPONSE"

- "LOCATION_RESTRICTED_WITHOUT_RESPONSE"

- "NOTIFICATION_ONLY"

- "NOTIFICATION_AND_VERIFICATION_ONLY"

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- "LOCATION_ALLOWED_WITH_NOTIFICATION": Location allowed with notification

- "LOCATION_ALLOWED_WITHOUT_NOTIFICATION": Location allowed without notification

- "LOCATION_ALLOWED_WITHOUT_RESPONSE": Location with notification and privacy verification; location allowed if no response

- "LOCATION_RESTRICTED_WITHOUT_RESPONSE": Location with notification and privacy verification; location restricted if no response

- "NOTIFICATION_ONLY": Notification only

- "NOTIFICATION_AND_VERIFICATION_ONLY": Notification and privacy verification only

UeAuth:

anyOf:

- type: string

enum:

- AUTHORIZED

- NOT_AUTHORIZED

- type: string

description: >

Possible values are

- AUTHORIZED: Indicates that the UE is authorized.

- NOT_AUTHORIZED: Indicates that the UE is not authorized.

DlDataDeliveryStatus:

anyOf:

- type: string

enum:

- BUFFERED

- TRANSMITTED

- DISCARDED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- BUFFERED: The first downlink data is buffered with extended buffering matching the source of the downlink traffic.

- TRANSMITTED: The first downlink data matching the source of the downlink traffic is transmitted after previous buffering or discarding of corresponding packet(s) because the UE of the PDU Session becomes ACTIVE, and buffered data can be delivered to UE.

- DISCARDED: The first downlink data matching the source of the downlink traffic is discarded because the Extended Buffering time, as determined by the SMF, expires or the amount of downlink data to be buffered is exceeded.

DlDataDeliveryStatusRm:

anyOf:

- $ref: '#/components/schemas/DlDataDeliveryStatus'

- $ref: '#/components/schemas/NullValue'

AuthStatus:

anyOf:

- type: string

enum:

- EAP_SUCCESS

- EAP_FAILURE

- PENDING

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- "EAP_SUCCESS": The NSSAA status is EAP-Success.

- "EAP_FAILURE": The NSSAA status is EAP-Failure.

- "PENDING": The NSSAA status is Pending.

#

# STRUCTURED DATA TYPES

#

Snssai:

type: object

properties:

sst:

type: integer

minimum: 0

maximum: 255

description: >

Unsigned integer, within the range 0 to 255, representing the Slice/Service Type. It indicates the expected Network Slice behaviour in terms of features and services.

Values 0 to 127 correspond to the standardized SST range. Values 128 to 255 correspond to the Operator-specific range. See clause 28.4.2 of 3GPP TS 23.003.

Standardized values are defined in clause 5.15.2.2 of 3GPP TS 23.501.

sd:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: >

3-octet string, representing the Slice Differentiator, in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the SD shall appear first in the string, and the character representing the 4 least significant bit of the SD shall appear last in the string.

This is an optional parameter that complements the Slice/Service type(s) to allow to differentiate amongst multiple Network Slices of the same Slice/Service type. This IE shall be absent if no SD value is associated with the SST.

description: When Snssai needs to be converted to string (e.g. when used in maps as key), the string shall be composed of one to three digits "sst" optionally followed by "-" and 6 hexadecimal digits "sd".

required:

- sst

PlmnId:

type: object

properties:

mcc:

$ref: '#/components/schemas/Mcc'

description: Mobile Country Code

mnc:

$ref: '#/components/schemas/Mnc'

description: Mobile Network Code

description: When PlmnId needs to be converted to string (e.g. when used in maps as key), the string shall be composed of three digits "mcc" followed by "-" and two or three digits "mnc".

required:

- mcc

- mnc

PlmnIdRm:

anyOf:

- $ref: '#/components/schemas/PlmnId'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "PlmnId" data type, but with the OpenAPI "nullable= true" property.

Tai:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

description: PLMN Identity

tac:

$ref: '#/components/schemas/Tac'

description: Tracking Area Code

nid:

$ref: '#/components/schemas/Nid'

description: Network Identifier, shall be present in case of SNPN, PlmnId together with Nid indicates the identity of the SNPN to which the TA belongs to.

required:

- plmnId

- tac

TaiRm:

anyOf:

- $ref: '#/components/schemas/Tai'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "Tai" data type, but with the OpenAPI "nullable= true" property.

Ecgi:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

description: PLMN Identity

eutraCellId:

$ref: '#/components/schemas/EutraCellId'

description: E-UTRA Cell Identity

nid:

$ref: '#/components/schemas/Nid'

description: Network Identifier

required:

- plmnId

- eutraCellId

EcgiRm:

anyOf:

- $ref: '#/components/schemas/Ecgi'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "Ecgi" data type, but with the OpenAPI "nullable= true" property.

Ncgi:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

description: PLMN Identity

nrCellId:

$ref: '#/components/schemas/NrCellId'

description: NR Cell Identity

nid:

$ref: '#/components/schemas/Nid'

description: Network Identifier, shall be present in case of SNPN, PlmnId together with Nid indicates the identity of the SNPN to which the NR cell belongs to.

required:

- plmnId

- nrCellId

NcgiRm:

anyOf:

- $ref: '#/components/schemas/Ncgi'

- $ref: '#/components/schemas/NullValue'

description: his data type is defined in the same way as the "Ncgi" data type, but with the OpenAPI "nullable= true" property.

UserLocation:

type: object

properties:

eutraLocation:

$ref: '#/components/schemas/EutraLocation'

description: E-UTRA user location

nrLocation:

$ref: '#/components/schemas/NrLocation'

description: NR user location

n3gaLocation:

$ref: '#/components/schemas/N3gaLocation'

description: Non-3GPP access user location

description: At least one of eutraLocation, nrLocation and n3gaLocation shall be present. Several of them may be present

EutraLocation:

type: object

properties:

tai:

$ref: '#/components/schemas/Tai'

description: Tracking Area Identity

ecgi:

$ref: '#/components/schemas/Ecgi'

description: E-UTRA Cell Identity

ignoreEcgi:

type: boolean

default: false

description: >

This flag when present shall indicate that the Ecgi shall be ignored

When present, it shall be set as follows

- true: ecgi shall be ignored.

- false (default): ecgi shall not be ignored.

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

description: >

The value represents the elapsed time in minutes since the last network contact of the mobile station.

Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful NG-RAN location reporting procedure with the eNB when the UE is in connected mode.

Any other value than "0" indicates that the location information is the last known one.

See 3GPP TS 29.002 clause 17.7.8.

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

description: The value represents the UTC time when the UeLocation information was acquired.

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

description: Refer to geographical Information. See 3GPP TS 23.032 clause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used.

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

description: Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used.

globalNgenbId:

$ref: '#/components/schemas/GlobalRanNodeId'

description: It indicates the global identity of the ng-eNodeB in which the UE is currently located. See  3GPP TS 38.413 clause 9.3.1.8.

globalENbId:

$ref: '#/components/schemas/GlobalRanNodeId'

description: It indicates the global identity of the eNodeB in which the UE is currently located. See 3GPP TS 36.413 clause 9.2.1.37. Either the "globalNgenbId" attribute or the "globalENbId" attribute shall be included in the "EutraLocation" data type.

required:

- tai

- ecgi

EutraLocationRm:

anyOf:

- $ref: '#/components/schemas/EutraLocation'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "EutraLocation" data type, but with the OpenAPI "nullable= true" property.

NrLocation:

type: object

properties:

tai:

$ref: '#/components/schemas/Tai'

description: Tracking Area Identity.

ncgi:

$ref: '#/components/schemas/Ncgi'

description: NR Cell Identity.

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

description: >

The value represents the elapsed time in minutes since the last network contact of the mobile station.

Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful NG-RAN location reporting procedure with the eNB when the UE is in connected mode.

Any other value than "0" indicates that the location information is the last known one.

See 3GPP TS 29.002 clause 17.7.8.

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

description: The value represents the UTC time when the UeLocation information was acquired

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

description: Refer to geographical Information. See 3GPP TS 23.032 clause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used.

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

description: Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used.

globalGnbId:

$ref: '#/components/schemas/GlobalRanNodeId'

description: It indicates the global identity of the gNodeB in which the UE is currently located. See 3GPP TS 38.413 clause 9.3.1.6.

required:

- tai

- ncgi

NrLocationRm:

anyOf:

- $ref: '#/components/schemas/NrLocation'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "NrLocation" data type, but with the OpenAPI "nullable= true" property.

N3gaLocation:

type: object

properties:

n3gppTai:

$ref: '#/components/schemas/Tai'

description: This IE shall be present over the 3GPP PLMN internal interfaces, but it shall not be present over the N5 interface. When present, it shall contain the TAI reported by the N3IWF, TNGF or W-AGF for the non-3GPP access.

n3IwfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: This IE shall contain the N3IWF identifier received over NGAP and shall be encoded as a string of hexadecimal characters. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the N3IWF ID shall appear first in the string, and the character representing the 4 least significant bit of the N3IWF ID shall appear last in the string.

ueIpv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

description: UE/N5CW device local IPv4 address (used to reach the N3IWF, TNGF or TWIF). The ueIPv4Addr or the ueIPv6Addr shall be present if the UE is accessing the 5GC via a trusted or untrusted non-3GPP access and the information is available.

ueIpv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

description: UE/N5CW device local IPv6 address (used to reach the N3IWF, TNGF or TWIF). The ueIPv4Addr or the ueIPv6Addr shall be present if the UE is accessing the 5GC via a trusted or untrusted non-3GPP access and the information is available.

portNumber:

$ref: '#/components/schemas/Uinteger'

description: UDP or TCP source port number. It shall be present if the UE is accessing the 5GC via a trusted or untrusted non-3GPP access and NAT is detected.

tnapId:

$ref: '#/components/schemas/TnapId'

description: This IE shall contain the TNAP Identifier, see clause 5.6.2 of 3GPP TS 23.501.

twapId:

$ref: '#/components/schemas/TwapId'

description: This IE shall contain the TWAP Identifier, see clause 4.2.8.5.3 of 3GPP TS 23.501.

hfcNodeId:

$ref: '#/components/schemas/HfcNodeId'

description: This IE shall contain the HFC Node Identifier received over NGAP. It shall be present for a 5G-CRG/FN-CRG accessing the 5GC via wireline access network.

gli:

$ref: '#/components/schemas/Gli'

description: This IE shall contain the Global Line Identifier. It shall be present for a 5G-BRG/FN-BRG accessing the 5GC via wireline access network.

w5gbanLineType:

$ref: '#/components/schemas/LineType'

description: This IE may be present for a 5G-BRG/FN-BRG accessing the 5GC via wireline access network. When present, it shall indicate the type of the wireline (DSL or PON).

gci:

$ref: '#/components/schemas/Gci'

description: This IE shall contain the Global Cable Identifier. It shall be present for the N5GC device accessing the 5GC via wireline access network. See clause 4.10a of 3GPP TS 23.316.

UpSecurity:

type: object

properties:

upIntegr:

$ref: '#/components/schemas/UpIntegrity'

description: This IE shall indicate whether UP integrity protection is required, preferred or not needed for all the traffic on the PDU Session.

upConfid:

$ref: '#/components/schemas/UpConfidentiality'

description: This IE shall indicate whether UP confidentiality protection is required, preferred or not needed for all the traffic on the PDU Session

required:

- upIntegr

- upConfid

UpSecurityRm:

anyOf:

- $ref: '#/components/schemas/UpSecurity'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "UpSecurity" data type, but with the OpenAPI "nullable= true" property.

NgApCause:

type: object

properties:

group:

$ref: '#/components/schemas/Uinteger'

description: >

This IE shall indicate the group of the NGAP cause. The value of this IE shall equal to the ASN.1 value of the specified NGAP cause group.

NGAP supports following cause groups defined as separate enumerations, as specified in clause 9.4.5 of 3GPP TS 38.413, with following values

0 – radioNetwork

1 – transport

2 – nas

3 – protocol

4 – misc

value:

$ref: '#/components/schemas/Uinteger'

description: This IE shall carry the NG AP cause value in specific cause group identified by the "group" attribute, as specified in clause 9.4.5 of 3GPP TS 38.413.

required:

- group

- value

BackupAmfInfo:

type: object

properties:

backupAmf:

$ref: '#/components/schemas/AmfName'

description: This IE shall contain the AMF name of the backup AMF related to the specific GUAMI(s) (see clause 5.21.2.3 of 3GPP TS 23.501). If no GUAMI is included in BackupAmfinfo, the AMF name of the backup AMF is related to all the GUAMI(s) supported by the AMF.

guamiList:

type: array

items:

$ref: '#/components/schemas/Guami'

minItems: 1

description: If present, this IE shall contain the list of GUAMI(s) (supported by the AMF) for which the backupAmf IE applies.

required:

- backupAmf

RefToBinaryData:

type: object

properties:

contentId:

type: string

description: This IE shall contain the value of the Content-ID header of the referenced binary body part.

required:

- contentId

RefToBinaryDataRm:

anyOf:

- $ref: '#/components/schemas/RefToBinaryData'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the " RefToBinaryData " data type, but with the OpenAPI "nullable= true" property.

RouteToLocation:

type: object

properties:

dnai:

$ref: '#/components/schemas/Dnai'

description: Identifies the location of the application

routeInfo:

$ref: '#/components/schemas/RouteInformation'

description: Includes the traffic routing information.

routeProfId:

type: string

nullable: true

description: Identifies the routing profile Id.

required:

- dnai

anyOf:

- required: [ routeInfo ]

- required: [ routeProfId ]

nullable: true

description: Either the "routeInfo" attribute or the "routeProfId" attribute shall be included in the "RouteToLocation" data type.

RouteInformation:

type: object

properties:

ipv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

description: Ipv4address of the tunnel end point in the data network.

ipv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

description: Ipv6 address of the tunnel end point in the data network.

portNumber:

$ref: '#/components/schemas/Uinteger'

description: UDP port number of the tunnel end point in the data network.

required:

- portNumber

nullable: true

description: At least one of the "ipv4Addr" attribute and the "ipv6Addr" attribute shall be included in the "RouteInformation" data type.

SubscribedDefaultQos:

type: object

required:

- 5qi

- arp

properties:

5qi:

$ref: '#/components/schemas/5Qi'

description: Default 5G QoS identifier see 3GPP TS 23.501 clause 5.7.2.7

arp:

$ref: '#/components/schemas/Arp'

description: Default Allocation and Retention Priority see 3GPP TS23.501 clause 5.7.2.7

priorityLevel:

$ref: '#/components/schemas/5QiPriorityLevel'

description: Defines the 5QI Priority Level When present, it contains the 5QI Priority Level value that overrides the standardized or pre-configured value as described in 3GPP TS 23.501

description: Provides the subsribed 5QI and the ARP, it may contain the priority level.

Area:

type: object

oneOf:

- required:

- tacs

- required:

- areaCode

properties:

tacs:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

description: List of TACs; shall be present if and only if areaCode is absent.

areaCode:

$ref: '#/components/schemas/AreaCode'

description: Area Code; shall be present if and only if tacs is absent

ServiceAreaRestriction:

type: object

properties:

restrictionType:

$ref: '#/components/schemas/RestrictionType'

description: >

string "ALLOWED_AREAS" or "NOT_ALLOWED_AREAS"

shall be present if and only if the areas attribute is present

areas:

type: array

items:

$ref: '#/components/schemas/Area'

description: >

A list of Areas.

These areas are:

- allowed areas if RestrictionType is "ALLOWED_AREAS"

- not allowed areas if RestrictionType is "NOT_ALLOWED_AREAS"

maxNumOfTAs:

$ref: '#/components/schemas/Uinteger'

description: Maximum number of allowed tracking areas for use when restrictionType indicates "ALLOWED_AREAS". This attribute shall be absent when attribute "restrictionType" takes the value "NOT_ALLOWED_AREAS".

maxNumOfTAsForNotAllowedAreas:

$ref: '#/components/schemas/Uinteger'

description: Maximum number of allowed tracking areas for use when restrictionType indicates "NOT_ALLOWED_AREAS". This attribute shall be absent when attribute "restrictionType" takes the value "ALLOWED_AREAS".

allOf:

#

# 1st condition: restrictionType and areas attributes shall be either both absent

#                or both present

#

- oneOf:

- not:

required: [ restrictionType ]

- required: [ areas ]

#

# 2nd condition: if restrictionType takes value NOT_ALLOWED_AREAS,

#                then maxNumOfTAs shall be absent

#

- anyOf:

- not:

required: [ restrictionType ]

properties:

restrictionType:

type: string

enum: [ NOT_ALLOWED_AREAS ]

- not:

required: [ maxNumOfTAs ]

#

# 3rd condition: if restrictionType takes value ALLOWED_AREAS,

#                then maxNumOfTAsForNotAllowedAreas shall be absent

#

- anyOf:

- not:

required: [ restrictionType ]

properties:

restrictionType:

type: string

enum: [ ALLOWED_AREAS ]

- not:

required: [ maxNumOfTAsForNotAllowedAreas ]

WirelineArea:

type: object

properties:

globalLineIds:

type: array

items:

$ref: '#/components/schemas/Gli'

minItems: 1

description: List of Global Line Identifiers, for a 5G-BRG accessing the 5GC via wireline access network

hfcNIds:

type: array

items:

$ref: '#/components/schemas/HfcNId'

minItems: 1

description: List of HFC Node Ids, for a 5G-CRG/FN-CRG is accessing the 5GC via wireline access network.

areaCodeB:

$ref: '#/components/schemas/AreaCode'

description: Area Code for for 5G-BRG accessing via wireline access network.

areaCodeC:

$ref: '#/components/schemas/AreaCode'

description: Area Code for 5G-CRG/FN-CRG is accessing via wireline access network.

description: One and only one of the "globLineIds", "hfcNIds", "areaCodeB" and "areaCodeC" attributes shall be included in a WirelineArea data structure

WirelineServiceAreaRestriction:

type: object

properties:

restrictionType:

$ref: '#/components/schemas/RestrictionType'

description: string "ALLOWED_AREAS" or "NOT_ALLOWED_AREAS"

areas:

type: array

items:

$ref: '#/components/schemas/WirelineArea'

description: >

A list of Areas.

These areas are

- allowed areas if RestrictionType is "ALLOWED_AREAS"

- not allowed areas if RestrictionType is "NOT_ALLOWED_AREAS"

description: >

The "restrictionType" attribute and the "areas" attribute shall be either both present or absent.

The empty array of areas is used when service is allowed/restricted nowhere.

PresenceInfo:

type: object

properties:

praId:

type: string

description: >

Represents an identifier of the Presence Reporting Area (see clause 28.10 of 3GPP TS 23.003. This IE shall be present if the Area of Interest subscribed or reported is a Presence Reporting Area or a Set of Core Network predefined Presence Reporting Areas.

When present, it shall be encoded as a string representing an integer in the following ranges:

0 to 8 388 607 for UE-dedicated PRA

8 388 608 to 16 777 215 for Core Network predefined PRA

Examples:

PRA ID 123 is encoded as "123"

PRA ID 11 238 660 is encoded as "11238660"

additionalPraId:

type: string

description: >

This IE may be present if the praId IE is present and if it contains a PRA identifier referring to a set of Core Network predefined Presence Reporting Areas.

When present, this IE shall contain a PRA Identifier of an individual PRA within the Set of Core Network predefined Presence Reporting Areas indicated by the praId IE.

presenceState:

$ref: '#/components/schemas/PresenceState'

description: Indicates whether the UE is inside or outside of the area of interest (e.g presence reporting area or the LADN area), or if the presence reporting area is inactive in the serving node.

trackingAreaList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

description: Represents the list of tracking areas that constitutes the area. This IE shall be present if the subscription or the event report is for tracking UE presence in the tracking areas. For non 3GPP access the TAI shall be the N3GPP TAI.

ecgiList:

type: array

items:

$ref: '#/components/schemas/Ecgi'

minItems: 1

description: Represents the list of EUTRAN cell Ids that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of EUTRAN cell Ids.

ncgiList:

type: array

items:

$ref: '#/components/schemas/Ncgi'

minItems: 1

description: Represents the list of NR cell Ids that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of NR cell Ids.

globalRanNodeIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

minItems: 1

description: Represents the list of NG RAN node identifiers that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of NG RAN node identifiers.

globaleNbIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

minItems: 1

description: Represents the list of eNodeB identifiers that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of eNodeB identifiers.

description: If the additionalPraId IE is present, this IE shall state the presence information of the UE for the individual PRA identified by the additionalPraId IE; If the additionalPraId IE is not present, this IE shall state the presence information of the UE for the PRA identified by the praId IE.

PresenceInfoRm:

type: object

properties:

praId:

type: string

description: >

Represents an identifier of the Presence Reporting Area (see clause 28.10 of 3GPP TS 23.003. This IE shall be present if the Area of Interest subscribed or reported is a Presence Reporting Area or a Set of Core Network predefined Presence Reporting Areas.

When present, it shall be encoded as a string representing an integer in the following ranges:

0 to 8 388 607 for UE-dedicated PRA

8 388 608 to 16 777 215 for Core Network predefined PRA

Examples:

PRA ID 123 is encoded as "123"

PRA ID 11 238 660 is encoded as "11238660"

additionalPraId:

type: string

description: >

This IE may be present if the praId IE is present and if it contains a PRA identifier referring to a set of Core Network predefined Presence Reporting Areas.

When present, this IE shall contain a PRA Identifier of an individual PRA within the Set of Core Network predefined Presence Reporting Areas indicated by the praId IE.

presenceState:

$ref: '#/components/schemas/PresenceState'

description: Indicates whether the UE is inside or outside of the area of interest (e.g presence reporting area or the LADN area), or if the presence reporting area is inactive in the serving node.

trackingAreaList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 0

description: Represents the list of tracking areas that constitutes the area. This IE shall be present if the subscription or the event report is for tracking UE presence in the tracking areas. For non 3GPP access the TAI shall be the N3GPP TAI.

ecgiList:

type: array

items:

$ref: '#/components/schemas/Ecgi'

minItems: 0

description: Represents the list of EUTRAN cell Ids that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of EUTRAN cell Ids.

ncgiList:

type: array

items:

$ref: '#/components/schemas/Ncgi'

minItems: 0

description: Represents the list of NR cell Ids that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of NR cell Ids.

globalRanNodeIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

description: Represents the list of NG RAN node identifiers that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of NG RAN node identifiers.

globaleNbIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

minItems: 1

description: Represents the list of eNodeB identifiers that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of eNodeB identifiers.

description: If the additionalPraId IE is present, this IE shall state the presence information of the UE for the individual PRA identified by the additionalPraId IE; If the additionalPraId IE is not present, this IE shall state the presence information of the UE for the PRA identified by the praId IE. This data type is defined in the same way as the "PresenceInfo" data type, but with the OpenAPI "nullable= true" property.

nullable: true

GlobalRanNodeId:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

description: Indicates the identity of the PLMN that the RAN node belongs to.

n3IwfId:

$ref: '#/components/schemas/N3IwfId'

description: This IE shall be included if the AN node represents a N3IWF. When present, this IE shall contain the identifier of the N3IWF

gNbId:

$ref: '#/components/schemas/GNbId'

description: This IE shall be included if the RAN Node Id represents a gNB. When present, this IE shall contain the identifier of the gNB.

ngeNbId:

$ref: '#/components/schemas/NgeNbId'

description: This IE shall be included if the RAN Node Id represents a NG-eNB. When present, this IE shall contain the identifier of an NG-eNB.

wagfId:

$ref: '#/components/schemas/WAgfId'

description: This IE shall be included if the AN node represents a W-AGF. When present, this IE shall contain the identifier of the W-AGF.

tngfId:

$ref: '#/components/schemas/TngfId'

description: This IE shall be included if the AN node represents a TNGF. When present, this IE shall contain the identifier of the TNGF.

nid:

$ref: '#/components/schemas/Nid'

description: Network Identifier shall be present in case of SNPN, PlmnId together with Nid indicates the identity of the SNPN to which the RanNode belongs to.

eNbId:

$ref: '#/components/schemas/ENbId'

description: This IE shall be included if the RAN Node Id represents an eNB. When present, this IE shall contain the identifier of an eNB.

oneOf:

- required: [ n3IwfId ]

- required: [ gNbId ]

- required: [ ngeNbId ]

- required: [ wagfId ]

- required: [ tngfId ]

- required: [ eNbId ]

description: One of the six attributes n3IwfId, gNbIdm, ngeNbId, wagfId, tngfId, eNbId shall be present. For UEs with 5GS subscription but without 5G NAS support, eNbId is used on N7 instead of n3IwfId, gNbIdm, ngeNbId.

required:

- plmnId

GNbId:

type: object

properties:

bitLength:

type: integer

minimum: 22

maximum: 32

description: Unsigned integer representing the bit length of the gNB ID as defined in clause 9.3.1.6 of 3GPP TS 38.413, within the range 22 to 32.

gNBValue:

type: string

pattern: '^[A-Fa-f0-9]{6,8}$'

description: >

This represents the identifier of the gNB.

The value of the gNB ID shall be encoded in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The padding 0 shall be added to make multiple nibbles, the most significant character representing the padding 0 if required together with the 4 most significant bits of the gNB ID shall appear first in the string, and the character representing the 4 least significant bit of the gNB ID shall appear last in the string.

required:

- bitLength

- gNBValue

AtsssCapability:

type: object

properties:

atsssLL:

type: boolean

default: false

description: >

Indicates the ATSSS-LL capability to support procedures related to Access Traffic Steering, Switching, Splitting (see clauses 4.2.10, 5.32 of 3GPP TS 23.501)

true: Supported

false (default): Not Supported

mptcp:

type: boolean

default: false

description: >

Indicates the MPTCP capability to support procedures related to Access Traffic Steering, Switching, Splitting (see clauses 4.2.10, 5.32 of 3GPP TS 23.501

true: Supported

false (default): Not Supported

rttWithoutPmf:

type: boolean

default: false

description: >

This IE is only used by the UPF to indicate whether the UPF supports RTT measurement without PMF (see clauses 5.32.2, 6.3.3.3 of 3GPP TS 23.501

true: Supported

false (default): Not Supported

PlmnIdNid:

type: object

required:

- mcc

- mnc

properties:

mcc:

$ref: '#/components/schemas/Mcc'

description: Mobile Country Code

mnc:

$ref: '#/components/schemas/Mnc'

description: Mobile Network Code

nid:

$ref: '#/components/schemas/Nid'

description: Network Identity; shall be present if PlmnIdNid identifies an SNPN; otherwise shall be absent.

SmallDataRateStatus:

type: object

properties:

remainPacketsUl:

type: integer

minimum: 0

description: When present, it shall contain the number of packets the UE is allowed to send uplink in the given time unit for the given PDU session (see clause 5.31.14.3 of 3GPP TS 23.501

remainPacketsDl:

type: integer

minimum: 0

description: When present it shall contain the number of packets the AF is allowed to send downlink in the given time unit for the given PDU session (see clause 5.31.14.3 of 3GPP TS 23.501

validityTime:

$ref: '#/components/schemas/DateTime'

description: When present, it shall indicate the period of time during which the small data rate control status will remain valid (see clause 5.31.14.3 of 3GPP TS 23.501.

remainExReportsUl:

type: integer

minimum: 0

description: When present, it shall indicate number of additional exception reports the UE is allowed to send uplink in the given time unit for  the given PDU session (see clause 5.31.14.3 of 3GPP TS 23.501.

remainExReportsDl:

type: integer

minimum: 0

description: When present, it shall indicate number of additional exception reports the AF is allowed to send downlink in the given time unit for the given PDU session (see clause 5.31.14.3 in 3GPP TS 23.501

ApnRateStatus:

type: object

properties:

remainPacketsUl:

type: integer

minimum: 0

description: When present, it shall contain the number of packets the UE is allowed to send uplink in the given time unit for the given APN (all PDN connections of the UE to this APN see clause 4.7.7.3 in 3GPP TS 23.401.

remainPacketsDl:

type: integer

minimum: 0

description: When present, it shall contain the number of packets the UE is allowed to send uplink in the given time unit for the given APN (all PDN connections of the UE to this APN see clause 4.7.7.3 in 3GPP TS 23.401.

validityTime:

$ref: '#/components/schemas/DateTime'

description: When present, it shall indicate the period of time during which the APN rate control status will remain valid.

remainExReportsUl:

type: integer

minimum: 0

description: When present, it shall indicate the number of additional exception reports the UE is allowed to send uplink in the given time unit for the given APN (all PDN connections of the UE to this APN, see clause 4.7.7.3 in 3GPP TS 23.401.

remainExReportsDl:

type: integer

minimum: 0

description: When present, it shall indicate the number of additional exception reports the AF is allowed to send downlink in the given time unit for the given APN (all PDN connections of the UE to this APN, see clause 4.7.7.3 in 3GPP TS 23.401.

HfcNodeId:

type: object

required:

- hfcNId

properties:

hfcNId:

$ref: '#/components/schemas/HfcNId'

description: HFC Node Id

HfcNodeIdRm:

anyOf:

- $ref: '#/components/schemas/HfcNodeId'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "HfcNodeId" data type, but with the OpenAPI "nullable= true" property

ScheduledCommunicationTime:

type: object

properties:

daysOfWeek:

type: array

items:

$ref: '#/components/schemas/DayOfWeek'

minItems: 1

maxItems: 6

description: Identifies the day(s) of the week. If absent, it indicates every day of the week.

timeOfDayStart:

$ref: '#/components/schemas/TimeOfDay'

description: Identifies the start time of the day.

timeOfDayEnd:

$ref: '#/components/schemas/TimeOfDay'

description: Identifies the end time of the day

ScheduledCommunicationTimeRm:

anyOf:

- $ref: '#/components/schemas/ScheduledCommunicationTime'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "ScheduledCommunicationTime" data type, but with the OpenAPI "nullable= true" property.

BatteryIndication:

type: object

properties:

batteryInd:

type: boolean

description: >

This IE shall indicate whether the UE is battery powered or not.

true: the UE is battery powered;

false or absent: the UE is not battery powered

replaceableInd:

type: boolean

description: >

This IE shall indicate whether the battery of the UE is replaceable or not.

true: the battery of the UE is replaceable;

false or absent: the battery of the UE is not replaceable.

rechargeableInd:

type: boolean

description: >

This IE shall indicate whether the battery of the UE is rechargeable or not.

true: the battery of UE is rechargeable;

false or absent: the battery of the UE is not rechargeable.

description: Parameters "replaceableInd" and "rechargeableInd" are only included if the value of Parameter "batteryInd" is true.

BatteryIndicationRm:

anyOf:

- $ref: '#/components/schemas/BatteryIndication'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "BatteryIndication" data type, but with the OpenAPI "nullable= true" property.

AcsInfo:

type: object

properties:

acsUrl:

$ref: '#/components/schemas/Uri'

description: This IE may contain the URL of the ACS, see BBF TR-069 or BBF TR-369.

acsIpv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

description: This IE may contain the IPv4 address of the ACS, see BBF TR-069 or BBF TR-369.

acsIpv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

description: This IE may contain the IPv6 address of the ACS, see BBF TR-069 or BBF TR-369

AcsInfoRm:

anyOf:

- $ref: '#/components/schemas/AcsInfo'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "AcsInfo" data type, but with the OpenAPI "nullable= true" property.

NrV2xAuth:

type: object

properties:

vehicleUeAuth:

$ref: '#/components/schemas/UeAuth'

description: This IE shall be present if available. When present, it shall indicate whether the UE is authorized as Vehicle UE.

pedestrianUeAuth:

$ref: '#/components/schemas/UeAuth'

description: This IE shall be present if available. When present, it shall indicate whether the UE is authorized as Pedestrian UE.

LteV2xAuth:

type: object

properties:

vehicleUeAuth:

$ref: '#/components/schemas/UeAuth'

description: This IE shall be present if available. When present, it shall indicate whether the UE is authorized as Vehicle UE.

pedestrianUeAuth:

$ref: '#/components/schemas/UeAuth'

description: This IE shall be present if available. When present, it shall indicate whether the UE is authorized as Pedestrian UE.

Pc5QoSPara:

type: object

required:

- pc5QosFlowList

properties:

pc5QosFlowList:

type: array

items:

$ref: '#/components/schemas/Pc5QosFlowItem'

description: This IE shall contain the set of PC5 flow(s).

pc5LinkAmbr:

$ref: '#/components/schemas/BitRate'

description: This IE shall be present if available. When present, it shall represent the PC5 Link Aggregated Bit Rates for all the Non-GBR QoS Flows (see clause 5.4.2.3 of 3GPP TS 23.287.

Pc5QosFlowItem:

type: object

required:

- pqi

properties:

pqi:

$ref: '#/components/schemas/5Qi'

description: PQI is a special 5QI (see clause 5.4.2.1 of 3GPP TS 23.287.

pc5FlowBitRates:

$ref: '#/components/schemas/Pc5FlowBitRates'

description: When present, it shall represent the PC5 Flow Bit Rates (see clause 5.4.2.2 of 3GPP TS 23.287.

range:

$ref: '#/components/schemas/Uinteger'

description: When present, it shall represent the Range in the unit of meters (see clause 5.4.2.4 of 3GPP TS 23.287.

Pc5FlowBitRates:

type: object

properties:

guaFbr:

$ref: '#/components/schemas/BitRate'

description: When present, it shall contain the guaranteed Bit Rate for the PC5 QoS flow.

maxFbr:

$ref: '#/components/schemas/BitRate'

description: When present, it shall contain the maximum Bit Rate for the PC5 QoS flow.

UtraLocation:

type: object

oneOf:

- required:

- cgi

- required:

- sai

- required:

- rai

description: Exactly one of cgi, sai or lai shall be present.

properties:

cgi:

$ref: '#/components/schemas/CellGlobalId'

description: Cell Global Identification. See 3GPP TS 23.003, clause 4.3.1

sai:

$ref: '#/components/schemas/ServiceAreaId'

description: Service Area Identifier. See 3GPP TS 23.003, clause12.5.

lai:

$ref: '#/components/schemas/LocationAreaId'

description: Location area identification. See 3GPP TS 23.003, clause 4.1

rai:

$ref: '#/components/schemas/RoutingAreaId'

description: Routing Area Identification. See 3GPP TS 23.003, clause 4.2.

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

description: The value represents the elapsed time in minutes since the last network contact of the mobile station.  Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful location reporting procedure the UE is in connected mode.Any other value than "0" indicates that the location information is the last known one.See 3GPP TS 29.002 clause 17.7.8.

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

description: The value represents the UTC time when the UeLocation information was acquired.

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

description: Refer to geographical Information.See 3GPP TS 23.032 clause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used.

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

description: Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used.

GeraLocation:

type: object

oneOf:

- required:

- cgi

- required:

- sai

- required:

- rai

description: Exactly one of cgi, sai or lai shall be present.

properties:

locationNumber:

type: string

description: Location number within the PLMN. See 3GPP TS 23.003, clause 4.5.

cgi:

$ref: '#/components/schemas/CellGlobalId'

description: Cell Global Identification. See 3GPP TS 23.003, clause 4.3.1.

sai:

$ref: '#/components/schemas/ServiceAreaId'

description: Service Area Identifier. See 3GPP TS 23.003, clause 12.5.

lai:

$ref: '#/components/schemas/LocationAreaId'

description: Location Area identification. See 3GPP TS 23.003, clause 4.1.

vlrNumber:

type: string

description: VLR number. See 3GPP TS 23.003 clause 5.1.

mscNumber:

type: string

description: MSC number. See 3GPP TS 23.003 clause 5.1.

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

description: The value represents the elapsed time in minutes since the last network contact of the mobile station.  Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful location reporting procedure the UE is in connected mode.Any other value than "0" indicates that the location information is the last known one.See 3GPP TS 29.002 clause 17.7.8.

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

description: The value represents the UTC time when the UeLocation information was acquired.

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

description: Refer to geographical Information.See 3GPP TS 23.032 clause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used.

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

description: Refers to Calling Geodetic Location.See ITU-T Recommendation Q.763 (1999) clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used.

CellGlobalId:

type: object

required:

- plmnId

- lac

- cellId

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

description: PLMN Identity

lac:

type: string

description: Location Area Code

cellId:

type: string

pattern: '^[A-Fa-f0-9]$'

description: Cell Identity

ServiceAreaId:

type: object

required:

- plmnId

- lac

- sac

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

description: PLMN Identity

lac:

type: string

pattern: '^[A-Fa-f0-9]$'

description: Location Area Code.

sac:

type: string

pattern: '^[A-Fa-f0-9]$'

description: Service Area Code.

LocationAreaId:

type: object

required:

- plmnId

- lac

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

description: PLMN Identity

lac:

type: string

pattern: '^[A-Fa-f0-9]$'

description: Location Area Code

RoutingAreaId:

type: object

required:

- plmnId

- lac

- rac

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

description: PLMN Identity

lac:

type: string

pattern: '^[A-Fa-f0-9]$'

description: Location Area Code

rac:

type: string

pattern: '^[A-Fa-f0-9]$'

description: Routing Area Code

DddTrafficDescriptor:

type: object

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

description: Ipv4 address of the source of downlink data.

ipv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

description: Ipv6 address of the source of downlink data.

portNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

description: Port number of the source of downlink data.

macAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

description: Source MAC address.

MoExpDataCounter:

type: object

required:

- counter

properties:

counter:

type: integer

description: Unsigned integer identifying the MO Exception Data Counter, as specified in clause 5.31.14.3 of 3GPP TS 23.501.

timeStamp:

$ref: '#/components/schemas/DateTime'

description: UTC time indicating the time at which the counter value increased from 0 to 1.

NssaaStatus:

type: object

required:

- snssai

- status

properties:

snssai:

$ref: '#/components/schemas/Snssai'

description: Subscribed S-NSSAI.

status:

$ref: '#/components/schemas/AuthStatus'

description: This flag when present shall indicate the NSSAA status of the related Snssai.

NssaaStatusRm:

anyOf:

- $ref: '#/components/schemas/NssaaStatus'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "NssaaStatus" data type, but with the OpenAPI "nullable= true" property.

TnapId:

type: object

properties:

ssId:

type: string

description: This IE shall be present if the UE is accessing the 5GC via a trusted WLAN access network.When present, it shall contain the SSID of the access point to which the UE is attached, that is received over NGAP, see IEEE Std 802.11-2012.

bssId:

type: string

description: When present, it shall contain the BSSID of the access point to which the UE is attached, that is received over NGAP, see IEEE Std 802.11-2012.

civicAddress:

$ref: '#/components/schemas/Bytes'

description: When present, it shall contain the civic address information of the TNAP to which the UE is attached, including the Location-Information Attribute and / or Location-Data Attribute as defined in IETF RFC 5580.

TnapIdRm:

anyOf:

- $ref: '#/components/schemas/TnapId'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "TnapId" data type, but with the OpenAPI "nullable= true" property.

TwapId:

type: object

required:

- ssId

properties:

ssId:

type: string

description: This IE shall contain the SSID of the access point to which the UE is attached, that is received over NGAP, see IEEE Std 802.11-2012.

bssId:

type: string

description: When present, it shall contain the BSSID of the access point to which the UE is attached, for trusted WLAN access, see IEEE Std 802.11-2012.

civicAddress:

$ref: '#/components/schemas/Bytes'

description: When present, it shall contain the civic address information of the TWAP to which the UE is attached, for trusted WLAN access. This IE shall include the Location-Information Attribute and / or Location-Data Attribute as defined in IETF RFC 5580.

TwapIdRm:

anyOf:

- $ref: '#/components/schemas/TwapId'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "TwapId" data type, but with the OpenAPI "nullable= true" property.

LineType:

anyOf:

- type: string

enum:

- DSL

- PON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- DSL: Identifies a DSL line

- PON: Identifies a PON line

LineTypeRm:

anyOf:

- $ref: '#/components/schemas/LineType'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "LineType" data type, but with the OpenAPI "nullable= true" property.

SnssaiExtension:

type: object

properties:

sdRanges:

type: array

items:

$ref: '#/components/schemas/SdRange'

minItems: 1

description: When present, it shall contain the range(s) of Slice Differentiator values supported for the Slice/Service Type value indicated in the sst attribute of the Snssai data type (see clause 5.4.4.2).

wildcardSd:

type: boolean

default: false

description: When present, it shall be set to "true", to indicate that all SD values are supported for the Slice/Service Type value indicated in the sst attribute of the Snssai data type (see clause 5.4.4.2).

description: Extensions to the Snssai data type, sdRanges and wildcardSd shall not be present simultaneously

SdRange:

description: A range of SDs (Slice Differentiators)

type: object

properties:

start:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: First value identifying the start of an SD range. This string shall be formatted as specified for the sd attribute of the Snssai data type in clause 5.4.4.2.

end:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: Last value identifying the end of an SD range. This string shall be formatted as specified for the sd attribute of the Snssai data type in clause 5.4.4.2.

description: A range of SDs (Slice Differentiators)

#

# Data types describing alternative data types or combinations of data types

#

ExtSnssai:

allOf:

- $ref: '#/components/schemas/Snssai'

- $ref: '#/components/schemas/SnssaiExtension'

description: The sdRanges and wildcardSd attributes shall be exclusive from each other. If one of these attributes is present, the sd attribute shall also be present and it shall contain one Slice Differentiator value within the range of SD (if the sdRanges attribute is present) or with any value (if the wildcardSd attribute is present).

#

# Data Types related to 5G QoS as defined in clause 5.5

#

#

# SIMPLE DATA TYPES

#

#

Qfi:

type: integer

minimum: 0

maximum: 63

description: Unsigned integer identifying a QoS flow, within the range 0 to 63.

QfiRm:

type: integer

minimum: 0

maximum: 63

nullable: true

description: This data type is defined in the same way as the "Qfi" data type, but with the OpenAPI "nullable= true" property.

5Qi:

type: integer

minimum: 0

maximum: 255

description: Unsigned integer representing a 5G QoS Identifier (see clause 5.7.2.1 of 3GPP TS 23.501, within the range 0 to 255

5QiRm:

type: integer

minimum: 0

maximum: 255

nullable: true

description: This data type is defined in the same way as the "5Qi" data type, but with the OpenAPI "nullable= true" property.

BitRate:

type: string

pattern: '^\d+(\.\d+)? (bps|Kbps|Mbps|Gbps|Tbps)$'

description: String representing a bit rate that shall be formatted see pattern

BitRateRm:

type: string

pattern: '^\d+(\.\d+)? (bps|Kbps|Mbps|Gbps|Tbps)$'

nullable: true

description: This data type is defined in the same way as the "BitRate" data type, but with the OpenAPI "nullable= true" property.

ArpPriorityLevelRm:

type: integer

minimum: 1

maximum: 15

nullable: true

description: This data type is defined in the same way as the "ArpPriorityLevel" data type, but with the OpenAPI "nullable= true" property.

ArpPriorityLevel:

type: integer

minimum: 1

maximum: 15

nullable: true

description: nullable true shall not be used for this attribute. Unsigned integer indicating the ARP Priority Level (see clause 5.7.2.2 of 3GPP TS 23.501, within the range 1 to 15.Values are ordered in decreasing order of priority, i.e. with 1 as the highest priority and 15 as the lowest priority.

5QiPriorityLevel:

type: integer

minimum: 1

maximum: 127

description: Unsigned integer indicating the 5QI Priority Level (see clauses 5.7.3.3 and 5.7.4 of 3GPP TS 23.501, within the range 1 to 127.Values are ordered in decreasing order of priority, i.e. with 1 as the highest priority and 127 as the lowest priority.

5QiPriorityLevelRm:

type: integer

minimum: 1

maximum: 127

nullable: true

description: This data type is defined in the same way as the "5QiPriorityLevel" data type, but with the OpenAPI "nullable= true" property.

PacketDelBudget:

type: integer

minimum: 1

description: Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP TS 23.501), expressed in milliseconds.

PacketDelBudgetRm:

type: integer

minimum: 1

nullable: true

description: This data type is defined in the same way as the "PacketDelBudget" data type, but with the OpenAPI "nullable= true" property

PacketErrRate:

type: string

pattern: '^([0-9]E-[0-9])$'

description: String representing Packet Error Rate (see clause 5.7.3.5 and 5.7.4 of 3GPP TS 23.501, expressed as a "scalar x 10-k" where the scalar and the exponent k are each encoded as one decimal digit

PacketErrRateRm:

type: string

pattern: '^([0-9]E-[0-9])$'

nullable: true

description: This data type is defined in the same way as the "PacketErrRate" data type, but with the OpenAPI "nullable= true" property.

PacketLossRate:

type: integer

minimum: 0

maximum: 1000

description: Unsigned integer indicating Packet Loss Rate (see clauses 5.7.2.8 and 5.7.4 of 3GPP TS 23.501), expressed in tenth of percent.

PacketLossRateRm:

type: integer

minimum: 0

maximum: 1000

nullable: true

description: This data type is defined in the same way as the "PacketLossRate" data type, but with the OpenAPI "nullable= true" property

AverWindow:

type: integer

minimum: 1

maximum: 4095

default: 2000

description: Unsigned integer indicating Averaging Window (see clause 5.7.3.6 and 5.7.4 of 3GPP TS 23.501), expressed in milliseconds.

AverWindowRm:

type: integer

maximum: 4095

default: 2000

minimum: 1

nullable: true

description: This data type is defined in the same way as the "AverWindow" data type, but with the OpenAPI "nullable= true" property.

MaxDataBurstVol:

type: integer

minimum: 1

maximum: 4095

description: Unsigned integer indicating Maximum Data Burst Volume (see clauses 5.7.3.7 and 5.7.4 of 3GPP TS 23.501), expressed in Bytes.

MaxDataBurstVolRm:

type: integer

minimum: 1

maximum: 4095

nullable: true

description: This data type is defined in the same way as the "MaxDataBurstVol" data type, but with the OpenAPI "nullable= true" property

SamplingRatio:

type: integer

minimum: 1

maximum: 100

description: Unsigned integer indicating Sampling Ratio (see clauses 4.15.1 of 3GPP TS 23.502), expressed in percent.

SamplingRatioRm:

type: integer

minimum: 1

maximum: 100

nullable: true

description: This data type is defined in the same way as the "SamplingRatio" data type, but with the OpenAPI "nullable= true" property.

RgWirelineCharacteristics:

$ref: '#/components/schemas/Bytes'

description: RG Level Wireline Access Characteristics(see BBF TR-456 and BBF TR-470). It shall be encoded as a string with format "byte" as defined in OpenAPI Specification, i.e.  base64 encoded characters, representing the RG-Level Wireline Access Characteristics encoded as specified in clause 7.5 of BBF TR-470.

RgWirelineCharacteristicsRm:

anyOf:

- $ref: '#/components/schemas/RgWirelineCharacteristics'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "RgWirelineCharacteristics" data type, but with the OpenAPI "nullable= true" property.

ExtMaxDataBurstVol:

type: integer

minimum: 4096

maximum: 2000000

description: Unsigned integer indicating Maximum Data Burst Volume (see clauses 5.7.3.7 and 5.7.4 of 3GPP TS 23.501), expressed in Bytes.

ExtMaxDataBurstVolRm:

type: integer

minimum: 4096

maximum: 2000000

nullable: true

description: This data type is defined in the same way as the "ExtMaxDataBurstVol" data type, but with the OpenAPI "nullable= true" property.

ExtPacketDelBudget:

type: integer

minimum: 1

description: Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP TS 23.501)), expressed in 0.01 milliseconds.

ExtPacketDelBudgetRm:

type: integer

minimum: 1

nullable: true

description: This data type is defined in the same way as the "ExtPacketDelBudget" data type, but with the OpenAPI "nullable= true" property.

#

# ENUMERATED DATA TYPES

#

PreemptionCapability:

anyOf:

- type: string

enum:

- NOT_PREEMPT

- MAY_PREEMPT

- type: string

description: The enumeration PreemptionCapability indicates the pre-emption capability of a request on other QoS flows. See clause 5.7.2.2 of 3GPP TS 23.501. It shall comply with the provisions defined in table 5.5.3.1-1.

PreemptionCapabilityRm:

anyOf:

- $ref: '#/components/schemas/PreemptionCapability'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "PreemptionCapability" enumeration, but with the OpenAPI "nullable= true" property.

PreemptionVulnerability:

anyOf:

- type: string

enum:

- NOT_PREEMPTABLE

- PREEMPTABLE

- type: string

description: The enumeration PreemptionVulnerability indicates the pre-emption vulnerability of the QoS flow to pre-emption from other QoS flows. See clause 5.7.2.2 of 3GPP TS 23.501. It shall comply with the provisions defined in table 5.5.3.2-1

PreemptionVulnerabilityRm:

anyOf:

- $ref: '#/components/schemas/PreemptionVulnerability'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "PreemptionVulnerability" enumeration, but with the OpenAPI "nullable= true" property.

ReflectiveQoSAttribute:

anyOf:

- type: string

enum:

- RQOS

- NO_RQOS

- type: string

description: The enumeration ReflectiveQosAttribute indicates whether certain traffic of the QoS flow may be subject to Reflective QoS (see clause 5.7.2.3 of 3GPP TS 23.501). It shall comply with the provisions defined in table 5.5.3.3-1.

ReflectiveQoSAttributeRm:

anyOf:

- $ref: '#/components/schemas/ReflectiveQoSAttribute'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "ReflectiveQosAttribute" enumeration, but with the OpenAPI "nullable= true" property.

NotificationControl:

anyOf:

- type: string

enum:

- REQUESTED

- NOT_REQUESTED

- type: string

description: The enumeration NotificationControl indicates whether notifications are requested from the RAN when the GFBR can no longer (or again) be fulfilled for a QoS Flow during the lifetime of the QoS Flow (see clause 5.7.2.4 of 3GPP TS 23.501). It shall comply with the provisions defined in table 5.5.3.5-1.

NotificationControlRm:

anyOf:

- $ref: '#/components/schemas/NotificationControl'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "NotificationControl" enumeration, but with the OpenAPI "nullable= true" property.

QosResourceType:

anyOf:

- type: string

enum:

- NON_GBR

- NON_CRITICAL_GBR

- CRITICAL_GBR

- type: string

description: The enumeration QosResourceType indicates whether a QoS Flow is non-GBR, delay critical GBR, or non-delay critical GBR (see clauses 5.7.3.4 and 5.7.3.5 of 3GPP TS 23.501). It shall comply with the provisions defined in table 5.5.3.6-1.

QosResourceTypeRm:

anyOf:

- $ref: '#/components/schemas/QosResourceType'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "QosResourceType" enumeration, but with the OpenAPI "nullable= true" property.

AdditionalQosFlowInfo:

anyOf:

- anyOf:

- type: string

enum:

- MORE_LIKELY

- type: string

- $ref: '#/components/schemas/NullValue'

description: The enumeration AdditionalQosFlowInfo provides additional QoS flow information (see clause 9.3.1.12 3GPP TS 38.413). It shall comply with the provisions defined in table 5.5.3.12-1.

#

#

# STRUCTURED DATA TYPES

#

Arp:

type: object

properties:

priorityLevel:

$ref: '#/components/schemas/ArpPriorityLevel'

description: Defines the relative importance of a resource request.

preemptCap:

$ref: '#/components/schemas/PreemptionCapability'

description: Defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level.

preemptVuln:

$ref: '#/components/schemas/PreemptionVulnerability'

description: Defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level.

required:

- priorityLevel

- preemptCap

- preemptVuln

ArpRm:

anyOf:

- $ref: '#/components/schemas/Arp'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "Arp" data type, but with the OpenAPI "nullable= true" property.

Ambr:

type: object

properties:

uplink:

$ref: '#/components/schemas/BitRate'

description: AMBR for uplink.

downlink:

$ref: '#/components/schemas/BitRate'

description: AMBR for downlink.

required:

- uplink

- downlink

AmbrRm:

anyOf:

- $ref: '#/components/schemas/Ambr'

- $ref: '#/components/schemas/NullValue'

description: This data type is defined in the same way as the "Ambr" data type, but with the OpenAPI "nullable= true" property.

Dynamic5Qi:

type: object

properties:

resourceType:

$ref: '#/components/schemas/QosResourceType'

description: Defines the 5QI resource type. See clause 5.5.3.6.??

priorityLevel:

$ref: '#/components/schemas/5QiPriorityLevel'

description: Defines the 5QI Priority Level.  See clause 5.5.2

packetDelayBudget:

$ref: '#/components/schemas/PacketDelBudget'

description: Defines the packet delay budget. See clause 5.5.2.

packetErrRate:

$ref: '#/components/schemas/PacketErrRate'

description: Defines the packet error rate. See clause 5.5.2.

averWindow:

$ref: '#/components/schemas/AverWindow'

description: Defines the averaging window. See clause 5.5.2. This IE shall be present only for a GBR QoS flow or a Delay Critical GBR QoS flow.

maxDataBurstVol:

$ref: '#/components/schemas/MaxDataBurstVol'

description: Defines the maximum data burst volume. See clause 5.5.2. This IE shall be present for a Delay Critical GBR QoS flow.

extMaxDataBurstVol:

$ref: '#/components/schemas/ExtMaxDataBurstVol'

description: Defines the maximum data burst volume. See clause 5.5.2.

extPacketDelBudget:

$ref: '#/components/schemas/ExtPacketDelBudget'

description: Defines the packet delay budget. See clause 5.5.2.

cnPacketDelayBudgetDl:

$ref: '#/components/schemas/ExtPacketDelBudget'

description: Defines the Core Network Packet Delay Budget for downlink. See clause 5.5.2.

cnPacketDelayBudgetUl:

$ref: '#/components/schemas/ExtPacketDelBudget'

description: Defines the Core Network Packet Delay Budget for uplink. See clause 5.5.2

required:

- resourceType

- priorityLevel

- packetDelayBudget

- packetErrRate

NonDynamic5Qi:

type: object

properties:

priorityLevel:

$ref: '#/components/schemas/5QiPriorityLevel'

description: Defines the 5QI Priority Level. See clause 5.5.2.When present, it contains the 5QI Priority Level value that overrides the standardized or pre-configured value.

averWindow:

$ref: '#/components/schemas/AverWindow'

description: Defines the averaging window. See clause 5.5.2.This IE may be present for a GBR QoS flow or a Delay Critical GBR QoS flow. When present, it contains the Averaging Window that overrides the standardized or pre-configured value.

maxDataBurstVol:

$ref: '#/components/schemas/MaxDataBurstVol'

description: Defines the maximum data burst volume. See clause 5.5.2.This IE may be present for a Delay Critical GBR QoS flow. When present, it contains the Maximum Data Burst Volume value that overrides the standardized or pre-configured value.

extMaxDataBurstVol:

$ref: '#/components/schemas/ExtMaxDataBurstVol'

description: Defines the maximum data burst volume. See clause 5.5.2.This IE may be present for a Delay Critical GBR QoS flow. When present, it contains the Maximum Data Burst Volume value that overrides the standardized or pre-configured value

cnPacketDelayBudgetDl:

$ref: '#/components/schemas/ExtPacketDelBudget'

description: Defines the Core Network Packet Delay Budget for downlink. See clause 5.5.2.

cnPacketDelayBudgetUl:

$ref: '#/components/schemas/ExtPacketDelBudget'

description: Defines the Core Network Packet Delay Budget for uplink. See clause 5.5.2.

minProperties: 0

#

# Data Types related to 5G Trace as defined in clause 5.6

#

#

# SIMPLE DATA TYPES

#

PhysCellId:

type: integer

minimum: 0

maximum: 1007

description: Integer value identifying the physical cell identity (PCI), as definition of "PhysCellId" IE in clause 6.3.2 of 3GPPTS 38.331.

ArfcnValueNR:

type: integer

minimum: 0

maximum: 3279165

description: Integer value indicating the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as definition of "ARFCN-ValueNR" IE in clause 6.3.2 of 3GPP TS 38.331.

#

#

# Enumerations

#

TraceDepth:

anyOf:

- type: string

enum:

- MINIMUM

- MEDIUM

- MAXIMUM

- MINIMUM_WO_VENDOR_EXTENSION

- MEDIUM_WO_VENDOR_EXTENSION

- MAXIMUM_WO_VENDOR_EXTENSION

- type: string

description: The enumeration TraceDepth defines how detailed information should be recorded in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.1-1

TraceDepthRm:

anyOf:

- $ref: '#/components/schemas/TraceDepth'

- $ref: '#/components/schemas/NullValue'

description: This enumeration is defined in the same way as the "TraceDepth" enumeration, but with the OpenAPI "nullable= true" property.

JobType:

anyOf:

- type: string

enum:

- IMMEDIATE_MDT_ONLY

- LOGGED_MDT_ONLY

- TRACE_ONLY

- IMMEDIATE_MDT_AND_TRACE

- RLF_REPORTS_ONLY

- RCEF_REPORTS_ONLY

- LOGGED_MBSFN_MDT

- type: string

description: The enumeration JobType defines Job Type in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.3-1.

ReportTypeMdt:

anyOf:

- type: string

enum:

- PERIODICAL

- EVENT_TRIGGED

- type: string

description: The enumeration ReportTypeMdt defines Report Type for logged MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.4-1.

MeasurementLteForMdt:

anyOf:

- type: string

enum:

- M1

- M2

- M3

- M4_DL

- M4_UL

- M5_DL

- M5_UL

- M6_DL

- M6_UL

- M7_DL

- M7_UL

- M8

- M9

- type: string

description: The enumeration MeasurementLteForMdt defines Measurements used for MDT in LTE in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.5-1.

MeasurementNrForMdt:

anyOf:

- type: string

enum:

- M1

- M2

- M3

- M4_DL

- M4_UL

- M5_DL

- M5_UL

- M6_DL

- M6_UL

- M7_DL

- M7_UL

- M8

- M9

- type: string

description: The enumeration MeasurementNrForMdt defines Measurements used for MDT in NR in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.6-1.

SensorMeasurement:

anyOf:

- type: string

enum:

- BAROMETRIC_PRESSURE

- UE_SPEED

- UE_ORIENTATION

- type: string

description: The enumeration SensorMeasurement defines sensor measurement type for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.7-1.

ReportingTrigger:

anyOf:

- type: string

enum:

- PERIODICAL

- EVENT_A2

- EVENT_A2_PERIODIC

- ALL_RRM_EVENT_TRIGGERS

- type: string

description: The enumeration ReportingTrigger defines Reporting Triggers for MDT in the trace. See 3GPP TS 32.42] for further description of the values. It shall comply with the provisions defined in table 5.6.3.8-1.

ReportIntervalMdt:

anyOf:

- type: string

enum:

- 120

- 240

- 480

- 640

- 1024

- 2048

- 5120

- 10240

- 60000

- 360000

- 720000

- 1800000

- 3600000

- type: string

description: The enumeration ReportIntervalMdt defines Report Interval for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.9-1.

ReportAmountMdt:

anyOf:

- type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- infinity

- type: string

description: The enumeration ReportAmountMdt defines Report Amount for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.10-1.

EventForMdt:

anyOf:

- type: string

enum:

- OUT_OF_COVERAG

- A2_EVENT

- type: string

description: The enumeration EventForMdt defines events triggered measurement for logged MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.11-1

LoggingIntervalMdt:

anyOf:

- type: string

enum:

- 128

- 256

- 512

- 1024

- 2048

- 3072

- 4096

- 6144

- type: string

description: The enumeration LoggingIntervalMdt defines Logging Interval for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.12-1.

LoggingDurationMdt:

anyOf:

- type: string

enum:

- 600

- 1200

- 2400

- 3600

- 5400

- 7200

- type: string

description: The enumeration LoggingIntervalMdt defines Logging Interval for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.12-1.

PositioningMethodMdt:

anyOf:

- type: string

enum:

- GNSS

- E_CELL_ID

- type: string

description: The enumeration LoggingDurationMdt defines Logging Duration for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.13-1.

CollectionPeriodRmmLteMdt:

anyOf:

- type: string

enum:

- 1024

- 1280

- 2048

- 2560

- 5120

- 10240

- 60000

- type: string

description: The enumeration CollectionPeriodRmmLteMdt defines Collection period for RRM measurements LTE for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.15-1.

MeasurementPeriodLteMdt:

anyOf:

- type: string

enum:

- 1024

- 1280

- 2048

- 2560

- 5120

- 10240

- 60000

- type: string

description: The enumeration MeasurementPeriodLteMdt defines Measurement period LTE for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.16-1.

ReportIntervalNrMdt:

anyOf:

- type: string

enum:

- 120

- 240

- 480

- 640

- 1024

- 2048

- 5120

- 10240

- 20480

- 40960

- 60000

- 360000

- 720000

- 1800000

- 3600000

- type: string

description: The enumeration ReportIntervalNrMdt defines Report Interval in NR for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.17-1

LoggingIntervalNrMdt:

anyOf:

- type: string

enum:

- 128

- 256

- 512

- 1024

- 2048

- 3072

- 4096

- 6144

- 320

- 640

- infinity

- type: string

description: The enumeration LoggingIntervalNrMdt defines Logging Interval in NR for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.18-1.

CollectionPeriodRmmNrMdt:

anyOf:

- type: string

enum:

- 1024

- 2048

- 5120

- 10240

- 60000

- type: string

description: The enumeration CollectionPeriodRmmNrMdt defines Collection period for RRM measurements NR for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.19-1

LoggingDurationNrMdt:

anyOf:

- type: string

enum:

- 600

- 1200

- 2400

- 3600

- 5400

- 7200

- type: string

description: The enumeration LoggingDurationMdt defines Logging Duration in NR for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.20-1.

#

# STRUCTURED DATA TYPES

#

TraceData:

type: object

nullable: true

properties:

traceRef:

type: string

pattern: '^[0-9]{3}[0-9]{2,3}-[A-Fa-f0-9]{6}$'

description: Trace Reference (see 3GPP TS 32.422).It shall be encoded as the concatenation of MCC, MNC and Trace ID as follows:<MCC><MNC>-<Trace ID>The Trace ID shall be encoded as a 3 octet string in hexadecimal representation. Each character in the Trace ID string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the Trace ID shall appear first in the string, and the character representing the 4 least significant bit of the Trace ID shall appear last in the string.

traceDepth:

$ref: '#/components/schemas/TraceDepth'

description: Trace Depth (see 3GPP TS 32.422).

neTypeList:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: List of NE Types (see 3GPP TS 32.422).It shall be encoded as an octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits shall appear first in the string, and the character representing the 4 least significant bit shall appear last in the string.Octets shall be coded according to 3GPP TS 32.422.

eventList:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: Triggering events (see 3GPP TS 32.422).It shall be encoded as an octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits shall appear first in the string, and the character representing the 4 least significant bit shall appear last in the string.Octets shall be coded according to 3GPP TS 32.422.

collectionEntityIpv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

description: IPv4 Address of the Trace Collection Entity (see 3GPP TS 32.422). At least one of the collectionEntityIpv4Addr or collectionEntityIpv6Addr attributes shall be present.

collectionEntityIpv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

description: IPv6 Address of the Trace Collection Entity (see 3GPP TS 32.422).At least one of the collectionEntityIpv4Addr or collectionEntityIpv6Addr attributes shall be present

interfaceList:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: List of Interfaces (see 3GPP TS 32.422).It shall be encoded as an octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits shall appear first in the string, and the character representing the 4 least significant bit shall appear last in the string.Octets shall be coded according to 3GPP TS 32.422. If this attribute is not present, all the interfaces applicable to the list of NE types indicated in the neTypeList attribute should be traced.

required:

- traceRef

- traceDepth

- neTypeList

- eventList

MdtConfiguration:

type: object

required:

- jobType

properties:

jobType:

$ref: '#/components/schemas/JobType'

description: This IE shall indicate the Job type for MDT, see 3GPP TS 32.422.

reportType:

$ref: '#/components/schemas/ReportTypeMdt'

description: This IE shall be present for logged MDT. When present, this IE shall indicate the report type for logged MDT, see 3GPP TS 32.422.

areaScope:

$ref: '#/components/schemas/AreaScope'

description: When present, this IE shall contain the area in Cells or Tracking Areas where the MDT data collection shall take place, see 3GPP TS 32.422.

measurementLteList:

type: array

items:

$ref: '#/components/schemas/MeasurementLteForMdt'

description: This IE shall be present if the Job type is configured for Immediate MDT or combined Immediate MDT and Trace.When present, this IE shall contain a list of the measurements that shall be collected for LTE.

measurementNrList:

type: array

items:

$ref: '#/components/schemas/MeasurementNrForMdt'

minItems: 1

description: This IE shall be present if the Job type is configured for Immediate MDT or combined Immediate MDT and Trace.When present, this IE shall contain a list of the measurements that shall be collected for NR.

sensorMeasurementList:

type: array

items:

$ref: '#/components/schemas/SensorMeasurement'

minItems: 1

description: When present, this IE shall include a list o the sensor measurements to be collected for UE if they are available.

reportingTriggerList:

type: array

items:

$ref: '#/components/schemas/ReportingTrigger'

minItems: 1

description: This IE shall be present if MeasurementList is configured for UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT or combined Immediate MDT and Trace.When present, this IE shall contain a list of the reporting triggers.For LTE and NR, this IE shall not have the combination of periodical, event based and event based periodic reporting at the same time.

reportInterval:

$ref: '#/components/schemas/ReportIntervalMdt'

description: This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT or combined Immediate MDT and Trace.When present, this IE shall indicate the interval between the periodical measurements to be taken when UE is in connected in LTE.

reportIntervalNr:

$ref: '#/components/schemas/ReportIntervalNrMdt'

description: This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in NR) and the jobType is configured for Immediate MDT or combined Immediate MDT and Trace.When present, this IE shall indicate the interval between the periodical measurements to be taken when UE is in connected in NR.

reportAmount:

$ref: '#/components/schemas/ReportAmountMdt'

description: This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT or combined Immediate MDT and Trace. When present, this IE shall indicate the number of measurement reports that shall be taken for periodical reporting while UE is in connected

eventThresholdRsrp:

type: integer

minimum: 0

maximum: 97

description: This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in LTE. When present, this IE shall indicate the Event Threshold for RSRP, and the value shall be between 0-97.

eventThresholdRsrpNr:

type: integer

minimum: 0

maximum: 127

description: This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in NR.When present, this IE shall indicate the Event Threshold for RSRP, and the value shall be between 0-127.

eventThresholdRsrq:

type: integer

minimum: 0

maximum: 34

description: This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in LTE.When present, this IE shall indicate the Event Threshold for RSRQ, and the value shall be between 0-34.

eventThresholdRsrqNr:

type: integer

minimum: 0

maximum: 127

description: This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in NR.When present, this IE shall indicate the Event Threshold for RSRQ, and the value shall be between 0-127.

eventList:

type: array

items:

$ref: '#/components/schemas/EventForMdt'

minItems: 1

description: This IE shall be present for event triggered measurement in the case of logged MDT.When present, this IE shall contain a list of events triggered measurement in NR.

loggingInterval:

$ref: '#/components/schemas/LoggingIntervalMdt'

description: This IE shall be present if the job type is configured for Logged MDT or Logged MBSFN MDT in LTE.When present, this IE shall contain the periodicity for logging MDT measurement results for periodic downlink pilot strength measurement in LTE when UE is in Idle.

loggingIntervalNr:

$ref: '#/components/schemas/LoggingIntervalNrMdt'

description: This IE shall be present if the job type is configured for Logged MDT or Logged MBSFN MDT in NR.When present, this IE shall contain the periodicity for logging MDT measurement results for periodic downlink pilot strength measurement in NR when UE is in Idle

loggingDuration:

$ref: '#/components/schemas/LoggingDurationMdt'

description: This IE shall be present if the job type parameter is configured for Logged MDT or Logged MBSFN MDT. When present, this IE shall indicate the validity time of MDT logged configuration for IDLE in LTE

loggingDurationNr:

$ref: '#/components/schemas/LoggingDurationNrMdt'

description: This IE shall be present if the job type parameter is configured for Logged MDT or Logged MBSFN MDT.When present, this IE shall indicate the validity time of MDT logged configuration for IDLE in NR.

positioningMethod:

$ref: '#/components/schemas/PositioningMethodMdt'

description: This IE may be present if the job type is set to Immediate MDT or Immediate MDT and Trace.When present, it shall indicate the positioning method that shall be used for the MDT job. For LTE the value "GNSS" may be selected only if the M1 measurement is selected in measurementList.

addPositioningMethodList:

type: array

items:

$ref: '#/components/schemas/PositioningMethodMdt'

minItems: 1

description: This IE may be present if positioningMethod is present. When present, it shall indicate a list of the additional positioning methods that shall be used for the MDT job.  For LTE, the value "GNSS" may be selected only if the M1 measurement is selected in measurementList.

collectionPeriodRmmLte:

$ref: '#/components/schemas/CollectionPeriodRmmLteMdt'

description: This IE shall be present if the job type is set to Immediate MDT or Immediate MDT and Trace and any of the "M4" or "M5" is contained in measurementList attribute in NR. When present, it shall contain the collection period that should be used to collect available measurement samples in case of RRM configured measurements. The same collection period should be used for all such measurements that are requested in the same MDT or combined Trace and MDT job.

collectionPeriodRmmNr:

$ref: '#/components/schemas/CollectionPeriodRmmNrMdt'

description: This IE shall be present if the job type is set to Immediate MDT or Immediate MDT and Trace and any of the "M4" or "M5" is contained in measurementList attribute in NR. When present, it shall contain the collection period that should be used to collect available measurement samples in case of RRM configured measurements. The same collection period should be used for all such measurements that are requested in the same MDT or combined Trace and MDT job.

measurementPeriodLte:

$ref: '#/components/schemas/MeasurementPeriodLteMdt'

description: This IE shall be present if the job type is set to Immediate MDT or Immediate MDT and Trace and either the value "M4_DL" or "M4_UL" or "M5_DL" or "M5_UL" is contained in measurementList attribute in LTE. When present, it shall contain the collection period that should be used for the Data Volume and Scheduled IP Throughput measurements made by the eNB. The same measurement period should be used for the UL and DL

mdtAllowedPlmnIdList:

type: array

items:

$ref: '#/components/schemas/PlmnId'

minItems: 1

maxItems: 16

description: When present, this IE shall contain the PLMNs where measurement collection, status indication and log reporting is allowed. E.g. the UE performs these actions for Logged MDT when the RPLMN is part of this set of PLMNs.

mbsfnAreaList:

type: array

items:

$ref: '#/components/schemas/MbsfnArea'

minItems: 1

maxItems: 8

description: When present, this IE shall contain MBSFN Area(s) for MBSFN measurement logging.

interFreqTargetList:

type: array

items:

$ref: '#/components/schemas/InterFreqTargetInfo'

minItems: 1

maxItems: 8

description: When present, this IE shall indicate Inter Frequency Target(s) for which the UE is requested to perform measurement logging.

AreaScope:

type: object

properties:

eutraCellIdList:

type: array

items:

$ref: '#/components/schemas/EutraCellId'

minItems: 1

description: When present, this IE shall contain a list of the E-UTRAN Cell Identifications where the MDT data collection shall take place.

nrCellIdList:

type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

description: When present, this IE shall contain a list of the NR Cell Identities where the MDT data collection shall take place.

tacList:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

description: When present, this IE shall contain a list of the tracking area codes where the MDT data collection shall take place

tacInfoPerPlmn:

type: object

additionalProperties:

$ref: '#/components/schemas/TacInfo'

minProperties: 1

description: A map (list of key-value pairs) where PlmnId converted to a string serves as key

TacInfo:

type: object

required:

- tacList

properties:

tacList:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

description: This IE shall contain a list of the tracking area codes.

MbsfnArea:

type: object

properties:

mbsfnAreaId:

type: integer

minimum: 0

maximum: 255

description: This IE shall contain the MBSFN Area ID.

carrierFrequency:

type: integer

minimum: 0

maximum: 262143

description: When present, this IE shall contain the Carrier Frequency (EARFCN).

InterFreqTargetInfo:

required:

- dlCarrierFreq

type: object

properties:

dlCarrierFreq:

$ref: '#/components/schemas/ArfcnValueNR'

description: This IE shall indicate the value of frequency for download for measurement logging.

cellIdList:

type: array

items:

$ref: '#/components/schemas/PhysCellId'

minItems: 1

maxItems: 32

description: When present, this IE shall contain a list of the physical cell identities where the UE is requested to perform measurement logging for the indicated frequency.

# Data Types related to 5G ODB as defined in clause 5.7

#

# SIMPLE DATA TYPES

#

#

#

# Enumerations

#

RoamingOdb:

anyOf:

- type: string

enum:

- OUTSIDE_HOME_PLMN

- OUTSIDE_HOME_PLMN_COUNTRY

- type: string

description: The enumeration RoamingOdb defines the Barring of Roaming as. See 3GPP TS 23.015 for further description. It shall comply with the provisions defined in table 5.7.3.1-1.

OdbPacketServices:

anyOf:

- anyOf:

- type: string

enum:

- ALL_PACKET_SERVICES

- ROAMER_ACCESS_HPLMN_AP

- ROAMER_ACCESS_VPLMN_AP

- type: string

- $ref: '#/components/schemas/NullValue'

description: The enumeration OdbPacketServices defines the Barring of Packet Oriented Services. See 3GPP TS 23.015 for further description. It shall comply with the provisions defined in table 5.7.3.2-1

#

# STRUCTURED DATA TYPES

#

OdbData:

type: object

properties:

roamingOdb:

$ref: '#/components/schemas/RoamingOdb'

description: Barring of Roaming.

#

# Data Types related to Charging as defined in clause 5.8

#

#

# SIMPLE DATA TYPES

#

#

ChargingId:

$ref: '#/components/schemas/Uint32'

description: Charging identifier allowing correlation of charging information.

ApplicationChargingId:

type: string

description: Application provided charging identifier allowing correlation of charging information.

RatingGroup:

$ref: '#/components/schemas/Uint32'

description: Identifier of a Rating Group.

ServiceId:

$ref: '#/components/schemas/Uint32'

description: Identifier of a Service.

#

# Enumerations

#

#

# STRUCTURED DATA TYPES

#

SecondaryRatUsageReport:

type: object

properties:

secondaryRatType:

$ref: '#/components/schemas/RatType'

description: Secondary RAT type.

qosFlowsUsageData:

type: array

items:

$ref: '#/components/schemas/QosFlowUsageReport'

minItems: 1

description: QoS flows usage data.

required:

- secondaryRatType

- qosFlowsUsageData

QosFlowUsageReport:

type: object

properties:

qfi:

$ref: '#/components/schemas/Qfi'

description: QoS Flow Indicator.

startTimeStamp:

$ref: '#/components/schemas/DateTime'

description: UTC time indicating the start time of the collection period of the included usage data for DL and UL.

endTimeStamp:

$ref: '#/components/schemas/DateTime'

description: UTC time indicating the end time of the collection period of the included usage data for DL and UL.

downlinkVolume:

$ref: '#/components/schemas/Int64'

description: Data usage for DL, encoding a number of octets.

uplinkVolume:

$ref: '#/components/schemas/Int64'

description: Data usage for UL, encoding a number of octets.

required:

- qfi

- startTimeStamp

- endTimeStamp

- downlinkVolume

- uplinkVolume

SecondaryRatUsageInfo:

type: object

properties:

secondaryRatType:

$ref: '#/components/schemas/RatType'

description: Secondary RAT type.

qosFlowsUsageData:

type: array

items:

$ref: '#/components/schemas/QosFlowUsageReport'

minItems: 1

description: QoS flows usage data.

pduSessionUsageData:

type: array

items:

$ref: '#/components/schemas/VolumeTimedReport'

minItems: 1

description: PDU session usage data.

required:

- secondaryRatType

VolumeTimedReport:

type: object

properties:

startTimeStamp:

$ref: '#/components/schemas/DateTime'

description: UTC time indicating the start time of the collection period of the included usage data for DL and UL.

endTimeStamp:

$ref: '#/components/schemas/DateTime'

description: UTC time indicating the end time of the collection period of the included usage data for DL and UL.

downlinkVolume:

$ref: '#/components/schemas/Int64'

description: Data usage for DL, encoding a number of octets.

uplinkVolume:

$ref: '#/components/schemas/Int64'

description: Data usage for UL, encoding a number of octets.

required:

- startTimeStamp

- endTimeStamp

- downlinkVolume

- uplinkVolume

#

# HTTP responses

#

responses:

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'401':

description: Unauthorized

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'405':

description: Method Not Allowed

'408':

description: Request Timeout

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'406':

description: 406 Not Acceptable

'409':

description: Conflict

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'410':

description: Gone

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'411':

description: Length Required

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'412':

description: Precondition Failed

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'413':

description: Payload Too Large

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'414':

description: URI Too Long

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'415':

description: Unsupported Media Type

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'429':

description: Too Many Requests

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'501':

description: Not Implemented

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'504':

description: Gateway Timeout

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

default:

description: Generic Error

* * * End of Changes * * * *