3GPP TSG-CT Meeting #92e	CP-211106

E-Meeting, 14th – 16th June 2021	was C4-213322

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0359 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscribed EE profile data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Authorizing MTC Provider information and/or AF ID during requesting Subscription to Notification of event occurrence service operation of Nudm_EE Service was introduced in the 29.503 from R16, this subscribed authorizion profiles for provisioning parameters should be stored in the UDR for the stateless UDMs from R16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add allowedMtcProvider to EeProfileData |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Monitoring (with a given Event Type) cannot be authorized per MTC provider. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2.20, 5.4.2.xx (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the Nudr_DataRepository API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Corrected nested cardinality for complying with the new agreed proposal. |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type |  | Clause defined |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| AuthenticationSubscription |  | 5.4.2.2 |  | A UE's authentication data |  |
| OperatorSpecificDataContainer |  | 5.4.2.3 |  | Container for operator specific data |  |
| SmfRegList |  | 5.4.2.4 |  | The list of all the SMF registrations of a UE |  |
| SubscriptionDataSubscriptions |  | 5.4.2.5 |  | A subscription to notifications. |  |
| DataChangeNotify |  | 5.4.2.6 |  | Container for data which have changed and notification was requested when changed. |  |
| IdentityData |  | 5.4.2.7 |  | Identity data corresponds to the provided ueId |  |
| ProvisionedDataSets |  | 5.4.2.8 |  |  |  |
| SorData |  | 5.4.2.9 |  | Used to store the status of the latest SOR data update |  |
| UpuData |  | 5.4.2.9A |  | Used to store the status of the latest UPU data update |  |
| NssaiAckData |  | 5.4.2.9B |  | Used to store the status of the latest NSSAI data update |  |
| CagAckData |  | 5.4.2.9C |  | Used to store the status of the latest CAG data update |  |
| AmfSubscriptionInfo |  | 5.4.2.19 |  | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |  |
| EeProfileData |  | 5.4.2.20 |  | Event Exposure Profile Data |  |
| ContextDataSets |  | 5.4.2.22 |  |  |  |
| SequenceNumber |  | 5.4.2.23 |  |  |  |
| MessageWaitingData |  | 5.4.2.24 |  |  |  |
| SmscData |  | 5.4.2.25 |  |  |  |
| SmfSubscriptionInfo |  | 5.4.2.26 |  | Information the UDR stores and retrieves related to active subscriptions at the SMF(s) |  |
| SmfSubscriptionItem |  | 5.4.2.27 |  | Contains info about a single SMF event subscription |  |
| AuthMethod |  | 5.4.3.3 |  |  |  |
| DataSetName |  | 5.4.3.4 |  |  |  |
| ContextDataSetName |  | 5.4.3.6 |  |  |  |
| SqnScheme |  | 5.4.3.7 |  |  |  |
| Sign |  | 5.4.3.8 |  |  |  |
| UeUpdateStatus |  | 5.4.3.9 |  |  |  |
| MtcProvider |  | 5.4.2.xx |  |  |  |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| ` | Reference | Comments |
| --- | --- | --- |
| AccessAndMobilitySubscriptionData | 3GPP TS 29.503 [6] | Access and Mobility Subscription Data |
| SmfSelectionSubscriptionData | 3GPP TS 29.503 [6] | SMF Selection Subscription Data |
| SessionManagementSubscriptionData | 3GPP TS 29.503 [6] | Session Management Subscription Data |
| Amf3GppAccessRegistration | 3GPP TS 29.503 [6] |  |
| AmfNon3GppAccessRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |
| SmfRegistration | 3GPP TS 29.503 [6] |  |
| SmsfRegistration | 3GPP TS 29.503 [6] |  |
| SmsManagementSubscriptionData | 3GPP TS 29.503 [6] |  |
| SupportedFeatures | 3GPP TS 29.571 [3] | Used to negotiate the applicability of the optional features |
| ProblemDetails | 3GPP TS 29.571 [3] | Detailed information about the status code. |
| PpData | 3GPP TS 29.503 [6] |  |
| SmsSubscriptionData | 3GPP TS 29.503 [6] |  |
| PatchItem | 3GPP TS 29.571 [3] | Data structure used for JSON patch. |
| SdmSubscription | 3GPP TS 29.503 [6] |  |
| EeSubscription | 3GPP TS 29.503 [6] |  |
| Uri | 3GPP TS 29.571 [3] |  |
| TraceData | 3GPP TS 29.571 [3] |  |
| Dnn | 3GPP TS 29.571 [3] | Data Network Name |
| Snssai | 3GPP TS 29.571 [3] | Single NSSAI |
| VarUeId | 3GPP TS 29.571 [3] | String represents the SUPI or GPSI. |
| NfInstanceId | 3GPP TS 29.571 [3] |  |
| NotifyItem | 3GPP TS 29.571 [3] | Common data type used for data change notification. |
| OdbData | 3GPP TS 29.571 [3] | Operator Determined Barring Data |
| EventType | 3GPP TS 29.503 [6] |  |
| ExtGroupId | 3GPP TS 29.503 [6] | External Group Identifier |
| PduSessionId | 3GPP TS 29.571 [3] |  |
| DateTime | 3GPP TS 29.571 [3] |  |
| UpuMac | 3GPP TS 29.509 [15] |  |
| 5GVnGroupConfiguration | 3GPP TS 29.503 [6] |  |
| PatchResult | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] |  |
| AppPortId | 3GPP TS 29.503 [6] |  |
| LcsPrivacyData | 3GPP TS 29.503 [6] | LCS Privacy Subscription Data |
| LcsMoData | 3GPP TS 29.503 [6] | LCS Mobile Originated Subscription Data |
| MtcProviderInformation | 3GPP TS 29.571 [3] |  |
| AuthorizationData | 3GPP TS 29.503 [6] |  |
| EnhancedCoverageRestrictionData | 3GPP TS 29.503 [6] |  |
| LocationInfo | 3GPP TS 29.503 [6] |  |
| V2xSubscriptionData | 3GPP TS 29.503 [6] | V2X Subscription Data |
| E164Number | 3GPP TS 29.503 [6] |  |
| NetworkNodeDiameterAddress | 3GPP TS 29.503 [6] |  |
| IpSmGwRegistration | 3GPP TS 29.503 [6] |  |
| LcsBroadcastAssistanceTypesData | 3GPP TS 29.503 [6] | LCS Broadcast Assistance Data Types |

* * * Next Change * * * *

5.4.2.20	Type: EeProfileData

# Table 5.4.2.20-1: Definition of type EeProfileData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| restrictedEventTypes | array(EventType) | O | 1..N | List of the event types that are restricted (if any) for the UE. The absence of this IE indicates that all event types are authorized for the UE. |
| supportedFeatures | SupportedFeatures | O | 0..1 |  |
| allowedMtcProvider | map(array(MtcProvider)) | O | 1..N(1..M) | A map (list of key-value pairs where EventType (see 3GPP TS 29.503 [6] serves as key) of MTC Provider. In addition to defined EventTypes, the key value "ALL" may be used to identify a map entry which contains a list of MtcProviders that are allowed monitoring all Event Types. The absence of this IE indicates that monitoring by any MTC provider is allowed for any non restricted event type. |

* * * Next Change * * * *

5.4.2.xx	Type: MtcProvider

# Table 5.4.2.xx-1: Definition of type MtcProvider

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mtcProviderInformation | MtcProviderInformation | O | 0..1 | Indicates MTC provider information for Monitoring Configuration authorization. |
| afId | string | O | 0..1 | ID of the AF. The string identifying the originating AF, formatted as described in the definition of type MonitoringConfiguration in 3GPP TS 29.503 [6]. |

* * * Next Change * * * *

A.2	Nudr_DataRepository API for Subscription Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29505_Subscription_Data.yaml".

**************text skipped for clarity******************

EeProfileData:

type: object

properties:

restrictedEventTypes:

type: array

items:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/EventType'

supportedFeatures:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

# allowedMtcProvider:

# description: A map (list of key-value pairs where EventType serves as key) of MTC provider lists. In addition to defined EventTypes, the key value "ALL" may be used to identify a map entry which contains a list of MtcProviders that are allowed monitoring all Event Types.

# type: object

# additionalProperties:

# type: array

# items:

# $ref: '#/components/schemas/MtcProvider'

# minItems: 1

# minProperties: 1

# MtcProvider:

# type: object

# properties:

# mtcProviderInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MtcProviderInformation'

# afId:

# type: string

**************text skipped for clarity******************

* * * End of Changes * * * *