3GPP TSG-CT WG1 Meeting #130-e	C1-213633

Electronic meeting, 20-28 May 2021	(was C1-213602)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.483 | CR | 0102 | rev | 2 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct MCVideo MOs R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, Airbus, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCImp-MCVIDEO-CT |  |  |  |  |  | Date: |  |  | 20 May 2021 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | Major discrepancies in the MCVideo user profile exist between its usage in TS 24.281 and its definition in TS 24.484. These discrepancies are being fixed in TS 24.484 CR0180. The Managed Objects (MOs) changes corresponding to those fixes need to be reflected in TS 24.483. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clean up the MCVideo user profile MOs. This CR specifically assures the addition of the private call parameters, but also removes unused MOs and adds other missing MOs. - The following unused MOs were deleted: AuthorisedAlias, AllowedModifyVideo, AllowedRenegotiateCodec, AllowedCameraControl, AllowedRemoteControl, AllowedDisplayRemoteUE, AllowedRemoteCamera, AllowedPushVideo, AllowedAutoSendNotify, NotifyList, VideoCategories, OnNetwork/PresenceStatus, OnNetwork/RemoteGroupChange, OnNetwork/DeletionPeriod, MaxSimultaneousVideoStreams/AllowedUnlimited, OnNetwork/AllowedAutoRecv, OnNetwork/AllowedAutoRecvEmergency, OnNetwork/AllowedAutoRecvImminentPeril, MandatoryReceiveGroups, AllowedRequestOverride, AllowedSelectOverride, AllowedOverrideGroupCall, MaxTimeSingleTransmit, - The following MOs were added where missing and for alignment with MCPTT: Under COMMON: MCVideoGroupCall/EmergencyAlert, MCVideoGroupCall/EmergencyCall, MCVideoGroupCall/ImminentPerilCall, MCVideoGroupCall/Priority, PrivateCall/AuthorisedAny, PrivateCall/UserList, PrivateCall/EmergencyAlert, RemoteGroupSelection, PrivateCall/AutoAnswer, PrivateCall/AllowedMediaProtection, PrivateCall/EmergencyCall, MCVideoGroupCall/MaxSimultaneousCallsN6 Under OnNETWORK: KMSURIList, EnabledParticipation Under OffNETWORK: KMSURIList, - MaxAffiliationsNc2 was changed to MaxAffiliationsN2 - The range of MaxSimultaneousVideoStreams was changed from 0-65535 to 1-65535. - ReceptionPriority was declared in 13.2.38M but was not used in TS 24.281. Since 13.2.38M has been used in the revision of TS 24.483 v14.8.0, this element is deleted. - Leaf nodes for AllowedRemoteInitiatedAmbientViewing amd AllowedRemoteInitiatedAmbientViewing were added as subclauses 13.2.87A1 and 13.2.87A2. These are referenced from TS 24.484 but are not defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to implement MCVideo. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1, 13.2.5, 13.2.14, 13.2.21, 13.2.22, 13.2.23, 13.2.24, 13.2.25, 13.2.26, 13.2.27, 13.2.28, 13.2.31, 13.2.32, 13.2.33, 13.2.34, 13.2.35, 13.2.36, 13.2.37, 13.2.38, 13.2.38A1 (new), 13.2.38A2 (new), 13.2.38A3 (new), 13.2.38A4 (new), 13.2.38A5 (new), 13.2.38A6 (new), 13.2.38A7 (new), 13.2.38D1 (new), 13.2.38D2 (new), 13.2.38D3 (new), 13.2.38D4 (new), 13.2.38D5 (new), 13.2.38G1 (new), 13.2.38G2 (new), 13.2.38G3 (new), 13.2.38G4 (new), 13.2.38G5 (new), 13.2.38G6 (new), 13.2.38G7 (new), 13.2.38I1 (new), 13.2.38I2 (new), 13.2.38I3 (new), 13.2.38I4 (new), 13.2.38I5 (new), 13.2.38I6 (new), 13.2.38I7 (new), 13.2.38I8 (new), 13.2.38I9 (new), 13.2.38M, 13.2.38N (new), 13.2.38O (new), 13.2.38P (new), 13.2.38Q (new), 13.2.38R (new), 13.2.38S (new), 13.2.38T (new), 13.2.38U (new), 13.2.38V (new), 13.2.38W (new), 13.2.38X (new), 13.2.38Y (new), 13.2.47, 13.2.50, 13.2.50A (new), 13.2.50B (new), 13.2.50C (new), 13.2.50D (new), 13.2.59, 13.2.62, 13.2.63, 13.2.64, 13.2.65, 13.2.66, 13.2.67, 13.2.69, 13.2.72, 13.2.74, 13.2.75, 13.2.76, 13.2.77, 13.2.78, 13.2.79, 13.2.80, 13.2.81, 13.2.82, 13.2.83, 13.2.84, 13.2.85, 13.2.86, 13.2.87, 13.2.87A (new), 13.2.87.A1 (new), 13.2.87.A2 (new), 13.2.87.D (new), 13.2.87.E (new), 13.2.87.F (new), 13.2.87.G (new), 13.2.87.H (new), 13.2.87.I (new), 13.2.87.J (new), 13.2.87.K (new), 13.2.87.L (new), 13.2.87.M (new), 13.2.87.N (new), 132.97, 13.2.100, 13.2.100A (new), 13.2.100B (new), 13.2.100C (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.484 CR0180 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes in TS 24.484 CR0180 are tightly tied to the contents of this CR. Both should be approved together. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: - Added blank lines below the new tables per the drafting rules. - Moved PrivateCall/EmergencyALert and RemoteGroupSelection from te Common part of Figure 13.1.1 to the OnNetwork part of Figure 13.1.2. - Added a plus sign for the OnNetwork/RemoteGroupSelection to indicate OneOrMore. - In Figure 13.1.1, added a plus sign for: node "<x>" node "/<x>/<x>/Common/PrivateCall/UserList/<x>" node "Common" - Corrected the coversheet to remove two instances of GroupServerInfo. - In Figure 13.1.3, removed the plus sign from "<x>/OffNetwork/ MCVideoGroupList/<x>/Entry/KMSURIList/<x>" - In Figure 13.1.3, changed the designation of "interior node" to "leaf node" by removing the box around GMSAppServId, IdMSTokenEndPoint, and KMSURI. - In Figure 13.1.1 for node "<x>/<x>" changed the annotation for the second <x> from "<x>*" to "<x>+" to indicate that there has to be "OneOrMore" instead of "ZeroOrMore", and made a parallel change in 13.2.5 in the table to "OneOrMore". - Added a space in 13.2.38A7 in 'UsePreConfigured'and 'DedicatedGroup' - Removed an extra space in 13.2.38D1 in "for the group". - Removed an extra forward slash in 13.2.38G2 - Corrected several occurrences of "MCVideoVideo" to "MCVideo" - Clarified the wording under the table in 13.2.38I1. - In 13.2.38I2, removed an extra space from the node name and added "/Common" in the table name. - New clauses 13.2.38I10, 13.2.38I11, 13.2.38I12, 13.2.38I13, 13.2.38I14, 13.2.38I15, 13.2.38I16, 13.2.38I17, 13.2.38I18 and 13.2.38I19, that is, the PrivateCall/EmergencyAlert, the RemoteGroupSelection and their subsequent nodes are part of the OnNetwork node and should not have been included within the Common part. A new clause 13.2.87B /<x>/<x>/OnNetwork/PrivateCall was added and these clauses were moved after clause 13.2.87B and renumbered to 13.2.87C, 13.2.87D, … - Removed a space from the table name in 13.2.38S. - Removed an extra "p" from the table name in 13.2.38X. - Moved (MaxSimultaneousCallsN6) clause 13.2.38Y to 13.2.38G7 - In 13.2.50A, 50B and 50C changed all occurrences of "GroupServerInfo" to "MCVideoGroupList" - In 13.2.100A, added "Entry/" to the table name. - Removed a space from the node name in 13.2.100B. Rev 2: - Added a node in 13.2.50A for <x>/Entry/KMSURIList - Added "<x>/Entry/" in 13.2.50B through 50D - Marked 13.2.38M as Void and renumbered 13.2.38M through X as 13.2.38N through Y - 13.2.87B through L are renumbered as 13.2.87D through N to make 13.2.87B and C available for the existing subclauses with those numbers in Rel-17. - Updated Figure 13.1.1 with a plus sign for node "<x>". |  |  |  |  |  |  |  |  |

*  *  *  *  *  FIRST CHANGE  *  *  *  *  *

## 13.1	General

The MCVideo user profile configuration Management Object (MO) is used to configure the MCVideo Client behaviour for the on-network or off-network MCVideo Service. The MCVideo user profile configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCVideo-user-profile:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCVideo user profile MO.

The following nodes and leaf objects are possible under the MCVideo user profile node as described in figure 13.1.1, figure 13.1.2 and figure 13.1.3:

Figure 13.1.1: The MCVideo user profile MO (1 of 3)

Figure 13.1.2: The MCVideo user profile MO (2 of 3)

Figure 13.1.3: The MCVideo user profile MO (3 of 3)

*  *  *  *  *  NEXT CHANGE  *  *  *  *  *

### 13.2.5	/<x>/<x>

Table 13.2.5.1: /<x>/<x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation and off-network operation. |  |  |  |  |

*  *  *  *  *  NEXT CHANGE  *  *  *  *  *

### 13.2.14	Void

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.21	Void

### 13.2.22	Void

### 13.2.23	Void

### 13.2.24	Void

### 13.2.25	Void

### 13.2.26	Void

### 13.2.27	Void

### 13.2.28	Void

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.31	Void

### 13.2.32	Void

### 13.2.33	Void

### 13.2.34	Void

### 13.2.35	Void

### 13.2.36	Void

### 13.2.37	Void

### 13.2.38	Void

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.38A1	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert

Table 13.2.38A1.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert

| <x>/Common/MCVideoGroupCall/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo emergency alert policy. |  |  |  |  |

### 13.2.38A2	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Authorised

Table 13.2.38A2.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Authorised

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to activate an MCVideo emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo emergency alert.

When set to "false" the MCVideo user is not authorised to make an MCVideo emergency alert.

### 13.2.38A3	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Cancel

Table 13.2.38A3.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Cancel

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Cancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel an MCVideo emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to cancel an MCVideo emergency alert.

When set to "false" the MCVideo user is not authorised to cancel an MCVideo emergency alert.

### 13.2.38A4	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry

Table 13.2.338A4.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCVideo group of an MCVideo emergency alert. |  |  |  |  |

### 13.2.38A5	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/ID

Table 13.2.38A5.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/ID

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCVideo group used upon certain criteria on initiation of an MCVideo emergency alert. |  |  |  |  |

### 13.2.38A6	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/DisplayName

Table 13.2.38A6.1: /<x>/<x>/Common/ MCVideoGroupCall/EmergencyAlert/Entry/DisplayName

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the ID. |  |  |  |  |

### 13.2.38A7	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/Usage

Table 13.2.38A7.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/Usage

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo emergency alert uses the ID. |  |  |  |  |

The valid values are 'LocallyDetermined', 'UseCurrentlySelectedGroup', 'UsePreConfigured' and 'DedicatedGroup'.

When set to 'LocallyDetermined' then if the MCVideo user selects an MCVideo ID then use that MCVideo ID for an on-network MCVideo emergency alert, if the MCVideo user does not select a MCVideo ID then use the MCVideo ID identified by the ID in subclause 13.2.38A5 for an on-network MCVideo emergency alert.

When set to 'UseCurrentlySelectedGroup' then if the MCVideo user has currently selected an MCVideo group then use that MCVideo group for an on-network MCVideo emergency alert, if the MCVideo user does not have a currently selected MCVideo group then use the MCVideo group identified by the ID in subclause 13.2.38A5 for an MCVideo emergency alert.

When set to 'UsePreConfigured' then use the ID identified by the ID in subclause 13.2.38A5 for an on-network MCVideo emergency alert.

When set to 'DedicatedGroup' then use the MCVideo group identified by the ID in subclause 13.2.38A5 for an MCVideo emergency alert.

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.38D1	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation

Table 13.2.38D1.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group used on initiation of an MCVideo emergency group call. |  |  |  |  |

### 13.2.38D2	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry

Table 13.2.38D2.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the group used on initiation of an MCVideo emergency group call. |  |  |  |  |

### 13.2.38D3	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/GroupID

Table 13.2.38D3.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/GroupID

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/GroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the group used upon certain criteria on initiation of an MCVideo emergency group call. |  |  |  |  |

### 13.2.38D4	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/
MCVideoGroupInitiation/Entry/DisplayName

Table 13.2.38D4.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/DisplayName

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the Group ID. |  |  |  |  |

### 13.2.38D5	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/Usage

Table 13.2.38D5.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/Usage

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo emergency group call uses the GroupID. |  |  |  |  |

The valid values are 'UseCurrentlySelectedGroup' and 'DedicatedGroup'.

When set to 'UseCurrentlySelectedGroup' then if the MCVideo user has currently selected an MCVideo group then use that MCVideo group for an on-network MCVideo emergency group call, if the MCVideo user does not have a currently selected MCVideo group then use the MCVideo group identified by the GroupID in subclause 13.2.38D3 for an MCVideo emergency group call.

When set to 'DedicatedGroup' then use the MCVideo group identified by the GroupID in subclause 13.2.38D3 for an MCVideo emergency group call.

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.38G1	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation

Table 13.2.38G1.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group used on initiation of an MCVideo imminent peril group call. |  |  |  |  |

### 13.2.38G2	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry

Table 13.2.38G2.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the group used on initiation of an imminent peril call. |  |  |  |  |

### 13.2.38G3	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
MCVideoGroupInitiation/Entry/GroupID

Table 13.2.38G3.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/GroupID

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/GroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the group used upon certain criteria on initiation of an MCVideo imminent peril group call. |  |  |  |  |

### 13.2.38G4	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
MCVideoGroupInitiation/DisplayName

Table 13.2.38G4.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/DisplayName

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the Group ID. |  |  |  |  |

### 13.2.38G5	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
MCVideoGroupInitiation/Entry/Usage

Table 13.2.38G5.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/Usage

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo imminent peril group call uses the GroupID. |  |  |  |  |

The valid values are 'UseCurrentlySelectedGroup' and 'DedicatedGroup'.

When set to 'UseCurrentlySelectedGroup' then if the MCVideo user has currently selected an MCVideo group then use that MCVideo group for an on-network MCVideo imminent peril group call, if the MCVideo user does not have a currently selected MCVideo group then use the MCVideo group identified by the GroupID in subclause 13.2.38G3 for an MCVideo imminent peril group call.

When set to 'DedicatedGroup' then use the MCVideo group identified by the GroupID in subclause 13.2.38G3 for an MCVideo imminent peril group call.

### 13.2.38G6	/<x>/<x>/Common/MCVideoGroupCall/Priority

Table 13.2.38G6.1: /<x>/<x>/Common/MCVideoGroupCall/Priority

| <x>/Common/MCVideoGroupCall/Priority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the priority of the MCVideo group calls. |  |  |  |  |

-	Values: 0-255

The MCVideo group call with the lowest Priority value shall be considered as the MCVideo group call having the lowest level among the MCVideo group calls.

### 13.2.38G7	/<x>/<x>/Common/MCVideoGroupCall/MaxSimultaneousCallsN6

Table 13.2.38G7.1: /<x>/<x>/Common/MCVideoGroupCall/MaxSimultaneousCallsN6

| <x>/Common/MCVideoGroupCall/MaxSimultaneousCallsN6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneously received MCVideo group calls (N6). |  |  |  |  |

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.38I1	/<x>/<x>/Common/PrivateCall/AuthorisedAny

Table 13.2.38I1.1: /<x>/<x>/Common/PrivateCall/AuthorisedAny

| <x>/Common/PrivateCall/AuthorisedAny |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCVideo private call to any MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo private call to any MCVideo user.

When set to "false" the MCVideo user is not authorised to make an MCVideo private call to any MCVideo user.

### 13.2.38I2	/<x>/<x>/Common/PrivateCall/UserList

Table 13.2.38I2.1: /<x>/<x>/Common/PrivateCall/UserList

| <x>/Common/PrivateCall/UserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCVideo user(s) who can be called in a MCVideo private call. |  |  |  |  |

### 13.2.38I3	/<x>/<x>/Common/PrivateCall/UserList/<x>

Table 13.2.38I3.1: /<x>/<x>/Common/PrivateCall/UserList/<x>

| <x>/Common/PrivateCall/UserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCVideo users who can be called in a MCVideo private call. |  |  |  |  |

### 13.2.38I4	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry

Table 13.2.38I4.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry

| <x>/Common/PrivateCall/UserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCVideo users who can be called in a private call. |  |  |  |  |

### 13.2.38I5	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID

Table 13.2.38I5.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID

| <x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.38I6	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DiscoveryGroupID

Table 13.2.38I6.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DiscoveryGroupID

| <x>/Common/PrivateCall/UserList/<x>/Entry/DiscoveryGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a discovery group ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The value is used as the discovery group ID in the ProSe discovery procedures as specified in 3GPP TS 23.303 [6].

### 13.2.38I7	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID

Table 13.2.38I7.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID

| <x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 13.2.38I8	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName

Table 13.2.38I8.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName

| <x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name. |  |  |  |  |

### 13.2.38I9	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/PrivateCallKMSURI

Table 13.2.38I9.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/PrivateCallKMSURI

| <x>/Common/PrivateCall/UserList/<x>/Entry/PrivateCallKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS associated with the MCVideoID. If the value is empty, the KMS leaf node present in the MCS UE initial configuration MO is used. |  |  |  |  |

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.38M	Void

### 13.2.38N	/<x>/<x>/Common/PrivateCall/AllowedMediaProtection

Table 13.2.38N.1: /<x>/<x>/Common/PrivateCall/AllowedMediaProtection

| <x>/Common/PrivateCall/AllowedMediaProtection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates authorisation to protect confidentiality and integrity of media for MCVideo private calls. |  |  |  |  |

When set to "true" the MCVideo user is authorised to protect confidentiality and integrity of media for MCVideo private calls.

When set to "false" the MCVideo user is not authorised to protect confidentiality and integrity of media for MCVideo private calls.

The default value is set to "true".

### 13.2.38O	/<x>/<x>/Common/PrivateCall/EmergencyCall

Table 13.2.38O.1: /<x>/<x>/Common/PrivateCall/EmergencyCall

| <x>/Common/PrivateCall/EmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo emergency call policy. |  |  |  |  |

### 13.2.38P	/<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised

Table 13.2.38P.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised

| <x>/Common/PrivateCall/EmergencyCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCVideo emergency private call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo emergency private call.

When set to "false" the MCVideo user is not authorised to make an MCVideo emergency private call.

### 13.2.38Q	/<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority

Table 13.2.38Q.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority

| <x>/Common/PrivateCall/EmergencyCall/CancelPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel emergency priority in an MCVideo emergency private call by an authorised MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to cancel an emergency priority in an MCVideo private call.

When set to "false" the MCVideo user is not authorised to cancel an emergency priority in an MPCTT private call.

### 13.2.38R	/<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient

Table 13.2.38R.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo private recipient for an MCVideo emergency private call. |  |  |  |  |

### 13.2.38S	/<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry

Table 13.2.38S.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCVideo private recipient for an MCVideo emergency private call. |  |  |  |  |

### 13.2.38T	/<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/ID

Table 13.2.38T.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/ID

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCVideo private recipient used upon certain criteria on initiation of an MCVideo emergency private call. |  |  |  |  |

### 13.2.38U	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient/Entry/DiscoveryGroupID

Table 13.2.38U.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DiscoveryGroupID

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DiscoveryGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the discovery group ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 13.2.38V	/<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/UserInfoID

Table 13.2.38V.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/UserInfoID

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The "UserInfoID" element identifies the MCVideo private recipient when making an off-network private call.

### 13.2.38W	/<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DisplayName

Table 13.2.38W.1: /<x>/<x>/Common/PrivatepCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DisplayName

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCVideo private recipient ID. |  |  |  |  |

### 13.2.38X	/<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/Usage

Table 13.2.38X.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/Usage

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo emergency private call uses the MCVideo private recipient ID. |  |  |  |  |

The valid values are 'LocallyDetermined' and 'UsePreConfigured'.

When set to 'LocallyDetermined' then if the MCVideo user selects an MCVideo ID then use that MCVideo ID for the MCVideo emergency private call, if the MCVideo user does not select a MCVideo ID then use the MCVideo ID identified by the MCVideo private recipient ID in subclause 13.2.38T for an on-network MCVideo emergency private call.

When set to 'UsePreConfigured' then use the MCVideo ID identified by the MCVideo private recipient ID in subclause 13.2.38T for an on-network MCVideo emergency private call.

When set to 'LocallyDetermined' then if the MCVideo user selects an MCVideo user then use the UserInfoID that corresponds to that MCVideo user for the MCVideo emergency private call, if the MCVideo user does not select a MCVideo user then use the User Info ID identified by the UserInfoID in subclause 13.2.38T for an off-network MCVideo emergency private call.

When set to 'UsePreConfigured' then use the User Info ID identified by the UserInfoID in subclause 13.2.38T for an off-network MCVideo emergency private call.

### 13.2.38Y	/<x>/<x>/Common/PrivateCall/AutoAnswer

Table 13.2.38Y.1: /<x>/<x>/Common/PrivateCall/AutoAnswer

| <x>/Common/PrivateCall/AutoAnswer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation of MCVideo user to force automatic answer for a MCVideo private call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to force automatic answer for a MCVideo private call.

When set to "false" the MCVideo user is not authorised to force automatic answer for a MCVideo private call.

*  *  *  *  *  NEXT CHANGE  *  *  *  *  *

### 13.2.47	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/GMSAppServList/<x>/GMSAppServId

Table 13.2.47.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/GMSAppServList/<x>/GMSAppServId

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/GMSAppServList/<x>/GMSAppServId |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the group management server hosting the MCVideo Group ID. |  |  |  |  |

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.50	/<x>/<x>/OnNetwork/MCVideoGroupList/
<x>/Entry/IdMSTokenEndPointList/<x>/IdMSTokenEndPoint

Table 13.2.50.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/IdMSTokenEndPointList/<x>/IdMSTokenEndPoint

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/IdMSTokenEndPointList/<x>/IdMSTokenEndPoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the identity management server hosting the MCVideo Group ID. |  |  |  |  |

### 13.2.50A	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList

Table 13.2.50A.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the list of KMS identities (URIs) for the groups contained in the MCVideoGroupList. |  |  |  |  |

### 13.2.50B	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>

Table 13.2.50B.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the KMS identity (URI) for a specific group contained in the MCVideoGroupList. |  |  |  |  |

### 13.2.50C	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/Entry

Table 13.2.50C.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/Entry

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the KMS identity (URI) for a specific group contained in the MCVideoGroupList. |  |  |  |  |

### 13.2.50D	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/Entry/KMSURI

Table 13.2.50D.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/Entry/KMSURI

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/Entry/KMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS identity (URI) for a specific group contained in the MCVideoGroupList. If the value is empty, the KMS identity (URI) (kms) present in the MCS UE initial configuration MO is used. |  |  |  |  |

*  *  *  *  *  NEXT CHANGE  *  *  *  *  *

### 13.2.59	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry

Table 13.2.59.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry

| <x>/OnNetwork/PresenceStatus/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCVideo IDs of MCVideo users that the configured MCVideo user is authorised to obtain presence status. |  |  |  |  |

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.62	Void

### 13.2.63	Void

### 13.2.64	Void

### 13.2.65	Void

### 13.2.66	Void

### 13.2.67	/<x>/<x>/OnNetwork/MaxAffiliationsN2

Table 13.2.67.1: /<x>/<x>/OnNetwork/MaxAffiliationsN2

| <x>/OnNetwork/MaxAffiliationsN2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of MCVideo groups that the MCVideo user is authorised to affiliate with. |  |  |  |  |

-	Values: 0-65535

*  *  *  *  *  NEXT CHANGE  *  *  *  *  *

### 13.2.69	/<x>/<x>/OnNetwork/AllowedPresenceStatus

Table 13.2.69.1: /<x>/<x>/OnNetwork/AllowedPresenceStatus

| <x>/OnNetwork/AllowedPresenceStatus |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the presence status on the network of this MCVideo user is available. |  |  |  |  |

When set to "true" the presence status on the network of this MCVideo user is available.

When set to "false" the presence status on the network of this MCVideo user is not available. This is the default if this leaf node is not present.

*  *  *  *  *  NEXT CHANGE  *  *  *  *  *

### 13.2.72	Void

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.74		/<x>/<x>/OnNetwork/MaxSimultaneousVideoStreams/MaxStreams

Table 13.2.74.1: /<x>/<x>/OnNetwork/MaxSimultaneousVideoStreams/MaxStreams

| <x>/OnNetwork/MaxSimultaneousVideoStreams/MaxStreams |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous video streams that can be received. |  |  |  |  |

-	Values: 1-65535

### 13.2.75	Void

### 13.2.76	Void

### 13.2.77	Void

### 13.2.78	Void

### 13.2.79	Void

### 13.2.80	Void

### 13.2.81	Void

### 13.2.82		Void

### 13.2.83	Void

### 13.2.84	Void

### 13.2.85	Void

### 13.2.86	Void

### 13.2.87	Void

### 13.2.87A	/<x>/<x>/OnNetwork/EnabledParticipation

Table 13.2.87A.1: /<x>/<x>/OnNetwork/EnabledParticipation

| <x>/OnNetwork/EnabledParticipation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is allowed to participate in MCVideo private calls that they are invited to. |  |  |  |  |

When set to "true" the MCVideo user is allowed to participate in MCVideo private calls that they are invited to.

When set to "false" the MCVideo user is not allowed to participate in MCVideo private calls that they are invited to.

### 13.2.87A1	/<x>/<x>/OnNetwork/AllowedRemoteInitiatedAmbientViewing

Table 13.2.87A1.1: /<x>/<x>/AllowedRemoteInitiatedAmbientViewing

| <x>/OnNetwork/AllowedRemoteInitiatedAmbientViewing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the presence status on the network of this MCVideo user is available. |  |  |  |  |

When set to "true" the MCVideo user is authorised to remotely initiate ambient viewing at another MCVideo user's device.

When set to "false" the MCVideo user is not authorised to remotely initiate ambient viewing at another MCVideo user's device.

### 13.2.87A2	/<x>/<x>/OnNetwork/AllowedLocallyInitiatedAmbientViewing

Table 13.2.87A2.1: /<x>/<x>/OnNetwork/AllowedLocallyInitiatedAmbientViewing

| <x>/OnNetwork/AllowedLocallyInitiatedAmbientViewing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the presence status on the network of this MCVideo user is available. |  |  |  |  |

When set to "true" the MCVideo user is authorised to locally initiate ambient viewing.

When set to "false" the MCVideo user is not authorised to locally initiate ambient viewing.

### 13.2.87D	/<x>/<x>/OnNetwork/PrivateCall

Table 13.2.87D.1: /<x>/<x>/OnNetwork/PrivateCall

| <x>/OnNetwork/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo private call configuration. |  |  |  |  |

### 13.2.87E	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert

Table 13.2.87E.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert

| <x>/OnNetwork/PrivateCall/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo private emergency alert policy. |  |  |  |  |

### 13.2.87F	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry

Table 13.2.87F.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of an MCVideo private emergency alert for on-network. |  |  |  |  |

### 13.2.87G	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID

Table 13.2.87G.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCVideo user ID used upon certain criteria on initiation of an MCVideo private emergency alert for on-network. |  |  |  |  |

### 13.2.87H	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/DisplayName

Table 13.2.87H.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/DisplayName

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the ID. |  |  |  |  |

### 13.2.87I	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage

Table 13.2.87I.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo private emergency alert uses the ID. |  |  |  |  |

The valid values are 'LocallyDetermined' and 'UsePreConfigured'.

When set to 'LocallyDetermined' then if the MCVideo user selects an MCVideo ID then use that MCVideo ID for an on-network MCVideo private emergency alert, if the MCVideo user does not select a MCVideo ID then use the MCVideo ID identified by the ID in subclause 13.2.38I12 for an on-network MCVideo private emergency alert.

When set to 'UsePreConfigured' then use the ID identified by the ID in subclause 13.2.38I12 for an on-network MCVideo private emergency alert.

### 13.2.87J	/<x>/<x>/OnNetwork/RemoteGroupSelection

Table 13.2.87J.1: /<x>/<x>/OnNetwork/RemoteGroupSelection

| <x>/OnNetwork/RemoteGroupSelection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the remote group selection configuration. |  |  |  |  |

### 13.2.87K	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>

Table 13.2.87K.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>

| <x>/OnNetwork/RemoteGroupSelection/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more remote group selection configuration elements. |  |  |  |  |

### 13.2.87L	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry

Table 13.2.87L.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCVideo users whose selected MCVideo group is allowed to be remotely changed by the MCVideo user. |  |  |  |  |

### 13.2.87M	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCVideoID

Table 13.2.87M.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCVideoID

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCVideoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a MCVideo ID of an MCVideo user whose selected MCVideo group is allowed to be remotely changed by the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.87N	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/DisplayName

Table 13.2.87N.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/DisplayName

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCVideo ID of the MCVideo user. |  |  |  |  |

*  *  *  *  *  NEXT CHANGE  *  *  *  *  *

### 13.2.97	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/GMSAppServList/<x>/GMSAppServId

Table 13.2.97.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/GMSAppServList/<x>/GMSAppServId

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/GMSAppServList/<x>/GMSAppServId |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the group management server hosting the MCVideo Group ID. |  |  |  |  |

*  *  *  *  *  NEXT CHANGES  *  *  *  *  *

### 13.2.100	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/
Entry/IdMSTokenEndPointList/<x>/IdMSTokenEndPoint

Table 13.2.100.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/IdMSTokenEndPointList/<x>/IdMSTokenEndPoint

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/IdMSTokenEndPointList/<x>/IdMSTokenEndPoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the identity management server hosting the MCVideo Group ID. |  |  |  |  |

### 13.2.100A	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList

Table 13.2.100A.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the list of KMS identities (URIs) for the groups contained in the off-network MCVideoGroupList. |  |  |  |  |

### 13.2.100B	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>

Table 13.2.100B.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the KMS identity (URI) for a specific group contained in the off-network MCVideoGroupList. |  |  |  |  |

### 13.2.100C	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/KMSURI

Table 13.2.100C.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/KMSURI

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/KMSURIList/<x>/KMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS identity (URI) for a specific group contained in the off-network MCVideoGroupList. If the value is empty, the KMS identity (URI) (kms) present in the MCS UE initial configuration MO is used. |  |  |  |  |

*  *  *  *  *  END CHANGES  *  *  *  *  *