3GPP TSG-CT WG1 Meeting #129-e	C1-212577

Electronic meeting, 19-23 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0195 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requested UE policies for 5G Prose |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE-requested 5PP provisioning procedure of 5G_ProSe has similar semantic as UE-requested V2X provisioning procedure - i.e to enable the UE to request some policies from PCF. However, to reuse the UE-requested V2X provisioning procedure for UE-requested 5PP provisioning procedure, the Requested UE policies IE needs to be extended with indications related to 5G Prose policies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Requested UE policies IE is extended with indications related to 5G Prose policies |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to reuse UE-requested V2X provisioning procedure for UE-requested 5PP provisioning procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: - indication for UE policies for 5G ProSe UE-to-UE relay is removed, it depends on SA2 decision Revision 2: - WI code on cover page corrected |  |  |  |  |  |  |  |  |

***** change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[3]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 24.386 "User Equipment (UE) to V2X control function; protocol aspects; Stage 3".

[6]	3GPP TS 24.501: "Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[7]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[10]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[11]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[12]	ETSI EN 302 636-3 v1.2.1: "Intelligent Transport Systems (ITS); Vehicular Communications; GeoNetworking; Part 3: Network Architecture".

[13]	IEEE 1609.3 2016: "IEEE Standard for Wireless Access in Vehicular Environments (WAVE) -- Networking Services".

[14]	IETF RFC 768: "User Datagram Protocol".

[15]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[16]	IETF RFC 4862: "Neighbor Discovery for IP version 6 (IPv6)".

[17]	ISO 29281-1 2013: "Intelligent transport systems -- Communication access for land mobiles (CALM) -- Non-IP networking -- Part 1: Fast networking & transport layer protocol (FNTP)".

[18]	ISO TS 17419 ITS-AID AssignedNumbers: http://standards.iso.org/iso/ts/17419/TS17419%20Assigned%20Numbers/TS17419_ITS-AID_AssignedNumbers.pdf

[19]	IETF RFC 1035: "DOMAIN NAMES - IMPLEMENTATION AND SPECIFICATION".

[20]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[21]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[22]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[23]	ISO/IEC 10118-3:2018: "IT Security techniques – Hash-functions – Part 3: Dedicated hash-functions".

[24]	CCSA YD/T 3707-2020: "Technical requirements of network layer of LTE-based vehicular communication".

[25]	IETF RFC 793: "Transmission Control Protocol."

[26]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[r24554]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

***** change *****

### 8.3.2	Requested UE policies

The purpose of the Requested UE policies information element is to enable the UE to request the PCF to provide certain UE policies or certain UE policy subsets.

The Requested UE policies information element is coded as shown in figure 8.3.2.1 and table 8.3.2.1.

The Requested UE policies is a type 4 information element with a minimum length of 3 octets and a maximum length of 4 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requested UE policies IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Requested UE policies contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 5PUNRI | 5PDCI | 5PDDI | V2XUUI | V2XPC5I | octet 3 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet 4* |

Figure 8.3.2.1: Requested UE policies information element

Table 8.3.2.1: Requested UE policies information element

| UE policies for V2X communication over PC5 indicator (V2XPC5I) (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | UE policies for V2X communication over PC5 not requested |
| 1 |  |  |  | UE policies for V2X communication over PC5 requested |
| UE policies for V2X communication over Uu indicator (V2XUUI) (octet 3, bit 2) |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | UE policies for V2X communication over Uu not requested |
| 1 |  |  |  | UE policies for V2X communication over Uu requested |
| UE policies for 5G ProSe direct discovery indicator (5PDDI) (octet 3, bit 3) (see NOTE) |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | UE policies for 5G ProSe direct discovery not requested |
| 1 |  |  |  | UE policies for 5G ProSe direct discovery requested |
| UE policies for 5G ProSe direct communications indicator (5PDCI) (octet 3, bit 4) (see NOTE) |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | UE policies for 5G ProSe direct communications not requested |
| 1 |  |  |  | UE policies for 5G ProSe direct communications requested |
| UE policies for 5G ProSe UE-to-network relay indicator (5PUNRI) (octet 3, bit 5) (see NOTE) |  |  |  |  |
| Bit |  |  |  |  |
| 5 |  |  |  |  |
| 0 |  |  |  | UE policies for 5G ProSe UE-to-network relay not requested |
| 1 |  |  |  | UE policies for 5G ProSe UE-to-network relay requested |
| Bit 6 to 8 of octet 3 and bits of octet 4 are spare and shall be coded as zero. |  |  |  |  |
| NOTE: Usage of this bit is not specified in the present specification and is specified in 3GPP TS 24.554 [r24554]. |  |  |  |  |