3GPP TSG-CT WG1 Meeting #130-e	C1-213835

Electronic meeting, 20-28 May 2021											was C1-213683, C1-213099

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 3219 | rev | 2 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE's handling of the indication of country of UE location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH-CT |  |  |  |  |  | Date: |  |  | 2021-05-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 23.502CR#2482 (S2-2101667) introduced the provision of an indication of country of UE location by the AMF to the UE in certain reject messages. So in 24.501, the handling of this received indication of country of UE location has to be specified. For instance, it has to be clear in CT1 NAS specifications if:- - if the indication of country of UE location is applcable only in NTN or in both TN and NTN; - if this indication of country of UE location is rceievd from AMF when not in NTN, how is it handled; - duration or length of time the received indication of country of UE location is valid This CR proposes - the indication of country of UE location is only applicable for use in NTN; - the indication of country of UE location is recived from a TN PLMN, it shall be disgarded; - the received the indication of country of UE location applies till UE successfully registers to another PLMN. The reasoning behind this proposal is that too early discard of the the indication of country of UE location can lead to the UE selecting back to the same NTN PLMN that provided the reject and the indication of country of UE location, even repeatedly. This will happen in following scenario:- UE is in NTN PLMN-A. There is only one NTN PLMN around but there are TN PLMNs. UE gets #79 with an the indication of country of UE location. UE does a search , and maybe even search in TN radio … UE find a TN PLMN but gets rejected with #11. Then UE has to search for another PLMN again. If the the indication of country of UE location was deleted upon the last search (that resulted in attempt to the TN PLMN), the next search by the UE will lead the UE to re-attempt the NTN PLMN-A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The indication of country of UE location:- - is only valid for NTN; - can only be received from NTN network, - when received valid for use till next successful initial registration to a NTN or when update by subsequent reject from a NTN network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unknown how to implement the use and handling of the received indication of country of UE location |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.XX	NAS over Non-Terrestrial Network

### 4.XX.1	General

### 4.XX.2	Handling of network's indication of country of UE location

The network provided indication of country of UE location is only applicable for a UE operating in satellite access mode.

The UE may receive an indication of country of UE location from the network in REGISTRATION REJECT, DEREGISTRATION REQUEST or SERVICE REJECT. If provided, the contents of the indication of country of UE location may be applied in proeedures described in 3GPP TS 23.122 [5].

If an indication of country of UE location is provided to the UE, that indication will be valid until the next successful initial registration to a PLMN through satellite access or when updated by the network.

If the UE receives an indication of country of UE location from a network not accessed through satellite access, the UE shall ignore the received indication.

Editor's note:	[5GSAT_ARCH-CT, CR#3219]. It is FFS if and how the HPLMN can influence the validity and use of the indication of country of UE location in the UE.

Editor's note:	[5GSAT_ARCH-CT, CR#3219]. The name and the encoding of the information element providing the country of the UE location is FFS.

* * * End of Changes * * * *