3GPP TSG-CT Meeting #92e	CP-211166

E-Meeting, 14th – 16th June 2021															(was C1-213968)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.183 | CR | 0075 | rev | 5 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new media feature tag g.3gpp.crs for indicating terminating UE support of the capabilities for CRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, China Mobile, China Telecom, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Incorporated, China Mobile, China Telecom, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2021-06-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CRS AS has no way to know whether the terminating UE supports the capability to play the CRS media ( i.e., play the received early media before terminating user answers the call) or not. If the CRS AS apply the CRS service using the gateway model for CRS, the CRS AS shall insert an Alert-info header field with an URN "urn:alert:service:crs" without a URL for CRS media in the initial INVITE request. On reception of 180 response to the initial INVITE request, the CRS AS may send an UPDATE request with SDP offer with a=content media-level attribute with a "g.3gpp.crs" value to update regular session media to CRS media. If terminating UE does not support the gateway model for CRS sercive, the terminating UE plays local ringing tone, and the UPDATE request will be recognized as a normal update of the regular session. Then the terminating UE sends a response with an SDP answer, and the response may be identified by the CRS AS as supporting or not supporting the CRS service. The signalling can be avoided, and the CRS AS can avoid applying the CRS media to the terminating UE when the response is identified as supporting the CRS service. To resolve this, it is propose to introduce a new g.3gpp.crs media feature tag. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A terminating UE shall indicate support of the gateway model for CRS in the 18x reponse by including a new g.3gpp.crs media feature tag with value "rs" in the Contact header field. The CRS AS that supports the gateway model for CRS service shall apply CRS media to the terminating UE based on the g.3gpp.crs media feature tag in the Contact header field in the 18x response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For the gateway model for CRS, needless signalings to apply the CRS media, and needless media operation applying the CRS media to the terminating UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.5.5.2.5.1, 4.5.5.3.6, 4.5.5.4.5, Annex X (New), X.1 (New), X.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 5 (CT#92-e): - Corrected revision counter in the CR coversheet |  |  |  |  |  |  |  |  |

# *** 1st change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[3]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[4]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[5]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[6]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Network Asserted Identity within Trusted Networks".

[7]	3GPP TS 24.623: "Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[8]	IETF draft-ietf-stir-passport-divert-08 (March  2020): "PASSporT Extension for Diverted Calls".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	OMA-TS-CPM_Message_Storage_Using_RESTFul_API-V1_0-20181025-D: "CPM Message Store using RESTFul API, Draft Version 1.0 – 25 Oct 2018",
http://member.openmobilealliance.org/ftp/Public_documents/COM/COM-CPM/Permanent_documents/OMA-TS-Message_Storage_Using_RESTFul_API-V1_0-20181025-D.zip.

[10]	OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C: "RESTful Network API for Network Message Storage, Candidate Version 1.0 – 28 May 2019",
http://member.openmobilealliance.org/ftp/Public_documents/ARCH/Permanent_documents/OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C.zip.

[11]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[12]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[13]	3GPP TS 24.175: "Management Object (MO) for Multi-Device and Multi-Identity in IMS; Stage 3".

[14]	3GPP TS 23.003: "Numbering, addressing and identification".

[x]	IETF RFC 3840: "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

# *** End of 1st change ***

# *** 2nd change ***

###### 4.5.5.2.5.1	General

If the UE supports playing CRS media using received early media in regular session, the UE shall follow the procedures specified in 3GPP TS 24.229 [3] for session termination, and additionally:

a)	the UE shall include a g.3gpp.crs media feature tag with value "rs" in a Contact header field when sending a SIP 18x response; and

b)	if the received initial INVITE request contains an Alert-Info header field with an URN "urn:alert:service:crs" and without a URL of CRS media, the UE shall play CRS media that is received from the network as ringing signal. The CRS media is identified by the a=content media-level attribute with a "g.3gpp.crs" value being in the SDP offer.

NOTE:	The UE plays local ringing tone if no UPDATE request with an SDP offer including a=content media-level attribute with a "g.3gpp.crs" value and no CRS media is received within a specific time.

# *** End of 2nd change ***

# *** 3rd change ***

##### 4.5.5.3.6	AS Actions for Gateway model

The AS performing the Gateway model shall follow the procedure as specified in RFC 3960 [10] and annex G in 3GPP TS 24.628 [11] with the additional procedures described in this subclause.

Upon receiving an initial INVITE request from the originating UE, the AS shall forward the initial INVITE request to the terminating UE with the following clarifications:

a)	insert an Alert-Info header field with an URN "urn:alert:service:crs"; and

b)	if no "precondition" option tag was received in the Supported header field of the incoming INVITE request, and if the AS uses precondition mechanism for providing CRS, add a "precondition" option tag to the Supported header field, insert precondition SDP parameters and indicate the status of local resource availability as specified in RFC 3312 [14].

Upon receiving the first reliable SIP 18x response to the initial INVITE request, and this 18x response contains a Contact header field parameter "+g.3gpp.crs" with value set to "rs", the AS:

a)	may contact the MRF to request CRS resource; and

b)	shall forward the reliable SIP 18x response to the originating UE. If no "precondition" option tag was included in the Supported header field of the INVITE request and if the precondition mechanism is used in the received 18x, before forwarding the 18x response, the AS shall remove the "precondition" option tag from the Require header field and remove precondition SDP parameters from the SDP answer.

Upon receiving the PRACK request of the first reliable 18x response from originating UE, the AS shall forward the PRACK request to the terminating UE and the AS may contact the MRF to request CRS resource, if it has not been previously requested and the g.3gpp.crs media feature tag with value "rs" was included in the Contact header field of the received 18x responses from the terminating UE.

When the video media feature tag is not included in the Contact header field of the previously received 18x response from the terminating UE and there is no video description in the SDP answer included in the 18x response, the AS shall not request video CRS resource from MRF, and shall not apply video CRS media to the terminating UE.

After receiving 180 (Ringing) response or receiving a SIP 200 (OK) response to the PRACK request of the first reliable SIP 18x response from terminating UE, when the g.3gpp.crs media feature tag with value "rs" was included in the Contact header field of the received 18x responses from the terminating UE, the AS may send the SDP offer of the CRS media in the UPDATE request as specified in RFC 3311 [12] with:

a)	P-Early-Media header field with a "sendrecv" value or a "sendonly" value;

b)	an SDP offer for the modification of the regular session, which is based on the CRS information received from the MRF and includes an a=content media-level attribute with a "g.3gpp.crs" value. The media types can include additional media types compared to the SDP answer of the previous 18x response from the terminating UE; and

c)	precondition mechanism as specified in RFC 3312 [14] if "precondition" option tag is included in the Require header field of a received 18x response.

When the g.3gpp.crs media feature tag with value "rs" was not included in the Contact header field of the previously received 18x responses from the terminating UE, the AS shall not request CRS resource from MRF, and shall not update regular session media to CRS media by UPDATE request towards terminating UE, and shall not apply CRS media to the terminating UE.

If the terminating UE requires the use of precondition mechanism, the AS shall not instruct the MRF to start applicable media for the CRS service before the terminating UE has indicated that preconditions are fulfilled. The point when the AS instruct the MRF to start applicable media for the CRS service is based on local policy.

Upon receiving a SIP 200 (OK) response to the INVITE request from the terminating UE, the AS shall instruct the MRF to stop media for the CRS service and update media for conversation. If the AS is going to update media with both originating side and terminating side, the AS shall:

a)	send an offerless re-INVITE request to the terminating side;

b)	upon receiving a SIP response to the re-INVITE request containing an SDP offer of the terminating side for the modification of the regular session, generate an UPDATE request as specified in RFC 3311 [12] to send an SDP offer for the modification of the regular session to the originating UE. The SDP offer shall only contain the media components which appeared both in the SDP offer contained in the SIP response to the re-INVITE request and the previously stored SDP offer in the initial INVITE request. The port number of the corresponding m-line shall be set to zero if it has been set to zero during previous SDP negotiation; and

c)	upon receiving a 200 (OK) response to the UPDATE request from the originating side, generate an SDP answer for the regular session to the terminating side, included in the ACK request associated with the re-INVITE request. The SDP answer shall be based on the SDP answer contained in the 200 (OK) response to the UPDATE request, and for the media components which do not appear in the SDP answer in the 200 (OK) response, set the port number of the corresponding m-line to zero.

Upon receiving a SIP 4xx, 5xx or 6xx response to the initial INVITE request from the terminating UE, the AS shall:

a)	instruct the MRF to stop the media for the CRS service; and

b)	forward the final response to the originating UE.

# *** End of 3rd change ***

# *** 4th change ***

##### 4.5.5.4.5	AS Actions for Gateway model

The procedures in subclause 4.5.5.3.6 shall apply with following additions:

Upon receiving the first SIP 18x response to the initial INVITE request, and this 18x response contains a Contact header field parameter "+g.3gpp.crs" with value set to "rs", if the received initial INVITE request contains an Alert-Info header field with an URN "urn:alert:service:crs" and without a URL of CRS media, the AS shall:

a)	decide which CRS service should have priority based on the operator policy and the called CRS service subscriber's preferences;

b)	if the CRS service provided by the AS serving the terminating UE has lower priority, then forward the 18x responses of the initial INVITE request and do not provide CRS service; and

c)	if the CRS service provided by the AS serving the terminating UE has higher priority, then forward the 18x responses of the initial INVITE request without the g.3gpp.crs media feature tag with value "rs" in the Contact header field of the 18x responses.

Upon receiving UPDATE request from originating side, if P-Early-Media header field with a "sendrecv" value or a "sendonly" value is included in the request, the AS shall:

a)	decide which CRS service should have priority based on the operator policy and the called CRS service subscriber's preferences;

b)	if the CRS service provided by the AS serving the terminating UE has lower priority, then forward the UPDATE request and do not provide CRS service; and

c)	if the CRS service provided by the AS serving the terminating UE has higher priority, then either:

1)	generate an SDP answer based on the received SDP offer, and set all port numbers of the media types to "0", then send the SDP answer to originating side in the 200 (OK) response to the UPDATE request;or

2)	send a 200 (OK) response to the UPDATE request to the originating side and include a P-Early-Media header field with an "inactive" value.

# *** End of 4th change ***

# *** 5th change ***

Annex X (normative):
Media feature tags defined within the current document

## X.1	General

This subclause describes the media feature tag definitions that are applicable for the CRS service.

## X.2	Definition of media feature tag g.3gpp.crs

Editor’s note [CR#0075, WI TEI17]: The media feature tag g.3gpp.crs should be registered immediately on approval of this specification. The ASN.1 Identifier will need to be updated once the IANA registration is completed.

Media feature-tag name: g.3gpp.crs

ASN.1 Identifier: 1.3.6.1.8.2.x

Summary of the media feature indicated by this tag: This media feature tag is for indicating terminating User Equipment (UE) support of the capabilities for the 3GPP Customized Ringing Signal (CRS) service.

Values appropriate for use with this feature tag: Token with an equality relationship. Values specified:

rs: The value is for indicating terminating UE supports the capability of playing CRS media (i.e., the received early media before the terminating user answers the call) as a ringing signal in regular session.

The feature tag is intended primarily for use in the following applications, protocols, services, or negotiation mechanisms: This feature tag is most useful in a communications application for describing the capabilities of a device, such as a phone.

Related standards or documents: 3GPP TS 24.183: IP Multimedia Subsystem (IMS) Customized Ringing Signal (CRS); Protocol specification.

Security Considerations: Security considerations for this media feature tag are discussed in subclause 11.1 of IETF RFC 3840.

# *** End of 5th change