3GPP TSG-CT WG4 Meeting #104-e	CP-211167

E-Meeting, 19th – 28th May 2021	was C4-213342

Source:	Huawei, CATT

Title:	Pseudo-CR on Data Model

Spec:	3GPP TS 29.555 v0.1.0

Agenda item:	6.2.6

Document for:	Decision

1. Introduction

This pCR provides the Data Model subclause of the TS.

2. Reason for Change

The Data Model subclause is needed, which is provided in this pCR.

3. Conclusions

Add the Data Model subclause.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.555 v0.1.0

* * * First Change * * * *

6.1.6	Data Model

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the N5g-ddnmf service based interface protocol.

Table 6.1.6.1-1: N5g-ddnmf specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AnnounceAuthReqData | 6.1.6.2.2 | Represents Data used to request the authorization to announce for a UE |  |
| AnnounceAuthRespData | 6.1.6.2.3 | Represents the obtained Announce Authorize Data for a UE |  |
| AnnounceDiscDataForOpen | 6.1.6.2.4 | Represents Data for open discovery used to request the authorization to announce for a UE |  |
| AnnounceDiscDataForRestricted | 6.1.6.2.5 | Represents Data for restricted discovery used to request the authorization to announce for a UE |  |
| AnnounceUpdateData | 6.1.6.2.6 | Represents Announce Authorize Data to update. |  |
| MonitorAuthReqData | 6.1.6.2.7 | Represents Data used to request the authorization to monitor for a UE |  |
| MonitorAuthRespData | 6.1.6.2.8 | Represents the obtained Monitor Authorize Data for a UE |  |
| MonitorDiscDataForOpen | 6.1.6.2.9 | Represents Data for open discovery used to request the authorization to monitor for a UE |  |
| MonitorDiscDataForRestricted | 6.1.6.2.10 | Represents Data for restricted discovery used to request the authorization to monitor for a UE |  |
| MonitorAuthDataForOpen | 6.1.6.2.11 | Represents the obtained Announce Authorize Data for open discovery for a UE |  |
| MonitorAuthDataForRestricted | 6.1.6.2.12 | Represents the obtained Announce Authorize Data for restricted discovery for a UE |  |
| MonitorUpdateData | 6.1.6.2.13 | Represents Monitor Authorize Data to update. |  |
| DiscoveryAuthReqData | 6.1.6.2.14 | Represents Data used to request the authorization for a discoverer UE |  |
| DiscoveryAuthRespData | 6.1.6.2.15 | Represents the obtained authorization Data for a discoverer UE. |  |
| DiscDataForRestricted | 6.1.6.2.16 | Represents Data for restricted discovery used to request the authorization for a discoverer UE |  |
| AuthDataForRestricted | 6.1.6.2.17 | Represents obtained authorization Data for restricted discovery for a discoverer UE |  |
| MatchReportReqData | 6.1.6.2.18 | Represents the Match Report information |  |
| MatchReportRespData | 6.1.6.2.19 | Represents Match Report Acknowledgement |  |
| MonitorUpdateResult | 6.1.6.2.20 | Represents the monitoring revocation results. |  |
| MatchInformation | 6.1.6.2.21 | Represents a report including a matching result, and the information that can be used for charging purpose |  |
| MatchInfoForOpen | 6.1.6.2.22 | Represents a report including a matching result, and the information that can be used for charging purpose for the open discovery type. |  |
| MatchInfoForRestricted | 6.1.6.2.23 | Represents a report including a matching result, and the information that can be used for charging purpose for the restricted discovery type. |  |
| DiscoveryType | 6.1.6.3.3 | Represents Discovery Type for ProSe Service |  |
| AuthorizationResult | 6.1.6.3.4 | Represents Authorization Result Type for ProSe Service |  |
| RevocationResult | 6.1.6.3.5 | Represents the monitoring Revocation Result for ProSe Service |  |

Table 6.1.6.1-2 specifies data types re-used by the N5g-ddnmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N<NF> service based interface.

Table 6.1.6.1-2: N5g-ddnmf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| VarUeId | 3GPP TS 29.571 [xy] | String represents the SUPI or GPSI. |  |
| DateTime | 3GPP TS 29.571 [xy] | DataTime |  |
| PlmnId | 3GPP TS 29.571 [xy] | Plmn ID |  |
| Supi | 3GPP TS 29.571 [xy] | SUPI |  |
| Uri | 3GPP TS 29.571 [xy] | URI |  |

6.1.6.2	Structured data types

6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.6.2.2	Type: AnnounceAuthReqData

Table 6.1.6.2.2-1: Definition of type AnnounceAuthReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. |  |
| openDiscData | AnnounceDiscDataForOpen | C | 0..1 | This IE shall contain the input data of announce authorisation for open discovery when present. If the value of discType is "OPEN", this IE shall be present. |  |
| restrictedDiscData | AnnounceDiscDataForRestricted | C | 0..1 | This IE shall contain the input data of announce authorisation for restricted discovery when present. If the value of discType is "RESTRICTED", this IE shall be present. |  |

6.1.6.2.3	Type: AnnounceAuthRespData

Table 6.1.6.2.3-1: Definition of type AnnounceAuthRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authResult | AuthorizationResult | M | 1 | This IE shall contain the authorization result. |  |

6.1.6.2.4	Type: AnnounceDiscDataForOpen

Table 6.1.6.2.4-1: Definition of type AnnounceDiscDataForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppId | ProseApplicationId | M | 1 | This IE shall contain the ProSe Application ID. |  |
| proseAppCode | ProseApplicationCode | M | 1 | This IE shall contain the ProSe Application Code |  |
| validityTimer | DateTime | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. |  |
| metaData | MetaData | O | 0..1 | This IE shall contain the metadata when present. |  |

6.1.6.2.5	Type: AnnounceDiscDataForRestricted

Table 6.1.6.2.5-1: Definition of type AnnounceDiscDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rpauid | Rpauid | M | 1 | This IE shall contain the RPAUID. |  |
| appId | ApplicationId | M | 1 | This IE shall contain the Application ID |  |
| proseRestrictedCode | ProseRestrictedCode | C | 0..1 | This IE shall contain the ProSe Restricted Code when present. (NOTE) |  |
| proseRestrictedPrefix | ProseRestrictedPrefix | C | 0..1 | This IE shall contain the ProSe Restricted Prefix when present. (NOTE) |  |
| metaData | MetaData | O | 0..1 | This IE shall contain the metadata when present. |  |
| codeSuffixPool | RestrictedCodeSuffixPool | O | 0..1 | This IE shall contain the Restricted Code Suffix pool when present. |  |
| NOTE: Either attribute proseRestrictedCode or attribute proseRestrictedPrefix shall be present. |  |  |  |  |  |

6.1.6.2.6	Type: AnnounceUpdateData

Table 6.1.6.2.6-1: Definition of type AnnounceUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service, only value "OPEN" is allowed. |  |
| validityTime | DateTime | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. If the value sets to a full zero (i.e., 0000-00-00T00:00:00), it indicates to revoke the authorization for the announcing in the PLMN. |  |
| proseAppCode | ProseApplicationCode | C | 0..1 | This IE shall contain the ProSe Application Code when present. If the ProSe Application Code is changed, this IE shall be present. If the value of attribute is a full zero, this IE shall be absent. |  |

6.1.6.2.7	Type: MonitorAuthReqData

Table 6.1.6.2.7-1: Definition of type MonitorAuthReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. |  |
| openDiscData | MonitorDiscDataForOpen | C | 0..1 | This IE shall contain the input data of monitor authorisation for open discovery when present. If the value of discType is "OPEN", this IE shall be present. |  |
| restrictedDiscData | MonitorDiscDataForRestricted | C | 0..1 | This IE shall contain the input data of monitor authorisation for restricted discovery when present. If the value of discType is "RESTRICTED", this IE shall be present. |  |

6.1.6.2.8	Type: MonitorAuthRespData

Table 6.1.6.2.8-1: Definition of type MonitorAuthRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authResult | AuthorizationResult | M | 1 | This IE shall contain the authorization result. |  |
| authDataOpen | MonitorAuthDataForOpen | C | 0..1 | This IE shall contain a list of the ProSe Application Codes when present. If the authResult is "OPEN", this IE shall be present. |  |
| authDataRestricted | MonitorAuthDataForRestricted | C | 0..1 | This IE shall contain the Prose Application Prefix when present. If the authResult is "OPEN", this IE shall be present. |  |

6.1.6.2.9	Type: MonitorDiscDataForOpen

Table 6.1.6.2.9-1: Definition of type MonitorDiscDataForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppIdNames | array(ProseApplicationIdName) | M | 1..N | This IE shall contain a list of ProSe Application ID names. |  |

6.1.6.2.10	Type: MonitorDiscDataForRestricted

Table 6.1.6.2.10-1: Definition of type MonitorDiscDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rpauid | Rpauid | M | 1 | This IE shall contain the RPAUID. |  |
| targetPduid | Pduid | M | 1 | This IE shall contain the Target PDUID. |  |
| appId | ApplicationId | M | 1 | This IE shall contain the Application ID |  |
| targetRpauid | Rpauid | M | 1 | This IE shall contain the Target RPAUID. |  |

6.1.6.2.11	Type: MonitorAuthDataForOpen

Table 6.1.6.2.11-1: Definition of type MonitorAuthDataForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppCodes | array(ProseApplicationCode) | C | 1..N | This IE shall contain a list of the ProSe Application Codes. (NOTE) |  |
| proseAppPrefix | ProseApplicationPrefix | C | 0..1 | This IE shall contain the Prose Application Prefix. (NOTE) |  |
| proseAppMasks | array(ProseApplicationMask) | M | 1..N | This IE shall contain a list of the ProSe Application Masks. |  |
| ttl | integer | M | 1 | This IE shall contain the TTL. If the value sets to zero, it indicates to revoke the authorization for the monitoring in the PLMN. |  |
| NOTE: Either attribute proseAppCodes or proseAppPrefix proseRestrictedPrefix shall be present if the discovery type is open. |  |  |  |  |  |

6.1.6.2.12	Type: MonitorAuthDataForRestricted

Table 6.1.6.2.12-1: Definition of type MonitorAuthDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseRestrictedCode | ProseRestrictedCode | M | 1 | This IE shall contain the ProSe Restricted Code. |  |
| validityTime | DateTime | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. |  |

6.1.6.2.13	Type: MonitorUpdateData

Table 6.1.6.2.13-1: Definition of type MonitorUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. |  |
| proseAppIdName | ProseApplicationIdName | M | 1 | This IE shall contain the ProSe Application ID Name. |  |
| ttl | integer | M | 1 | This IE shall contain the TTL. |  |
| monitorUpdateResultCallbackRef | Uri | O | 0..1 | A URI provided by 5G DDNMF to receive (implicitly subscribed) notifications on the monitoring revocation results. |  |

6.1.6.2.14	Type: DiscoveryAuthReqData

Table 6.1.6.2.14-1: Definition of type DiscoveryAuthReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. Only value "RESTRICTED" is allowed. |  |
| restrictedDiscData | DiscDataForRestricted | C | 0..1 | This IE shall contain the input data of authorisation for restricted discovery when present. If the value of discType is "RESTRICTED", this IE shall be present. |  |

6.1.6.2.15	Type: DiscoveryAuthRespData

Table 6.1.6.2.15-1: Definition of type DiscoveryAuthRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authResult | AuthorizationResult | M | 1 | This IE shall contain the authorization result. |  |
| authDataRestricted | AuthDataForRestricted | C | 0..1 | This IE shall contain the authorized data for restricted discovery when present. If the discovery type in request is "RESTRICTED", this IE shall be present. |  |

6.1.6.2.16	Type: DiscDataForRestricted

Table 6.1.6.2.16-1: Definition of type MonitorDiscDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rpauid | Rpauid | M | 1 | This IE shall contain the RPAUID. |  |
| targetPduid | Pduid | M | 1 | This IE shall contain the Target PDUID. |  |
| appId | ApplicationId | M | 1 | This IE shall contain the Application ID |  |
| targetRpauid | Rpauid | M | 1 | This IE shall contain the Target RPAUID. |  |

6.1.6.2.17	Type: AuthDataForRestricted

Table 6.1.6.2.17-1: Definition of type AuthDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseQueryCodes | array(ProseQueryCode) | M | 1..N | This IE shall contain a list of the authorised ProSe Restricted Code. |  |
| proseRespCode | ProseResponseCode | M | 1 | This IE shall contain the authorised ProSe Respond Code. |  |
| validityTime | DateTime | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. |  |

6.1.6.2.18	Type: MatchReportReqData

Table 6.1.6.2.18-1: Definition of type MatchReportReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. Only value "OPEN" is allowed. |  |
| proseAppCodes | array(ProseApplicationCode) | C | 1..N | This IE shall contain a list of the ProSe Application Codes when present. If the value of DiscoveryType is "OPEN", this IE shall be present. |  |
| moniteredPlmnId | PlmnId | C | 0..1 | This IE shall contain the Monitored PLMN ID when present. |  |

6.1.6.2.19	Type: MatchReportRespData

Table 6.1.6.2.19-1: Definition of type MatchReportRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppIdNames | array(ProseApplicationIdName) | C | 1..N | This IE shall contain a list of ProSe Application ID names when present. If the discovery type in the request is "OPEN", this IE shall be present. |  |
| validityTime | DateTime | C | 0..1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire when present. If the discovery type in the request is "OPEN", this IE shall be present. |  |
| metaData | MetaData | O | 0..1 | This IE shall contain the metadata when present. |  |
| metaDataIndexMasks | array(MetaDataIndexMask) | O | 1..N | This IE shall contain a list of the Meta Data Index Masks when present. |  |

6.1.6.2.20	Type: MonitorUpdateResult

Table 6.1.6.2.20-1: Definition of type MonitorUpdateResult

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. Only value "RESTRICTED" is allowed. |  |
| proseRestrictedCode | ProseRestrictedCode | M | 1 | This IE shall contain the ProSe Restricted Code. |  |
| appId | ApplicationId | M | 1 | This IE shall contain the Application ID |  |
| bannedRpauid | Rpauid | M | 1 | This IE shall contain the Banned RPAUID. |  |
| bannedPduid | Pduid | M | 1 | This IE shall contain the Banned PDUID. |  |
| revocationResult | RevocationResult | M | 1 | This IE shall contain the monitoring revocation results. |  |

6.1.6.2.21	Type: MatchInformation

Table 6.1.6.2.21-1: Definition of type MatchInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. |  |
| openMatchInfoForOpen | MatchInfoForOpen | C | 0..1 | This IE shall contain the match report information for open discovery type when present. If the value of discType is "OPEN", this IE shall be present. |  |
| restrictedMatchInfo | MatchInfoForRestricted | C | 0..1 | This IE shall contain the match report information for restricted discovery when present. If the value of discType is "RESTRICTED", this IE shall be present. |  |

6.1.6.2.22	Type: MatchInfoForOpen

Table 6.1.6.2.22-1: Definition of type MatchInfoForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | This IE shall contain the SUPI of UE. |  |
| appId | array(ApplicationId) | M | 1 | This IE shall contain a list of the Application IDs. |  |

6.1.6.2.23	Type: MatchInfoForRestricted

Table 6.1.6.2.23-1: Definition of type MatchInfoForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | This IE shall contain the SUPI of UE. |  |
| rpauid | Rpauid | M | 1 | This IE shall contain the RPAUID. |  |
| targetPduid | Pduid | M | 1 | This IE shall contain the Target PDUID. |  |
| proseRestrictedCode | ProseRestrictedCode | M | 1 | This IE shall contain the ProSe Restricted Code. |  |

6.1.6.3	Simple data types and enumerations

6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| DiscoveryEntryId |  | string |  | Discovery Entry ID |  |  |  |
| ProseApplicationId |  | string |  | ProSe Application ID |  |  |  |
| ProseApplicationCode |  | string |  | ProSe Application Code |  |  |  |
| Rpauid |  | string |  | RPAUID |  |  |  |
| ApplicationId |  | string |  | Application ID |  |  |  |
| ProseRestrictedCode |  | string |  | ProSe Restricted Code |  |  |  |
| ProseRestrictedPrefix |  | string |  | ProSe Restricted Prefix |  |  |  |
| MetaData | string | metadata |  |
| RestrictedCodeSuffixPool | string | Restricted Code Suffix pool |  |
| ProseApplicationIdName | string | ProSe Application ID name |  |
| Pduid | string | PDUID |  |
| ProseApplicationPrefix | string | Prose Application Prefix |  |
| ProseApplicationMask | string | Prose Application Mask |  |
| ProseQueryCode | string | ProSe Query Code |  |
| ProseResponseCode | string | ProSe Response Code |  |
| MetaDataIndexMask | string | Meta Data Index Mask |  |

6.1.6.3.3	Enumeration: DiscoveryType

The enumeration DiscoveryType represents Discovery Type for ProSe Service. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.3-1: Enumeration DiscoveryType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "OPEN" |  | Discovery type is "open". |  |  |  |
| "RESTRICTED" | Discovery type is "restricted". |  |

6.1.6.3.4	Enumeration: AuthorizationResult

The enumeration AuthorizationResult represents Authorization Result Type for ProSe Service. It shall comply with the provisions defined in table 6.1.5.3.4-1.

Table 6.1.6.3.4-1: Enumeration DiscoveryType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SUCCESSFUL" | The authorisation is successful. |  |

6.1.6.3.5	Enumeration: RevocationResult

The enumeration AuthorizationResult represents the monitoring Revocation Result for ProSe Service. It shall comply with the provisions defined in table 6.1.5.3.5-1.

Table 6.1.6.3.5-1: Enumeration RevocationResult

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SUCCESSFUL" | The Monitoring Revocation is successful. |  |
| "FAILED" | The Monitoring Revocation is failed. |  |

6.1.6.4	Data types describing alternative data types or combinations of data types

None.

6.1.6.5	Binary data

None.

* * * End of Changes * * * *