3GPP TSG-CT WG4 Meeting #104-e	CP-211168

E-Meeting, 19th – 28th May 2021 	was C4-213463, C4-213343

Source:	Huawei, CATT

Title:	Pseudo-CR on N5g-ddnmf_Discovery API

Spec:	3GPP TS 29.555 v0.1.0

Agenda item:	6.2.6

Document for:	Decision

1. Introduction

This pCR provides the N5g-ddnmf_Discovery API subclause of the TS.

2. Reason for Change

The N5g-ddnmf_Discovery API is needed, which is provided in this pCR.

3. Conclusions

Add the N5g-ddnmf_Discovery API.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.555 v0.1.0

* * * First Change * * * *

A.2	N5g-ddnmf_Discovery API

openapi: 3.0.0

info:

title: N5g-ddnmf_Discovery API

version: 1.0.0-alpha.1

description: |

N5g-ddnmf_Discovery Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.555 V0.1.0; 5G System; 5G Direct Discovery Name Management Services; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.555/

servers:

- url: '{apiRoot}/n5g-ddnmf-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- n5g-ddnmf-disc

paths:

/{ueId}/announce-authorize/{discEntryId}:

put:

summary: Obtain the authorization to announce for a UE

operationId: ObtainAnnounceAuth

tags:

- Obtain the authorization to announce for a UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AnnounceAuthReqData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/AnnounceAuthRespData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/announce-authorize/{discEntryId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: Update the authorization for announcing for a UE

operationId: UpdateAnnounceAuth

tags:

- Update the authorization for announcing for a UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AnnounceUpdateData'

required: true

responses:

'204':

description: Expected response to a valid request

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/monitor-authorize/{discEntryId}:

put:

summary: Obtain the authorization to monitor for a UE

operationId: ObtainMonitorAuth

tags:

- Obtain the authorization to monitor for a UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MonitorAuthReqData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/MonitorAuthRespData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/monitor-authorize/{discEntryId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: Update the authorization for monitoring for a UE

operationId: UpdateMonitorAuth

tags:

- Update the authorization for monitoring for a UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MonitorUpdateData'

required: true

responses:

'204':

description: Expected response to a valid request

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

callbacks:

MonitorUpdateResult:

'{request.body#/monitorUpdateResultCallbackRef}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MonitorUpdateResult'

responses:

'204':

description: Successful Notification response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/discovery-authorize/{discEntryId}:

put:

summary: Obtain the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery

operationId: ObtainDiscAuth

tags:

- Obtain the authorization for a discoverer UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveryAuthReqData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveryAuthRespData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/discovery-authorize/{discEntryId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/match-report:

post:

summary: Obtain the information about the indicated discovery code from the 5G DDNMF

operationId: MatchReport

tags:

- Obtain the information about the indicated discovery code

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MatchReportReqData'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/MatchReportRespData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

n5g-ddnmf-disc: Access to the N5g-ddnmf_Discovery API

schemas:

# COMPLEX TYPES:

AnnounceAuthReqData:

type: object

description: Represents Data used to request the authorization to announce for a UE

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

openDiscData:

$ref: '#/components/schemas/AnnounceDiscDataForOpen'

restrictedDiscData:

$ref: '#/components/schemas/AnnounceDiscDataForRestricted'

AnnounceAuthRespData:

type: object

description: Represents the obtained Announce Authorize Data for a UE

required:

- authResult

properties:

authResult:

$ref: '#/components/schemas/AuthorizationResult'

AnnounceDiscDataForOpen:

type: object

description: Represents Data for open discovery used to request the authorization to announce for a UE

required:

- proseAppId

- proseAppCode

- validityTime

properties:

proseAppId:

$ref: '#/components/schemas/ProseApplicationId'

proseAppCode:

$ref: '#/components/schemas/ProseApplicationCode'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

metaData:

$ref: '#/components/schemas/MetaData'

AnnounceDiscDataForRestricted:

type: object

description: Represents Data for restricted discovery used to request the authorization to announce for a UE

required:

- rpauid

- appId

properties:

rpauid:

$ref: '#/components/schemas/Rpauid'

appId:

$ref: '#/components/schemas/ApplicationId'

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

proseRestrictedPrefix:

$ref: '#/components/schemas/ProseRestrictedPrefix'

metaData:

$ref: '#/components/schemas/MetaData'

codeSuffixPool:

$ref: '#/components/schemas/RestrictedCodeSuffixPool'

AnnounceUpdateData:

type: object

description: Represents Announce Authorize Data to update

required:

- discType

- validityTime

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

proseAppCode:

$ref: '#/components/schemas/ProseApplicationCode'

MonitorAuthReqData:

type: object

description: Represents Data used to request the authorization to monitor for a UE

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

openDiscData:

$ref: '#/components/schemas/MonitorDiscDataForOpen'

restrictedDiscData:

$ref: '#/components/schemas/MonitorDiscDataForRestricted'

MonitorAuthRespData:

type: object

description: Represents the obtained Monitor Authorize Data for a UE

required:

- authResult

properties:

authResult:

$ref: '#/components/schemas/AuthorizationResult'

authDataOpen:

$ref: '#/components/schemas/MonitorAuthDataForOpen'

authDataRestricted:

$ref: '#/components/schemas/MonitorAuthDataForRestricted'

MonitorDiscDataForOpen:

type: object

description: Represents Data for open discovery used to request the authorization to monitor for a UE

required:

- proseAppIdNames

properties:

proseAppIdNames:

type: array

items:

$ref: '#/components/schemas/ProseApplicationIdName'

minItems: 1

MonitorDiscDataForRestricted:

type: object

description: Represents Data for restricted discovery used to request the authorization to monitor for a UE

required:

- rpauid

- targetPduid

- appId

- targetRpauid

properties:

rpauid:

$ref: '#/components/schemas/Rpauid'

targetPduid:

$ref: '#/components/schemas/Pduid'

appId:

$ref: '#/components/schemas/ApplicationId'

targetRpauid:

$ref: '#/components/schemas/Rpauid'

MonitorAuthDataForOpen:

type: object

description: Represents the obtained Announce Authorize Data for open discovery for a UE

required:

- proseAppMasks

- ttl

properties:

proseAppCodes:

type: array

items:

$ref: '#/components/schemas/ProseApplicationCode'

minItems: 1

proseAppPrefix:

$ref: '#/components/schemas/ProseApplicationPrefix'

proseAppMasks:

type: array

items:

$ref: '#/components/schemas/ProseApplicationMask'

minItems: 1

ttl:

type: integer

MonitorAuthDataForRestricted:

type: object

description: Represents the obtained Announce Authorize Data for restricted discovery for a UE

required:

- proseRestrictedCode

- validityTime

properties:

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

MonitorUpdateData:

type: object

description: Represents Monitor Authorize Data to update.

required:

- discType

- proseAppIdName

- ttl

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

proseAppIdName:

$ref: '#/components/schemas/ProseApplicationIdName'

ttl:

type: integer

monitorUpdateResultCallbackRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

DiscoveryAuthReqData:

type: object

description: Represents Data used to request the authorization for a discoverer UE.

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

restrictedDiscData:

$ref: '#/components/schemas/DiscDataForRestricted'

DiscoveryAuthRespData:

type: object

description: Represents the obtained authorization Data for a discoverer UE

required:

- authResult

properties:

authResult:

$ref: '#/components/schemas/AuthorizationResult'

authDataRestricted:

$ref: '#/components/schemas/AuthDataForRestricted'

DiscDataForRestricted:

type: object

description: Represents Data for restricted discovery used to request the authorization for a discoverer UE

required:

- rpauid

- targetPduid

- appId

- targetRpauid

properties:

rpauid:

$ref: '#/components/schemas/Rpauid'

targetPduid:

$ref: '#/components/schemas/Pduid'

appId:

$ref: '#/components/schemas/ApplicationId'

targetRpauid:

$ref: '#/components/schemas/Rpauid'

AuthDataForRestricted:

type: object

description: Represents obtained authorization Data for restricted discovery for a discoverer UE

required:

- proseQueryCodes

- proseRespCode

- validityTime

properties:

proseQueryCodes:

items:

$ref: '#/components/schemas/ProseQueryCode'

minItems: 1

proseRespCode:

$ref: '#/components/schemas/ProseResponseCode'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

MatchReportReqData:

type: object

description: Represents the Match Report information

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

proseAppCodes:

items:

$ref: '#/components/schemas/ProseApplicationCode'

minItems: 1

moniteredPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

MatchReportRespData:

type: object

description: Represents Match Report Acknowledgement

properties:

proseAppIdNames:

items:

$ref: '#/components/schemas/ProseApplicationIdName'

minItems: 1

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

metaData:

$ref: '#/components/schemas/MetaData'

metaDataIndexMasks:

items:

$ref: '#/components/schemas/MetaDataIndexMask'

minItems: 1

MonitorUpdateResult:

type: object

description: Represents the monitoring revocation results.

required:

- discType

- proseRestrictedCode

- appId

- bannedRpauid

- bannedPduid

- revocationResult

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

appId:

$ref: '#/components/schemas/ApplicationId'

bannedRpauid:

$ref: '#/components/schemas/Rpauid'

bannedPduid:

$ref: '#/components/schemas/Pduid'

revocationResult:

$ref: '#/components/schemas/RevocationResult'

MatchInformation:

type: object

description: Represents a report including a matching result, and the information that can be used for charging purpose.

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

openMatchInfoForOpen:

$ref: '#/components/schemas/MatchInfoForOpen'

restrictedMatchInfo:

$ref: '#/components/schemas/MatchInfoForRestricted'

MatchInfoForOpen:

type: object

description: Represents a report including a matching result, and the information that can be used for charging purpose for the open discovery type.

required:

- supi

- appId

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

appId:

items:

$ref: '#/components/schemas/ApplicationId'

minItems: 1

MatchInfoForRestricted:

type: object

description: Represents a report including a matching result, and the information that can be used for charging purpose for the restricted discovery type.

required:

- supi

- rpauid

- targetPduid

- proseRestrictedCode

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

rpauid:

$ref: '#/components/schemas/Rpauid'

targetPduid:

$ref: '#/components/schemas/Pduid'

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

# SIMPLE TYPES:

DiscoveryEntryId:

description: Contains the Discovery Entry ID.

type: string

ProseApplicationId:

description: Contains the ProSe Application ID.

type: string

ProseApplicationCode:

description: Contains the ProSe Application Code.

type: string

Rpauid:

description: Contains the RPAUID.

type: string

ApplicationId:

description: Contains the Application ID.

type: string

ProseRestrictedCode:

description: Contains the ProSe Restricted Code.

type: string

ProseRestrictedPrefix:

description: Contains the ProSe Restricted Prefix.

type: string

MetaData:

description: Contains the metadata.

type: string

RestrictedCodeSuffixPool:

description: Contains the Restricted Code Suffix pool.

type: string

ProseApplicationIdName:

description: Contains the ProSe Application ID name.

type: string

Pduid:

description: Contains the PDUID.

type: string

ProseApplicationPrefix:

description: Contains the Prose Application Prefix.

type: string

ProseApplicationMask:

description: Contains the Prose Application Mask.

type: string

ProseQueryCode:

description: Contains the ProSe Query Code.

type: string

ProseResponseCode:

description: Contains the ProSe Response Code.

type: string

MetaDataIndexMask:

description: Contains the Meta Data Index Mask.

type: string

# ENUMS:

DiscoveryType:

anyOf:

- type: string

enum:

- OPEN

- RESTRICTED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- OPEN: Discovery type is "open".

- RESTRICTED: Discovery type is "restricted".

AuthorizationResult:

anyOf:

- type: string

enum:

- SUCCESSFUL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- SUCCESSFUL: The authorisation is successful.

RevocationResult:

anyOf:

- type: string

enum:

- SUCCESSFUL

- FAILED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- SUCCESSFUL: The Monitoring Revocation is successful.

- FAILED: The Monitoring Revocation is failed.

* * * End of Changes * * * *