3GPP TSG-CT Meeting #92e	CP-211171

# E-Meeting, 14th – 16th June 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.562 | CR | 0071 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS User Profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 2021-06-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As described in clause 6.2.3.1 of TS 29.562, the IMS User Profile includes ifcs, service-level-trace-info and prority-levels. It is recommended to use a single GET request with query parameters rather than issuing multiple GET requests as described in the NOTE in clause 6.2.3.4.3.1 of TS 29.562: NOTE: The retrieval of these data sets can also be achieved by sending individual GET requests to the corresponding sub-resources under the {imsUeId} resource. When multiple data sets need to be retrieved by the NF Service consumer, it is recommended to use a single GET request with query parameters rather than issuing multiple GET requests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add attributes chargingInfo, serviceLevelTraceInfo and servicePriorityLevelList to ImsProfileData data type - Add Enumeration values TRACE_DATA and PRIORITY_DATA to Enumeration DataSetName |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2.4; 6.2.6.3.3; A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the Nhss-imsSDM API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: The following OpenAPI content is incorrect: chargingInfo: $ref: '#/components/schemas/ChargingInfo' serviceLevelTraceInfo: $ref: '#/components/schemas/ServiceLevelTraceInformation' servicePriorityLevelList: type: array items: $ref: '#/components/schemas/NameSpacePriority' minItems: 1 uniqueItems: true it needs to be replaced with: chargingInfo: $ref: '#/components/schemas/ChargingInfo' serviceLevelTraceInfo: $ref: '#/components/schemas/ServiceLevelTraceInformation' servicePriorityLevelList: type: array items: $ref: '#/components/schemas/NameSpacePriority' minItems: 1 uniqueItems: true Also, the following OpenAPI part is incorrect: DataSetNames: type: array minItems: 1 uniqueItems: true properties: dataSetName: $ref: '#/components/schemas/DataSetName' It needs to be replaced with: DataSetNames: type: array items: $ref: '#/components/schemas/DataSetName' minItems: 1 uniqueItems: true The correction needs be done in both CRs, Rel-16 and Rel-17 mirror. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.2.6.2.4	Type: ImsProfileData

Table 6.2.6.2.4-1: Definition of type ImsProfileData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supportedFeatures | SupportedFeatures | O | 0..1 | See clause 6.1.8 |
| imsServiceProfiles | array(ImsServiceProfile) | M | 1..N | IMS service profile shared by several public identities when they belong to an IRS. |
| chargingInfo | ChargingInfo | O | 0..1 | A response body containing the Charging Information for the user. |
| serviceLevelTraceInfo | string | O | 0..1 | IMS Service Level Trace information. Syntax described in 3GPP TS 24.323 [24]. |
| servicePriorityLevelList | array(NameSpacePriority) | O | 0..N | List of namespaces and their associated priorities allowed for the user. |
| maxAllowedSimulReg | integer | O | 0..1 | Maximum number of allowed simultaneous registrations for the Public User Identity. |

* * * Next Change * * * *

6.2.6.3.3	Enumeration: DataSetName

The enumeration DataSetName represents a Set of Data within the IMS User Profile. It shall comply with the provisions defined in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration DataSetName

| Enumeration value | Description |
| --- | --- |
| "CHARGING_DATA" | Charging Information Data |
| "IFC_DATA" | Initial Filter Criteria Data |
| "TRACE_DATA" | IMS service level trace information |
| "PRIORITY_DATA" | Service Priority Levels |

* * * Next Change * * * *

A.3	Nhss_imsSDM API

***not shown for clarity***

/{imsUeId}/ims-data/profile-data/profile-data:

get:

summary: Retrieve the complete IMS profile for a given IMS public identity (and public identities in the same IRS)

operationId: GetProfileData

tags:

- IMS Profile Data Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:profile-data:read

parameters:

- name: imsUeId

in: path

description: IMS Identity. In this case it shall be an IMS public identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: dataset-names

in: query

description: Datasets to be retrieved

schema:

$ref: '#/components/schemas/DataSetNames'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsProfileData'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

***not shown for clarity***

***not shown for clarity***

ImsProfileData:

type: object

required:

- imsServiceProfiles

properties:

imsServiceProfiles:

type: array

items:

$ref: '#/components/schemas/ImsServiceProfile'

chargingInfo:

$ref: '#/components/schemas/ChargingInfo'

serviceLevelTraceInfo:

$ref: '#/components/schemas/ServiceLevelTraceInformation'

servicePriorityLevelList:

type: array

items:

$ref: '#/components/schemas/NameSpacePriority'

minItems: 1

uniqueItems: true

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

maxAllowedSimulReg:

type: integer

***not shown for clarity***

***not shown for clarity***

ScscfSelectionAssistanceInformation:

type: object

properties:

scscfCapabilityList:

$ref: '#/components/schemas/ScscfCapabilityList'

scscfNames:

type: array

items:

type: string

minItems: 1

anyOf:

- required: [scscfCapabilityList]

- required: [scscfNames]

DataSetNames:

type: array

items:

$ref: '#/components/schemas/DataSetName'

minItems: 1

uniqueItems: true

***not shown for clarity***

***not shown for clarity***

UserStateCs:

anyOf:

- type: string

enum:

- CAMEL_BUSY

- NETWORK_DETERMINED_NOT_REACHABLE

- ASSUMED_IDLE

- NOT_PROVIDED_FROM_VLR

- type: string

DataSetName:

anyOf:

- type: string

enum:

- CHARGING_DATA

- IFC_DATA

- TRACE_DATA

- PRIORITY_DATA

- type: string

***not shown for clarity***

* * * End of Changes * * * *