3GPP TSG-CT TSG Meeting #92-e	CP-211175

# E-Meeting, 14th – 16h June 2021															Revision of C4-213219

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.509 | CR | 0123 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data Type Descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2021-06-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the requirement from clause 5.3.9 of 3GPP TS 29.501: - "description: <description>", where <description> is the description of the data type in the table defining the structured data type. The "description" attribute should be provided for all data types, specially if they are frequently reused from the same or other OpenAPI specification files; the "description" attribute shall always be provided for data types defined as maps, with a clear indication of the values (strings) used as key of the map. It is proposed to add a "description" field in all data types defined under "components/schemas" section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a "description" field for the data types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API definition does not comply with style guides. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.2.6.1, A.2, A.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI files of the Nausf_UEAuthentication API, Nausf_SoRProtection API and Nausf_UPUProtection API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Fix the issue discovered during implementing CRs, i.e. Revert the change on EapPayload. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nausf service based interface protocol.

Table 6.1.6.1-1: Nausf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationInfo | 6.1.6.2.2 | Contains the UE id (i.e. SUCI or SUPI) and the Serving Network Name. |
| UEAuthenticationCtx | 6.1.6.2.3 | Contains the information related to the resource generated to handle the UE authentication. It contains at least the UE id, Serving Network, the Authentication Method and related EAP information or related 5G-AKA information. |
| 5gAuthData | 6.1.6.2.4 | Contains 5G authentication related information. |
| Av5gAka | 6.1.6.2.5 | Contains Authentication Vector for method 5G AKA. |
| ConfirmationData | 6.1.6.2.6 | Contains the "RES*" generated by the UE. |
| DeregistrationInfo | 6.1.6.2.11 | Contains the UE id (i.e. SUPI). |
| EapSession | 6.1.6.2.7 | Contains information related to the EAP session. |
| ConfirmationDataResponse | 6.1.6.2.8 | Contains the result of the authentication. |
| RgAuthenticationInfo | 6.1.6.2.9 | Contains the UE id (i.e. SUCI) and the authenticated indication. |
| RgAuthCtx | 6.1.6.2.10 | Contains the UE id (i.e. SUPI) and the authentication indication. |
| EapPayload | 6.1.6.3.2 | Contains the EAP packet. |
| ResStar | 6.1.6.3.2 | Contains the RES*. |
| Kseaf | 6.1.6.3.2 | Contains the Kseaf. |
| HxresStar | 6.1.6.3.2 | Contains the HXRES*. |
| Suci | 6.1.6.3.2 | Contains the SUCI. |
| AuthType | 6.1.6.3.3 | Indicates the authentication method used. |
| AuthResult | 6.1.6.3.4 | Indicates the result of the authentication. |

Table 6.1.6.1-2 specifies data types re-used by the Nausf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nausf service based interface.

Table 6.1.6.1-2: Nausf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| LinksValueSchema | 3GPP TS 29.571 [10] | 3GPP Hypermedia link |
| ProblemDetails | 3GPP TS 29.571 [10] | Common Data Type used in response bodies |
| Supi | 3GPP TS 29.571 [10] |  |
| Uri | 3GPP TS 29.571 [10] |  |
| ResynchronizationInfo | 3GPP TS 29.503 [12] |  |
| SupiOrSuci | 3GPP TS 29.571 [10] |  |
| ServingNetworkName | 3GPP TS 29.503 [12] |  |
| Autn | 3GPP TS 29.503 [12] |  |
| Rand | 3GPP TS 29.503 [12] |  |
| Pei | 3GPP TS 29.571 [10] |  |
| TraceData | 3GPP TS 29.571 [10] |  |
| NfGroupId | 3GPP TS 29.571 [10] |  |
| CagId | 3GPP TS 29.571 [10] |  |
| SupportedFeatures | 3GPP TS 29.571 [10] | Supported Features |

* * * Next Change * * * *

6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nausf-SORProtection service based interface protocol.

Table 6.2.6.1-1: Nausf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SorInfo | 6.2.6.2.2 | Contains the Steering Information |
| SorSecurityInfo | 6.2.6.2.3 | Contains the material generated for securing of SoR. It contains at least the SoR-MAC-IAUSF and CounterSoR. |
| SteeringInfo | 6.2.6.2.4 | Contains a combination of one PLMN identity and zero or more access technologies. |
| SteeringContainer | 6.2.6.2.5 | Contains the information sent to UE. |
| SorMac | 6.2.6.3.2 | MAC value for protecting SOR procedure (SoR-MAC-IAUSF and SoR-XMAC-IUE) |
| CounterSor | 6.2.6.3.2 | CounterSoR |
| AckInd | 6.2.6.3.2 | Contains indication whether the acknowledgement from UE is needed |
| SecuredPacket | 6.2.6.3.2 | Contains a secure packet. |
| AccessTech | 6.2.6.3.3 | Access Technology |
| SorHeader | 6.2.6.3.2 | Contains the SoR Header. |
| SorTransparentInfo | 6.2.6.3.2 | Contains steering information encoded as transparent containers. |

Table 6.2.6.1-2 specifies data types re-used by the Nausf-SORProtection service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nausf service based interface.

Table 6.2.6.1-2: Nausf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| PlmnId | 3GPP TS 29.571 [10] | PLMN ID |
| SupportedFeatures | 3GPP TS 29.571 [10] | Supported Features |

* * * Next Change * * * *

A.2	Nausf_UEAuthentication API

openapi: 3.0.0

info:

version: 1.1.2

title: AUSF API

description: |

AUSF UE Authentication Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

schemas:

AuthenticationInfo:

description: Contains the UE id (i.e. SUCI or SUPI) and the Serving Network Name.

type: object

properties:

supiOrSuci:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupiOrSuci'

servingNetworkName:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

resynchronizationInfo:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ResynchronizationInfo'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

pattern: '^[0-9]{1,4}$'

cellCagInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CagId'

minItems: 1

n5gcInd:

type: boolean

default: false

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supiOrSuci

- servingNetworkName

UEAuthenticationCtx:

description: Contains the information related to the resource generated to handle the UE authentication. It contains at least the UE id, Serving Network, the Authentication Method and related EAP information or related 5G-AKA information.

type: object

properties:

authType:

$ref: '#/components/schemas/AuthType'

5gAuthData:

oneOf:

- $ref: '#/components/schemas/Av5gAka'

- $ref: '#/components/schemas/EapPayload'

_links:

type: object

description: A map(list of key-value pairs) where member serves as key

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

servingNetworkName:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

required:

- authType

- 5gAuthData

- _links

Av5gAka:

description: Contains Authentication Vector for method 5G AKA.

type: object

required:

- rand

- hxresStar

- autn

properties:

rand:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

hxresStar:

$ref: '#/components/schemas/HxresStar'

autn:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Autn'

ConfirmationData:

description: Contains the "RES*" generated by the UE.

type: object

required:

- resStar

properties:

resStar:

$ref: '#/components/schemas/ResStar'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ConfirmationDataResponse:

description: Contains the result of the authentication.

type: object

properties:

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

kseaf:

$ref: '#/components/schemas/Kseaf'

required:

- authResult

EapSession:

description: Contains information related to the EAP session.

type: object

properties:

eapPayload:

$ref: '#/components/schemas/EapPayload'

kSeaf:

$ref: '#/components/schemas/Kseaf'

_links:

type: object

description: A map(list of key-value pairs) where member serves as key

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eapPayload

DeregistrationInfo:

description: Contains the UE id (i.e. SUPI).

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supi

RgAuthenticationInfo:

description: Contains the UE id (i.e. SUCI) and the authenticated indication.

type: object

properties:

suci:

$ref: '#/components/schemas/Suci'

authenticatedInd:

type: boolean

default: false

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- suci

- authenticatedInd

RgAuthCtx:

description: Contains the UE id (i.e. SUPI) and the authentication indication.

type: object

properties:

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

authInd:

type: boolean

default: false

required:

- authResult

AuthResult:

description: Indicates the result of the authentication.

type: string

enum:

- AUTHENTICATION_SUCCESS

- AUTHENTICATION_FAILURE

- AUTHENTICATION_ONGOING

EapPayload:

type: string

format: base64

description: contains an EAP packet

nullable: true

Kseaf:

description: Contains the Kseaf.

type: string

pattern: '[A-Fa-f0-9]{64}'

ResStar:

description: Contains the RES*.

type: string

pattern: '[A-Fa-f0-9]{32}'

nullable: true

HxresStar:

description: Contains the HXRES*.

type: string

pattern: "[A-Fa-f0-9]{32}"

Suci:

description: Contains the SUCI.

type: string

pattern: '^(suci-(0-[0-9]{3}-[0-9]{2,3}|[1-7]-.+)-[0-9]{1,4}-(0-0-.+|[a-fA-F1-9]-([1-9]|[1-9][0-9]|1[0-9]{2}|2[0-4][0-9]|25[0-5])-[a-fA-F0-9]+)|.+)$'

AuthType:

description: Indicates the authentication method used.

anyOf:

- type: string

enum:

- 5G_AKA

- EAP_AKA_PRIME

- EAP_TLS

- type: string

* * * Next Change * * * *

A.3	Nausf_SoRProtection API

openapi: 3.0.0

info:

version: 1.2.0-alpha.2

title: Nausf_SoRProtection Service

description: |

AUSF SoR Protection Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

schemas:

# COMPLEX TYPES:

#

SorInfo:

description: Contains the Steering Information.

type: object

properties:

steeringContainer:

$ref: '#/components/schemas/SteeringContainer'

ackInd:

$ref: '#/components/schemas/AckInd'

sorHeader:

$ref: '#/components/schemas/SorHeader'

sorTransparentInfo:

$ref: '#/components/schemas/SorTransparentInfo'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ackInd

SorSecurityInfo:

description: Contains the material generated for securing of SoR. It contains at least the SoR-MAC-IAUSF and CounterSoR.

type: object

properties:

sorMacIausf:

$ref: '#/components/schemas/SorMac'

counterSor:

$ref: '#/components/schemas/CounterSor'

sorXmacIue:

$ref: '#/components/schemas/SorMac'

required:

- sorMacIausf

- counterSor

SteeringContainer:

description: Contains the information sent to UE.

oneOf:

- type: array

items:

$ref: '#/components/schemas/SteeringInfo'

minItems: 1

- $ref: '#/components/schemas/SecuredPacket'

SteeringInfo:

description: Contains a combination of one PLMN identity and zero or more access technologies.

type: object

properties:

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

accessTechList:

type: array

items:

$ref: '#/components/schemas/AccessTech'

minItems: 1

required:

- plmnId

# SIMPLE TYPES:

#

SorMac:

description: MAC value for protecting SOR procedure (SoR-MAC-IAUSF and SoR-XMAC-IUE).

type: string

pattern: '^[A-Fa-f0-9]{32}$'

CounterSor:

description: CounterSoR.

type: string

pattern: '^[A-Fa-f0-9]{4}$'

AckInd:

description: Contains indication whether the acknowledgement from UE is needed.

type: boolean

SecuredPacket:

description: Contains a secure packet.

type: string

format: base64

SorHeader:

description: Contains the SoR Header.

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

SorTransparentInfo:

description: Contains steering information encoded as transparent containers.

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

# ENUMS:

AccessTech:

description: Access Technology.

anyOf:

- type: string

enum:

- NR

- EUTRAN_IN_WBS1_MODE_AND_NBS1_MODE

- EUTRAN_IN_NBS1_MODE_ONLY

- EUTRAN_IN_WBS1_MODE_ONLY

- UTRAN

- GSM_AND_ECGSM_IoT

- GSM_WITHOUT_ECGSM_IoT

- ECGSM_IoT_ONLY

- CDMA_1xRTT

- CDMA_HRPD

- GSM_COMPACT

- type: string

* * * Next Change * * * *

A.4	Nausf_UPUProtection API

openapi: 3.0.0

info:

version: 1.1.2

title: Nausf_UPUProtection Service

description: |

AUSF UPU Protection Service

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

schemas:

# COMPLEX TYPES:

UpuInfo:

description: Contains the UE parameters update Information.

type: object

properties:

upuDataList:

type: array

items:

$ref: '#/components/schemas/UpuData'

minItems: 1

upuAckInd:

$ref: '#/components/schemas/UpuAckInd'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- upuDataList

- upuAckInd

UpuSecurityInfo:

description: Contains the material generated for securing of UPU. It contains at least the UPU-MAC-IAUSF and CounterUPU.

type: object

properties:

upuMacIausf:

$ref: '#/components/schemas/UpuMac'

counterUpu:

$ref: '#/components/schemas/CounterUpu'

upuXmacIue:

$ref: '#/components/schemas/UpuMac'

required:

- upuMacIausf

- counterUpu

UpuData:

description: Contains UE parameters update data set (e.g., the updated Routing ID Data or the Default configured NSSAI).

type: object

properties:

secPacket:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SecuredPacket'

defaultConfNssai:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

routingId:

$ref: 'TS29544_Nspaf_SecuredPacket.yaml#/components/schemas/RoutingId'

oneOf:

- required: [secPacket]

- required: [defaultConfNssai]

# SIMPLE TYPES:

UpuMac:

description: MAC value for protecting UPU procedure (UPU-MAC-IAUSF and UPU-MAC-IUE).

type: string

pattern: '^[A-Fa-f0-9]{32}$'

CounterUpu:

description: CounterUPU.

type: string

pattern: '^[A-Fa-f0-9]{4}$'

UpuAckInd:

description: Contains the indication of whether the acknowledgement from UE is needed.

type: boolean

* * * End of Changes * * * *