3GPP TSG-CT WG4 Meeting #104-e	CP-211176

# E-Meeting, 19th – 28th May 2021														   was C4-213060, 3449

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.540 | CR | 0081 | rev | 2 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Redirect Response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per agreement in CT4#103E meeting, the HTTP 307/308 response shall not be regarded as a error case. Thus, it should not use "application/problem+json" media type, and the returned data structure shall be replaced from ProblemDetails to RedirectResponse. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the HTTP response of 307 / 308. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The response of HTTP 307 / 308 is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.3.3.1, 6.1.3.3.3.2, 6.1.3.3.4.2.2, 6.1.6.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI file TS29540_Nsmsf_SMService. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#2: - Fix error in the OpenAPI. Rev#1: - Fix the error in the coversheet. - Correct the description to 307/308 response. - Correct the OpenAPI, i.e. change content type to application+json for 307/308 response. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.3.3.3.1	PUT

This method creates an individual resource of UE Context for SMS in the SMSF, or updates the indicated resource of UE Context for SMS in the SMSF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeSmsContextData | M | 1 | Representation of the UE Context for SMS to be created in the SMSF, or to be updated in the SMSF. |

Table 6.1.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeSmsContextData | M | 1 | 201 Created | This case represents the successful creation of an UE Context for SMS. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| n/a |  |  | 204 No Content | This case represents the successful update of an UE Context for SMS. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if this is a redirection triggered by an SCP to the same target resource via another SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if this is a redirection triggered by an SCP to the same target resource via another SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | When used to represent the failure of creation / update of an UE Context for SMS, the "cause" attribute of the "ProblemDetails" shall be set to one of the following application error codes: - SERVICE_NOT_ALLOWED, if SMS service is not allowed for the given service user; |
| ProblemDetails | O | 0..1 | 404 Not Found | When used to represent the failure of creation / update of an UE Context for SMS, the "cause" attribute of the "ProblemDetails" shall be set to one of the following application error codes: - USER_NOT_FOUND, if the provided subscriber identifier is invalid or the service user is not found from UDM; |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponses may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsmsf-sms/<apiVersion>/ue-contexts/{supi} |
| ETag | string | O | 0..1 | Entity Tag, containing a strong validator, as described in IETF RFC 7232[18], clause 2.3. |

Table 6.1.3.3.3.1-5: Headers supported by the 204 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ETag | string | O | 0..1 | Entity Tag, containing a strong validator, as described in IETF RFC 7232 [18], clause 2.3. |

Table 6.1.3.3.3.1-6: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.3.1-7: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

6.1.3.3.3.2	DELETE

This method deletes an individual resource of UE Context for SMS in the SMSF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents a successful deletion of an UE Context for SMS. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if this is a redirection triggered by an SCP to the same target resource via another SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if this is a redirection triggered by an SCP to the same target resource via another SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | When used to represent an unsuccessful deletion of an UE Context for SMS, the "cause" attribute of the "ProblemDetails" may be used to include one of the following application error codes: - CONTEXT_NOT_FOUND, if the UE context for SMS to be operated is invalid or not found in SMSF. |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponses may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.3.2-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| If-Match | String | O | 0..1 | Validator for conditional requests, as described in IETF RFC 7232 [18], clause 3.1 |

Table 6.1.3.3.3.2-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set |
| 3gpp-Sbi-Target-Nf-Id | String | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.3.2-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

6.1.3.3.4.2.2	Operation Definition

This custom operation is used to send a SMS payload to an individual UEContext resource in the SMSF.

This operation shall support the request data structures specified in table 6.1.3.3.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.2.2-2.

Table 6.1.3.3.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmsRecordData | M | 1 | Representation of the SMS Record to be created in the SMSF. |

Table 6.1.3.3.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmsRecordDeliveryData | M | 1 | 200 OK | This case represents the successful of sending SMS record in uplink direction, with necessary response data. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents an unsuccessful delivery of SMS payload. The "cause" attribute may be used to indicate one of the following application errors: - SMS_PAYLOAD_MISSING, if the expected SMS payload content is missing; - SMS_PAYLOAD_ERROR, if error exists in the SMS payload content. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This case represents an unsuccessful delivery of SMS payload. The "cause" attribute may be used to indicate one of the following application errors: - SERVICE_NOT_ALLOWED, if SMS service is not allowed for the given service user; |
| ProblemDetails | O | 0..1 | 404 Not Found | This case represents an unsuccessful delivery of SMS payload. The "cause" attribute may be used to indicate one of the following application errors: - CONTEXT_NOT_FOUND, if the UE context for SMS to be operated is invalid or not found in SMSF. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponses may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmsf service based interface protocol.

Table 6.1.6.1-1: Nsmsf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| UeSmsContextData | 6.1.6.2.2 | Information used for activating SMS service for a service user, or updating the parameters for SMS service. |
| SmsRecordData | 6.1.6.2.3 | Information within request message invoking UplinkSMS service operation, for delivering SMS payload. |
| SmsRecordDeliveryData | 6.1.6.2.5 | Information for result of invoking UplinkSMS service operation. |
| RecordId | 6.1.6.3.2 | Record ID used to identify a message carrying SMS payload. |
| SmsDeliveryStatus | 6.1.6.3.3 | Status of SMS delivery attempts. |

Table 6.1.6.1-2 specifies data types re-used by the Nsmsf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmsf service based interface.

Table 6.1.6.1-2: Nsmsf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [10] | Common Data Type used in response bodies |
| RedirectResponse | 3GPP TS 29.571 [10] | Redirect Response |
| Supi | 3GPP TS 29.571 [6] | Subscription Permanent Identifier |
| Gpsi | 3GPP TS 29.571 [6] | General Public Subscription Identifier |
| Pei | 3GPP TS 29.571 [6] | Permanent Equipment, it contains an IMEI or IMEISV. |
| Guami | 3GPP TS 29.571 [6] | Globally Unique AMF Identifier |
| AccessType | 3GPP TS 29.571 [6] | Access Type (3GPP or non-3GPP access) |
| UserLocation | 3GPP TS 29.571 [6] | User location information |
| TimeZone | 3GPP TS 29.571 [6] | User time zone information |
| NfInstanceId | 3GPP TS 29.571 [6] | NF Instance ID |
| RefToBinaryData | 3GPP TS 29.571 [6] | Information for indicating the binary content of SMS payload. |
| TraceData | 3GPP TS 29.571 [6] | Trace control and configuration parameters |
| BackupAmfInfo | 3GPP TS 29.571 [6] | Backup AMF Information |
| NfGroupId | 3GPP TS 29.571 [6] | Network Function Group Id |
| RatType | 3GPP TS 29.571 [6] | RAT Type |
| SupportedFeatures | 3GPP TS 29.571 [6] | Supported Features |

* * * Next Change * * * *

A.2	Nsmsf_SMService API

********TEXT SKIPPED********

paths:

/ue-contexts/{supi}:

put:

summary: Activate SMS Service for a given UE

operationId: SMServiceActivation

tags:

- Activation of SMS service

parameters:

- name: supi

in: path

required: true

description: Subscriber Permanent Identifier (SUPI)

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UeSmsContextData'

required: true

responses:

'201':

description: UE Context for SMS is created in SMSF

content:

application/json:

schema:

$ref: '#/components/schemas/UeSmsContextData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsmsf-sms/<apiVersion>/ue-contexts/{supi}'

required: true

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in IETF RFC 7232, 2.3

schema:

type: string

'204':

description: UE Context for SMS is updated in SMSF

headers:

ETag:

description: Entity Tag, containing a strong validator, as described in IETF RFC 7232, 2.3

schema:

type: string

'307':

description: temporary redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

description: Invalid Service Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Unable to create/update UE Context for SMS in SMSF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Unable to found subscription for service user or UE Context for SMS in SMSF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

delete:

summary: Deactivate SMS Service for a given UE

operationId: SMServiceDeactivation

tags:

- Deactivation of SMS service

parameters:

- name: supi

in: path

required: true

description: Subscriber Permanent Identifier (SUPI)

schema:

type: string

- name: If-Match

in: header

description: Validator for conditional requests, as described in IETF RFC 7232, 3.1

schema:

type: string

responses:

'204':

description: UE Context for SMS is deleted from SMSF

'307':

description: temporary redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

description: Invalid Service Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Unable to found UE Context for SMS in SMSF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/ue-contexts/{supi}/sendsms:

post:

summary: Send SMS payload for a given UE

operationId: SendSMS

tags:

- Send SMS payload in uplink direction

parameters:

- name: supi

in: path

required: true

description: Subscriber Permanent Identifier (SUPI)

schema:

type: string

requestBody:

content:

multipart/related:

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/SmsRecordData'

binaryPayload:

type: string

format: binary

encoding:

jsonData:

contentType: application/json

binaryPayload:

contentType: application/vnd.3gpp.sms

headers:

Content-Id:

schema:

type: string

required: true

responses:

'200':

description: SMS payload is received by SMSF, and is being delivered out

content:

application/json:

schema:

$ref: '#/components/schemas/SmsRecordDeliveryData'

'307':

description: temporary redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'308':

description: permanent redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the same SMSF or SMSF (service) set'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: ' Identifier of the target NF (service) instance ID towards which the request is redirected'

schema:

type: string

'400':

description: Invalid Service Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Unable to deliver SMS at SMSF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Unable to found UE Context for SMS in SMSF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

********TEXT SKIPPED********

* * * End of Changes * * * *