3GPP TSG-CT Meeting #92e	CP-211183

# E-Meeting, 14th – 16th June 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.193 | CR | 0047 | rev | 2 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of steering mode indicator for load-balancing steering mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-06-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 23.501 has been updated with the introducion of steering mode indicator for load-balancing steering mode (CR2654 in S2-2101108) which is already implemented in TS 23.501. Hence, TS 24.193 needs to be updated to define the stage 3 details of the new requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Steering mode indicator for load-balancing steering mode is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of stage 3 for stage 2 requirements on load-balancing steering mode indicator. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.1, 6.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Version 1 was agreed at CT1#130 (in C1-213905). Version 2 proposes that the autonomous load-balance indicator is off by default (i.e., encoded as zero "0"). This aligns with the legacy behaviour since there is no autonomous load-balancing performed. Note 4 is amended so that only load balancing steering mode is considered. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

* * * Next Change * * * *

#### 6.1.3.1	Definition of ATSSS rules

The ATSSS rules are defined in 3GPP TS 23.501 [2] and is set of one or more ATSSS rules, where a rule is composed of:

a)	a precedence value of the ATSSS rule identifying the precedence of the ATSSS rule;

b)	a traffic descriptor matching a service data flow (SDF); and

c)	an access selection descriptor including:

1)	a steering functionality:

A)	MPTCP, the UE steers the SDF by using the MPTCP functionality; or

B)	ATSSS-LL functionality, the UE steers the SDF by using the ATSSS-LL functionality; and

NOTE:	If the included steering functionality is not supported by the UE, the UE ignores this ATSSS rule, and proceeds with the evaluation of the ATSSS rule with the next smallest precedence, if available.

2)	a steering mode:

A)	active-standby, the UE steers the SDF by using the active access if the active access is available. If the active access is not available and the standby access is available, the UE steers the SDF by using the standby access;

B)	smallest delay, the UE steers the SDF by using the access network with the smallest RTT. If there is only one access available, the UE steers the SDF by using the available access. This steering mode is only applicable to non-GBR SDF;

C)	load balancing, the UE steers the SDF across both the 3GPP access and the non-3GPP access with a given precentage if both accesses are available. If there is only one access available, the UE steers the SDF by using the available access. This steering mode is only applicable to non-GBR SDF; or

D)	priority based, the UE steers the SDF over the access with high priority unless the access with high priority is congested or unavailable, when the UE steers the SDF over both the access with high priority and the access with low priority. This steering mode is only applicable to non-GBR SDF.

3)	a steering mode indicator:

A)	autonomous load-balance indicator (ALB), this indicator is only applicable to load balancing steering mode. The UE may ignore the information provided in the steering mode information (i.e. percentages of the SDF traffic transmitted over 3GPP access and non-3GPP access) and the UE may autonomously determine its own percentages for traffic splitting, in a way that maximizes the aggregated bandwidth in the uplink direction. The UPF may apply a similar behaviour in the downlink direction;

NOTE:	The UE is expected to determine its own percentages for traffic splitting by performing measurements across both the 3GPP access and the non-3GPP access.

* * * Next Change * * * *

#### 6.1.3.2	Encoding of ATSSS rules

The ATSSS rules are encoded as shown in figure 6.1.3.2-1, figure 6.1.3.2-2 and figure 6.1.3.2-3 and table 6.1.3.2-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATSSS rule 1 |  |  |  |  |  |  |  | octet a+1 octet s |
| ATSSS rule 2 |  |  |  |  |  |  |  | octet s+1 octet t |
| … |  |  |  |  |  |  |  | octet t+1 octet u |
| ATSSS rule n |  |  |  |  |  |  |  | octet u+1 octet b |

Figure 6.1.3.2-1: ATSSS parameter contents including one or more ATSSS rules

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ATSSS rule |  |  |  |  |  |  |  | octet a+1 octet a+2 |
| Precedence value of ATSSS rule |  |  |  |  |  |  |  | octet a+3 |
| Length of traffic descriptor |  |  |  |  |  |  |  | octet a+4 octet a+5 |
| Traffic descriptor |  |  |  |  |  |  |  | octet a+6 octet s-4 |
| Access selection descriptor |  |  |  |  |  |  |  | octet s-3 octet s* |

Figure 6.1.3.2-2: ATSSS rule

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of access selection descriptor |  |  |  |  |  |  |  | octet s-3 |
| Steering functionality |  |  |  |  |  |  |  | octet s-2 |
| Steering mode |  |  |  |  |  |  |  | octet s-1 |
| Steering mode information |  |  |  |  |  |  |  | octet s* |
| Steering mode indicator |  |  |  |  |  |  |  |  | octet s+1* |  |

Figure 6.1.3.2-3: Access selection descriptor

| Length of steering mode indicator |  |  |  |  |  |  |  |  | octet t |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | ALB | octet t+1 |  |

Figure 6.1.3.2-4: Steering mode indicator

Table 6.1.3.2-1: ATSSS parameter contents including an ATSSS rule

| Precedence value of an ATSSS rule (octet a+3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The precedence value of an ATSSS rule field shall be used to specify the precedence of the ATSSS rule among all ATSSS rules. This field shall include the binary encoded value of the precedence value in the range from 0 to 255 (decimal). The higher the value of the precedence value field, the lower the precedence of the ATSSS rule is. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Traffic descriptor (octets a+6 to s-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The traffic descriptor field is, as defined in table 5.2.1 in 3GPP TS 24.526 [5], of variable size and contains a variable number (at least one) of traffic descriptor components (NOTE 3). Each traffic descriptor component shall be encoded as a sequence of one octet traffic descriptor component type identifier and a traffic descriptor component value field. The traffic descriptor component type identifier shall be transmitted first. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Traffic descriptor component type identifier Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  |  | 1 |  |  |  |  |  |  |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | Match-all type |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | OS Id + OS App Id type (NOTE 1) |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | IPv4 remote address type |  |  |  |
| 0 |  |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | IPv6 remote address/prefix length type |  |  |  |
| 0 |  |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | Protocol identifier/next header type |  |  |  |
| 0 |  |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | Single remote port type |  |  |  |
| 0 |  |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | Remote port range type |  |  |  |
| 0 |  |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 1 |  | 0 |  |  |  |  |  | IP 3 tuple type |  |  |  |
| 0 |  |  | 1 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | Security parameter index type |  |  |  |
| 0 |  |  | 1 |  | 1 |  | 1 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | Type of service/traffic class type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | Flow label type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | Destination MAC address type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 1 |  |  |  |  |  | 802.1Q C-TAG VID type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  | 0 |  |  |  |  |  | 802.1Q S-TAG VID type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  | 1 |  |  |  |  |  | 802.1Q C-TAG PCP/DEI type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  |  | 0 |  |  |  |  |  | 802.1Q S-TAG PCP/DEI type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  |  | 1 |  |  |  |  |  | Ethertype type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | DNN type |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | Destination FQDN |  |  |  |
| 1 |  |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 1 |  | 0 |  |  |  |  |  | Regular expression |  |  |  |
| 1 |  |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | OS App Id type |  |  |  |
| All other values are spare. If received they shall be interpreted as unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length of access selection descriptor (octet s-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  |  | 1 |  |  |  |  |  |  |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 1 |  |  |  |  |  | If the steering mode is smallest delay |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  | 0 |  |  |  |  |  | If the steering mode is not smallest delay |  |  |  |
| All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steering functionality (octet s-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The steering functionality field shall be encoded by one octet (octet s-2) as follows |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  |  | 1 |  |  |  |  |  |  |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | UE's supported steering functionality (NOTE 2) |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 0 |  |  |  |  |  | MPTCP functionality |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 1 |  |  |  |  |  | ATSSS-LL functionality |  |  |  |
| All other values are spare. If the UE does not support the received encoded steering functionality in the ATSSS rule, the UE shall ignore the ATSSS rule. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steering mode (octet s-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The steering mode descriptor field shall be encoded by one octet (octet s-1) as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  |  | 1 |  |  |  |  |  |  |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | Active-standby |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 0 |  |  |  |  |  | Smallest delay |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 1 |  |  |  |  |  | Load balancing |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  | 0 |  |  |  |  |  | Priority based |  |  |  |
| All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steering mode information (octet s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If the steering mode is defined as active-standby, octet s shall be defined as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  |  | 2 |  |  |  | 1 |  |  |  |  |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  |  | 1 |  |  |  |  |  | Active 3GPP and no standby |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  | 0 |  |  |  |  |  | Active 3GPP and non-3GPP standby |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  | 1 |  |  |  |  |  | Active non-3GPP and no standby |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 0 |  |  |  | 0 |  |  |  |  |  | Active non-3GPP and 3GPP standby |  |
| All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If the steering mode is defined as smallest delay, octet s shall not be encoded. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If the steering mode is defined as load balancing, octet s shall be encoded to show the percentage of the SDF traffic transmitted over 3GPP access and non-3GPP access as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  |  | 1 |  |  |  |  |  |  |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | 100% over 3GPP and 0% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 0 |  |  |  |  |  | 90% over 3GPP and 10% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 1 |  |  |  |  |  | 80% over 3GPP and 20% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  | 0 |  |  |  |  |  | 70% over 3GPP and 30% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  | 1 |  |  |  |  |  | 60% over 3GPP and 40% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  |  | 0 |  |  |  |  |  | 50% over 3GPP and 50% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  |  | 1 |  |  |  |  |  | 40% over 3GPP and 60% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  | 30% over 3GPP and 70% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | 20% over 3GPP and 80% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  |  | 0 |  |  |  |  |  | 10% over 3GPP and 90% over non-3GPP |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  |  | 1 |  |  |  |  |  | 0% over 3GPP and 100% over non-3GPP |  |  |  |
| All other values are spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If the steering mode is defined as priority-based, octet s shall be encoded as: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  |  | 1 |  |  |  |  |  |  |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 1 |  |  |  |  |  | 3GPP is high priority access |  |  |  |
| 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 0 |  |  |  |  |  | non-3GPP is high priority access |  |  |  |
| All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steering mode indicator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The steering mode indicator provides information to adjust the traffic steering. The following indicators exist (NOTE 4). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALB (autonomous load-balance indicator) (octet t+2, bit 1) is set as follows: Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  | Autonomous load-balance indicator is off |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | Autonomous load-balance indicator is on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: For "OS Id + OS App Id type", the traffic descriptor component value field does not specify the OS version number or the version number of the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 2: This value shall be set by the SMF if the UE supports only one steering functionality. The SMF knows the UE's supported steering functionality during the MA PDU session establishment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: Traffic descriptor components of an ATSSS rule are not required to be the same as the traffic descriptor components, defined in table 5.2.1 in 3GPP TS 24.526 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: If the value is received for a steering mode other than load balancing, it shall be ignored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |