3GPP TSG-CT3 Meeting #116e	C3-213150

# E-Meeting, 19th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.486 | CR | 0050 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reception report for downlink message delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI17 |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in 24.486, the VAE server can report the result of the downlink message delivery to the NF service consmer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Defines the procedure of reception report of downlink message delivery. - Update the Open API file. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NF service consumer don’t know the result of the downlink message delivery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.2.4.2, 6.1.55, 6.1.5.x(new), 6.1.5.x.1(new)， 6.1.5.x.2(new), 6.1.6.1, 6.2.6.3.x(new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature to the OpenAPI file of VAE_MessageDelivery API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	3GPP TS 23.286: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows".

[5]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[8]	3GPP TR 21.900: "Technical Specification Group working methods".

[11]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[12]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[13]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[14]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[15]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[16]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[17]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[18]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[19]	3GPP TS 29.116: "Representational state transfer over xMB reference point between Content Provider and BM-SC".

[20]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[21]	IETF RFC 6455: "The Websocket Protocol".

[22]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[23]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[24]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[x]	3GPP TS 24.486: " Vehicle-to-Everything (V2X) Application Enabler (VAE) layer; Protocol aspects; stage 3".

# *** Next Change ***

5.2.2.4.2	Deliver Downlink Message

Figure 5.2.2.4.2-1: V2X message delivery

When the NF service consumer (e.g. V2X application specific server) needs to send the message to the V2X UE, the NF service consumer shall send the HTTP POST method as step 1of the figure 5.2.2.4.2-1 to request to create an "Individual Message Delivery".

The NF service consumer shall include V2xMessageDeliveryData data structure in the payload body of the HTTP POST to request a creation of representation of the "Individual Message Delivery" resource. The "Individual Message Delivery" resource is created as described below.

The NF service consumer within the MessageDeliveryData data structure shall include:

-	Either the V2X UE ID within the "ueId" attribute or the V2X Group ID within the "groupId" attribute;

-	V2X message payload carried by the V2X message within the "payload" attribute;

and may include:

-	The duration within the "duration" attribute; and

-	The geographical area identifier within the "geoId" attribute.

When the VAE Server receives the HTTP POST request from the NF service consumer, the VAE server shall make an authorization based on the information received from the NF service consumer.  If the authorization is successful, the VAE Server shall create a new resource, which represents "Individual Message Delivery", addressed by a URI as defined in clause 6.1.3.5.2 and contains a VAE Server created resource identifier. The VAE Server shall respond to the NF service consumer with a 201 Created message, including Location header field containing the URI for the created resource.

The NF service consumer shall use the URI received in the Location header in subsequent requests to the VAE Server to refer to the "Individual Message Delivery".

Upon receipt of the HTTP DELETE message from the NF service consumer, the VAE Server shall check if the Individual Message Delivery resource identified by the URI already exists. If the resource exists, the VAE Server shall delete the resource and respond to the NF service consumer with a 204 No Content success message.

When the message delivery duration expires, the VAE server may remove the associated Individual Message Delivery resource locally.

If errors occur when processing the HTTP POST or DELETE request, the VAE Server shall apply error handling procedures as specified in subclause 6.1.7.

After the VAE Server responded to the NF service consumer, the VAE Server shall invoke the procedure defined in subclause 6.5.2.4 or 6.5.2.5 of 3GPP TS 24.486 [x] to send the message to the VAE Client.

When the VAE Server received the reception report from the VAE Client as defined in subclause 6.5.2.2 of 3GPP TS 24.486 [x], the VAE Server shall send an HTTP POST message to the NF service consumer identified by the notification URI received during the message delivery subscribed if the "ReceptionReport" feature is supported. Upon receipt of the request, the SCS/AS shall acknowledge the notification with an HTTP 204 No Content response.

# *** Next Change ***

6.1.5.5	Methods

The notifications provided by the VAE_MessageDelivery Service are specified in this subclause.

Table 6.1.5.5-1: Notifications

|  |  | Callback URI |  |  | HTTP method or custom operation | Description (service operation) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink Message Delivery |  | {notifUri} |  |  | POST | Uplink Message Delivery. |  |
| Reception Report of Downlink Message Delivery | {notifUri} | POST | Send notifications about the result of the downlink Message delivery |

# *** Next Change ***

6.1.5.x	Reception Report of Downlink Message Delivery

6.1.5.x.1	Description

This notification is used by the VAE Server to report the result of downlink message delivery to NF service consumer.

6.1.5.x.2	Operation Definition

This operation shall support the request data structures specified in table 6.1.5.x.2-1 and the response data structure and response codes specified in table 6.1.5.x.2-2.

Table 6.1.5.x.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Result | M | 1 | Contains the result of downlink messagege delivery. |

Table 6.1.5.x.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The reception report is successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during reception report of downlink data delivery. The response shall include a Location header field containing an alternative URI representing the end point of an alternative V2X application specific server where the uplink message should be sent. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [22] with the difference: SCEF is replaced by the VAE Server and the SCS/AS is replaced by the V2X application specific server. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during reception report of downlink data delivery. The response shall include a Location header field containing an alternative URI representing the end point of an alternative V2X application specific server where the uplink message should be sent. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [22] with the difference: SCEF is replaced by the VAE Server and the SCS/AS is replaced by the V2X application specific server. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 6.1.5.x.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative V2X application specific server towards which the uplink message should be redirected. |

Table 6.1.5.x.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative V2X application specific server towards which the uplink message should be redirected. |

# *** Next Change ***

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the VAE_MessageDelivery API.

Table 6.1.6.1-1: VAE_MessageDelivery specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DownlinkMessageDeliveryData | 6.1.6.2.2 | Contains the downlink V2X message delivery data |  |
| GeoId | 6.1.6.3.2 | Geographical area identifier |  |
| MessageDeliverySubscriptionData | 6.1.6.2.3 | Contains the V2X message delivery subscription data |  |
| Result | 6.1.6.3.x | Contains the result of the downlink message delivery | ReceptionReport |
| UplinkMessageDeliveryData | 6.1.6.2.4 | Contains the uplink V2X message delivery data |  |
| V2xGroupId | 6.1.6.3.2 | The group ID for which the V2X message is addressed |  |
| V2xServiceID | 6.1.6.3.2 | The V2X service ID to which the V2X message belongs to |  |
| V2xUeId | 6.1.6.3.2 | Identifier of the destination V2X UE |  |
| V2xMessagePayload | 6.1.6.3.2 | V2X message payload carried by the V2X message |  |

Table 6.1.6.1-2 specifies data types re-used by the VAE_MessageDelivery service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the VAE_MessageDelivery service based interface.

Table 6.1.6.1-2: VAE_MessageDelivery re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Bytes | 3GPP TS 29.571 [11] | String with format "byte" as defined in OpenAPI Specification [6], i.e, base64-encoded characters |  |
| DateTime | 3GPP TS 29.571 [11] | String with format "date-time" as defined in OpenAPI Specification [6]. |  |
| SupportedFeatures | 3GPP TS 29.571 [11] |  |  |
| TestNotification | 3GPP TS 29.122 [22] | Represents a notification that can be sent to test whether a chosen notification mechanism works. | Notification_test_event |
| Uri | 3GPP TS 29.571 [11] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [22] | Pepresents configuration for the delivery of notifications over Websockets. | Notification_websocket |

# *** Next Change ***

6.2.6.3.x	Enumeration: Result

Table 6.2.6.3.x-1: Enumeration Result

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESS | Indicates that the downlink message delivery is successful. |  |
| FAIL | Indicates that the downlink message delivery is failed. |  |

# *** Next Change ***

A.2	VAE_MessageDelivery API

openapi: 3.0.0

info:

version: 1.1.0-alpha.1

title: VAE_MessageDelivery

description: |

API for VAE Message Delivery Service

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.486 V17.0.0 V2X Application Enabler (VAE) Services

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.486/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/vae-message-delivery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

paths:

/subscriptions:

post:

summary: Create a new Individual Message Delivery Data Subscription resource

operationId: CreateIndividualMessageDeliveryDataSubscription

tags:

- Message Delivery Data Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliverySubscriptionData'

responses:

'201':

description: The subscription was created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliverySubscriptionData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

uplinkMessageDelivery:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UplinkMessageDeliveryData'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

receptReportOfDownlinkMesageDelivery:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Result'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Get an existing individual Message Delivery Subscription resource

operationId: ReadIndividualMessageDeliverySubscription

tags:

- Individual Message Delivery Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Individual Message Delivery Subscription

required: true

schema:

type: string

responses:

'200':

description: The subscription information is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliverySubscriptionData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an individual Message Delivery Subscription resource

operationId: DeleteMessageDeliverySubscription

tags:

- Individual Message Delivery Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Individual Message Delivery Subscription

required: true

schema:

type: string

responses:

'204':

description: The subscription was terminated successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}/message-deliveries:

post:

summary: VAE Message delivery resource create service Operation

tags:

- message deliveries collection (Collection)

operationId: CreateDownlinkMessageDelivery

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Individual Message Delivery Subscription

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/DownlinkMessageDeliveryData'

required: true

responses:

'201':

description: Downlink Message Delivery Resource Created

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/DownlinkMessageDeliveryData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/subscriptions/{subscriptionId}/message-deliveries/{dlDeliveryId}:

get:

summary: VAE Message delivery resource Read service Operation

tags:

- Individual downlink message delivery (Document)

operationId: ReadIndividualDownlinkMessageDelivery

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Individual Message Delivery Subscription

required: true

schema:

type: string

- name: dlDeliveryId

in: path

description: Identifier of a downlink messge delivery resource

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/DownlinkMessageDeliveryData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: VAE Message delivery resource delete service Operation

tags:

- Individual message delivery (Document)

operationId: DeleteMessageDelivery

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Individual Message Delivery Subscription

required: true

schema:

type: string

- name: dlDeliveryId

in: path

required: true

description: Unique ID of the message delivery to be deleted

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DownlinkMessageDeliveryData:

description: Contains the downlink V2X message delivery data.

type: object

properties:

ueId:

$ref: '#/components/schemas/V2xUeId'

groupId:

$ref: '#/components/schemas/V2xGroupId'

duration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

geoId:

$ref: '#/components/schemas/GeoId'

payload:

$ref: '#/components/schemas/V2xMessagePayload'

required:

- payload

MessageDeliverySubscriptionData:

description: Represents the V2X message delivery subscription data.

type: object

properties:

appSerId:

$ref: '#/components/schemas/AppServerId'

serviceId:

$ref: '#/components/schemas/V2xServiceId'

geoId:

$ref: '#/components/schemas/GeoId'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by the NF service consumer to request the VAE server to send a test notification as defined in clause 6.1.5.3. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- appSerId

- serviceId

- notifUri

UplinkMessageDeliveryData:

description: Represents the uplink V2X message delivery data.

type: object

properties:

resourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ueId:

$ref: '#/components/schemas/V2xUeId'

geoId:

$ref: '#/components/schemas/GeoId'

payload:

$ref: '#/components/schemas/V2xMessagePayload'

required:

- resourceUri

- ueId

- payload

Result:

description: Contains the result of downlink message delivery.

anyOf:

- type: string

enum:

- SUCCESS

- FAIL

- type: string

AppServerId:

description: Represents the V2X application specific server identifier.

type: string

V2xUeId:

description: Represents the identifier of the V2X UE.

type: string

V2xGroupId:

description: Represents the group ID for which a V2X message is addressed.

type: string

V2xServiceId:

description: Represents the V2X service ID to which a V2X message belongs.

type: string

GeoId:

description: Represents a geographical area identifier.

type: string

V2xMessagePayload:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'