3GPP TSG-CT Meeting #92-e	CP-211318

# E-Meeting, 14th – 16th June 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0920 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAS security context storage in multiple registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | IDEMIA |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-06-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This is a mirror CR for Rel-17 related to CR#0193 as presented for approval in CP-211182 In the LS (C1-210015, S3-203534) from SA3 to CT6, SA3 has identified a security issue which results from the storage of NAS security context on the USIM as specified in TS 31.102. SA3 requires that “when UE is registered over both access (i.e. 3GPP access and non-3GPP access) to two different PLMNs simultaneously, for each PLMN the UE is registered, both NAS count pairs (one pair for 3GPP access and another pair for non-3GPP access) for each NAS security context shall be storated in the USIM. This is because even if a UE is currently not connected to one PLMN over non-3GPP access, the NAS COUNTs for that access may be non-zero from a previous use of the security context on that access. NAS COUNTs shall not be reset without rekeying, as otherwise keystream will be reused”. This contribution proposes to udpate the EF5GS3GPPNSC and EF5GSN3GPPNSC to align with SA3’s requirement. Specifically, each of the EF for storing NAS security contexts is extended to 2 records, and the second record is extended to include PLMN identifier. It is also pointed out stage 2 requirement on 5G-GUTI (clause 5.3.2.4 in TS 23.501), i.e., “When the 3GPP access and the non-3GPP access for the same UE are served by the same PLMN, the AMF assigns the same 5G-GUTI for use over both accesses. Such a 5G-GUTI may be assigned or re-assigned over any of the 3GPP and Non-3GPP accesses. The 5G-GUTI is assigned upon a successful registration of the UE, and is valid over both 3GPP and Non-3GPP access to the same PLMN for the UE”. To align with the GUTI requirement, when the ME receives a new GUTI from PLMN-1, the ME needs to update both EF5GS3GPPLOCIand EF5GSN3GPPLOCI to ensure that the two accesses share the new GUTI if the UE is regsistered to PLMN-1 over both acccesses. Since TS 31.102 provides storage for GUTI, and it is up to the ME implementation to ensure the two accesses of one PLMN shares the same GUTI when the UE is registered over both accesses to this PLMN, so TS 31.102 needs no changes on GUTI storage. Since both the GUTI and the NAS security context have PLMN identifier, they can be associated with each other by checking the PLMN identifier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update EF5GS3GPPNSC and EF5GSN3GPPNSC to allow for NAS security context storage for both accesses for two PLMNs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security issue (e.g. keystream reuse) exists, and misalignment with TS 33.501 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.8, 4.11.1, 4.4.11.4, 4.4.11.5, 5.2.32 Annex D, (new) Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1. Cover page corrections |  |  |  |  |  |  |  |  |

******************************* Start of Change 1******************************

### 4.2.8	EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, (X ≥ 1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n1 to n8 |  |  | M |  | 1 byte |
| 2 | Services n9 to n16 |  |  | O |  | 1 byte |
| 3 | Services n17 to n24 |  |  | O |  | 1 byte |
| 4 | Services n25 to n32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n(8X-7) to n(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | Local Phone Book |
|  | Service n°2: | Fixed Dialling Numbers (FDN) |
|  | Service n°3: | Extension 2 |
|  | Service n°4: | Service Dialling Numbers (SDN) |
|  | Service n°5: | Extension3 |
|  | Service n°6: | Barred Dialling Numbers (BDN) |
|  | Service n°7: | Extension4 |
|  | Service n°8: | Outgoing Call Information (OCI and OCT) |
|  | Service n°9: | Incoming Call Information (ICI and ICT) |
|  | Service n°10: | Short Message Storage (SMS) |
|  | Service n°11: | Short Message Status Reports (SMSR) |
|  | Service n°12: | Short Message Service Parameters (SMSP) |
|  | Service n°13: | Advice of Charge (AoC) |
|  | Service n°14: | Capability Configuration Parameters 2 (CCP2) |
|  | Service n°15: | Cell Broadcast Message Identifier |
|  | Service n°16: | Cell Broadcast Message Identifier Ranges |
|  | Service n°17: | Group Identifier Level 1 |
|  | Service n°18: | Group Identifier Level 2 |
|  | Service n°19: | Service Provider Name |
|  | Service n°20: | User controlled PLMN selector with Access Technology |
|  | Service n°21: | MSISDN |
|  | Service n°22: | Image (IMG) |
|  | Service n°23: | Support of Localised Service Areas (SoLSA) |
|  | Service n°24: | Enhanced Multi-Level Precedence and Pre-emption Service |
|  | Service n°25: | Automatic Answer for eMLPP |
|  | Service n°26: | RFU |
|  | Service n°27: | GSM Access |
|  | Service n°28: | Data download via SMS-PP |
|  | Service n°29: | Data download via SMS-CB |
|  | Service n°30: | Call Control by USIM |
|  | Service n°31: | MO-SMS Control by USIM |
|  | Service n°32: | RUN AT COMMAND command |
|  | Service n°33: | shall be set to '1' |
|  | Service n°34: | Enabled Services Table |
|  | Service n°35: | APN Control List (ACL) |
|  | Service n°36: | Depersonalisation Control Keys |
|  | Service n°37: | Co-operative Network List |
|  | Service n°38: | GSM security context |
|  | Service n°39: | CPBCCH Information |
|  | Service n°40: | Investigation Scan |
|  | Service n°41: | MexE |
|  | Service n°42: | Operator controlled PLMN selector with Access Technology |
|  | Service n°43: | HPLMN selector with Access Technology |
|  | Service n°44: | Extension 5 |
|  | Service n°45: | PLMN Network Name |
|  | Service n°46: | Operator PLMN List |
|  | Service n°47: | Mailbox Dialling Numbers |
|  | Service n°48: | Message Waiting Indication Status |
|  | Service n°49: | Call Forwarding Indication Status |
|  | Service n°50: | Reserved and shall be ignored |
|  | Service n°51: | Service Provider Display Information |
|  | Service n°52 | Multimedia Messaging Service (MMS) |
|  | Service n°53 | Extension 8 |
|  | Service n°54 | Call control on GPRS by USIM |
|  | Service n°55 | MMS User Connectivity Parameters |
|  | Service n°56 | Network's indication of alerting in the MS (NIA) |
|  | Service n°57 | VGCS Group Identifier List (EFVGCS and EFVGCSS) |
|  | Service n°58 | VBS Group Identifier List (EFVBS and EFVBSS) |
|  | Service n°59 | Pseudonym |
|  | Service n°60 | User Controlled PLMN selector for I-WLAN access |
|  | Service n°61 | Operator Controlled PLMN selector for I-WLAN access |
|  | Service n°62 | User controlled WSID list |
|  | Service n°63 | Operator controlled WSID list |
|  | Service n°64 | VGCS security |
|  | Service n°65 | VBS security |
|  | Service n°66 | WLAN Reauthentication Identity |
|  | Service n°67 | Multimedia Messages Storage |
|  | Service n°68 | Generic Bootstrapping Architecture (GBA) |
|  | Service n°69 | MBMS security |
|  | Service n°70 | Data download via USSD and USSD application mode |
|  | Service n°71 | Equivalent HPLMN |
|  | Service n°72 | Additional TERMINAL PROFILE after UICC activation |
|  | Service n°73 | Equivalent HPLMN Presentation Indication |
|  | Service n°74 | Last RPLMN Selection Indication |
|  | Service n°75 | OMA BCAST Smart Card Profile |
|  | Service n°76 | GBA-based Local Key Establishment Mechanism |
|  | Service n°77 | Terminal Applications |
|  | Service n°78 | Service Provider Name Icon |
|  | Service n°79 | PLMN Network Name Icon |
|  | Service n°80 | Connectivity Parameters for USIM IP connections |
|  | Service n°81 | Home I-WLAN Specific Identifier List |
|  | Service n°82 | I-WLAN Equivalent HPLMN Presentation Indication |
|  | Service n°83 | I-WLAN HPLMN Priority Indication |
|  | Service n°84 | I-WLAN Last Registered PLMN |
|  | Service n°85 | EPS Mobility Management Information |
|  | Service n°86 | Allowed CSG Lists and corresponding indications |
|  | Service n°87 | Call control on EPS PDN connection by USIM |
|  | Service n°88 | HPLMN Direct Access |
|  | Service n°89 | eCall Data |
|  | Service n°90 | Operator CSG Lists and corresponding indications |
|  | Service n°91 | Support for SM-over-IP |
|  | Service n°92 | Support of CSG Display Control |
|  | Service n°93 | Communication Control for IMS by USIM |
|  | Service n°94 | Extended Terminal Applications |
|  | Service n°95 | Support of UICC access to IMS |
|  | Service n°96 | Non-Access Stratum configuration by USIM |
|  | Service n°97 | PWS configuration by USIM |
|  | Service n°98 | RFU |
|  | Service n°99 | URI support by UICC |
|  | Service n°100 | Extended EARFCN support |
|  | Service n°101 | ProSe |
|  | Service n°102 | USAT Application Pairing |
|  | Service n°103 | Media Type support |
|  | Service n°104 | IMS call disconnection cause |
|  | Service n°105 | URI support for MO SHORT MESSAGE CONTROL |
|  | Service n°106 | ePDG configuration Information support |
|  | Service n°107 | ePDG configuration Information configured |
|  | Service n°108 | ACDC support |
|  | Service n°109 | Mission Critical Services |
|  | Service n°110 | ePDG configuration Information for Emergency Service support |
|  | Service n°111 | ePDG configuration Information for Emergency Service configured |
|  | Service n°112 | eCall Data over IMS |
|  | Service n°113 | URI support for SMS-PP DOWNLOAD as defined in 3GPP TS 31.111 [12] |
|  | Service n°114 | From Preferred |
|  | Service n°115 | IMS configuration data |
|  | Service n°116 | TV configuration |
|  | Service n°117 | 3GPP PS Data Off |
|  | Service n°118 | 3GPP PS Data Off Service List |
|  | Service n°119 | V2X |
|  | Service n°120 | XCAP Configuration Data |
|  | Service n°121 | EARFCN list for MTC/NB-IOT UEs |
|  | Service n°122 | 5GS Mobility Management Information |
|  | Service n°123 | 5G Security Parameters |
|  | Service n°124 | Subscription identifier privacy support |
|  | Service n°125 | SUCI calculation by the USIM |
|  | Service n°126 | UAC Access Identities support |
|  | Service n°127 | Control plane-based steering of UE in VPLMN |
|  | Service n°128 | Call control on PDU Session by USIM |
|  | Service n°129 | 5GS Operator PLMN List |
|  | Service n°130 | Support for SUPI of type NSI or GLI or GCI |
|  | Service n°131 | 3GPP PS Data Off separate Home and Roaming lists |
|  | Service n°132 | Support for URSP by USIM |
|  | Service n°133 | 5G Security Parameters extended |
|  | Service n°134 | MuD and MiD configuration data |
|  | Service n°135 | Support for Trusted non-3GPP access networks by USIM |
|  | Service n°xxx | Support for multiple records of NAS security context storage for multiple registration |

The EF shall contain at least one byte. Further bytes may be included, but if the EF includes an optional byte, then it is mandatory for the EF to also contain all bytes before that byte. Other services are possible in the future and will be coded on further bytes in the EF. The coding falls under the responsibility of the 3GPP.

******************************* Start of Change 2******************************

#### 4.4.11.1	Introduction

This clause describes the files that are specific for 5GS.

DF5GS shall be present at the ADFUSIM level if any of the following services are "available" in EFUST (USIM Service Table):

| Service n°122 | 5GS Mobility Management Information |
| --- | --- |
| Service n°123 | 5G Security Parameters |
| Service n°124 | Subscription identifier privacy support |
| Service n°125 | SUCI calculation by the USIM |
| Service n°126 | UAC Access Identities support |
| Service n°127 | Control plane-based steering of UE in VPLMN |
| Service n°128 | Call control on PDU Session by USIM |
| Service n°129 | 5GS Operator PLMN List |
| Service n°130 | Support for SUPI of type NSI or GLI or GCI |
| Service n°132 | Support for URSP by USIM |
| Service n°133 | 5G Security Parameters extended |
| Service n°134 Service n°135 Service n°136 | MuD and MiD configuration data Support for Trusted non-3GPP access networks by USIM Pre-configured CAG information list |
| Service n°xxx | Support for multiple records of NAS security context storage for multiple registration |

******************************* Start of Change 3******************************

#### 4.4.11.4	EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

If service n°122 is "available" in EFUST, this file shall be present.

This EF contains the 5GS 3GPP access NAS security context as defined in TS 24.501 [104], consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. This EF also contains the EPS NAS security algorithms to be used when the UE goes to EPS, either by means of connected mode handover in a network that supports N26 interface, or by the means of idle mode mobility performed by the UE from 5GS to EPS, as specified in TS 33.501 [105].

If Service n°xxx is not "available" in EFUST, the file shall contain one record.

If service n°xxx is "available" in EFUST, the file shall contain two records. As specified in TS 33.501 [105] clause 6.3.2.1, in case of multiple registrations in different PLMNs, each record shall store 5GS 3GPP access NAS security context for each different PLMN (Refer to Annex X for examples),

| Identifier: '4F03' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '03' |  |  |  |  |  |  |
| Record size: X bytes (X≥57) |  |  | Update activity: high |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | 5GS NAS Security Context TLV Object |  |  | M |  | X bytes |

5GS NAS Security Context tags

| Description | Tag Value |
| --- | --- |
| 5GS NAS Security Context Tag | 'A0' |

5GS NAS Security Context information

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| 5GS NAS Security Context Tag | 'A0' | M | 1 |
| Length (length of all subsequent data) | Y | M | Note 1 |
| ngKSI Tag | '80' | M | 1 |
| Length | K | M | Note 1 |
| ngKSI | -- | M | K |
| KAMF Tag | '81' | M | 1 |
| Length | L | M | Note 1 |
| KAMF | -- | M | L |
| Uplink NAS count Tag | '82' | M | 1 |
| Length | M | M | Note 1 |
| Uplink NAS count | -- | M | M |
| Downlink NAS count Tag | '83' | M | 1 |
| Length | N | M | Note 1 |
| Downlink NAS count | -- | M | N |
| Identifiers of selected NAS integrity and encryption algorithms Tag | '84' | M | 1 |
| Length | S | M | Note 1 |
| Identifiers of selected NAS integrity and encryption algorithms | -- | M | S |
| Identifiers of selected EPS NAS integrity and encryption algorithms for use after mobility to EPS Tag | '85' | M | 1 |
| Length | U | M | Note 1 |
| Identifiers of selected EPS NAS integrity and encryption algorithms for use after mobility to EPS | -- | M | U |
| PLMN identifier tag | '86' | C (see Note 2) | 1 |
| Length | 3 | C (see Note 2) | Note 1 |
| PLMN identifier | -- | C (see Note 2) | 3 |
| Note 1: The length is coded according to ISO/IEC 8825-1 [35] Note 2: If service n°xxx is "available", PLMN identifier does only apply to record 2 and is mandatory for record 2 |  |  |  |

-	ngKSI Tag '80'

Contents:

The ngKSI (Key Set Identifier in 5G) as defined in TS 33.501 [105] is coded on 1 byte.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ngKSI |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | bits b4 to b8 are coded 0 |

-	KAMF Tag '81'

Contents:

The KAMF as defined in TS 33.501 [105] is coded on 32 bytes. The ME shall treat any KAMF values stored in this EF as invalid if the ngKSI indicates that no KAMF is available or if the length indicated in the KAMF TLV is set to '00',

Coding:

The most significant bit of KAMF is the most significant bit of the 1st byte of this TLV value field. The least significant bit of KAMF is the least significant bit of the last byte of this TLV value field.

-	Uplink NAS count Tag '82'

Contents:

The uplink NAS count as defined in TS 33.501 [105] is coded on 4 bytes.

Coding:

The most significant bit of the uplink NAS count is the most significant bit of the 1st byte of this TLV value field. The least significant bit of the uplink NAS count is the least significant bit of the last byte of this TLV value field.

-	Downlink NAS count Tag '83'

Contents:

The downlink NAS count as defined in TS 33.501 [105] is coded on 4 bytes.

Coding:

The most significant bit of the downlink NAS count is the most significant bit of the 1st byte of this TLV value field. The least significant bit of the downlink NAS count is the least significant bit of the last byte of this TLV value field.

-	Identifiers of selected NAS integrity and encryption algorithms Tag '84'

Contents:

The identifiers of selected NAS integrity and encryption algorithms as defined in TS 33.501 [105] and TS 24.501 [104]. In this release the identifiers of selected NAS integrity and encryption algorithms are coded as 4-bit identifiers.

Coding:

Coding is same as the content of the NAS security algorithms information element defined in TS 24.501 [104].

Byte 1 of this TLV value field: first byte of the value part of the NAS security algorithms information element

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MSB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-	Identifiers of selected EPS NAS algorithms for use after mobility to EPS Tag '85'

Contents:

The identifiers of selected EPS NAS integrity and encryption algorithms to be used when the UE goes to EPS, either by means of connected mode handover in a network that supports N26 interface, or by the means of idle mode mobility performed by the UE from 5GS to EPS, as specified in TS 33.501 [105] and TS 24.501 [104]. These identifiers are coded as 4-bit identifiers.

Coding:

Coding is same as the content of the EPS NAS security algorithms information element specified in TS 24.301 [51].

Byte 1 of this TLV value field: first byte of the value part of the EPS NAS security algorithms information element in TS 24.301 [51].

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MSB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-	PLMN identifier Tag '86'

Contents:

The PLMN identifier comprising MCC and MNC defined in TS 23.003 [25] is coded on 3 bytes.

Coding:

The PLMN identifier is coded according to TS 24.008 [9].

Unused bytes shall be set to 'FF'.

In order to mark the stored 5GS NAS security context as invalid:

-	the record bytes shall be set to 'FF', or

-	the ngKSI is set to '07', or

-	the length indicated in the KAMF TLV is set to '00'.

************************** Start of Change 4***************************

#### 4.4.11.5	EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)

If Service n°122 is "available" in EFUST, this file shall be present.

This EF contains the 5GS non-3GPP access NAS security context as defined in TS 24.501 [104], consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. This EF also contains the EPS NAS security algorithms to be used when the UE goes to EPS, either by means of connected mode handover in a network that supports N26 interface, or by the means of idle mode mobility performed by the UE from 5GS to EPS, as specified in TS 33.501 [105].

If Service n°xxx is not "available" in EFUST, the file shall contain one record.

If service n°xxx is "available" in EFUST, the file shall contain two records. As specified in TS 33.501 [105] clause 6.3.2.1, in case of multiple registrations in different PLMNs, each record shall store 5GS 3GPP access NAS security context for each different PLMN (Refer to Annex X for examples).

| Identifier: '4F04' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| Record size: X bytes (X≥57) |  |  | Update activity: high |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | 5GS NAS Security Context TLV Object |  |  | M |  | X bytes |

For content and coding see clause 4.4.11.4 for EF5GS3GPPNSC.

************************** Start of Change 5***************************

### 5.2.32	5GS NAS Security Context

Requirement:	Service n°122 "available".

Request:	The ME performs the reading procedure with EF5GS3GPPNSC and EF5GSN3GPPNSC.

If service n°xxx is "available" in addition, the ME shall associate the NAS security context to the appropriate PLMN by parsing the two records and by retrieving the matching PLMN identifier of the second record.

5GS NAS Security Context in the first record shall be associated with 5G GUTI in the 5G LOCI file for the same access type (e.g. EF5GS3GPPNSC) and the 5GS NAS Security Context in the second record shall be associated with the 5G GUTI as indicated in the PLMN Identifier.

Editor’s Note: 	How multiple registrations for the case that a previous NAS security context exists, has to be defined for the NAS COUNT values in TS 24.501 [104]

In order to prevent UICC memory wear out due to excessive writing, the update of 5GS NAS security context shall be according to the rules and procedures specified in TS 33.501 [104].

************************** Start of Change 6***************************

Annex D (informative):
Tags defined in 31.102

| Tag | Name of Data Element | Usage |
| --- | --- | --- |
| '43' | Full name for network IEI | PLMN Network Name (EFPNN) |
| '45' | Short name for network IEI | PLMN Network Name (EFPNN) |
| '53' | MBMS Data Object | AUTHENTICATE command parameter, in MBMS security context |
| '53' | MBMS operation response Data Object The following tags are encapsulated within '53' 'DB' successful MBMS operation tag | Response to AUTHENTICATE command, in MBMS security context |
| '73' | Key Derivation Data Object The following tags are encapsulated within '73' '80' Local Key Establishment Control tag '81' Counter limit tag '82' Request MAC tag '83' NAF_ID tag '84' Terminal_ID tag '85' Terminal_appli_ID_tag '86' UICC_appli_ID tag '87' RANDx tag 'A0' Key Identifier tag | AUTHENTICATE command parameter, in Local Key Establishment security context |
| '73' | Key Derivation Operation Response Object The following tags are encapsulated within '73' '80' Local Key Establishment Control tag '82' Request MAC tag | Response to AUTHENTICATE command, in Local Key Establishment security context |
| '73' | Key Availability Check Data Object The following tags are encapsulated within '73' '80' Local Key Establishment Control tag 'A0' Key Identifier tag | AUTHENTICATE command parameter in Local Key Establishment security context |
| '80' | NAF_ID tag | GBA NAF List (EFGBANL) |
| '80' | NAF Key Centre address tag | NAF Key Centre Address (EFNAFKCA) |
| '80' | Icon Tag (Icon link is URI) | Service Provider Name Icon (EFSPNI) |
| '80' | Reauthentication Identity tag | WLAN Reauthentication Identity (EFWRI) |
| '80' | NAS signalling priority Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '80' | MMS Implementation tag | MMS User Preference (EFMMSUP) Multimedia Messages List (EFMML) |
| '80' | IARI TLV TAG | UICC IARI (EFUICCIARI) |
| '80' | Graphics CSG Type tag (Icon link is URI) | CSG Type (EFCSGT) |
| '80' | HNB Name tag | Home NodeB Name (EFHNBN) |
| '80' | PLMN Additional information tag | PLMN Network Name (EFPNN) |
| '80' | ICE Free Format Label tag | In Case of Emergency – Free Format (EFICE-FF) |
| '80' | HPLMN ProSe Function tag | Address of the HPLMN ProSe Function (EFPROSEFUNC) |
| '80' | ProSe Group Counter | Counter for ProSe group (EFPROSE_GC) |
| '80' | ProSe ServerAddress tag | Server address for usage information reports (EFPROSE_UIRC) |
| '80' | Home ePDG Identifier TLV | Home ePDG Identifier (EFePDGId) |
| '80' | ePDG Selection Information TLV | ePDG Selection Information (EFePDGSelection) |
| '80' | Emergency ePDG Identifier TLV | Emergency ePDG Identifier (EFePDGIdEm) |
| '80' | ePDG Selection Information for Emergency Services TLV | ePDG Selection Information for Emergency Services (EFePDGSelectionEm) |
| '80' | XCAP_conn_params_policy TLV TAG | EFXCAPConfigData |
| '80' | Serving network name TLV tag | EFTN3GPPSNN |
| '80' | IMS configuration data encoding | EFIMSConfigData |
| '81' | IMS configuration data | EFIMSConfigData |
| '80' | KAUSF tag | EF5GAUTHKEYS |
| '80' | Protection Scheme Identifier List data object tag | Protection Scheme Identifier List (EFSUCI_Calc_Info) |
| '80' | Network Specific Identifier TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |
| '81' | Global Line Identifier Tag TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |
| '82' | Global Cable Identifier TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |
| '80' | MuD_and_MiD_configuration_data encoding | EFMuDMiDConfigData |
| '81' | MuD_and_MiD_configuration_data | EFMuDMiDConfigData |
| '81' | B-TID tag | GBA NAF List (EFGBANL) |
| '81' | Icon Tag (Icon link is record number) | Service Provider Name Icon (EFSPNI) |
| '81' | Master key tag | WLAN Reauthentication Identity (EFWRI) |
| '81' | Time Stamp counter tag | MBMS User Key (EFMUK) |
| '81' | MMS User preference profile name tag | MMS User Preference (EFMMSUP) |
| '81' | Login Tag | Network Connectivity Parameters for USIM IP connections (EFNCP-IP) |
| '81' | NMO I Behaviour Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '81' | Graphics CSG Type tag (Icon link is record number) | CSG Type (EFCSGT) |
| '81' | ICE Free Format Content tag | In Case of Emergency – Free Format (EFICE-FF) |
| '81' | MM File Identifier / SFI tag | Multimedia Messages List (EFMML) |
| '81' | ProSe CollectionPeriod tag | Collection Period Parameter (EFPROSE_UIRC) |
| '82' | Counter tag | WLAN Reauthentication Identity (EFWRI) |
| '82' | MMS User Preference information tag | MMS User Preference (EFMMSUP) |
| '82' | Password Tag | Network Connectivity Parameters for USIM IP connections (EFNCP-IP) |
| '82' | Attach with IMSI Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '82' | MM Content Data Object Tag | Multimedia Messages List (EFMML) |
| '82' | ProSe ReportingWindow tag | Reporting Window Parameter (EFPROSE_UIRC) |
| '82' | KSEAF for non-3GPP access tag | EF5GAUTHKEYS |
| '80' | Home Network Public Key Identifier tag | Home Network Public Key Identifier (EFSUCI_Calc_Info) |
| '81' | KSEAF for 3GPP access tag | EF5GAUTHKEYS |
| '83' | Data Destination Address Range Tag | Network Connectivity Parameters for USIM IP connections (EFNCP-IP) |
| '83' | Minimum Periodic Search Timer Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '83' | MM Size tag | Multimedia Messages List (EFMML) |
| '83' | SOR counter tag | EF5GAUTHKEYS |
| '84' | UE parameter update counter tag | EF5GAUTHKEYS |
| '83' | ProSe ReportGroupParameters tag | Reporting Parameter for Goups (EFPROSE_UIRC) |
| '81' | Home Network Public Key tag | Home Network Public Key (EFSUCI_Calc_Info) |
| '84' | Bearer Description Tag | Network Connectivity Parameters for USIM IP connections (EFNCP-IP) |
| '84' | Extended access barring Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '84' | MM Status tag | Multimedia Messages List (EFMML) |
| '84' | ProSe ReportTimeStampsFirstTransmissionAndReception tag | Reporting Parameter (EFPROSE_UIRC) |
| '85' | Timer T3245 Behaviour Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '85' | MM Alpha Identifier tag | Multimedia Messages List (EFMML) |
| '85' | ProSe ReportDataTransmitted tag | Reporting Parameter for transmitted Data (EFPROSE_UIRC) |
| '86' | Override NAS signalling low priority Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '86' | ProSe ReportDataReceived tag | Reporting Parameter for received Data (EFPROSE_UIRC) |
| '86' | PLMN-ID tag | PLMN-ID (EF5GS 3GPP Access NAS Security Context) |
| '87' | Override Extended access barring Tag | Non Access Stratum Configuration (EFNASCONFIG) |

….

************************** Start of Change 7***************************

Annex X (informative):
Examples of NAS security contexts management in multiple registrations

|  | Scenario 1 |  | 5GS3GPPLOCI | 5GS3GPPNSC |  | 5GSN3GPPLOCI | 5GSN3GPPNSC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Record 1 | Record 2 |  | Record 1 | Record 2 |
| 1 |  | UE is authenticated to PLMN-A with dual connection (3GPP and non-3GPP) | GUTI-A1 | 3gppNSC-A1 | - | GUTI-A1 | n3gppNSC-A1 | - |
| 2 |  | UE switches from PLMN-A to PLMN-B on non-3GPP |  |  |  | Use GUTI-A1 from 5GSN3GPPLOCI and n3gppNSC-A1 from 5GSN3GPPNSC |  |  |
| 3 |  | UE successful REGISTRATION to PLMN-B on non-3GPP | GUTI-A1 | 3gppNSC-A1 | - | GUTI-B1 | n3gppNSC-B1 | n3gppNSC-A1 |
| 4 |  | UE successful REGISTRATION to PLMN-B over 3GPP | GUTI-B1 | 3gppNSC-B1 | - | GUTI-B1 | n3gppNSC-B1 | - |
| 5 |  | UE switches-off |  |  |  |  |  |  |
| 6 |  | UE stores to USIM the 3GPP Security Context (PLMN-B) and the non-3GPP Security Context (PLMN-B) | GUTI-B1 | 3gppNSC-B1 | - | GUTI-B1 | n3gppNSC-B1 | - |
| 7 |  | UE clears all Security Contexts |  |  |  |  |  |  |
| 8 |  | UE switches-on back |  |  |  |  |  |  |
| 9 |  | UE reads from USIM the 3GPP Security Context (PLMN-B) and the non-3GPP Security Context (PLMN-B) | GUTI-B1 | 3gppNSC-B1 | - | GUTI-B1 | n3gppNSC-B1 | - |
| 10 |  | UE tries to reconnect to PLMN-C on non-3GPP Security Context |  |  |  | Use GUTI-B1 from 5GSN3GPPLOCI and n3gppNSC-B1 from 5GSN3GPPNSC |  |  |
| 11 |  | UE gets REGISTRATION successful on PLMN-C over non-3GPP | GUTI-B1 | 3gppNSC-B1 | - | GUTI-C1 | n3gppNSC-C1 | n3gppNSC-B1 |
| 12 |  | UE tries to reconnect to PLMN-C on 3GPP | Use GUTI-C1 from 5GS3GPPLOCI |  |  |  |  |  |
| 13 |  | UE gets REGISTRATION successful on PLMN-C over 3GPP | GUTI-C2 | 3gppNSC-C1 |  | GUTI-C2 | n3gppNSC-C1 | - |
| 14 |  | UE switches-off |  |  |  |  |  |  |
| 15 |  | UE stores to USIM the 3GPP Security Context (PLMN-C) and the non-3GPP Security Context (PLMN-C) | GUTI-C2 | 3gppNSC-C1 |  | GUTI-C2 | n3gppNSC-C1 | - |
| 16 |  | UE switches to PLMN-D over non-3GPP | GUTI-C2 | 3gppNSC-C1 |  | GUTI-D1 | n3gppNSC-D1 | n3gppNSC-C1 |
| 17 |  | UE switches to PLMN-D over 3GPP | GUTI-D2 | 3gppNSC-D1 | - | GUTI-D2 | n3gppNSC-D1 | - |
| 18 |  | UE switches to PLMN-C over non-3GPP | GUTI-D2 | 3gppNSC-D1 | - | GUTI-C3 | n3gppNSC-C2 | n3gppNSC-D1 |