3GPP TSG-CT WG1 Meeting #131-e	C1-214935

# E-meeting, 19-27 August 2021																   was C1-214516

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.546 | CR | 0007 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | add VAL UE Information to configuration management procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA6, CR S6-211483 has been agreed – which adds new generic IE to contain VAL UE specific information. This container will be used by the SCM-S to provide different configuration based on different device type or any other VAL UE specific information. It is required to support new IE in stage#3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new parameter while retrieving VAL UE configuration data |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage#3 specification will not be aligned to stage#2 specification. Further, the SCM-S will not be able to provide device specific approprite configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1, A.2 (NEW), A.2.1 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 6.2.3.1	Client procedure

Upon receiving a request from the VAL user to retrieve a VAL UE configuration data, the SCM-C shall send an HTTP GET request to the SCM-S according to procedures specified in IETF RFC 4825 [3] "Fetch a Document". In HTTP GET request, the SCM-C:

a)	shall set the Request-URI to a XCAP URI identifying the XML document to be retrieved. In the Request-URI:

1)	the "XCAP Root" is set to the URI of the SCM-S;

2)	the "auid" is set to specific VAL service identity; and

3)	the document selector is set to a document URI pointing to the VAL UE configuration document;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [6]; and

c)	may include the parameters specified in clause A.2.1 serialized into a JavaScript Object Notation (JSON) structure as specified in IETF RFC 7159 [7].

***** Next change *****

# A.2	Retrieve VAL UE configuration data

## A.2.1	Client side parameters

The SGM-C shall convey the following parameters, if available, while sending request to retrieve a VAL UE configuration data.

Table A.1.2-1: Client side parameters to retrieve VAL UE configuration data

| Parameter | Description |
| --- | --- |
| VAL UE Information | OPTIONAL. Represents additional UE related information required to identify the configuration data (e.g. device type, device vendor, etc). |

***** End of change *****