3GPP TSG-CT Meeting #93-e	CP-212161

# E-Meeting, 13th – 15th September 2021	(was C4-214669)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0372 | rev | 2 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NIDD Authorization data storage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 23.502 in chapter 4.25.6 specifies a mechanism to notify NEF about authorization updates performed in UDM, so that if an already authorized identity, S-NSSAI ir DNN is not allowed to be used for NIDD configuration, UDM is able to revoke or refresh this authorization. TS 29.503 includes NIDD Authorization Data Update Notification operation in chapter 5.7.2.3.2 whose callback reference is provided as part of NIDD Authorization Data Retrieval operation (authUpdateCallbackUri in AuthorizationInfo information element in chapter 6.6.6.2.6). However, TS 29.505 does not provide a operation to keep authUpdateCallbackUri so that it can be later on used when the authorization is changed to notify NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new resource is included in 29.505 to allow the storage and retrieval of the active authorizations data (NiddAuthorizations) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Notifications of NIDD Authorization changes will not be executed, not providing SA2 functionality described in chapter 4.25.6 in TS 23.502. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.X (new), 5.4.1, 5.4.2.X (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible corrections with impacts on the following APIs: - TS29504_Nudr_DR.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 2 (to CT#93-e): Correct the reference to AuthorizationInfo (it is defined in Nudm_NIDDAU, and not in Nudm_EE) |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 5.2	Resources

### 5.2.1	Overview

# Figure 5.2.1-1: Resource URI sub-level structure for subscription data

# Figure 5.2.1-2: Resource URI sub-level structure for subscription data (cont.)

# Figure 5.2.1-3: Resource URI sub-level structure for subscription data (cont.)

Table 5.2.1-1 provides an overview of the resources and applicable HTTP methods.

# Table 5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| AuthenticationSubscription | /subscription-data/{ueId}/authentication-data/authentication-subscription | GET | Retrieve a UE's authentication subscription data |
|  |  | PATCH | Update a UE's authentication subscription data Updates shall be limited to the sequenceNumber attribute. Attempts to patch any other attribute shall be rejected by the UDR. |
| AuthenticationSoR | /subscription-data/{ueId}/ue-update-confirmation-data/sor-data | PUT | Store UE's SOR acknowledgement information (SoR-XMAC-IUE) |
|  |  | GET | Retrieve UE's SoR acknowledgement information |
| AuthenticationUPU | /subscription-data/{ueId}/ue-update-confirmation-data/upu-data | PUT | Store UE Parameter Update acknowledgement information (UPU-XMAC-IUE) |
|  |  | GET | Retrieve UE Parameter Update acknowledgement information |
| SubscribedSNSSAIs | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais | PUT | Store UE-acknowledgement info for change of subscribed S-NSSAIs |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed S-NSSAIs |
| SubscribedCAG | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag | PUT | Store UE-acknowledgement info for change of subscribed CAG |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed CAG |
| AuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status | PUT | Store a UE's authentication status |
|  |  | GET | Retrieve a UE's authentication status |
|  |  | DELETE | Delete a UE's authentication status |
| IndividualAuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status/{servingNetworkName} | PUT | When the feature "PerUePerSnAuthStatus" is supported, store a UE's Individual authentication status |
|  |  | GET | When the feature "PerUePerSnAuthStatus" is supported, retrieve a UE's Individual authentication status |
|  |  | DELETE | When the feature "PerUePerSnAuthStatus" is supported, delete a UE's Individual authentication status |
| ProvisionedData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data | GET | Retrieve the UE's subscribed Provisioned Data |
| AccessAndMobilitySubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/am-data | GET | Retrieve the UE's subscribed Access and Mobility Data |
| SmfSelectionSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data | GET | Retrieve the UE's subscribed SMF Selection Data |
| SessionManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data | GET | Retrieve the UE's subscribed SM Subscription Data |
| ContextData | /subscription-data/{ueId}/context-data | GET | Retrieve the UE's context Data |
| Amf3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-3gpp-access | PUT | Create and Update the AMF registration for 3GPP access |
|  |  | PATCH | Modify the AMF registration for 3GPP access |
|  |  | GET | Retrieve the AMF registration information for 3GPP access |
| AmfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-non-3gpp-access | PUT | Update the AMF registration for non 3GPP access |
|  |  | PATCH | Modify the AMF registration for non 3GPP access |
|  |  | GET | Retrieve the AMF registration information for non 3GPP access |
| SmfRegistrations | /subscription-data/{ueId}/context-data/smf-registrations | GET | Retrieve the list of the SMF registrations |
| IndividualSmfRegistration | /subscription-data/{ueId}/context-data /smf-registrations/{pduSessionId} | PUT | Store an individual SMF registration identified by PDU Session Id |
|  |  | DELETE | Delete an individual SMF registration |
|  |  | GET | Retrieve individual SMF registration |
| OperatorSpecificData | /subscription-data/{ueId}/operator-specific-data | GET | retrieve the operator specific subscription data of a UE |
|  |  | PATCH | modify the operator specific subscription data of a UE |
| OperatorDeterminedBarringData | /subscription-data/{ueId}/operator-determined-barring-data | GET | Retrieve the UE's Operator Determined Barring |
| SMSManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data | GET | Retrieve the UE's subscribed SMS management subscription data. |
| Smsf3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-3gpp-access | PUT | Create or Update the SMSF registration |
|  |  | DELETE | Delete the SMSF registration for 3GPP access |
|  |  | GET | Retrieve the SMSF registration information |
| SmsfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-non-3gpp-access | PUT | Create or Update the SMSF registration for non 3GPP access |
|  |  | DELETE | Delete the SMSF registration for non 3GPP access |
|  |  | GET | Retrieve the SMSF registration information for non 3GPP access |
| IpSmGwRegistration | /subscription-data/{ueId}/context-data/ip-sm-gw | PUT | Create or Update the IP-SM-GW registration |
|  |  | DELETE | Delete the IP-SM-GW registration |
|  |  | PATCH | Modify the IP-SM-GW registration |
|  |  | GET | Retrieve the IP-SM-GW registration information |
| MessageWaitingData | /subscription-data/{ueId}/context-data/mwd | PUT | Create or Update the SMS Message Waiting Data |
|  |  | DELETE | Delete the SMS Message Waiting Data |
|  |  | PATCH | Modify the SMS Message Waiting Data |
|  |  | GET | Retrieve the SMS Message Waiting Data |
| SdmSubscriptions | /subscription-data/{ueId}/context-data/sdm-subscriptions | GET | Retrieve SDM subscriptions |
|  |  | POST | Create an individual SDM subscription |
| IndividualSdmSubscription | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId} | PUT | Update an individual SDM subscription |
|  |  | DELETE | Delete an individual SDM subscription |
|  |  | PATCH | Update an individual SDM Subscription |
|  |  | GET | Retrieve an individual SDM subscription |
| EeSubscriptions | /subscription-data/{ueId}/context-data/ee-subscriptions | GET | Retrieve EE subscriptions |
|  |  | POST | Create an EE subscription |
| IndividualEeSubscription | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription |
|  |  | DELETE | Delete an individual EE subscription |
|  |  | PATCH | Update an individual EE subscription |
|  |  | GET | Retrieve an individual EE subscription |
| AmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions | PUT | Store information related to the Amf-EE-Subscription response |
|  |  | DELETE | Delete the Amf-EE-subscriptions |
|  |  | GET | Retrieve AMF-subscriptions |
|  |  | PATCH | Update AMF-subscriptions |
| SmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/smf-subscriptions | PUT | Store information related the a received Smf-EE-Subscription response |
|  |  | DELETE | Delete the Smf-EE-subscriptions |
|  |  | GET | Retrieve SMF-subscriptions |
|  |  | PATCH | Update SMF-subscriptions |
| HssSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/hss-subscriptions | PUT | Store information related to the Hss-EE-Subscriptions |
|  |  | DELETE | Delete the Hss-EE-subscriptions |
|  |  | GET | Retrieve Hss-EE-subscriptions |
|  |  | PATCH | Update Hss-EE-subscriptions |
| EeProfileData | /subscription-data/{ueId}/ee-profile-data | GET | Retrieve the UE's subscribed EE profile data. |
| ProvisionedParamenterData | /subscription-data/{ueId}/pp-data | PATCH | Update of provisioned parameters. |
|  |  | GET | Retrieves the UE's provisioned parameters. |
| PpProfileData | /subscription-data/{ueId}/pp-profile-data | GET | Retrieve the UE's subscribed PP profile data. |
| ProvisionedParamenterDataEntry | /subscription-data/{ueId}/pp-data-store/{afInstanceId} | PUT | Create a Provisioned Parameter Data Entry |
|  |  | DELETE | Delete a Provisioned Parameter Data Entry |
|  |  | GET | Retrieve a Provisioned Parameter Data Entry |
| SMSSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-data | GET | Retrieve the UE's subscribed SMS subscription data. |
| SubscriptionDataSubscriptions | /subscription-data/subs-to-notify | GET | Retrieve existing subscriptions |
|  |  | POST | Create a subscription, i.e. subscribe a node to receive notification for change of data |
|  |  | DELETE | Deletes multiple subscriptions for a given UE. |
| IndividualSubscriptionDataSubscription | /subscription-data/subs-to-notify/{subsId} | DELETE | Delete the subscription identified by {subsId}, i.e. unsubscribe a node to receive notification for change of data |
|  |  | PATCH | Update an individual Subscription to notification |
|  |  | GET | Retrieve an individual Subscription to notification |
| EeGroupSubscriptions | /subscription-data/group-data/{ueGroupId}/ee-subscriptions | GET | Retrieve EE subscriptions for groups of UEs |
|  |  | POST | Create an EE subscription for groups of UEs |
| IndividualEeGroupSubscription | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription for a group of UEs |
|  |  | DELETE | Delete an individual EE subscription for a group of UEs |
|  |  | PATCH | Update an individual EE subscription for a group of UEs |
|  |  | GET | Retrieve an individual EE subscription for a group of UEs |
| EeGroupProfileData | /subscription-data/group-data/{ueGroupId}/ee-profile-data | GET | Retrieve the subscribed EE profile data for a group of UEs. |
| TraceData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/trace-data | GET | Retrieve the UE's trace configuration data |
| IdentityData | /subscription-data/{ueId}/identity-data | GET | Retrieve identity data that corresponds to the provided ueId |
| SharedData (Collection) | /subscription-data/shared-data | GET | Retrieve shared data |
| GroupIdentifiers | /subscription-data/group-data/group-identifiers | GET | Retrieve group identifiers and the UE identifiers belong to the group identifiers. |
| 5GVnGroups | /subscription-data/group-data/5g-vn-groups | GET | Retrieve 5G VN Groups |
| Individual5GVnGroup | /subscription-data/group-data/5g-vn-groups/{externalGroupId} | PUT | Create a 5G VN Group |
|  |  | PATCH | Update a 5G VN Group |
|  |  | DELETE | Delete a 5G VN Group |
|  |  | GET | Retrieve a 5G VN Group |
| 5GVnGroupsInternal | /subscription-data/group-data/5g-vn-groups/internal | GET | Retrieve 5G VN Group Data based on Internal Group Identifier(s) |
| Pp5gVnGroupProfileData | /subscription-data/group-data/5g-vn-groups/pp-profile-data | GET | Retrieve the UE's subscribed PP profile data for accessing 5G VN Groups service operations. |
| LcsPrivacySubscriptionData | /subscription-data/{ueId}/lcs-privacy-data | GET | Retrieve the UE's subscribed LCS privacy Subscription Data |
| LcsMobileOriginatedSubscriptionData | /subscription-data/{ueId}/lcs-mo-data | GET | Retrieve the UE's subscribed LCS Mobile Originated Subscription Data |
| NiddAuthorizationData | /subscription-data/{ueId}/nidd-authorization-data | GET | Retrieve the UE's NIDD Authorization Data |
| CoverageRestrictionData | /subscription-data/{ueId}/coverage-restriction-data | GET | Retrieve the UE's subscribed enhanced Coverage Restriction Data |
| Location | /subscription-data/{ueId}/context-data/location | GET | Retrieve the UE's Location Information |
| V2xSubscriptionData | /subscription-data/{ueId}/v2x-data | GET | Retrieve the UE's subscribed V2X Data |
| ProseSubscriptionData | /subscription-data/{ueId}/prose-data | GET | Retrieve the UE's subscribed ProSe Service Data |
| LcsBroadcastAssistanceSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-bca-data | GET | Retrieve the UE's subscribed LCS Broadcast Assistance subscription data |
| NiddAuthorizations | /subscription-data/{ueId}/context-data/nidd-authorizations | PUT | Store information related to the NIDD Authorization |
|  |  | DELETE | Delete the NIDD Authorizations |
|  |  | GET | Retrieve NIDD Authorizations |
|  |  | PATCH | Update a specific NIDD Authorization |

* * * Next Change * * * *

### 5.2.X	Resource: NiddAuthorizations

#### 5.2.X.1	Description

This resource represents information relevant to NIDD authorizations. After successful NIDD authorization, the UDM needs to store within the UDR related information (e.g. authUpdateCallbackUri), so that it is possible to notify the UDM consumer later on if something has changed affecting these authorizations.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.X.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/nidd-authorizations

# This resource shall support the resource URI variables defined in table 5.2.X.2-1.

# Table 5.2.X.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |

#### 5.2.X.3	Resource Standard Methods

##### 5.2.X.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.X.3.1-1.

# Table 5.2.X.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.X.3.1-2 and the response data structure, and response codes specified in table 5.2.X.3.1-3.

# Table 5.2.X.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NiddAuthorizationInfo | M | 1 | NIDD Authorization Data |

# Table 5.2.X.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful update of this resource, an empty response body shall be returned. |
| NiddAuthorizationInfo | M | 1 | 201 Created | Upon successful creation of this resource, a representation of the created resource containing an array of the NiddAuthorizationData as allocated by the UDR shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.X.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.X.3.2-1.

# Table 5.2.X.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.X.3.2-2 and the response data structure, and response codes specified in table 5.2.X.3.2-3.

# Table 5.2.X.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.X.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.X.3.3	GET

# This method shall support the URI query parameters specified in table 5.2.X.3.3-1.

# Table 5.2.X.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.X.3.3-2 and the response data structure and response codes specified in table 5.2.X.3.3-3.

# Table 5.2.X.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.X.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NiddAuthorizationInfo | M | 1 | 200 OK | Upon success, a response body containing SMF Subscription Infos shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.X.3.4	PATCH

This method is used to modify the NIDD active authorizations in the UDR.

This method shall support the URI query parameters specified in table 5.2.X.3.4-1.

# Table 5.2.X.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.X.3.2-2 and the response data structures, and response codes specified in table 5.2.X.3.4-3.

# Table 5.2.X.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the provisioned parameter data |

# Table 5.2.X.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

* * * Next Change * * * *

### 5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationSubscription | 5.4.2.2 | A UE's authentication data |
| OperatorSpecificDataContainer | 5.4.2.3 | Container for operator specific data |
| SmfRegList | 5.4.2.4 | The list of all the SMF registrations of a UE |
| SubscriptionDataSubscriptions | 5.4.2.5 | A subscription to notifications. |
| DataChangeNotify | 5.4.2.6 | Container for data which have changed and notification was requested when changed. |
| IdentityData | 5.4.2.7 | Identity data corresponds to the provided ueId |
| ProvisionedDataSets | 5.4.2.8 | Contains the provisioned data sets |
| SorData | 5.4.2.9 | Used to store the status of the latest SOR data update |
| UpuData | 5.4.2.9A | Used to store the status of the latest UPU data update |
| NssaiAckData | 5.4.2.9B | Used to store the status of the latest NSSAI data update |
| CagAckData | 5.4.2.9C | Used to store the status of the latest CAG data update |
| AmfSubscriptionInfo | 5.4.2.19 | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |
| EeProfileData | 5.4.2.20 | Event Exposure Profile Data |
| ContextDataSets | 5.4.2.22 | Contains the context data sets |
| SequenceNumber | 5.4.2.23 | Contains the SQN |
| MessageWaitingData | 5.4.2.24 | Message Waiting Data list |
| SmscData | 5.4.2.25 | Addresses of SM-Service Center entities with SMS wating to be delivered to the UE |
| SmfSubscriptionInfo | 5.4.2.26 | Information the UDR stores and retrieves related to active subscriptions at the SMF(s) |
| SmfSubscriptionItem | 5.4.2.27 | Contains info about a single SMF event subscription |
| MtcProvider | 5.4.2.28 | MTC provider information |
| HssSubscriptionInfo | 5.4.2.29 | Information the UDR stores related to active subscriptions at the HSS(s) |
| HssSubscriptionItem | 5.4.2.30 | Contains info about a single HSS event subscription |
| EeGroupProfileData | 5.4.2.31 | The Event Exposure Profile Data for a group of UEs |
| Pp5gVnGroupProfileData | 5.4.2.32 | The Paramter Provision Profile Data for 5G VN groups |
| PpProfileData | 5.4.2.33 | The Paramter Provision Profile Data |
| AllowedMtcProviderInfo | 5.4.2.34 | Allowed MTC Providers or AFs Information to provision parameters for UE. |
| AuthMethod | 5.4.3.3 | Contains the Authentication Method |
| DataSetName | 5.4.3.4 | The name of data set |
| ContextDataSetName | 5.4.3.6 | The name of context data set |
| SqnScheme | 5.4.3.7 | Scheme for generation of Sequence Numbers |
| Sign | 5.4.3.8 | Sign of the DIF value |
| UeUpdateStatus | 5.4.3.9 | Status of the procedure |
| PpDataType | 5.4.3.10 |  |
| NiddAuthorizationInfo | 5.4.2.X | Information the UDR stores and retrieves related to active NIDD authorizations |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| Data type |  | Reference |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| AccessAndMobilitySubscriptionData |  | 3GPP TS 29.503 [6] |  | Access and Mobility Subscription Data |  |
| SmfSelectionSubscriptionData |  | 3GPP TS 29.503 [6] |  | SMF Selection Subscription Data |  |
| SessionManagementSubscriptionData |  | 3GPP TS 29.503 [6] |  | Session Management Subscription Data |  |
| Amf3GppAccessRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| AmfNon3GppAccessRegistration |  | 3GPP TS 29.503 [6] |  | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |  |
| SmfRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| SmsfRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| SmsManagementSubscriptionData |  | 3GPP TS 29.503 [6] |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [3] |  | Used to negotiate the applicability of the optional features |  |
| ProblemDetails |  | 3GPP TS 29.571 [3] |  | Detailed information about the status code. |  |
| PpData |  | 3GPP TS 29.503 [6] |  |  |  |
| PpDataEntry |  | 3GPP TS 29.503 [6] |  | Provisioned Parameter Data Entry |  |
| SmsSubscriptionData |  | 3GPP TS 29.503 [6] |  |  |  |
| PatchItem |  | 3GPP TS 29.571 [3] |  | Data structure used for JSON patch. |  |
| SdmSubscription |  | 3GPP TS 29.503 [6] |  |  |  |
| EeSubscription |  | 3GPP TS 29.503 [6] |  |  |  |
| Uri |  | 3GPP TS 29.571 [3] |  |  |  |
| TraceData |  | 3GPP TS 29.571 [3] |  |  |  |
| Dnn |  | 3GPP TS 29.571 [3] |  | Data Network Name |  |
| Snssai |  | 3GPP TS 29.571 [3] |  | Single NSSAI |  |
| VarUeId |  | 3GPP TS 29.571 [3] |  | String represents the SUPI or GPSI. |  |
| NfInstanceId |  | 3GPP TS 29.571 [3] |  |  |  |
| NotifyItem |  | 3GPP TS 29.571 [3] |  | Common data type used for data change notification. |  |
| OdbData |  | 3GPP TS 29.571 [3] |  | Operator Determined Barring Data |  |
| EventType |  | 3GPP TS 29.503 [6] |  |  |  |
| ExtGroupId |  | 3GPP TS 29.503 [6] |  | External Group Identifier |  |
| PduSessionId |  | 3GPP TS 29.571 [3] |  |  |  |
| DateTime |  | 3GPP TS 29.571 [3] |  |  |  |
| UpuMac |  | 3GPP TS 29.509 [15] |  |  |  |
| 5GVnGroupConfiguration |  | 3GPP TS 29.503 [6] |  |  |  |
| PatchResult |  | 3GPP TS 29.571 [3] |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [3] |  |  |  |
| AppPortId |  | 3GPP TS 29.503 [6] |  |  |  |
| LcsPrivacyData |  | 3GPP TS 29.503 [6] |  | LCS Privacy Subscription Data |  |
| LcsMoData |  | 3GPP TS 29.503 [6] |  | LCS Mobile Originated Subscription Data |  |
| MtcProviderInformation |  | 3GPP TS 29.571 [3] |  |  |  |
| AuthorizationData |  | 3GPP TS 29.503 [6] |  |  |  |
| EnhancedCoverageRestrictionData |  | 3GPP TS 29.503 [6] |  |  |  |
| LocationInfo |  | 3GPP TS 29.503 [6] |  |  |  |
| V2xSubscriptionData |  | 3GPP TS 29.503 [6] |  | V2X Subscription Data |  |
| E164Number |  | 3GPP TS 29.503 [6] |  |  |  |
| NetworkNodeDiameterAddress |  | 3GPP TS 29.503 [6] |  |  |  |
| IpSmGwRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| LcsBroadcastAssistanceTypesData |  | 3GPP TS 29.503 [6] |  | LCS Broadcast Assistance Data Types |  |
| ContextInfo |  | 3GPP TS 29.503 [6] |  |  |  |
| NfGroupId |  | 3GPP TS 29.571 [3] |  |  |  |
| ProseSubscriptionData |  | 3GPP TS 29.503 [6] |  | ProSe Service Subscription Data |  |
| PlmnId |  | 3GPP TS 29.571 [3] |  | PLMN ID |  |
| GroupIdentifiers |  | 3GPP TS 29.503 [6] |  | External or Internal Group Identifier with a list of group members |  |
| AuthorizationInfo |  | 3GPP TS 29.503 [6] |  | Contains NIDD Authorization Information |  |

* * * Next Change * * * *

#### 5.4.2.X	Type: NiddAuthorizationInfo

# Table 5.4.2.26-1: Definition of type NiddAuthorizationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| niddAuthorizationList | array(AuthorizationInfo) | M | 1..N | List of NiddAuthorizationInfo items |

* * * Next Change * * * *

## A.2	Nudr_DataRepository API for Subscription Data

(... text not shown for clarity ...)

/subscription-data/{ueId}/context-data/nidd-authorizations:

put:

summary: Create NIDD Authorization Info

operationId: CreateNIDDAuthorizationInfo

tags:

- NIDD Authorization Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NiddAuthorizationInfo'

required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

'201':

description: Upon successful creation, the created resource shall be returned

content:

application/json:

schema:

$ref: '#/components/schemas/NiddAuthorizationInfo'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete NIDD Authorization Info

operationId: RemoveNiddAuthorizationInfo

tags:

- NIDD Authorization Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Expected response to a successful subscription removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify NIDD Authorization Info

operationId: ModifyNiddAuthorizationInfo

tags:

- NIDD Authorization Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve NIDD Authorization Info

operationId: GetNiddAuthorizationInfo

tags:

- NIDD Authorization Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/SmfSubscriptionInfo'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

(... text not shown for clarity ...)

NiddAuthorizationInfo:

description: Information related to active NIDD Authorizations

type: object

required:

- niddAuthorizationList

properties:

niddAuthorizationList:

type: array

items:

$ref: 'TS29503_Nudm_NIDDAU.yaml#/components/schemas/AuthorizationInfo'

minItems: 1

* * * End of Changes * * * *