3GPP TSG-CT Meeting #93-e	CP-212183

# E-Meeting, 13th – 15th September 2021										   Revision of C4-214587

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.562 | CR | 0080 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Retrieval of multiple Repository Data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2021-09-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The IMS-AS cannot retrieve multiple Repository Data with a single GET request. (equivalent functionality via Sh is available) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure, resources data types and yaml definitions to support retrieval of multiple Repository Data |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS-AS needs to retrieve Repository data one by one. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.2.8.x (new), 6.2.3.1, 6.2.3.xx (new), 6.2.6.1, 6.2.6.2.yy (new), A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible corrections with impacts on the following OpenAPI specifications: - TS29562_Nhss_imsSDM.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: Missing data type definition for RepositoryDataList added to OpenAPI |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

###### 5.3.2.2.8.x	Repository Data Retrieval for multiple service indications

Figure 5.3.2.2.8.x-1 shows a scenario where the NF service consumer (IMS-AS) sends a request to the HSS to receive the UE's Repository Data for multiple Service Indications. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/repository-data) and query parameters (e.g. list of service indications, supported-features).

Figure 5.3.2.2.8.x-1: Repository Data Retrieval for multiple service indications

1.	The NF service consumer (IMS-AS) sends a GET request to the resource representing the UE's Repository Data, with query parameters indicating the supported-features and list of service indications.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's Repository Data for the requested Service Indications as relevant for the requesting NF service consumer.

2b.	If there is no valid subscription data or Repository Data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

* * * Next Change * * * *

#### 6.2.3.1	Overview

Figure 6.2.3.1-1: Resource URI structure of the Nhss_imsSDM API

Figure 6.2.3.1-2: Resource URI structure of the Nhss_imsSDM API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Repository Data (Document) | /{imsUeId}/repository-data/{serviceIndication} | GET | Retrieve repository data for a service indication |
|  |  | DELETE | Delete repository data for a service indication |
|  |  | PUT | Update repository data for a service indication |
| Repository Data List (Document) | /{imsUeId}/repository-data | GET | Retrieve multiple repository data for a list of service indications |
| IMS Associated Identities (Document) | /{imsUeId}/identities/ims-associated-identities | GET | Retrieve the UE's IMS associated identities (implicit and alias) |
| MSISDNs (Document) | /{imsUeId}/identities/msisdns | GET | Retrieve the UE's MSISDNs (basic and additional) |
| PrivateIdentities (Document) | /{imsUeId}/identities/private-identities | GET | Retrieve the UE's Private Identities (IMPI and IMSI) |
| ImeiSvInformation (Document) | /{imsUeId}/identities/imeisv | GET | Retrieve the UE's IMEISV |
| IMS Profile Data (Document) | /{imsUeId}/ims-data/profile-data | GET | Retrieve the UE's subscribed Profile |
| Initial Filter Criteria (Document) | /{imsUeId}/ims-data/ profile-data/ifcs | GET | Retrieve the UE's subscribed Initial Filter Criteria |
| Service Level Trace Information (Document) | /{imsUeId}/ims-data/profile-data/service-level-trace-info | GET | Retrieve the UE's subscribed Service Level Trace Information |
| Service Priority Information (Document) | /{imsUeId}/ims-data/ profile-data/priority-levels | GET | Retrieve the UE's subscribed Service Priority Levels |
| Charging Information (Document) | /{imsUeId}/ims-data/ profile-data/chargingInfo | GET | Retrieve the UE's subscribed Charging Information |
| IMS Location Data (Document) | /{imsUeId}/ims-data/location-data/server-name | GET | Retrieve the UE's S-CSCF name |
| S-CSCF Capabilities (Document) | /{imsUeId}/ims-data/location-data/scscf-capabilities | GET | Retrieve the UE's S-CSCF capabilities (mandatory and/or optional) |
| S-CSCF Selection Assistance Information (Document) | /{imsUeId}/ims-data/location-data/scscf-selection-assistance-info | GET | Retrieve the UE's S-CSCF Selection Assistance Information |
| IMS Registration Status (Document) | /{imsUeId}/ims-data/ registration-status | GET | Retrieve the UE's IMS registration status. |
| Subscriptions (Collection) | /{imsUeId}/subscriptions | POST | Create a subscription |
| Individual subscription (Document) | /{imsUeId}/subscriptions/ {subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |
|  |  | PATCH | Modify the sdm-subscription identified by {subscriptionId} |
| PS Location Information(Document) | /{imsUeId}/access-data/ ps-domain/location-data | GET | Retrieve the UE's location in PS domain |
| CS Location Information(Document) | /{imsUeId}/access-data/ cs-domain/location-data | GET | Retrieve the UE's location in CS domain |
| PsUserState (Document) | /{imsUeId}/access-data/ ps-domain/user-state | GET | Retrieve the UE's state in PS domain |
| CsUserState (Document) | /{imsUeId}/access-data/ cs-domain/user-state | GET | Retrieve the UE's state in CS domain |
| TadsInfo (Document) | /{imsUeId}/access-data/ ps-domain/tads-info | GET | Retrieve the UE's T-ADS information |
| UeReachabilityIpSubscriptions (Collection) | /{imsUeId}/access-data/ ps-domain/ue-reach-subscriptions | POST | Create a subscription to UE Reachability for IP |
| UE reachability individual subscription (Document) | /{imsUeId}/access-data/ ps-domain/ue-reach-subscriptions/ {subscriptionId} | DELETE | Delete the subscription to UE reachability for IP identified by {subscriptionId}, i.e. unsubscribe |
|  |  | PATCH | Modify the subscription to UE reachability for IP identified by {subscriptionId} |
| IpAddress (Document) | /{imsUeId}/access-data/ ps-domain/ip-address | GET | Retrieve the UE's stored IP Address Secure Binding Information |
| Csrn (Document) | /{imsUeId}/access-data/ cs-domain/csrn | GET | Retrieve a CSRN for the UE |
| ReferenceAccessLocation (Document) | /{imsUeId}/access-data/wireline-domain/reference-location | GET | Retrieve the UE's subscribed Reference Access Location |
| SRVCC Data (Document) | /{imsUeId}/srvcc-data | GET | Retrieve the UE's STN-SR and SRVCC capability |
|  |  | PATCH | Update the UE's STN-SR. |
| PSI Activation State (Document) | /{imsUeId}/service-data/ psi-status | GET | Retrieve a PSI activation status |
|  |  | PATCH | Update a PSI activation status |
| Dsai (Document) | /{imsUeId}/service-data/dsai | GET | Retrieve DSAI data (i.e. DSAI tags and status) associated to an Application Server |
|  |  | PATCH | Update DSAI data associated to an Application Server |
| SMSRegistrationInfo (Document) | /{imsUeId}/service-data/ sms-registration-info | GET | Retrieve the SMS Registration Information (e.g. IP-SM-GW address) |
|  |  | PUT | Creates or updates the SMS Registration Information (e.g. IP-SM-GW address) |
|  |  | DELETE | Deletes the SMS Registration Information (e.g. IP-SM-GW address) |
| SharedData (Collection) | /shared-data | GET | Retrieve shared data |
| SharedDataSubscriptions (Collection) | /shared-data-subscriptions | POST | Create a subscription |
| Shared Data Individual Subscription (Document) | /shared-data-subscriptions/{subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |
|  |  | PATCH | Modify the subscription identified by {subscriptionId} |

* * * Next Change * * * *

#### 6.2.3.xx	Resource: Repository Data List

##### 6.2.3.xx.1	Description

This resource represents the Repository Data. It is used by the service consumer (e.g. IMS-AS) to  retrieve multiple repository data for a given list of service indications.

##### 6.2.3.xx.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/repository-data

This resource shall support the resource URI variables defined in table 6.2.3.19.2-1.

Table 6.2.3.xx.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.xx.3	Resource Standard Methods

###### 6.2.3.xx.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.xx.3.1-1.

Table 6.2.3.19.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features |  | SupportedFeatures |  | O |  | 0..1 |  | see 3GPP TS 29.500 [4] clause 6.6 |  |
| service-indications | array(ServiceIndication) | M | 1..N |  |

This method shall support the request data structures specified in table 6.2.3.xx.3.1-2 and the response data structures and response codes specified in table 6.2.3.xx.3.1-3.

Table 6.2.3.xx.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.xx.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RepositoryDataList | M | 1 | 200 OK | A response body containing the Repository Data List for the requested Service Indications. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same HSS (service) set. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same HSS (service) set. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.xx.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.xx.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

* * * Next Change * * * *

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nhss_imsSDM service API.

Table 6.2.6.1-1: Nhss_imsSDM specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ScscfCapabilityList | 6.2.6.2.2 | Information about mandatory and optional S-CSCF capabilities |
| ImsProfileData | 6.2.6.2.4 | User's IMS profile data |
| RepositoryData | 6.2.6.2.7 | Repository Data for the requested Service Indication |
| MsisdnList | 6.2.6.2.8 | List of MSISDNs associated to the IMS public Identity |
| PublicIdentities | 6.2.6.2.9 | IMS Public Identities which belong to the same Implicit Registration Set (if any) than the requested IMS Public Identity |
| PublicIdentity | 6.2.6.2.10 | IMS Public Identity and the related data (Alias Group Id, IRS default indication, Identity Type) |
| ImsSdmSubscription | 6.2.6.2.11 | A subscription to notifications of data change |
| ImsRegistrationStatus | 6.2.6.2.12 | Registration status of the user. |
| PriorityLevels | 6.2.6.2.13 | Namespaces and priority levels allowed for the IMS public Identity. |
| Ifcs | 6.2.6.2.14 | List of IFCs associated to the IMS public Identity |
| Ifc | 6.2.6.2.15 |  |
| TriggerPoint | 6.2.6.2.16 |  |
| Spt | 6.2.6.2.17 |  |
| HeaderSipRequest | 6.2.6.2.18 |  |
| SdpDescription | 6.2.6.2.19 |  |
| ApplicationServer | 6.2.6.2.20 |  |
| ImsLocationData | 6.2.6.2.21 | IMS Location Data (S_CSCF name) |
| ServiceLevelTraceInformation | 6.2.6.2.22 | IMS Service Level Trace Information |
| PsLocation | 6.2.6.2.23 | Location data in PS domain. |
| SgsnLocationData | 6.2.6.2.24 |  |
| MmeLocationData | 6.2.6.2.25 |  |
| AmfLocationData | 6.2.6.2.26 |  |
| TwanLocationData | 6.2.6.2.27 |  |
| CsLocation | 6.2.6.2.28 | Location data in CS domain. |
| CsgInformation | 6.2.6.2.29 |  |
| SrvccData | 6.2.6.2.30 | UE SRVCC capability (if available) and STN-SR (if subscribed) |
| PsiActivationState | 6.2.6.2.31 | Public Service Identity activation state |
| ImeiSvInformation | 6.2.6.2.34 | IMEI(SV) information |
| IpAddress | 6.2.6.2.35 | IPv4 address, IPv6 address or IPv6 prefix and/or IPv6 interface identifier |
| TadsInformation | 6.2.6.2.36 | T-ADS Information |
| UeReachabilitySubscription | 6.2.6.2.37 | A subscription to notifications of UE reachability for IP. |
| UeReachabilityNotification | 6.2.6.2.38 | A notification of UE reachability for IP. |
| PsUserState | 6.2.6.2.39 | User state in PS domain |
| CsUserState | 6.2.6.2.40 | User state in CS domain |
| Csrn | 6.2.6.2.41 | CS domain routeing number |
| ReferenceLocationInformation | 6.2.6.2.42 | Reference Location Information for the user in fixed access networks. |
| SmsRegistrationInfo | 6.2.6.2.43 | SMS Registration Information (IP-SM-GW number and SC address) |
| IpSmGwAddress | 6.2.6.2.44 | IP-SM-GW number and diameter URI/realm |
| ImsAssociatedIdentities | 6.2.6.2.45 |  |
| DsaiTagInformation | 6.2.6.2.46 | DSAI tag information (list of DSAI tags activation state for an Application Server) |
| DsaiTagStatus | 6.2.6.2.47 | DSAI tag status (DSAI tag and activation state) |
| CreatedUeReachabilitySubscription | 6.2.6.2.48 | A subscription to notifications of UE reachability for IP. |
| PrivateIdentities | 6.2.6.2.49 | IMS Private Identities |
| PrivateIdentity | 6.3.6.2.50 | IMS Private Identity and the related data (e.g. Identity Type) |
| ScscfSelectionAssistanceInformation | 6.2.6.2.51 | This information shall be used by the I-CSCF to select an S-CSCF for the UE |
| ChargingInfo | 6.2.6.2.52 | Diameter addresses of the charging function |
| RepositoryDataList | 6.2.6.2.yy | List of Repository Data for the requested Service Indications |

Table 6.2.6.1-2 specifies data types re-used by the Nhss_imsSDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_imsSDM.

Table 6.2.6.1-2: Nhss_imsSDM re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uri | 3GPP TS 29.571 [16] | Uniform Resource Identifier |
| SupportedFeatures | 3GPP TS 29.571 [16] | see 3GPP TS 29.500 [4] clause 6.6 |
| ModificationNotification | 3GPP TS 29.503 [15] | Payload of the notification after a data change |
| EutraLocation | 3GPP TS 29.571 [16] | E-UTRAN user location |
| NrLocation | 3GPP TS 29.571 [16] | NR user location |
| PlmnId | 3GPP TS 29.571 [16] | PLMN Identity |
| TimeZone | 3GPP TS 29.571 [16] | Time Zone and Daylight Saving Time |
| UtraLocation | 3GPP TS 29.571 [16] | UTRAN user location |
| GeraLocation | 3GPP TS 29.571 [16] | GERAN user location |
| DiameterIdentity | 3GPP TS 29.571 [16] | Diameter URI |
| NfInstanceId | 3GPP TS 29.571 [16] | String uniquely identifying a NF instance |
| RatType | 3GPP TS 29.571 [16] | RAT type |
| StnSr | 3GPP TS 29.571 [16] | Session Transfer Number for SRVCC |
| PatchItem | 3GPP TS 29.571 [16] | Patch Item |
| PatchResult | 3GPP TS 29.571 [16] | Patch Result |
| Ipv4Addr | 3GPP TS 29.571 [16] | IPv4 address |
| Ipv6Addr | 3GPP TS 29.571 [16] | IPv6 address |
| Ipv6Prefix | 3GPP TS 29.571 [16] | IPv6 address prefix |
| DateTime | 3GPP TS 29.571 [16] | UTC time |
| AccessType | 3GPP TS 29.571 [16] | Type of access |
| ProblemDetails | 3GPP TS 29.571 [16] | Response body of error response messages. |
| RedirectResponse | 3GPP TS 29.571 [16] | Response body of redirect response messages. |

* * * Next Change * * * *

##### 6.2.6.2.yy	Type: RepositoryDataList

Table 6.2.6.2.yy-1: Definition of type RepositoryDataList

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| repositoryDataMap | map(RepositoryData) | M | 1..N | A map (list of key-value pairs where ServiceIndication serves as key) of RepositoryData. |

* * * Next Change * * * *

## A.3	Nhss_imsSDM API

openapi: 3.0.0

***************text not shown for clarity***********

/shared-data:

get:

summary: retrieve shared data

operationId: GetSharedData

tags:

- Retrieval of shared data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-data:read

parameters:

- name: shared-data-ids

in: query

description: List of shared data ids

required: true

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/SharedDataId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SharedData'

minItems: 1

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/repository-data:

get:

summary: Retrieve the repository data associated to an IMPU and service indication list

operationId: GetRepositoryDataServIndList

tags:

- Repository data list

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:repository-data:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: serviceIndicationList

in: query

description: Identifiers of a services related data

required: true

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/ServiceIndication'

minItems: 1

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryDataList'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

***************text not shown for clarity***********

DataSetNames:

type: array

items:

$ref: '#/components/schemas/DataSetName'

minItems: 1

uniqueItems: true

RepositoryDataList:

description: List of Repository Data for the requested Service Indications

type: object

required:

- repositoryDataMap

properties:

repositoryDataMap:

description: A map (list of key-value pairs where ServiceIndication serves as key) of RepositoryData

type: object

additionalProperties:

$ref: '#/components/schemas/RepositoryData'

# SIMPLE TYPES:

***************text not shown for clarity***********

* * * End Of Change * * * *