3GPP TSG-CT Meeting #93-e	CP-212234

# E-Meeting, 13th – 15th September 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.550 | CR | 0023 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Redirect Response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, 5G_eSBA |  |  |  |  |  | Date: |  |  | 2021-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per agreement in CT4#103E meeting, the HTTP 307/308 response shall not be regarded as a error case. Thus, it should not use "application/problem+json" media type, and the returned data structure shall be replaced from ProblemDetails to RedirectResponse. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the HTTP response of 307 / 308. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The response of HTTP 307 / 308 is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.3.1, 6.1.3.3.3.1, 6.1.6.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI file TS29550_Nsoraf |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: - Correct the CR version number - Add location header Rev 2: - API update to correct implementation issues |  |  |  |  |  |  |  |  |

* * * First Change * * * *

###### 6.1.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | See 3GPP TS 29.500 [4] clause 6.6. |  |
| plmn-id | PlmnId | M | 1 | PLMN identity of the PLMN serving the UE. |  |
| access-type | AccessType | O | 0..1 | Access type used by the UE. |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SorInformation | M | 1 | 200 OK | Upon success, a response with "200 OK" status code and a response body containing the SoR information as requested by the NF consumer (e.g. UDM) shall be returned by the SOR-AF. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a URI pointing to a resource located on alternative SOR-AF or the initial target SOR-AF when this is a redirection triggered by an SCP via another SCP. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a URI pointing to a resource located on alternative SOR-AF or the initial target SOR-AF when this is a redirection triggered by an SCP via another SCP. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be set to one of the following application errors: - USER_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponses may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Cache-Control | string | M | 1 | The Cache-Control HTTP header is set to the value "no-cache" instructing the NF consumer (e.g. UDM) to not cache the received SoR information. |

Table 6.1.3.2.3.1-x: Headers supported by the 307 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to a resource located on alternative SOR-AF or the initial target SOR-AF when this is a redirection triggered by an SCP via another SCP. |

Table 6.1.3.2.3.1-y: Headers supported by the 308 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to a resource located on alternative SOR-AF or the initial target SOR-AF when this is a redirection triggered by an SCP via another SCP. |

* * * Next Change * * * *

###### 6.1.3.3.3.1	PUT

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SorAckInfo | M | 1 | Contains an indication on the reception status of the acknowledgment of successful reception of SoR information by the UE. |

Table 6.1.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, a response with "204 No Content" status code shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a URI pointing to a resource located on alternative SOR-AF or the initial target SOR-AF when this is a redirection triggered by an SCP via another SCP. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a URI pointing to a resource located on alternative SOR-AF or the initial target SOR-AF when this is a redirection triggered by an SCP via another SCP. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be set to the following application error: - USER_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponses may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.3.1-x: Headers supported by the 307 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to a resource located on alternative SOR-AF or the initial target SOR-AF when this is a redirection triggered by an SCP via another SCP. |

Table 6.1.3.3.3.1-y: Headers supported by the 308 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to a resource located on alternative SOR-AF or the initial target SOR-AF when this is a redirection triggered by an SCP via another SCP. |

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsoraf_SOR service based interface protocol.

Table 6.1.6.1-1: Nsoraf specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SorInformation | 6.1.6.2.2 | Contains the SoR information to be conveyed to the UE. |  |
| SorAckInfo | 6.1.6.2.3 | Contains an indication to the SOR-AF on the reception status of the acknowledgment of successful reception of SoR Information by the UE. |  |
| SorAckStatus | 6.1.6.3.3 | Contains the reception status of the acknowledgment of successful reception of SoR Information by the UE. |  |

Table 6.1.6.1-2 specifies data types re-used by the Nsoraf_SOR service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsoraf service based interface.

Table 6.1.6.1-2: Nsoraf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| PlmnId | 3GPP TS 29.571 [16] | PLMN Identity |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Common data type used in response bodies |  |
| RedirectResponse | 3GPP TS 29.571 [10] | Redirect Response |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | see 3GPP TS 29.500 [4] clause 6.6 |  |
| SteeringContainer | 3GPP TS 29.503 [15] | Contains the SoR Information |  |
| Supi | 3GPP TS 29.571 [16] | Contains the SUPI information. |  |
| DateTime | 3GPP TS 29.571 [16] | Date Time |  |
| AccessType | 3GPP TS 29.571 [16] | Access type (e.g. 3GPP) |  |

* * * Next Change * * * *

## A.2	Nsoraf_SOR API

********TEXT SKIPPED********

paths:

/{supi}/sor-information:

get:

summary: retrieve the steering of roaming information for a UE

operationId: GetSorInformation

tags:

- SoR Information Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

description: serving PLMN ID

required: true

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: access-type

in: query

description: Access type used by the UE

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SorInformation'

headers:

Cache-Control:

description: Cache-Control (as described in RFC 7234) with value "no-cache" to indicate that the returned SoR information should not be cached

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/sor-information/sor-ack:

put:

summary: SoR Acknowledgment Reception Notification

operationId: SorAckInfo

tags:

- Providing the reception status of the acknowledgement of Steering of Roaming information reception by the UE

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SorAckInfo'

responses:

'204':

description: Successful reception of the indication

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

********TEXT SKIPPED********

* * * End of Changes * * * *