3GPP TSG-CT Meeting #94-e	CP-213191

# E-Meeting, 13th – 15th December 2021													 was C4-216347

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0408 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Roaming Information in EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2021-12-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The roaming status change is the specific monitoring event and kept synchronized between UDM and HSS, see the clause of TS 23.632, in the current definition, there is no interface to synchronize the roaming status change in the EPC domain to the UDM by the HSS and no interface provided to the UDM to store this information in the UDR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds the resource to store the roaming information in the EPC, and PUT and GET methods are added on the new resource. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirement synchronization of the roaming status change in the EPC domain to the UDM by the HSS is not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.xx(new), 5.4.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatibile new feature to the OpenAPI specification files of TS29505_Nudr_DataRepository API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: corrects Open API syntax errors detected during CR implementation. |  |  |  |  |  |  |  |  |

* * * Begin of Changes * * * *

5.2	Resources

5.2.1	Overview

# Figure 5.2.1-1: Resource URI sub-level structure for subscription data

# Figure 5.2.1-2: Resource URI sub-level structure for subscription data (cont.)

# Figure 5.2.1-3: Resource URI sub-level structure for subscription data (cont.)

Table 5.2.1-1 provides an overview of the resources and applicable HTTP methods.

# Table 5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| AuthenticationSubscription | /subscription-data/{ueId}/authentication-data/authentication-subscription | GET | Retrieve a UE's authentication subscription data |
|  |  | PATCH | Update a UE's authentication subscription data Updates shall be limited to the sequenceNumber attribute. Attempts to patch any other attribute shall be rejected by the UDR. |
| AuthenticationSoR | /subscription-data/{ueId}/ue-update-confirmation-data/sor-data | PUT | Store UE's SOR acknowledgement information (SoR-XMAC-IUE) |
|  |  | GET | Retrieve UE's SoR acknowledgement information |
| AuthenticationUPU | /subscription-data/{ueId}/ue-update-confirmation-data/upu-data | PUT | Store UE Parameter Update acknowledgement information (UPU-XMAC-IUE) |
|  |  | GET | Retrieve UE Parameter Update acknowledgement information |
| SubscribedSNSSAIs | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais | PUT | Store UE-acknowledgement info for change of subscribed S-NSSAIs |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed S-NSSAIs |
| SubscribedCAG | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag | PUT | Store UE-acknowledgement info for change of subscribed CAG |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed CAG |
| AuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status | PUT | Store a UE's authentication status |
|  |  | GET | Retrieve a UE's authentication status |
|  |  | DELETE | Delete a UE's authentication status |
| IndividualAuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status/{servingNetworkName} | PUT | When the feature "PerUePerSnAuthStatus" is supported, store a UE's Individual authentication status |
|  |  | GET | When the feature "PerUePerSnAuthStatus" is supported, retrieve a UE's Individual authentication status |
|  |  | DELETE | When the feature "PerUePerSnAuthStatus" is supported, delete a UE's Individual authentication status |
| ProvisionedData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data | GET | Retrieve the UE's subscribed Provisioned Data |
| AccessAndMobilitySubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/am-data | GET | Retrieve the UE's subscribed Access and Mobility Data |
| SmfSelectionSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data | GET | Retrieve the UE's subscribed SMF Selection Data |
| SessionManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data | GET | Retrieve the UE's subscribed SM Subscription Data |
| ContextData | /subscription-data/{ueId}/context-data | GET | Retrieve the UE's context Data |
| Amf3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-3gpp-access | PUT | Create and Update the AMF registration for 3GPP access |
|  |  | PATCH | Modify the AMF registration for 3GPP access |
|  |  | GET | Retrieve the AMF registration information for 3GPP access |
| AmfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-non-3gpp-access | PUT | Update the AMF registration for non 3GPP access |
|  |  | PATCH | Modify the AMF registration for non 3GPP access |
|  |  | GET | Retrieve the AMF registration information for non 3GPP access |
| SmfRegistrations | /subscription-data/{ueId}/context-data/smf-registrations | GET | Retrieve the list of the SMF registrations |
| IndividualSmfRegistration | /subscription-data/{ueId}/context-data /smf-registrations/{pduSessionId} | PUT | Store an individual SMF registration identified by PDU Session Id |
|  |  | DELETE | Delete an individual SMF registration |
|  |  | GET | Retrieve individual SMF registration |
| OperatorSpecificData | /subscription-data/{ueId}/operator-specific-data | GET | retrieve the operator specific subscription data of a UE |
|  |  | PATCH | modify the operator specific subscription data of a UE |
| OperatorDeterminedBarringData | /subscription-data/{ueId}/operator-determined-barring-data | GET | Retrieve the UE's Operator Determined Barring |
| SMSManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data | GET | Retrieve the UE's subscribed SMS management subscription data. |
| Smsf3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-3gpp-access | PUT | Create or Update the SMSF registration |
|  |  | DELETE | Delete the SMSF registration for 3GPP access |
|  |  | GET | Retrieve the SMSF registration information |
| SmsfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-non-3gpp-access | PUT | Create or Update the SMSF registration for non 3GPP access |
|  |  | DELETE | Delete the SMSF registration for non 3GPP access |
|  |  | GET | Retrieve the SMSF registration information for non 3GPP access |
| IpSmGwRegistration | /subscription-data/{ueId}/context-data/ip-sm-gw | PUT | Create or Update the IP-SM-GW registration |
|  |  | DELETE | Delete the IP-SM-GW registration |
|  |  | PATCH | Modify the IP-SM-GW registration |
|  |  | GET | Retrieve the IP-SM-GW registration information |
| MessageWaitingData | /subscription-data/{ueId}/context-data/mwd | PUT | Create or Update the SMS Message Waiting Data |
|  |  | DELETE | Delete the SMS Message Waiting Data |
|  |  | PATCH | Modify the SMS Message Waiting Data |
|  |  | GET | Retrieve the SMS Message Waiting Data |
| SdmSubscriptions | /subscription-data/{ueId}/context-data/sdm-subscriptions | GET | Retrieve SDM subscriptions |
|  |  | POST | Create an individual SDM subscription |
| IndividualSdmSubscription | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId} | PUT | Update an individual SDM subscription |
|  |  | DELETE | Delete an individual SDM subscription |
|  |  | PATCH | Update an individual SDM Subscription |
|  |  | GET | Retrieve an individual SDM subscription |
| HssSdmSubscriptionInfo | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId}/hss-sdm-subscriptions | PUT | Store information related to the Hss-SDM-Subscriptions |
|  |  | DELETE | Delete the Hss-SDM-subscriptions |
|  |  | GET | Retrieve Hss-SDM-subscriptions |
|  |  | PATCH | Update Hss-SDM-subscriptions |
| EeSubscriptions | /subscription-data/{ueId}/context-data/ee-subscriptions | GET | Retrieve EE subscriptions |
|  |  | POST | Create an EE subscription |
| IndividualEeSubscription | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription |
|  |  | DELETE | Delete an individual EE subscription |
|  |  | PATCH | Update an individual EE subscription |
|  |  | GET | Retrieve an individual EE subscription |
| AmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions | PUT | Store information related to the Amf-EE-Subscription response |
|  |  | DELETE | Delete the Amf-EE-subscriptions |
|  |  | GET | Retrieve AMF-subscriptions |
|  |  | PATCH | Update AMF-subscriptions |
| SmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/smf-subscriptions | PUT | Store information related the a received Smf-EE-Subscription response |
|  |  | DELETE | Delete the Smf-EE-subscriptions |
|  |  | GET | Retrieve SMF-subscriptions |
|  |  | PATCH | Update SMF-subscriptions |
| HssSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/hss-subscriptions | PUT | Store information related to the Hss-EE-Subscriptions |
|  |  | DELETE | Delete the Hss-EE-subscriptions |
|  |  | GET | Retrieve Hss-EE-subscriptions |
|  |  | PATCH | Update Hss-EE-subscriptions |
| EeProfileData | /subscription-data/{ueId}/ee-profile-data | GET | Retrieve the UE's subscribed EE profile data. |
| ProvisionedParamenterData | /subscription-data/{ueId}/pp-data | PATCH | Update of provisioned parameters. |
|  |  | GET | Retrieves the UE's provisioned parameters. |
| PpProfileData | /subscription-data/{ueId}/pp-profile-data | GET | Retrieve the UE's subscribed PP profile data. |
| ProvisionedParameterDataEntries | /subscription-data/{ueId}/pp-data-store | GET | Retrieve a Provisioned Parameter Data of multiple Entries |
| ProvisionedParameterDataEntry | /subscription-data/{ueId}/pp-data-store/{afInstanceId} | PUT | Create a Provisioned Parameter Data Entry |
|  |  | DELETE | Delete a Provisioned Parameter Data Entry |
|  |  | GET | Retrieve a Provisioned Parameter Data Entry |
| SMSSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-data | GET | Retrieve the UE's subscribed SMS subscription data. |
| SubscriptionDataSubscriptions | /subscription-data/subs-to-notify | GET | Retrieve existing subscriptions |
|  |  | POST | Create a subscription, i.e. subscribe a node to receive notification for change of data |
|  |  | DELETE | Deletes multiple subscriptions for a given UE. |
| IndividualSubscriptionDataSubscription | /subscription-data/subs-to-notify/{subsId} | DELETE | Delete the subscription identified by {subsId}, i.e. unsubscribe a node to receive notification for change of data |
|  |  | PATCH | Update an individual Subscription to notification |
|  |  | GET | Retrieve an individual Subscription to notification |
| EeGroupSubscriptions | /subscription-data/group-data/{ueGroupId}/ee-subscriptions | GET | Retrieve EE subscriptions for groups of UEs |
|  |  | POST | Create an EE subscription for groups of UEs |
| IndividualEeGroupSubscription | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription for a group of UEs |
|  |  | DELETE | Delete an individual EE subscription for a group of UEs |
|  |  | PATCH | Update an individual EE subscription for a group of UEs |
|  |  | GET | Retrieve an individual EE subscription for a group of UEs |
| EeGroupProfileData | /subscription-data/group-data/{ueGroupId}/ee-profile-data | GET | Retrieve the subscribed EE profile data for a group of UEs. |
| TraceData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/trace-data | GET | Retrieve the UE's trace configuration data |
| IdentityData | /subscription-data/{ueId}/identity-data | GET | Retrieve identity data that corresponds to the provided ueId |
| SharedData (Collection) | /subscription-data/shared-data | GET | Retrieve shared data |
| IndividualSharedData (Document) | /subscription-data/shared-data/{sharedDataId} | GET | Retrieve the individual Shared Data |
| GroupIdentifiers | /subscription-data/group-data/group-identifiers | GET | Retrieve group identifiers and the UE identifiers belong to the group identifiers. |
| 5GVnGroups | /subscription-data/group-data/5g-vn-groups | GET | Retrieve 5G VN Groups |
| Individual5GVnGroup | /subscription-data/group-data/5g-vn-groups/{externalGroupId} | PUT | Create a 5G VN Group |
|  |  | PATCH | Update a 5G VN Group |
|  |  | DELETE | Delete a 5G VN Group |
|  |  | GET | Retrieve a 5G VN Group |
| 5GVnGroupsInternal | /subscription-data/group-data/5g-vn-groups/internal | GET | Retrieve 5G VN Group Data based on Internal Group Identifier(s) |
| Pp5gVnGroupProfileData | /subscription-data/group-data/5g-vn-groups/pp-profile-data | GET | Retrieve the UE's subscribed PP profile data for accessing 5G VN Groups service operations. |
| LcsPrivacySubscriptionData | /subscription-data/{ueId}/lcs-privacy-data | GET | Retrieve the UE's subscribed LCS privacy Subscription Data |
| LcsMobileOriginatedSubscriptionData | /subscription-data/{ueId}/lcs-mo-data | GET | Retrieve the UE's subscribed LCS Mobile Originated Subscription Data |
| NiddAuthorizationData | /subscription-data/{ueId}/nidd-authorization-data | GET | Retrieve the UE's NIDD Authorization Data |
| CoverageRestrictionData | /subscription-data/{ueId}/coverage-restriction-data | GET | Retrieve the UE's subscribed enhanced Coverage Restriction Data |
| Location | /subscription-data/{ueId}/context-data/location | GET | Retrieve the UE's Location Information |
| V2xSubscriptionData | /subscription-data/{ueId}/v2x-data | GET | Retrieve the UE's subscribed V2X Data |
| ProseSubscriptionData | /subscription-data/{ueId}/prose-data | GET | Retrieve the UE's subscribed ProSe Service Data |
| LcsBroadcastAssistanceSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-bca-data | GET | Retrieve the UE's subscribed LCS Broadcast Assistance subscription data |
| NiddAuthorizations | /subscription-data/{ueId}/context-data/nidd-authorizations | PUT | Store information related to the NIDD Authorization |
|  |  | DELETE | Delete the NIDD Authorizations |
|  |  | GET | Retrieve NIDD Authorizations |
|  |  | PATCH | Update a specific NIDD Authorization |
| RoamingInfo | /subscription-data/{ueId}/context-data/roaming-information | GET | Retrieve the Roaming information in EPC domain in the AMF 3GPP Registration context |
|  |  | POST | Update or create the Roaming Information in EPC domain in the AMF 3GPP Registration context |

* * * Next Change * * * *

5.2.xx	Resource: RoamingInfo

5.2.xx.1	Description

This resource is used to represent the Roaming information in the EPC domain to be stored in the UDR by the UDM.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

5.2.xx.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/roaming-information

This resource shall support the resource URI variables defined in table 5.2.xx.2-1.

# Table 5.2.xx.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

5.2.xx.3	Resource Standard Methods

5.2.xx.3.1	POST

This method shall support the URI query parameters specified in table 5.2.xx.3.1-1.

# Table 5.2.xx.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.xx.3.1-2 and the response data structures and response codes specified in table 5.2.xx.3.1-3.

# Table 5.2.xx.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RoamingInfo | M | 1 | The AMF registration for 3GPP access is replaced with the received information. |

# Table 5.2.xx.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| RoamingInfo | M | 1 | 201 Created | Upon success, a response body containing a representation of the resource of the created Roaming Information in the EPC domain shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

5.2.xx.4.2	GET

# This method shall support the URI query parameters specified in table 5.2.xx.4.1-1.

# Table 5.2.xx.4.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.xx.4.2-2 and the response data structures and response codes specified in table 5.2.xx.4.2-3.

# Table 5.2.xx.4.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.xx.4.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RoamingInfo | M | 1 | 200 OK | Upon success, a response body containing the Roaming Information in the EPC domain shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

* * * Next Change * * * *

5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationSubscription | 5.4.2.2 | A UE's authentication data |
| OperatorSpecificDataContainer | 5.4.2.3 | Container for operator specific data |
| SmfRegList | 5.4.2.4 | The list of all the SMF registrations of a UE |
| SubscriptionDataSubscriptions | 5.4.2.5 | A subscription to notifications. |
| DataChangeNotify | 5.4.2.6 | Container for data which have changed and notification was requested when changed. |
| IdentityData | 5.4.2.7 | Identity data corresponds to the provided ueId |
| ProvisionedDataSets | 5.4.2.8 | Contains the provisioned data sets |
| SorData | 5.4.2.9 | Used to store the status of the latest SOR data update |
| UpuData | 5.4.2.9A | Used to store the status of the latest UPU data update |
| NssaiAckData | 5.4.2.9B | Used to store the status of the latest NSSAI data update |
| CagAckData | 5.4.2.9C | Used to store the status of the latest CAG data update |
| AmfSubscriptionInfo | 5.4.2.19 | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |
| EeProfileData | 5.4.2.20 | Event Exposure Profile Data |
| ContextDataSets | 5.4.2.22 | Contains the context data sets |
| SequenceNumber | 5.4.2.23 | Contains the SQN |
| MessageWaitingData | 5.4.2.24 | Message Waiting Data list |
| SmscData | 5.4.2.25 | Addresses of SM-Service Center entities with SMS waiting to be delivered to the UE |
| SmfSubscriptionInfo | 5.4.2.26 | Information the UDR stores and retrieves related to active subscriptions at the SMF(s) |
| SmfSubscriptionItem | 5.4.2.27 | Contains info about a single SMF event subscription |
| MtcProvider | 5.4.2.28 | MTC provider information |
| HssSubscriptionInfo | 5.4.2.29 | Information the UDR stores related to active subscriptions at the HSS(s) |
| HssSubscriptionItem | 5.4.2.30 | Contains info about a single HSS event subscription |
| EeGroupProfileData | 5.4.2.31 | The Event Exposure Profile Data for a group of UEs |
| Pp5gVnGroupProfileData | 5.4.2.32 | The Parameter Provision Profile Data for 5G VN groups |
| PpProfileData | 5.4.2.33 | The Parameter Provision Profile Data |
| AllowedMtcProviderInfo | 5.4.2.34 | Allowed MTC Providers or AFs Information to provision parameters for UE. |
| GroupIdentifiers | 5.4.2.35 | External or Internal Group Identifier with a list of group members |
| AuthorizationData | 5.4.2.36 | NIDD Authorization Information |
| NiddAuthorizationInfo | 5.4.2.37 | Information the UDR stores and retrieves related to active NIDD authorizations |
| PpDataEntryList | 5.4.3.38 | A list of the Provisioned Parameters entries. |
| AuthMethod | 5.4.3.3 | Contains the Authentication Method |
| DataSetName | 5.4.3.4 | The name of data set |
| ContextDataSetName | 5.4.3.6 | The name of context data set |
| SqnScheme | 5.4.3.7 | Scheme for generation of Sequence Numbers |
| Sign | 5.4.3.8 | Sign of the DIF value |
| UeUpdateStatus | 5.4.3.9 | Status of the procedure |
| PpDataType | 5.4.3.10 | The data type of the Provisioned Parameters |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| Data type |  |  | Reference |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AccessAndMobilitySubscriptionData |  |  | 3GPP TS 29.503 [6] |  |  | Access and Mobility Subscription Data |  |  |
| SmfSelectionSubscriptionData |  |  | 3GPP TS 29.503 [6] |  |  | SMF Selection Subscription Data |  |  |
| SessionManagementSubscriptionData |  |  | 3GPP TS 29.503 [6] |  |  | Session Management Subscription Data |  |  |
| Amf3GppAccessRegistration |  |  | 3GPP TS 29.503 [6] |  |  | The complete set of information relevant to the AMF where the UE has registered via 3GPP access. |  |  |
| AmfNon3GppAccessRegistration |  |  | 3GPP TS 29.503 [6] |  |  | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |  |  |
| SmfRegistration |  |  | 3GPP TS 29.503 [6] |  |  | The complete set of information relevant to an SMF serving the UE |  |  |
| SmsfRegistration |  |  | 3GPP TS 29.503 [6] |  |  | The complete set of information relevant to the SMSF serving the UE. |  |  |
| SmsManagementSubscriptionData |  |  | 3GPP TS 29.503 [6] |  |  | SMS Management Subscription Data |  |  |
| SupportedFeatures |  |  | 3GPP TS 29.571 [3] |  |  | Used to negotiate the applicability of the optional features |  |  |
| ProblemDetails |  |  | 3GPP TS 29.571 [3] |  |  | Detailed information about the status code. |  |  |
| PpData |  |  | 3GPP TS 29.503 [6] |  |  | Parameter Provision Data |  |  |
| PpDataEntry |  |  | 3GPP TS 29.503 [6] |  |  | Provisioned Parameter Data Entry |  |  |
| SmsSubscriptionData |  |  | 3GPP TS 29.503 [6] |  |  | SMS Subscription Data |  |  |
| PatchItem |  |  | 3GPP TS 29.571 [3] |  |  | Data structure used for JSON patch. |  |  |
| SdmSubscription |  |  | 3GPP TS 29.503 [6] |  |  | A subscription to notifications |  |  |
| EeSubscription |  |  | 3GPP TS 29.503 [6] |  |  | A subscription to Notifications of event exposure |  |  |
| Uri |  |  | 3GPP TS 29.571 [3] |  |  | URI |  |  |
| TraceData |  |  | 3GPP TS 29.571 [3] |  |  | Trace Data |  |  |
| Dnn |  |  | 3GPP TS 29.571 [3] |  |  | Data Network Name |  |  |
| Snssai |  |  | 3GPP TS 29.571 [3] |  |  | Single NSSAI |  |  |
| VarUeId |  |  | 3GPP TS 29.571 [3] |  |  | String represents the SUPI or GPSI. |  |  |
| NfInstanceId |  |  | 3GPP TS 29.571 [3] |  |  | NF Instance Identifier |  |  |
| NotifyItem |  |  | 3GPP TS 29.571 [3] |  |  | Common data type used for data change notification. |  |  |
| OdbData |  |  | 3GPP TS 29.571 [3] |  |  | Operator Determined Barring Data |  |  |
| EventType |  |  | 3GPP TS 29.503 [6] |  |  | Event Type |  |  |
| ExtGroupId |  |  | 3GPP TS 29.503 [6] |  |  | External Group Identifier |  |  |
| PduSessionId |  |  | 3GPP TS 29.571 [3] |  |  | PDU Session ID which identifies a PDU session |  |  |
| DateTime |  |  | 3GPP TS 29.571 [3] |  |  | Date Time |  |  |
| UpuMac |  |  | 3GPP TS 29.509 [15] |  |  | MAC value for protecting UPU procedure (UPU-MAC-IAUSF and UPU-MAC-IUE) |  |  |
| 5GVnGroupConfiguration |  |  | 3GPP TS 29.503 [6] |  |  | 5G VN Group Configuration data |  |  |
| PatchResult |  |  | 3GPP TS 29.571 [3] |  |  | Patch Result |  |  |
| SupportedFeatures |  |  | 3GPP TS 29.571 [3] |  |  | Supported Features |  |  |
| AppPortId |  |  | 3GPP TS 29.503 [6] |  |  | Application Port Identifier |  |  |
| LcsPrivacyData |  |  | 3GPP TS 29.503 [6] |  |  | LCS Privacy Subscription Data |  |  |
| LcsMoData |  |  | 3GPP TS 29.503 [6] |  |  | LCS Mobile Originated Subscription Data |  |  |
| MtcProviderInformation |  |  | 3GPP TS 29.571 [3] |  |  | An ID uniquely identifying MTC provider information |  |  |
| EnhancedCoverageRestrictionData |  |  | 3GPP TS 29.503 [6] |  |  | Enhanced Coverage Restriction Data |  |  |
| LocationInfo |  |  | 3GPP TS 29.503 [6] |  |  | Location Information |  |  |
| V2xSubscriptionData |  |  | 3GPP TS 29.503 [6] |  |  | V2X Subscription Data |  |  |
| E164Number |  |  | 3GPP TS 29.503 [6] |  |  | E164Number |  |  |
| NetworkNodeDiameterAddress |  |  | 3GPP TS 29.503 [6] |  |  | Diameter Address of an Network Node |  |  |
| IpSmGwRegistration |  |  | 3GPP TS 29.503 [6] |  |  | The complete set of information relevant to the IP-SM-GW where the UE has registered. |  |  |
| LcsBroadcastAssistanceTypesData |  |  | 3GPP TS 29.503 [6] |  |  | LCS Broadcast Assistance Data Types |  |  |
| ContextInfo |  |  | 3GPP TS 29.503 [6] |  |  | Contains the headers received by an NF. |  |  |
| NfGroupId |  |  | 3GPP TS 29.571 [3] |  |  | NF Group ID |  |  |
| ProseSubscriptionData |  |  | 3GPP TS 29.503 [6] |  |  | ProSe Service Subscription Data |  |  |
| PlmnId |  |  | 3GPP TS 29.571 [3] |  |  | PLMN ID |  |  |
| AppPortId |  |  | 3GPP TS 29.503 [6] |  |  | Application Port Id |  |  |
| AuthorizationInfo |  |  | 3GPP TS 29.503 [6] |  |  | Contains NIDD Authorization Information |  |  |
| RoamingInfoUpdate |  |  | 3GPP TS 29.503 [6] |  |  | Roaming Information Update in EPC domain |  |  |

* * * Next Change * * * *

A.2	Nudr_DataRepository API for Subscription Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29505_Subscription_Data.yaml".

(… text not shown for clarity …)

/subscription-data/{ueId}/context-data/mwd:

put:

summary: Create the Message Waiting Data of the UE

operationId: CreateMessageWaitingData

tags:

- Message Waiting Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MessageWaitingData'

# required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/MessageWaitingData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/mwd'

required: true

schema:

type: string

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

delete:

summary: To remove the Message Waiting Data of the UE

operationId: DeleteMessageWaitingData

tags:

- Message Waiting Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the Message Waiting Data of the UE

operationId: ModifyMessageWaitingData

tags:

- Message Waiting Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the Message Waiting Data of the UE

operationId: QueryMessageWaitingData

tags:

- Message Waiting Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/MessageWaitingData'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/roaming-information:

put:

summary: Update the Roaming Information in the EPC domain

operationId: UpdateRoamingInformation

tags:

- Update the Roaming Information in the EPC domain (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/RoamingInfo'

# required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/RoamingInfo'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/roaming-information'

required: true

schema:

type: string

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

get:

summary: Retrieves the Roaming Information in the EPC domain

operationId: QueryRoamingInformation

tags:

- Retrieve the Roaming Information in the EPC domain (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/RoamingInfo'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

(… text not shown for clarity …)

SessionManagementSubscriptionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SessionManagementSubscriptionData'

Amf3GppAccessRegistration:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/Amf3GppAccessRegistration'

AmfNon3GppAccessRegistration:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/AmfNon3GppAccessRegistration'

SmfRegistration:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/SmfRegistration'

SmsfRegistration:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/SmsfRegistration'

LocationInfo:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/LocationInfo'

RoamingInfo:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/RoamingInfoUpdate'

SmsManagementSubscriptionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SmsManagementSubscriptionData'

SmsSubscriptionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SmsSubscriptionData'

(… text not shown for clarity …)

* * * End of Changes * * * *