3GPP TSG-CT3 Meeting #118e	C3-215071

# E-Meeting, 11th – 15th October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.507 | CR | 0180 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Direct access to SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When an SNPN-enabled UE is registered in the SNPN, direct access to the SNPN is specified for 3GPP access only, as defined in TS 23.501, clauses 5.30.2.0 and 5.30.2.4.1. Similar statement should be added in this specification. Furthermore, specification does not clearly indicate that the serving network can be a PLMN or an SNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.2.2.1: - added note indicating that when an SNPN-enabled UE is registered in an SNPN, direct access to SNPN is specified for 3GPP access only. - specified that serving network identity can be the PLMN Identifier or the SNPN Identifier and added note indicating that the SNPN Identifier consists of the PLMN Identifier and the NID. Clause 5.6.1: description of the PlmnIdNid data type updated to indicate that the PLMN Identifier or the SNPN Identifier identifies the network and added that the SNPN Identifier consists of the PLMN Identifier and the NID. Clause 5.6.2.3: added that the serving network can be a PLMN or an SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1, 5.6.1, 5.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file of the Npcf_AMPolicyControl API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 4.2.2.1	General

The procedure in the present subclause is applicable when the NF service consumer (e.g. AMF) creates an AM policy association when the UE registers to the network, and when the AMF is relocated (between the different AMF sets) and the new AMF selects a new PCF. The procedure for the case where the AMF is relocated and the new AMF selects the old PCF is defined in subclause 4.2.3.1.

The creation of an AM policy association only applies for normally registered UEs, i.e., it does not apply for Emergency Registered UEs.

Figure 4.2.2.1-1 illustrates the creation of a policy association.

Figure 4.2.2.1-1: Creation of a policy association

When a UE registers and a UE context is being established, the AMF can obtain Service Area Restrictions, RFSP index, subscribed UE-AMBR, subscribed UE-Slice-MBR(s) and GPSI(s) from the UDM during the Access and Mobility Subscription Data retrieval procedure and the allowed NSSAI from local configuration or from the NSSF during the slice selection procedure and shall decide based on local policies whether to request policies from the PCF.

To request policies from the PCF, the NF service consumer (e.g. AMF) shall send an HTTP POST request with: "{apiRoot}/npcf-am-policy-control/v1/policies" as Resource URI and the PolicyAssociationRequest data structure as request body that shall include:

-	Notification URI encoded as "notificationUri" attribute;

-	SUPI encoded as "supi" attribute; and

-	if the feature "SliceSupport" or the feature "DNNReplacementControl" is supported in the NF service consumer and the UE is registered via a 3GPP access, the allowed NSSAI in the 3GPP access encoded in the "allowedSnssais" attribute;

and that shall include when available:

-	GPSI encoded as "gpsi" attribute;

-	if the feature "MultipleAccessTypes" is not supported, the access type encoded as "accessType" attribute;

NOTE n1:	In this Release, for SNPN-enabled UE registered in the SNPN, direct access to the SNPN is specified for 3GPP access only.

-	Permanent Equipment Identifier (PEI) encoded as "pei" attribute;

-	User Location Information encoded as "userLoc" attribute;

-	UE Time Zone encoded as "timeZone" attribute;

-	the identifier of the serving network (the PLMN Identifier or the SNPN Identifier) encoded as "servingPlmn" attribute;

NOTE n2:	The SNPN Identifier consists of the PLMN Identifier and the NID.

-	if the feature "MultipleAccessTypes" is not supported, the RAT type encoded as "ratType" attribute;

-	Service Area Restrictions (see subclause 4.2.2.3.1) derived from the Service Area Restrictions obtained from the UDM by mapping any service areas denoted by geographical information into Tracking Area Identities (TAIs) and encoded as "servAreaRes" attribute;

-	RFSP index (see subclause 4.2.2.3.2) as obtained from the UDM encoded as "rfsp" attribute;

-	a list of Internal Group Identifiers encoded as "groupIds" attribute;

-	if the NF service consumer is an AMF, the GUAMI encoded as "guami" attribute;

-	if the NF service consumer is an AMF, the name of a service produced by the AMF that expects to receive information within Npcf_AMPolicyControl_UpdateNotify service operation encoded as "serviceName" attribute;

-	Alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

-	Alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

-	Alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

-	trace control and configuration parameters information encoded as "traceReq" attribute;

-	if the feature "UE-AMBR_Authorization" is supported in the NF service consumer, the subscribed UE-AMBR (see subclause 4.2.2.3.3) in the "ueAmbr" attribute;

-	if the feature "DNNReplacementControl" is supported, the mapping of each S-NSSAI of the Allowed NSSAI to the corresponding S-NSSAI of the HPLMN encoded in the "mappingSnssais" attribute; and

-	if the feature "UE-Slice-MBR_Authorization" is supported in the AMF, the subscribed UE-Slice-MBR(s) for the allowed NSSAI (see subclause 4.2.2.3.5) in the "ueSliceMbrs" attribute.

Upon the reception of this HTTP POST request, the PCF shall:

-	assign a policy association ID;

-	determine the applicable policy (taking into consideration and optionally modifying the possibly received UE-AMBR, UE-Slice-MBR(s), Service Area Restrictions and/or RFSP index);

-	for the successful case, send a HTTP "201 Created" response with the URI for the created resource in the "Location" header field

NOTE n3:	The assigned policy association ID is part of the URI for the created resource and is thus associated with the SUPI.

and the PolicyAssociation data type as response body including:

-	conditionally AMF Access and Mobility Policy (see subclause 4.2.2.3), i.e.:

a)	if the PCF received the "servAreaRes" attribute in the request, Service Area Restrictions encoded as "servAreaRes" attribute; and/or

b)	if the PCF received the "rfsp" attribute in the request, RAT Frequency Selection Priority (RFSP) Index encoded as "rfsp" attribute; and/or

c)	if the feature "UE-AMBR_Authorization" is supported and the PCF received the "ueAmbr" attribute in the request, the authorized UE-AMBR encoded as "ueAmbr" attribute; and/or

d)	if the feature "UE-Slice-MBR_Authorization" is supported and the PCF received the "ueSliceMbrs" attribute in the request, the authorized UE-Slice-MBR(s) for the allowed NSSAI encoded as "ueSliceMbrs" attribute;

-	optionally one or several of the following Policy Control Request Trigger(s) encoded as "triggers" attribute (see subclause 4.2.3.2):

a)	Location change (tracking area); and

b)	Change of UE presence in PRA; and

c)	if the "SliceSupport" feature or the "DNNReplacementControl" feature is supported, change of allowed NSSAI; and

d)	if the "DNNReplacementControl" feature is supported, change of SMF selection information; and

-	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided, the presence reporting areas for which reporting is required encoded as "pras" attribute;

NOTE n4:	If the PCF uses a Presence Reporting Area identifier referring to a Set of Core Network predefined Presence Reporting Areas as defined in 3GPP TS 23.501 [2], the PCF includes the identifier of this Presence Reporting Area set within the "praId" attribute.

-	if the Policy Control Request Trigger "Change of SMF selection information" is provided, the SMF selection information representing the conditions upon which the AMF shall request a DNN replacement (see subclause 4.2.2.3.4) encoded as "smfSelInfo" attribute;

-	if errors occur when processing the HTTP POST request, apply error handling procedures as specified in subclause 5.7 and according to the following provisions:

-	if the user information received within the "supi" attribute is unknown, the PCF shall reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "USER_UNKNOWN";

-	if the PCF is, due to incomplete, erroneous or missing information in the request, not able to provision an AM policy decision, the PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_REQUEST_PARAMETERS".

-	if the PCF rejects the AM policy association establishment, the NF service consumer shall apply the policy retrieved from the UDM if available; otherwise, the NF service consumer shall apply the operator configured policy.

If the PCF received a GUAMI, the PCF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [14], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF set.

If the PCF received a "traceReq" attribute, it shall perform trace procedures as defined in 3GPP TS 32.422 [18].

The PCF may retrieve AF requirements on Access and Mobility policies from the UDR as specified in 3GPP TS 29.519 [17] and consider them for determining the Access and Mobility policies to be provisioned.

# *** Next Change ***

### 5.6.1	General

This subclause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Npcf_AMPolicyControl service based interface protocol.

Table 5.6.1-1: Npcf_AMPolicyControl specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| CandidateForReplacement | 5.6.2.8 | Contains the list of candidate DNNs for replacement per S-NSSAI. | DNNReplacementControl |
| PolicyAssociation | 5.6.2.2 | Description of a policy association that is returned by the PCF when a policy Association is created, or read. |  |
| PolicyAssociationReleaseCause | 5.6.3.4 | The cause why the PCF requests the termination of the policy association. |  |
| PolicyAssociationRequest | 5.6.2.3 | Information that NF service consumer provides when requesting the creation of a policy association. |  |
| PolicyAssociationUpdateRequest | 5.6.2.4 | Information that NF service consumer provides when requesting the update of a policy association. |  |
| PolicyUpdate | 5.6.2.5 | Updated policies that the PCF provides in a notification or in the reply to an Update Request. |  |
| RequestTrigger | 5.6.3.3 | Enumeration of possible Request Triggers. |  |
| SmfSelectionData | 5.6.2.7 | Includes the SMF Selection information that may be replaced by the PCF. | DNNReplacementControl |
| TerminationNotification | 5.6.2.6 | Request to terminate a policy Association that the PCF provides in a notification. |  |
| AmRequestedValueRep | 5.6.2.9 | Contains the current applicable values corresponding to the policy control request triggers. | ImmediateReport |

Table 5.6.1-2 specifies data types re-used by the Npcf_AMPolicyControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_AMPolicyControl service based interface.

Table 5.6.1-2: Npcf_AMPolicyControl re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AccessType |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| Ambr |  | 3GPP TS 29.571 [11] |  | Aggregated Maximum Bit Rate. |  | UE-AMBR_Authorization |  |
| Dnn |  | 3GPP TS 29.571 [11] |  | DNN |  | DNNReplacementControl |  |
| Fqdn |  | 3GPP TS 29.510 [13] |  | FQDN |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [11] |  | Generic Public Subscription Identifier |  |  |  |
| GroupId |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| Guami |  | 3GPP TS 29.571 [11] |  | Globally Unique AMF Identifier |  |  |  |
| Ipv4Addr |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| MappingOfSnssai |  | 3GPP TS 29.531 [24] |  | Identifies the mapping of an S-NSSAI of the Allowed NSSAI to the corresponding S-NSSAI of the HPLMN. |  | DNNReplacementControl |  |
| Pei |  | 3GPP TS 29.571 [11] |  | Permanent Equipment Identifier |  |  |  |
| PlmnIdNid |  | 3GPP TS 29.571 [11] |  | Identifies the network: PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). |  |  |  |
| PresenceInfo |  | 3GPP TS 29.571 [11] |  | Presence reporting area information |  |  |  |
| PresenceInfoRm |  | 3GPP TS 29.571 [11] |  | This data type is defined in the same way as the "PresenceInfo" data type, but with the OpenAPI "nullable: true" property. |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| RedirectResponse |  | 3GPP TS 29.571 [11] |  | Contains redirection related information. |  | ES3XX |  |
| Uri |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| UserLocation |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| RatType |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| RfspIndex |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| ServiceAreaRestriction |  | 3GPP TS 29.571 [11] |  | Within the areas attribute, only tracking area codes shall be included. |  |  |  |
| ServiceName |  | 3GPP TS 29.510 [13] |  | Name of the service instance. |  |  |  |
| SliceMbr |  | 3GPP TS 29.571 [11] |  | Per UE per Slice-Maximum Bit Rate. |  | UE-Slice-MBR_Authorization |  |
| Snssai |  | 3GPP TS 29.571 [11] |  | Identifies a S-NSSAI included in the Allowed NSSAI. |  | SliceSupport |  |
| Supi |  | 3GPP TS 29.571 [11] |  | Subscription Permanent Identifier |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [11] |  | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |  |  |
| TimeZone |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| TraceData |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| WirelineServiceAreaRestriction |  | 3GPP TS 29.571 [11] |  |  |  | WirelineWirelessConvergence |  |

# *** Next Change ***

#### 5.6.2.3	Type PolicyAssociationRequest

Table 5.6.2.3-1: Definition of type PolicyAssociationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationUri | Uri | M | 1 | Identifies the recipient of Notifications sent by the PCF. |  |
| altNotifIpv4Addrs | array(Ipv4Addr) | O | 1..N | Alternate or backup IPv4 Address(es) where to send Notifications. |  |
| altNotifIpv6Addrs | array(Ipv6Addr) | O | 1..N | Alternate or backup IPv6 Address(es) where to send Notifications. |  |
| altNotifFqdns | array(Fqdn) | O | 1..N | Alternate or backup FQDN(s) where to send Notifications. |  |
| supi | Supi | M | 1 | Subscription Permanent Identifier. |  |
| gpsi | Gpsi | C | 0..1 | Generic Public Subscription Identifier. Shall be provided when available. |  |
| accessType | AccessType | C | 0..1 | The Access Type where the served UE is camping. Shall be provided when available. |  |
| accessTypes | array(AccessType) | C | 1..N | The Access Types where the served UE is camping. Shall be provided when available. | MultipleAccessTypes |
| pei | Pei | C | 0..1 | The Permanent Equipment Identifier of the served UE. Shall be provided when available. |  |
| userLoc | UserLocation | C | 0..1 | The location of the served UE. Shall be provided when available. |  |
| timeZone | TimeZone | C | 0..1 | The time zone where the served UE is camping. Shall be provided when available. |  |
| servingPlmn | PlmnIdNid | C | 0..1 | The serving network (a PLMN or an SNPN) where the served UE is camping. For the SNPN the NID together with the PLMN ID identifies the SNPN. Shall be provided when available. |  |
| ratType | RatType | C | 0..1 | The 3GPP RAT Type where the served UE is camping. Shall be provided when available. |  |
| ratTypes | array(RatType) | C | 1..N | The 3GPP and non-3GPP RAT Types where the served UE is camping. Shall be provided when available. | MultipleAccessTypes |
| groupIds | array(GroupId) | C | 1..N | List of Internal Group Identifiers of the served UE. Shall be provided when available. |  |
| servAreaRes | ServiceAreaRestriction | C | 0..1 | Service Area Restriction as part of the AMF Access and Mobility Policy. Shall be provided when available. |  |
| wlServAreaRes | WirelineServiceAreaRestriction | O | 0..1 | Wireline Service Area Restriction as part of the AMF Access and Mobility Policy as determined by the PCF | WirelineWirelessConvergence |
| rfsp | RfspIndex | C | 0..1 | RFSP Index as part of the AMF Access and Mobility Policy. Shall be provided when available. |  |
| ueAmbr | Ambr | C | 0..1 | UE-AMBR as part of the AMF Access and Mobility Policy. Shall be provided when available. | UE-AMBR_Authorization |
| ueSliceMbrs | map(SliceMbr) | C | 1..N | One or more subscribed UE-Slice-MBR(s). Shall be provided when available. The key of the map is the S-NSSAI to which the UE-Slice-MBR belongs. | UE-Slice-MBR_Authorization |
| allowedSnssais | array(Snssai) | C | 1..N | Represents the Allowed NSSAI in the 3GPP access and includes the S-NSSAIs values the UE can use in the serving PLMN. It shall be included if the feature "SliceSupport" or the feature "DNNReplacementControl" is supported in the AMF. | SliceSupport, DNNReplacementControl |
| mappingSnssais | array(MappingOfSnssai) | C | 1..N | The mapping of each S-NSSAI of the Allowed NSSAI to the corresponding S-NSSAI of the HPLMN. It shall be included if available. If the feature "MultipleAccessTypes" is supported, this attribute contains also the mapping of the Allowed NSSAI in the non-3GPP access to the corresponding S-NSSAI of the HPLMN. | DNNReplacementControl |
| n3gAllowedSnssais | array(Snssai) | C | 1..N | Represents the Allowed NSSAI in the non-3GPP access and includes the S-NSSAIs values the UE can use in the serving PLMN. It shall be included if the feature "MultipleAccessTypes" and, the feature "SliceSupport" or "DNNReplacementControl" are supported in the AMF and the UE is registered in the non-3GPP access. | SliceSupport, MultipleAccessTypes, DNNReplacementControl |
| guami | Guami | C | 0..1 | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as service consumer. |  |
| serviceName | ServiceName | O | 0..1 | If the NF service consumer is an AMF, it should provide the name of a service produced by the AMF that makes use of information received within the Npcf_AMPolicyControl_UpdateNotify service operation. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the features supported by the service consumer. |  |
| traceReq | TraceData | C | 0..1 | Trace control and configuration parameters information defined in 3GPP TS 32.422 [18] shall be included if trace is required to be activated. |  |

# *** End of Changes ***