3GPP TSG-CT Meeting #94e	CP-213257

# E-Meeting, 13th – 15th December 2021								 			 (Revision of C3-216392)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0426 | rev | 3 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The OpenAPI file for AMInfluence service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_DCAMP |  |  |  |  |  | Date: |  |  | 2021-12-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The OpenAPI file for the AMInfluence service is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the OpenAPI file for the AMInfluence service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OpenAPI file for AMInfluence API is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR introduces a new OpenAPI file for AMInfluence API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of C3-216392: Remove the extra string "Nnef_" in "TS29522_Nnef_AmPolicyAuthorization.yaml". |  |  |  |  |  |  |  |  |

# *** First Change ***

A.X	AMInfluence API

openapi: 3.0.0

info:

title: AMInfluence

version: 1.0.0-alpha.1

description: |

AMInfluence API Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V17.4.0; 5G System; Network Exposure Function Northbound APIs.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/

servers:

- url: '{apiRoot}/3gpp-am-influence/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials: []

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

get:

summary: Read all of the active subscriptions for the AF.

tags:

- AM Influence Subscription

responses:

'200':

description: OK (Successful get all of the active subscriptions for the AF).

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AmInfluSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Create a new subscription to AM influence.

operationId: CreateAMInfluenceSubcription

tags:

- AM Influence Subscription

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AmInfluSub'

responses:

'201':

description: Create a new Individual AM Influence Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/AmInfluSub'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-am-influence/v1/{afId}/subscriptions/{subscriptionId}.'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AmInfluEventNotif'

minItems: 1

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

parameters:

- name: afId

in: path

description: Identifier of the AF.

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource.

required: true

schema:

type: string

get:

summary: Read an active subscription identified by the subscriptionId.

tags:

- Individual AM Influence Subscription

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/AmInfluSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update/Replace an existing subscription resource.

tags:

- Individual AM Influence Subscription

requestBody:

description: Parameters to update/replace the existing subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AmInfluSub'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/AmInfluSub'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Update/Replace an existing subscription resource.

tags:

- Individual AM Influence Subscription

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AmInfluSubPatch'

responses:

'200':

description: OK. The subscription was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/AmInfluSub'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing subscription.

tags:

- Individual AM Influence Subscription

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes: {}

schemas:

AmInfluSub:

description: Represents an AM influence subscription.

type: object

properties:

afTransId:

type: string

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

anyUeInd:

type: boolean

description: Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false".

dnnSnssaiInfos:

type: array

items:

$ref: '#/components/schemas/DnnSnssaiInformation'

minItems: 1

description: Each of the element identifies a (DNN, S-NSSAI) combination.

afAppIds:

type: array

items:

type: string

minItems: 1

description: Each of the element identifies an application.

highThruInd:

type: boolean

geoAreas:

type: array

items:

$ref: 'TS29522_AmPolicyAuthorization.yaml#/components/schemas/GeographicalArea'

minItems: 1

description: Identifies geographic areas of the user where the request is applicable.

policyDuration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

subscribedEvents:

type: array

items:

$ref: '#/components/schemas/AmInfluEvent'

minItems: 1

description: Indicates one or more AM influence related events.

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

requestTestNotification:

type: boolean

description: Set to true by the AF to request the NEF to send a test notification as defined in subclause 5.2.5.3 of 3GPP TS 29.122. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afTransId

anyOf:

- required: [highThruInd]

- required: [geoAreas]

oneOf:

- required: [gpsi]

- required: [externalGroupId]

- required: [anyUeInd]

AmInfluSubPatch:

description: Represents parameters to request the modification of an AM influence subscription resource.

type: object

properties:

highThruInd:

type: boolean

geoAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

description: Identifies geographic areas of the user where the request is applicable.

policyDuration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSecRm'

dnnSnssaiInfos:

type: array

items:

$ref: '#/components/schemas/DnnSnssaiInformation'

minItems: 1

description: Each of the element identifies a (DNN, S-NSSAI) combination.

afAppIds:

type: array

items:

type: string

minItems: 1

description: Each of the element identifies an application.

subscribedEvents:

type: array

items:

$ref: '#/components/schemas/AmInfluEvent'

minItems: 1

description: Indicates one or more AM influence related events.

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

afTransId:

type: string

description: Identifies an NEF Northbound interface transaction, generated by the AF.

AmInfluEventNotif:

description: Represents an AM influence event notification.

type: object

properties:

afTransId:

type: string

event:

$ref: '#/components/schemas/AmInfluEvent'

geoAreas:

type: array

items:

$ref: 'TS29522_AmPolicyAuthorization.yaml#/components/schemas/GeographicalArea'

minItems: 1

description: Identifies geographic areas of the user where the request is applicable.

required:

- event

- afTransId

DnnSnssaiInformation:

description: Represents a (DNN, SNSSAI) combination.

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

# ENUMERATIONS DATA TYPES

#

AmInfluEvent:

description: Represents the service area coverage outcome event.

anyOf:

- type: string

enum:

- SERVICE_AREA_COVRG_OUTCOME

- type: string

*** End of Changes ***