3GPP TSG-CT WG4 Meeting #107-bis-e	C4-220393

# E-Meeting, 17th – 21st January 2022                                                           was C4-220163

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.281 | CR | 0116 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Detecting of the restart of a GTP-U entity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EGTPUR |  |  |  |  |  | Date: |  |  | 2022-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As specified in clause 18A of 3GPP TS 23.007, a GTP-U entity may optionally support the detection of peer GTP-U entity restart. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify relevant stage 3 requirement for GTP-U protocol with defining a new IE type for Recovery Time Stamp. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A restarted GTP-U entity sends massive amount of GTP-U Error Indication messages, which further leads massive signalling over N4/Sx interface for the peer user plane function to report the receiving of GTP-U Error Indication and subsequent massive PFCP Session Modification signalling for affected PFCP sessions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.2.1, 7.2.2, 7.3.1, 8.1, 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.007 CR 0378 TS 23.527 CR 0039 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.007: "Restoration procedures".

[4]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[7]	3GPP TS 29.274: "3GPP Evolved Packet System; Evolved GPRS Tunnelling Protocol for EPS (GTPv2)".

[8]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[9]	IETF RFC 768 (STD 0006): "User Datagram Protocol", J. Postel.

[10]	IETF RFC 791 (STD 0005): "Internet Protocol", J. Postel.

[11]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[12]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[13]	3GPP TS 23.121: "Architectural requirements for Release 1999".

[14]	3GPP TS 43.129: "Packet-switched handover for GERAN A/Gb mode; Stage 2".

[15]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[16]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[17]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[18]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description; Stage 2".

[19]	IETF RFC 4604 (2006): "Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast".

[20]	IETF RFC 4607 (2006): "Source-Specific Multicast for IP".

[21]	3GPP TS 33.102: "3G Security; Security architecture".

[22]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE): Security architecture ".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[25]	3GPP TS 36.425: "E-UTRAN X2 interface user plane protocol".

[26]	IETF RFC 2474, "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[27]	3GPP TS 36.465: "Evolved Universal Terrestrial Radio Access (E-UTRAN) and Wireless LAN (WLAN) Xw interface user plane protocol".

[28]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[29]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[30]	3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[31]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[32]	3GPP TS 33.250: "Security assurance specification for the PGW network product class".

[33]	3GPP TS 23.527: "5G System; Restoration Procedures".

[34]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[35]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[36]	IETF RFC 8200: "Internet Protocol, Version 6 (IPv6) Specification"

[37]	IETF RFC 6437: "IPv6 Flow Label Specification".

[38]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[39]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[40]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[xx]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification".

* * * Next Change * * * *

### 7.2.1	Echo Request

A GTP-U peer may send an Echo Request on a path to the other GTP-U peer to find out if it is alive (see clause Path Failure). Echo Request messages may be sent for each path in use. A path is considered to be in use if at least one PDP context, EPS Bearer, PDU Session, MBMS UE context, or MBMS bearer context uses the path to the other GTP-U peer. When and how often an Echo Request message may be sent is implementation specific but an Echo Request shall not be sent more often than every 60 s on each path. This doesn't prevent resending an Echo Request with the same sequence number according to the T3-RESPONSE timer.

Even if there is no path in use, a GTP-U peer shall be prepared to receive an Echo Request at any time and it shall reply with an Echo Response.  The optional Private Extension contains vendor or operator specific information.

Handling of the Recovery Time Stamp is specified in 3GPP TS 23.007 [3] and 3GPP TS 23.527 [33].

Table 7.2.1-1: Information Elements in an Echo Request

| Information element | Presence requirement | Reference |
| --- | --- | --- |
| Recovery Time Stamp | Optional | 8.x |
| Private Extension | Optional | 8.6 |

For the GTP-U tunnel setup between two nodes for forwarding user traffic, e.g. between eNodeBs for direct forwarding over X2, Echo Request path maintenance message shall not be sent except if the forwarded data and the normal data are sent over the same path.

* * * Next Change * * * *

### 7.2.2	Echo Response

The message shall be sent as a response to a received Echo Request.

The Restart Counter value in the Recovery information element shall not be used, i.e. it shall be set to zero by the sender and shall be ignored by the receiver. The Recovery information element is mandatory due to backwards compatibility reasons.

Handling of the Recovery Time Stamp is specified in 3GPP TS 23.007 [3] and 3GPP TS 23.527 [33].

The optional Private Extension contains vendor or operator specific information.

Table 7.2.2-1: Information Elements in an Echo Response

| Information element | Presence requirement | Reference |
| --- | --- | --- |
| Recovery | Mandatory | 8.2 |
| Recovery Time Stamp | Optional | 8.x |
| Private Extension | Optional | 8.6 |

* * * Next Change * * * *

### 7.3.1	Error Indication

When a GTP-U node receives a G-PDU for which no EPS Bearer context, PDP context, PDU Session, MBMS Bearer context, or RAB exists, the GTP-U node shall discard the G-PDU. If the TEID of the incoming G-PDU is different from the value 'all zeros' the GTP-U node shall also return a GTP error indication to the originating node. GTP entities may include the "UDP Port" extension header (Type 0x40), in order to simplify the implementation of mechanisms that can mitigate the risk of Denial-of-Service attacks in some scenarios.

Handling of the received Error Indication and the Recovery Time Stamp is specified in 3GPP TS 23.007 [3] and 3GPP TS 23.527 [33].

The information element Tunnel Endpoint Identifier Data I shall be the TEID fetched from the G-PDU that triggered this procedure.

The information element GTP-U Peer Address shall be the destination address (e.g. destination IP address, MBMS Bearer Context) fetched from the original user data message that triggered this procedure. A GTP-U Peer Address can be a GGSN, SGSN, RNC, PGW, SGW, ePDG, eNodeB, TWAN, MME, gNB, N3IWF, or UPF address. The TEID and GTP-U peer Address together uniquely identify the related PDP context, RAB, PDU session or EPS bearer in the receiving node.

The optional Private Extension contains vendor or operator specific information.

Table 7.3.1-1: Information Elements in an Error Indication

| Information element | Presence requirement | Reference |
| --- | --- | --- |
| Tunnel Endpoint Identifier Data I | Mandatory | 8.3 |
| GTP-U Peer Address | Mandatory | 8.4 |
| Recovery Time Stamp | Optional | 8.x |
| Private Extension | Optional | 8.6 |

* * * Next Change * * * *

## 8.1	Information Element Types

A GTP-U Signalling message may contain several information elements. The TLV (Type, Length, Value) or TV (Type, Value) encoding format shall be used for the GTP information elements. The information elements shall be sorted, with the Type fields in ascending order, in the signalling messages. The Length field contains the length of the information element excluding the Type and Length field.

For all the length fields, bit 8 of the lowest numbered octet is the most significant bit and bit 1 of the highest numbered octet is the least significant bit.

Within information elements, certain fields may be described as spare. These bits shall be transmitted with the value defined for them. To allow for future features, the receiver shall not evaluate these bits.

The most significant bit in the Type field is set to 0 when the TV format is used and set to 1 for the TLV format.

Figure 8.1-1: Type field for TV and TLV format

The complete range of information element types defined for GTPv1 is defined in 3GPP TS 29.060 [6]. The table below includes those applicable to GTP user plane.

Table 8.1-1: Information Elements

| IE Type Value | Format | Information Element | Reference |
| --- | --- | --- | --- |
| 0-13 | TV | Reserved in 3GPP TS 29.060 [6] |  |
| 14 | TV | Recovery | 8.2 |
| 15 | TV | Reserved in 3GPP TS 29.060 [6] |  |
| 16 | TV | Tunnel Endpoint Identifier Data I | 8.3 |
| 17-132 | TV/TLV | Reserved in 3GPP TS 29.060 [6] |  |
| 133 | TLV | GSN Address. See NOTE 1. | 8.4 |
| 134-140 | TLV | Reserved in 3GPP TS 29.060 [6] |  |
| 141 | TLV | Extension Header Type List | 8.5 |
| 142-229 | TLV | Reserved in 3GPP TS 29.060 [6] |  |
| 230 | TLV | GTP-U Tunnel Status Information | 8.7 |
| 23x | TLV | Recovery Time Stamp | 8.x |
| 23y-237 | TLV | Spare. For future use. |  |
| 238-254 | TLV | Reserved in 3GPP TS 29.060 [6] |  |
| 255 | TLV | Private Extension | 8.6 |
| NOTE 1: This IE is named as " GTP-U Peer Address" in the rest of this specification. |  |  |  |

* * * Next Change * * * *

### 8.x	Recovery Time Stamp

The Recovery Time Stamp IE is coded as shown in Figure 8.x-1. It indicates the UTC time when the GTP-U entity started. Octets 4 to 7 are encoded in the same format as the first four octets of the 64-bit timestamp format as defined in clause 6 of IETF RFC 5905 [xx].

NOTE:	The encoding is defined as the time in seconds relative to 00:00:00 on 1 January 1900.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 | Type = aa (decimal) |  |  |  |  |  |  |  |  |
|  | 2 to 3 | Length = n |  |  |  |  |  |  |  |  |
|  | 4 to 7 | Recovery Time Stamp value |  |  |  |  |  |  |  |  |
|  | 8 to (n+3) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.x-1: Recovery Time Stamp

* * * End of Changes * * * *