3GPP TSG-CT Meeting #95e	CP-220110

Electronic Meeting, 14 – 23 March 2022

Title:	Summary for Stage 3 aspects of eMCCI

Type:	Work Item Summary

Agenda Item:	20

Source:	Rapporteur (Mike Dolan, FirstNet)

Contact:	Michael.Dolan@firstnet.gov

This document is a contribution to TR 21.917 ("Release 17 Description; Summary of Rel-17 Work Items"). It is meant to be endorsed by the plenary.

Summary based on the input provided by FirstNet in CP-22xxxx.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 890003 | CT aspects of Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI_CT | C1 | CP-202241 | Mike Dolan, FirstNet |

Introduction

Stage 3 aspects in eMCCI_CT are based on the stage 2 work for eMCCI. This work item covers enhancements to interworking 3GPP mission critical systems with LMR systems in Rel-17 to provide support for a conference event package, affiliation on behalf of a set of users, and private call floor control.

Description

The Rel-17 IWF (interworking function) now supports the ability for the IWF to handle subscriptions to events that may occur in the LMR system, and for the IWF to be able to subscribe on behalf of LMR users to events in the 3GPP mission critical system (MCPTT and MCData).

Affiliation on behalf of a set of users is now handled from the IWF toward the 3GPP mission critical system to potentially reduce the signalling load due to repetitive messaging.

Private call floor control is now supported between an MCPTT system and an IWF.

References

[1]	TS 29.379: “Mission Critical Push To Talk (MCPTT) call control interworking with Land Mobile Radio (LMR) systems”

[2]	TS 29.380: “Mission Critical Push To Talk (MCPTT) media plane control interworking with Land Mobile Radio (LMR) systems”

[3]	TS 29.582: “Mission Critical Data (MCData) interworking with Land Mobile Radio (LMR) systems”