3GPP TSG-CT Meeting #95e	CP-220111

Electronic Meeting, 14 – 23 March 2022

Title:	Summary for Stage 3 aspects of enh3MCPTT

Type:	Work Item Summary

Agenda Item:	20

Source:	Rapporteur (Mike Dolan, FirstNet)

Contact:	Michael.Dolan@firstnet.gov

This document is a contribution to TR 21.917 ("Release 17 Description; Summary of Rel-17 Work Items"). It is meant to be endorsed by the plenary.

Summary based on the input provided by FirstNet in CP-220021.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 900039 | Stage 3 aspects of enh3MCPTT | enh3MCPTT-CT | C1 | CP-213071 | Mike Dolan, FirstNet |

Introduction

Stage 3 aspects of enh3MCPTT are based on the stage 2 work for enh3MCPTT. This WI covers work on enhancements to the MCPTT feature in Rel-17 to provide support for an MCPTT user logged into multiple devices, support for z-axis location measurements, preconfigured regroup support, and unicast media stop/resume.

Description

The Rel-17 system now supports multiple registrations for the same mission critical user, such as might happen if a public safety person uses a tablet, an in-vehicle computer, and a smartphone.

Location services reports can now include z-axis information in support of situational awareness, such as might be needed to determine what floor of a building a firefighter is on.

User and group regroups can now be based on a group that is to be used only for preconfigured regroups. Clients in the device will reject attempts to make group calls directly to groups marked for preconfigured group use only. In addition, the preconfigured group capabilities have been added to MCVideo and MCData.

For MCPTT, it is now possible for a client to indicate to the server that it wants to stop/resume receiving media for a given group call. This may be needed to preserve radio resources when the MCPTT user is participating in another higher priority group call.

Finally, a number of issues that were found during ETSI Plugtest events have been fixed or clarified.

References

List of related CRs: select "TSG Status = Approved" in: 
https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=900039

[1]	TS 24.281: “Mission Critical Video (MCVideo) signalling control”

[2]	TS 24.282: “Mission Critical Data (MCData) signalling control”

[3]	TS 24.379: “Mission Critical Push To Talk (MCPTT) call control”

[4]	TS 24.380: “Mission Critical Push To Talk (MCPTT) media plane control”

[5]	TS 24.483: “Mission Critical Services (MCS) Management Object (MO)”

[6]	TS 24.484: “Mission Critical Services (MCS) configuration management”