3GPP TSG-CT Meeting #95-e	CP-220125

# E-Meeting, 14th – 16th March 2022													Revision of C4-221484

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0335 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS Session Status subscriptions and notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The definition of the eventsSubscription IE in the MbsSession data type is FFS. TS 29.532 and TS 29.522 are not aligned on the definition of the MBS session status subscription and notification. The MB-SMF and NEF APIs should be aligned on the contents of an MBS session status subscription and notification, to expose the same information to AFs on the Nmb13 (MB-SMF <> AF) and N33 (NEF <> AF) reference points, avoid inconsistencies (now and in future) causing implementation and interoperability issues, and to simplify the NEF implementation. It is proposed to define common data types in TS 29.571 for MBS Session Status subscriptions and notifications and use these data types in both the MB-SMF and NEF APIs, to ensure a uniform and consistent encoding of MBS session creation responses in both APIs. Attributes that are not desired to be returned in the representation of the MBS session in the creation response can be defined with the Write-Only property. Those that are only returned in the response can be defined with the Read-Only property. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Common data types are defined for MBS Session Status subscriptions and notifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent information defined for MBS session status subscriptions and notifications in the MB-SMF and NEF APIs, over the Nmb13 (MB-SMF <> AF) and N33 (NEF <> AF) reference points, causing NEF and AF implementation complexity and the perception of a poor quality design by 3GPP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.3.y (new), 5.9.4.6, 5.9.4.u (new), 5.9.4.v (new), 5.9.4.w (new), 5.9.4.x (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the following APIs: - TS29522_MBSSession API - TS29532_Nmbsmf_MBSSession API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: - BROADCAST_STATUS is renamed BROADCAST_DELIVERY_STATUS. - Huawei added as co-source (merge of C4-221122) Rev. 2: The following errors (found during the implementation of the CR agreed at CT4#108-e) have been corrected in the OpenAPI definition: - typo corrected in the data type of AreaSessionId (extra "s" removed) - StatusEventType corrected to MbsSessionEventType in one place |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.9.3.y	Enumeration: MbsSessionEventType

Table 5.9.3.y-1: Enumeration MbsSessionEventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "MBS_REL_TMGI_EXPIRY" | Subscription to be notified about the MBS session release due to TMGI expiry. |  |
| "BROADCAST_DELIVERY_STATUS" | Subscription to be notified about the MBS session broadcast delivery status. |  |

* * * Next Change * * * *

#### 5.9.4.6	Type: MbsSession

Table 5.9.4.6-1: Definition of type MbsSession

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | C | 0..1 | MBS session identifier (TMGI and/or SSM, and NID for an SNPN) (NOTE) |
| tmgiAllocReq | boolean | C | 0..1 | TMGI allocation request indication. This IE shall be present if the mbsSessionId IE is absent. This IE may also be present if the mbsSessionId IE is present and it does not contain a TMGI. When present, it shall be set as follows: - true: a TMGI is requested to be allocated - false (default): no TMGI is requested to be allocated Write-Only: true (NOTE) |
| serviceType | MbsServiceType | M | 1 | Service Type (either multicast or broadcast service) |
| ingressAddrReq | boolean | O | 0..1 | Ingress transport address request indication (for unicast transport over N6mb/Nmb9). When present, it shall be set as follows: - true: an ingress transport address is requested - false (default): no request Write-Only: true |
| mbsServiceArea | MbsServiceArea | O | 0..1 | MBS Service Area |
| dnn | Dnn | O | 0..1 | DNN |
| snssai | Snssai | O | 0..1 | S-NSSAI |
| activationTime | DateTime | O | 0..1 | MBS session activation time |
| terminationTime | DateTime | O | 0..1 | MBS session termination time |
| qosInformation | FFS | O | 0..1 | QoS information |
| mbsSessionSubsc | MbsSessionSubscription | O | 0..1 | Subscription to one or more MBS session events |
| activityStatus | MbsSessionActivityStatus | O | 0..1 | Session activity status (active or inactive) This IE may be provided if the serviceType indicates a multicast MBS session. |
| anyUeInd | Boolean | O | 0..1 | Indication that any UE may join the MBS session. This IE may be provided if the serviceType indicates a multicast MBS session. When present, it shall be set as follows: - true: any UE may join the MBS session - false (default): the MBS session is not open to any UE |
| NOTE: At least one of the mbsSessionId IE and tmgiAllocReq IE shall be present. Both may be present if the mbsSessionId IE does not contain a TMGI (i.e. if it only contains a SSM). |  |  |  |  |

Editor's Note: the definition of the qosInformation attribute is FFS.

* * * Next Change * * * *

#### 5.9.4.u	Type: MbsSessionSubscription

Table 5.9.4.u-1: Definition of type MbsSessionSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | C | 0..1 | Identifier of the MBS Session for which the subscription is created. This IE shall be present, except for an MBS session subscription request that is sent within an MBS session creation request. |
| areaSessionId | AreaSessionId | C | 0..1 | Area Session ID, for a location dependent MBS session, identifying the part of the MBS session in an MBS service area for which the subscription is created. It shall be present for a location dependent MBS session, except for an MBS session subscription request that is sent within an MBS session creation request |
| eventList | array(MbsSessionEvent) | M | 1..N | List of MBS session events subscribed |
| notifyUri | Uri | M | 1 | URI where the NF service consumer requests to receive MBS session notifications. Write-Only: true |
| notifyCorrelationId | string | O | 0..1 | Notification Correlation ID Write-Only: true |
| expiryTime | DateTime | O | 0..1 | When present in an MBS Session subscription creation request, it shall indicate the time up to which the subscription is desired to be kept active and after which the subscribed events shall stop generating notifications. When present in a subscription response, it shall indicate the expiry time after which the subscription becomes invalid. |
| nfcInstanceId | NfInstanceId | C | 0..1 | NF Instance ID of the NF Service Consumer This IE shall be present if available. Write-Only: true |
| mbsSessionSubscUri | Uri | C | 0..1 | This IE shall be present in the response to an MBS session creation request that includes a subscription to events about the MBS session and the subscription was created successfully. When present, it shall contain the URI of the individual subscription resource. Read-Only: true (NOTE) |
| NOTE: When an MBS session status subscription is created separately (i.e. after) an MBS session creation, the Location header returned in the MBS session status subscription creation response contains the URI of the created subscription. |  |  |  |  |

* * * Next Change * * * *

#### 5.9.4.v	Type: MbsSessionEventReportList

Table 5.9.4.v-1: Definition of type MbsSessionEventReportList

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eventReportList | Array(MbsSessionEventReport) | M | 1..N | List of MBS session events to report |
| notifyCorrelationId | string | C | 0..1 | Notification Correlation ID. This IE shall be present if a Notification Correlation ID is available in the subscription. |

* * * Next Change * * * *

#### 5.9.4.w	Type: MbsSessionEvent

Table 5.9.4.w-1: Definition of type MbsSessionEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventType | MbsSessionEventType | M | 1 | MBS session event type |  |

* * * Next Change * * * *

#### 5.9.4.x	Type: MbsSessionEventReport

Table 5.9.4.x-1: Definition of type MbsSessionEventReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventType | MbsSessionEventType | M | 1 | MBS session event type |  |
| timeStamp | DateTime | C | 0..1 | This IE shall contain the time at which the event is generated. This IE should be present, if available. |  |

* * * Next Change * * * *

## A.2	Data related to Common Data Types

openapi: 3.0.0

info:

version: '1.3.0-alpha.4'

title: 'Common Data Types'

description: |

Common Data Types for Service Based Interfaces.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

#

# Enumerations

#

#

MbsServiceType:

description: Indicates the type of an MBS session

anyOf:

- type: string

enum:

- MULTICAST

- BROADCAST

- type: string

MbsSessionActivityStatus:

description: Indicates the MBS session's activity status

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

MbsSessionEventType:

description: MBS Session Event Type

anyOf:

- type: string

enum:

- MBS_REL_TMGI_EXPIRY

- BROADCAST_DELIVERY_STATUS

- type: string

[…]

MbsSession:

description: Individual MBS session

type: object

properties:

mbsSessionId:

$ref: '#/components/schemas/MbsSessionId'

tmgiAllocReq:

type: boolean

default: false

writeOnly: true

serviceType:

$ref: '#/components/schemas/MbsServiceType'

ingressAddrReq:

type: boolean

default: false

writeOnly: true

mbsServiceArea:

$ref: '#/components/schemas/MbsServiceArea'

dnn:

$ref: '#/components/schemas/Dnn'

snssai:

$ref: '#/components/schemas/Snssai'

activationTime:

$ref: '#/components/schemas/DateTime'

terminationTime:

$ref: '#/components/schemas/DateTime'

mbsSessionSubsc:

$ref: '#/components/schemas/MbsSessionSubscription'

activityStatus:

$ref: '#/components/schemas/MbsSessionActivityStatus'

anyUeInd:

type: boolean

default: false

required:

- serviceType

anyOf:

- required: [ mbsSessionId ]

- required: [ tmgiAllocReq ]

MbsSessionSubscription:

description: MBS session subscription

type: object

properties:

mbsSessionId:

$ref: '#/components/schemas/MbsSessionId'

areaSessionId:

$ref: '#/components/schemas/AreaSessionId'

eventList:

type: array

items:

$ref: '#/components/schemas/MbsSessionEvent'

minItems: 1

notifyUri:

$ref: '#/components/schemas/Uri'

notifyCorrelationId:

type: string

expiryTime:

$ref: '#/components/schemas/DateTime'

nfcInstanceId:

$ref: '#/components/schemas/NfInstanceId'

mbsSessionSubscUri:

$ref: '#/components/schemas/Uri'

readOnly: true

required:

- eventList

- notifyUri

MbsSessionEventReportList:

description: MBS session event report list

type: object

properties:

eventReportList:

type: array

items:

$ref: '#/components/schemas/MbsSessionEventReport'

minItems: 1

notifyCorrelationId:

type: string

required:

- eventReportList

MbsSessionEvent:

description: MBS session event

type: object

properties:

eventType:

$ref: '#/components/schemas/MbsSessionEventType'

required:

- eventType

MbsSessionEventReport:

description: MBS session event report

type: object

properties:

eventType:

$ref: '#/components/schemas/MbsSessionEventType'

timeStamp:

$ref: '#/components/schemas/DateTime'

required:

- eventType

[…]

* * * End of Changes * * * *