3GPP TSG-CT3 Meeting #119bis-e	C3-220178

E-Meeting, 17th – 21th January 2022											(Revision of C3-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.486 | CR | 0078 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to VAE_PC5ProvisioningRequirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2022-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some descriptions of VAE_PC5ProvisioningRequirement API shall be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make some correction for VAE_PC5ProvisioningRequirement API service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.10.2.2.1, 5.10.2.2.2, 5.10.2.3.2, 6.9.2.1, 6.9.3.1, 6.9.3.2, 6.9.3.2.2, 6.9.3.3.3.3, 6.9.6.1, 6.9.6.2.2, 6.9.3.3.3.3, 6.9.6.1, 6.9.6.2.2, 6.9.6.2.3, 6.9.7.1, 6.9.7.2, 6.9.7.3, 6.9.8, A.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the OpenAPI file of VAE_PC5ProvisioningRequirement API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.1	Introduction

The table 5.1-1 shows the services provided by the VAE server and corresponding Service Operations:

Table 5.1-1 List of services provided by the VAE Server

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| VAE_MessageDelivery | Deliver_DL_Message | Request/Response | V2X application specific server |
|  | Deliver_UL_Message | Subscribe/Notify | V2X application specific server |
|  | V2X_MessageDelivery_Subscribe |  | V2X application specific server |
|  | V2X_MessageDelivery_Unsubscribe |  | V2X application specific server |
| VAE_FileDistribution | Distribute_File | Request/ Response | V2X application specific server |
| VAE_ApplicationRequirement | Reserve_NetworkResource | Subscribe/Notify | V2X application specific server |
|  | Notify_NetworkResource |  |  |
| VAE_DynamicGroup | Configure_DynamicGroup | Subscribe/Notify | V2X application specific server |
| VAE_ServiceContinuity | Query_ServiceContinuity | Request/Response | VAE server |
| VAE_HDMapDynamicInfo | Subscribe_HDMapDynamicInfo | Subscribe/Notify | V2X application specific server |
| VAE_SessionOrientedService API | Establish_Session | Subscribe/Notify | V2X application specific server |
|  | Notify_Establish_Session |  |  |
|  | Update_Session |  |  |
|  | Notify_Update_Session |  |  |
|  | Terminate_Session |  |  |
|  | Notify_Terminate_Session |  |  |
| VAE_V2VConfigRequirement API | Request_V2VConfigRequirement | Request/Response | V2X application specific server |
| VAE_PC5ProvisioningRequirement | Config_PC5ProvisioningRequirement | Subscribe/Notify | V2X application specific server |
|  | Notify_PC5ProvisioningRequirement |  |  |

Table 5.1-2 summarizes the corresponding APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| VAE_MessageDelivery | 6.1 | VAE Message Delivery Service | TS29486_VAE_MessageDelivery.yaml | vae-message-delivery | A.2 |
| VAE_FileDistribution | 6.2 | VAE File Distribution Service | TS29486_VAE_FileDistribution.yaml | vae-file-distribution | A.3 |
| VAE_ApplicationRequirement | 6.3 | VAE Application Requirement Provision Service | TS29486_VAE_ApplicationRequirement.yaml | vae-app-req | A.4 |
| VAE_DynamicGroup | 6.4 | VAE Configure Dynamic Group Information Service | TS29486_VAE_DynamicGroup.yaml | vae-dynamic-group | A.5 |
| VAE_ServiceContinuity | 6.5 | VAE Service Continuity Service | TS29486_VAE_ServiceContinuity.yaml | vae-service-continuity | A.6 |
| VAE_HDMapDynamicInfo | 6.6 | VAE_HDMapDynamicInfo Service | TS29486_VAE_HDMapDynamicInfo.yaml | vae-hdmap-dynamic-info | A.7 |
| VAE_SessionOrientedService API | 6.7 | VAE_SessionOrientedService | TS29486_VAE_SessionOrientedService.yaml | vae-session-oriented-service | A.8 |
| VAE_V2VConfigRequirement API | 6.8 | VAE_SessionOrientedService | TS29486_VAE_V2VConfigRequirement.yaml | vae-v2v-config-req | A.9 |
| VAE_PC5ProvisioningRequirement | 6.9 | VAE_PC5ProvisioningRequirement | TS29486_VAE_PC5ProvisioningRequirement.yaml | vae-pc5-prov-req | A.10 |

# *** Next Change ***

5.10.1	Service Description

This API enables the V2X application specific server to communicate with the VAE server to request from VAE server the PC5 provisioning service in multi-operator V2X scenarios.

# *** Next Change ***

5.10.2.2.1	General

The Config_PC5ProvisioningRequirement service operation is used by the V2X application specific server to provide a V2X PC5 provisioning requirement to the VAE server.

# *** Next Change ***

5.10.2.2.2	Config_PC5ProvisioningRequirement

Figure 5.10.2.2.2-1: Config_PC5ProvisioningRequirement

When the NF service consumer (e.g. V2X application specific server) needs to provide a V2X PC5 provisioning requirement to the VAE server, the NF service consumer shall send the POST method as step 1 of the figure 5.10.2.2.2-1 to request to create an "Individual PC5 Provisioning Requirement Subscription".

The NF service consumer shall include ProvisioningRequirement data structure in the payload body of the HTTP POST to request a creation of representation of the "Individual PC5 Provisioning Requirement Subscription" resource. The "Individual PC5 Provisioning Requirement Subscription" resource is created as described below.

The NF service consumer within the ProvisioningRequirement data structure shall include:

-	notification URI within the "notifUri" attribute;

-	either the remote V2X UE ID within the "ueId" attribute or the V2X group ID within the "groupId" attribute;

-	the V2X service ID within the "serviceId" attribute;

-	application QoS requirements for the session within the "appQosReq" attribute;

and may include:

-	the PLMN ID list within the "plmnList" attribute.

When the VAE Server receives the HTTP POST request from the NF service consumer, the VAE server shall make an authorization based on the information received from the NF service consumer.  If the authorization is successful, the VAE Server shall create a new resource, which represents "Individual PC5 Provisioning Requirement Subscription", addressed by a URI as defined in clause 6.9.3.3.2 and contains a VAE Server created resource identifier. The VAE Server shall respond to the NF service consumer with a 201 Created message, including Location header field containing the URI for the created resource.

The NF service consumer shall use the URI received in the Location header in subsequent requests to the VAE Server to refer to the "Individual PC5 Provisioning Requirement Subscription".

After the VAE Server responded to the NF service consumer, the VAE Server may invoke the procedure defined in 3GPP TS 24.486 [28] to send a PC5 provisioning status request to VAE client(within the multi-operator V2X service) to receive up-to-date information on the per PLMN provisioning policies/ parameters.

The NF service consumer may include the ProvisioningRequirement data structure in the payload body of the HTTP PUT to update the "Individual PC5 Provisioning Requirement Subscription" resource. The remote V2X UE ID, the V2X service ID and the V2X service ID shall remain unchanged from previous values When the VAE Server receives the HTTP PUT request from the NF service consumer, the VAE server shall make an authorization based on the information received from the NF service consumer. If the authorization is successful, the VAE Server shall update the "Individual PC5 Provisioning Requirement Subscription" and respond to the NF service consumer with a 200 OK or 204 No Content status code. After the VAE Server responded to the NF service consumer, the VAE Server shall invoke the procedure defined in 3GPP TS 24.486 [28] to provide the upated infomation to the VAE client.

Upon receipt of the HTTP DELETE message from the NF service consumer, the VAE Server shall check if the "Individual PC5 Provisioning Requirement Subscription" resource identified by the URI already exists. If the resource exists, the VAE Server shall delete the resource and respond to the NF service consumer with a 204 No Content success message. After the VAE Server responded to the NF service consumer, the VAE Server shall invoke the procedure defined in 3GPP TS 24.486 [28] to delete the PC5 provisioning status request from the VAE client.

If errors occur when processing the HTTP POST, HTTP PUT or HTTP DELETE request, the VAE Server shall apply error handling procedures as specified in subclause 6.9.7.

# *** Next Change ***

5.10.2.3.2	Notify_PC5ProvisioningRequirement

Figure 5.10.2.3.2-1: Notify_PC5ProvisioningRequirement

After the VAE Server determines the updated PC5 provisioning policies/parameters to be jointly used across the V2X-UEs within the multi-operator V2X service, the VAE Server shall send an HTTP POST request with "{notifUri}" as previously provided by the NF service consumer within the corresponding subscription as URI and Notification data structure as request body that shall include:

-	resource URI of the Individual PC5 Provisioning Requirement Subscription related to the notification within the "resourceUri" attribute;

-	the result of V2X PC5 provisioning requirement within the "result" attribute.

Upon the reception of the HTTP POST message, if the NF service consumer successfully processed and accepted the received HTTP POST request, the NF service consumer shall send an "204 No Content" HTTP response for a succesfull processing.

If errors occur when processing the HTTP POST request, the VAE Server shall send an HTTP error response as specified in subclause 6.9.7.

# *** Next Change ***

6.9.2.1	General

Support of HTTP/1.1 (IETF RFC 7230 [12], IETF RFC 7231 [13], IETF RFC 7232 [14], IETF RFC 7233 [15], IETF RFC 7234 [16] and IETF RFC 7235 [17]) over TLS (IETF RFC 5246 [18]) is mandatory and support of HTTP/2 as specified in clause 5 of 3GPP TS 29.500 [2] is recommended. A V2X application specific server desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 7540 [5].

HTTP/2, shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [2].

An OpenAPI [6] specification of HTTP messages and content bodies for the VAE_PC5ProvisioningRequirement is contained in Annex A.10.

# *** Next Change ***

6.9.3.1	Overview

Figure 6.9.3.1-1: Resource URI structure of the VAE_PC5ProvisioningRequirement API

Table 6.9.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.9.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PC5 Provisioning Requirement Subscriptions | /subscriptions | POST | Create a new Individual PC5 Provisioning Requirement Subscription. |
| Individual PC5 Provisioning Requirement Subscription | /subscriptions/{subscriptionId} | GET | Read an Individual PC5 Provisioning Requirement Subscription. |
|  |  | PUT | Update an Individual PC5 Provisioning Requirement Subscription. |
|  |  | DELETE | Delete an Individual PC5 Provisioning Requirement Subscription. |

# *** Next Change ***

6.9.3.2	Resource: PC5 Provisioning Requirement Subscriptions

# *** Next Change ***

6.9.3.2.2	Resource Definition

Resource URI: {apiRoot}/vae-pc5-prov-req/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.9.3.2.2-1.

Table 6.9.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.9.1 |
| apiVersion | string | See clause 6.9.1 |

# *** Next Change ***

6.9.3.3.3.3	DELETE

This method shall support the URI query parameters specified in table 6.9.3.3.3.3-1.

Table 6.9.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.9.3.3.3.3-2 and the response data structures and response codes specified in table 6.9.3.3.3.3-3.

Table 6.9.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.9.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Individual PC5 Provisioning Requirement Subscription resource was successfully deleted |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during the Individual PC5 Provisioning Requirement Subscription resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative VAE Server. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [22] with the difference: SCEF is replaced by the VAE Server and the SCS/AS is replaced by the V2X application specific server. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during the Individual PC5 Provisioning Requirement Subscription resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative VAE Server. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [22] with the difference: SCEF is replaced by the VAE Server and the SCS/AS is replaced by the V2X application specific server. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

Table 6.9.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative VAE Server. |

Table 6.9.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative VAE Server. |

# *** Next Change ***

6.9.6.1	General

This clause specifies the application data model supported by the API.

Table 6.9.6.1-1 specifies the data types defined for the VAE_PC5ProvisioningRequirement API.

Table 6.9.6.1-1: VAE_PC5ProvisioningRequirement specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| Notification | 6.9.6.2.3 |  |  |
| ProvisioningRequirement | 6.9.6.2.2 |  |  |

Table 6.9.6.1-2 specifies data types re-used by the VAE_PC5ProvisioningRequirement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the VAE_PC5ProvisioningRequirement service based interface.

Table 6.9.6.1-2: VAE_PC5ProvisioningRequirement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AppplicationQosRequirement | 6.7.6.2.4 |  |  |
| AppServerId | 6.1.6.3.2 | Identity of the V2X application specific server. |  |
| PlmnId | 3GPP TS 29.571 [11] |  |  |
| Result | 6.2.6.3.3 |  |  |
| SupportedFeatures | 3GPP TS 29.571 [11] |  |  |
| TestNotification | 3GPP TS 29.122 [22] | Represents a notification that can be sent to test whether a chosen notification mechanism works. | Notification_test_event |
| Uri | 3GPP TS 29.571 [11] | URI. |  |
| V2xGroupId | 6.1.6.3.2 | The group ID |  |
| V2xServiceId | 6.1.6.3.2 | The V2X service ID. |  |
| V2xUeId | 6.1.6.3.2 | Identifier of the destination V2X UE |  |
| WebsockNotifConfig | 3GPP TS 29.122 [22] | Pepresents configuration for the delivery of notifications over Websockets. | Notification_websocket |

# *** Next Change ***

6.9.6.2.2	Type: ProvisioningRequirement

Table 6.9.6.2.2-1: Definition of type ProvisioningRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | V2xUeId | C | 0..1 | Identity of the V2X UE for which V2X application requirement is initiated. |  |
| groupId | V2xGroupId | C | 0..1 | Identity of the V2X group for which the V2X application requirement is initiated. |  |
| notifUri | Uri | M | 1 | Identifies the recipient of notification sent by the VAE server. |  |
| serviceId | V2xServiceId | M | 1 | The V2X service ID for which application requirement corresponds to. |  |
| appQosReq | AppplicationQosRequirement | O | 0..1 | The application QoS requirements for the V2X service. |  |
| plmnList | array(PlmnId) | O | 1..N | The list of the PLMN identities for the PLMNs which offer the V2X service |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by the NF service consumer to request the VAE server to send a test notification as defined in clause 6.3.5.3. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 6.3.5.4. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the features supported by the service consumer. It shall be included in the first interaction. |  |
| NOTE: Either "ueId" attribute or "groupId" attribute shall be present. |  |  |  |  |  |

# *** Next Change ***

6.9.6.2.3	Type: Notification

Table 6.9.6.2.3-1: Definition of type Notification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceUri | Uri | M | 1 | The resource URI of the Individual PC5 Provisioning Requirement Subscription related to the notification. |  |
| result | Result | M | 1 | The result indicating success or failure to provisioning of QoS requirement. |  |

# *** Next Change ***

6.9.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [2].

For the VAE_PC5ProvisioningRequirement Service API, HTTP error responses shall be supported as specified in subclause 4.8 of 3GPP TS 29.501 [3].

Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [2] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [2].

In addition, the requirements in the following subclauses are applicable for the VAE_PC5ProvisioningRequirement Service API.

# *** Next Change ***

6.9.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the VAE_PC5ProvisioningRequirement API.

# *** Next Change ***

6.9.7.3	Application Errors

The application errors defined for the VAE_PC5ProvisioningRequirement service are listed in table 6.9.7.3-1.

Table 6.9.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

# *** Next Change ***

6.9.8	Feature negotiation

The optional features in table 6.9.8-1 are defined for the VAE_PC5ProvisioningRequirement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [2].

Table 6.9.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | The testing of notification connection is supported according to clause 6.9.5.3. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 6.9.5.4. This feature requires that the Notification_test_event feature is also supported. |

# *** Next Change ***

A.10	VAE_PC5ProvisioningRequirement

openapi: 3.0.0

info:

version: 1.0.0-alpha.1

title: VAE_PC5ProvisioningRequirement

description: |

API for VAE_PC5ProvisioningRequirement

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.486 V17.3.0 V2X Application Enabler (VAE) Services

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.486/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/vae-pc5-prov-req/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

paths:

/subscriptions:

post:

summary: VAE_PC5 Provisioning Requirement resource create service Operation

tags:

- PC5 provisioning requirement subscriptions collection (Document)

operationId: Create

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ProvisioningRequirement'

required: true

responses:

'201':

description: PC5 Provisioning Requirement Subscription Resource Created

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/ProvisioningRequirement'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyResutOfMultiOperationPC5Provisioning:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Notification'

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: VAE PC5 Provisioning Requirement Subscription resource read service Operation

tags:

- Individual PC5 Provisioning Requirement Subscription (Document)

operationId: ReadPC5ProvisioningRequirementSubscription

parameters:

- name: subscriptionId

in: path

description: Identifier of an PC5 Provisioning Requirement Subscription resource

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/ProvisioningRequirement'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing subscription resource

tags:

- VAE PC5 Provisioning Requirement Subscription resource put service Operation

operationId: UpdatePC5ProvisioningRequirementSubscription

parameters:

- name: subscriptionId

in: path

description: Identifier of an PC5 Provisioning Requirement Subscription resource

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ProvisioningRequirement'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ProvisioningRequirement'

'204':

description: No Content (Successful update of the subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: VAE PC5 Provisioning Requirement Subscription resource delete service Operation

tags:

- Individual PC5 Provisioning Requirement Subscription (Document)

operationId: DeletePC5ProvisioningRequirementSubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the PC5 Provisioning Requirement Subscription to be deleted

schema:

type: string

responses:

'204':

description: The subscription was terminated successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ProvisioningRequirement:

description: Represents an Individual PC5 Provisioning Requirement Subscription resource.

type: object

properties:

ueId:

$ref: 'TS29486_VAE_MessageDelivery.yaml#/components/schemas/V2xUeId'

groupId:

$ref: 'TS29486_VAE_MessageDelivery.yaml#/components/schemas/V2xGroupId'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

serviceId:

$ref: 'TS29486_VAE_MessageDelivery.yaml#/components/schemas/V2xServiceId'

appQosReq:

$ref: 'TS29486_VAE_SessionOrientedService.yaml#/components/schemas/AppplicationQosRequirement'

plmnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

requestTestNotification:

type: boolean

description: Set to true by the NF service consumer to request the VAE server to send a test notification as defined in clause 6.3.5.3. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- serviceId

- notifUri

Notification:

description: Represents a notificaton of result of PC5 Provisioning Requirement.

type: object

properties:

resourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

result:

$ref: 'TS29486_VAE_MessageDelivery.yaml#/components/schemas/Result'

required:

- resourceUri

- result

# *** End of Changes ***