3GPP TSG-CT Meeting #95-e	CP-220214

# E-Meeting, 14th – 16th March 2022													Revision of C4-221145

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0684 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N2 MBS Session Management Information in Namf_MBSBroadcast API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) R3-221166 was agreed during RAN3 #114-bis-e, defining MBS Session Management messages and MB-SMF IEs, along similar principles as NGAP Session Management messages and SMF. Corresponding editor's notes in TS 29.518 can be removed and the content of N2 MBS SM Info aligned on TS 38.413. 2) As designed in the AMF and SMF APIs in earlier releases, it is proposed to encode the NGAP IE type in the MBSBroadcast service messages when they encapsulate an N2 MBS Session Management container, to help the receiver know which information (MB-SMF IE) is encapsulated in the binary body part of the HTTP message. Note for instance that a ContextStatusNotify request may encapsulate different MB-SMF IEs (MBS Session Information Response Transfer, MBS Session Information Failure Transfer). 3) TS 29.518 contains the following editor's notes: Editor's Note: whether the ContextCreateRspData contains MBS Session Information Response Transfer IE is FFS. Editor's note: It is FFS whether ngapData should be included and how many ngapData (and binary body part) can be included in the ContextUpdateRspData. TS 23.247 specifies: The AMF transfers the Namf_MBSBroadcast_ContextCreate Response () to the MB-SMF. The AMF should respond success when it receives the first success response from the NG-RAN(s). Accordingly, the AMF should return the Context Create response when it gets the first successful response from the NG-RANs. The AMF may have received an N2 MBS SM container in this first successful response from the NG-RAN(s), including e.g. an NG-RAN DL F-TEID to use for unicast transport over N3mb. The AMF may have also already received unsuccessful responses from other NG-RAN nodes, e.g. including an MBS Session Information Failure Transfer container to pass to the MB-SMF. The AMF should forward the received N2 MBS Session Management containers immediately in the Create response, rather than having to send them in a subsequent ContextStatusNotify request. Not doing so could delay the start of the delivery of the MBS data to the NG-RAN and creates unnecessary extra signaling. Similar considerations apply to the Context Update Response. If after sending the Context Create response or Context Update response, the AMF receives additional N2 MBS SM info containers from other NG-RANs to pass to the MB-SMF, the AMF sends a ContextStatusNotify request as currently specified in TS 29.518. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) N2 MBS SM Information passed in the MBSBroadcast service messages are encoded as NGAP MB-SMF IEs. 2) A new ngapIeType IE is defined to signal the MB-SMF IE that is encoded in the binary body part. 3) The Context Create response and Update Context response are extended to enable carrying up to 10 N2 MBS Session Management containers (like already defined in the ContextStatusNotify Request). Editor's notes are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6, 6.5, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new backward compatible features to the OpenAPI definition of the MBSBroadcast API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: corrects the following OpenAPI errror found during the implementation of the CR agreed at CT4#108-e: The N2MbsSmInfo data type being defined within the Namf_MBSBroadcast API, references to this data type shall not include a reference to TS 29.571: $ref: 'TS29571_CommonData.yaml#/components/schemas/N2MbsSmInfo' |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.6.2.2	ContextCreate

The ContextCreate service operation shall be used by the NF Service Consumer (e.g. MB-SMF) to request the AMF to create a broadcast MBS session context.

It is used in the following procedures:

-	MBS Session Start for Broadcast (see clause 7.3.1 of 3GPP TS 23.247 [55]);

-	Support for Local Broadcast Service (see clause 7.3.4 of 3GPP TS 23.247 [55]).

There shall be only one broadcast MBS session context per MBS session, or per MBS session and Area Session ID for an MBS session with Local Broadcast service.

The NF Service Consumer (e.g. MB-SMF) shall create a broadcast MBS session context by using the HTTP POST method as shown in Figure 5.6.2.2-1.

Figure 5.6.2.2-1: Broadcast MBS session context creation

1.	The NF Service Consumer shall send a POST request targeting the Broadcast MBS session contexts collection resource of the AMF. The payload body of the POST request shall contain the following information:

-	MBS Session ID (i.e. TMGI);

-	Area Session ID, if this is a Location dependent broadcast MBS service;

-	MBS service area;

-	N2 MBS Session Management container (see MBS Session Information Setup Request Transfer IE in 3GPP TS 38.413 [12]); and

-	Notification URI where to be notified about the status change of the broadcast MBS session context.

The NF Service Consumer may also include the maxResponseTime IE in the request to indicate the maximum response time to receive information about the completion of the Broadcast MBS session establishment.

2a.	On success, "201 Created" shall be returned. The AMF should respond success when it receives the first successful response from the NG-RAN(s). The 201 Created response may contain one or more N2 MBS Session Management containers, if additional information (e.g. MBS Session Information Response Transfer IE or MBS Session Information Failure Transfer IE in 3GPP TS 38.413 [12]) needs to be transferred to the MB-SMF.

Upon receipt of subsequent responses from other NG-RANs after sending the 201 Created response, if additional information (e.g. MBS Session Information Response Transfer IE or MBS Session Information Failure Transfer IE in 3GPP TS 38.413 [12]) needs to be transferred to the MB-SMF, the AMF shall transfer such information by sending one or more Namf_MBSBroadcast_ContextStatusNotify requests to the MB-SMF. A Namf_MBSBroadcast_ContextStatusNotify request may include a list of N2 MBS Session Management containers received from different NG-RANs.

When the AMF receives the response from all NG-RANs, the AMF shall include an indication of the completion of the operation in the Namf_MBSBroadcast_ContextStatusNotify request. If the AMF does not receive responses from all NG-RAN nodes before the maximum response time elapses since the reception of the Namf_MBSBroadcast_ContextCreate Request, then the AMF should send one Namf_MBSBroadcast_ContextStatusNotify request indicating the incompletion of the Broadcast MBS session establishment.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.5.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.5.3.2.3.1-3.

* * * Next Change * * * *

#### 5.6.2.3	ContextUpdate

The ContextUpdate service operation shall be used by the NF Service Consumer (e.g. MB-SMF) to request the AMF to update a broadcast MBS session context.

It is used in the following procedures:

-	MBS Session Update for Broadcast (see clause 7.3.3 of 3GPP TS 23.247 [55]).

The NF Service Consumer (e.g. MB-SMF) shall update a broadcast MBS session context by using the HTTP POST method as shown in Figure 5.6.2.3-1.

Figure 5.6.2.3-1: Broadcast MBS session context update

1.	The NF Service Consumer shall send a POST request targeting the individual Broadcast MBS session context resource to be updated in the AMF. The payload body of the POST request may contain the following information:

-	N2 MBS Session Management container (see MBS Session Information Modify Request Transfer IE in 3GPP TS 38.413 [12]);

-	Notification URI, if the NF Service Consumer wishes to modify the notification URI where to be notified about the status change of the broadcast MBS session context;

-	updated MBS service area.

The NF Service Consumer may also include the maxResponseTime IE in the request to indicate the maximum response time to receive information about the completion of the Broadcast MBS session update.

2a.	On success, "200 OK" shall be returned if additional information  needs to be returned in the response. The 200 OK response may contain one or more N2 MBS Session Management containers, if such information (e.g. MBS Session Information Response Transfer IE or MBS Session Information Failure Transfer IE in 3GPP TS 38.413 [12]) needs to be transferred to the MB-SMF. If the AMF received the NG-RAN responses from all involved NG-RAN(s), the AMF should include an indication of completion of the operation in all NG-RANs.

2b.	On success, "204 No Content" shall be returned if no additional information needs to be returned in the response.

In both 2a and 2b cases, upon receipt of subsequent responses from other NG-RANs after sending the 200 OK response or the 204 No Content response, if additional information (e.g. MBS Session Information Response Transfer IE or MBS Session Information Failure Transfer IE in 3GPP TS 38.413 [12]) needs to be transferred to the MB-SMF, the AMF shall transfer such information by sending one or more Namf_MBSBroadcast_ContextStatusNotify requests to the MB-SMF. A Namf_MBSBroadcast_ContextStatusNotify request may include a list of N2 MBS Session Management containers received from different NG-RANs.

When the AMF receives the response from all NG-RANs, the AMF shall include an indication of the completion of the operation in the Namf_MBSBroadcast_ContextStatusNotify request. If the AMF does not receive responses from all NG-RAN nodes before the maximum response time elapses since the reception of the Namf_MBSBroadcast_ContextUpdate Request, then the AMF should send one Namf_MBSBroadcast_ContextStatusNotify request indicating the incompletion of the Broadcast MBS session update.

2c.	On failure or redirection, one of the HTTP status code listed in Table 6.5.3.2.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.5.3.2.4.2.2-2.

* * * Next Change * * * *

#### 5.6.2.5	ContextStatusNotify

The ContextStatusNotify service operation shall be used by the AMF to notify status change of a broadcast MBS session context to the NF Service Consumer (e.g. MB-SMF).

It is used in the following procedures:

-	MBS Session Start for Broadcast (see clause 7.3.1 of 3GPP TS 23.247 [55]);

-	MBS Session Update for Broadcast (see clause 7.3.3 of 3GPP TS 23.247 [55]).

The AMF shall notify status change of a broadcast MBS session context to the NF Service Consumer (e.g. MB-SMF) by using the HTTP POST method as shown in Figure 5.6.2.5-1.

Figure 5.6.2.5-1: Broadcast MBS session context status change notification

1.	The AMF shall send a POST request targeting the notification URI received from the NF Service Consumer. The payload body of the POST request shall contain the following information:

-	MBS Session ID (i.e. TMGI);

-	Area Session ID, if this is a Location dependent broadcast MBS service;

-	one or more N2 MBS Session Management containers, if N2 MBS Session Management information has been received from one or more NG-RANs that needs to be transferred to the NF Service Consumer;

-	the operationStatus IE indicating the completion of the Broadcast MBS session establishment or update, if the NF Service Consumer has requested to establish or update the Broadcast MBS session context and a response has been received from all NG-RANs; and

-	the operationStatus IE indicating the incompletion of the Broadcast MBS session establishment or update, if the NF Service Consumer has requested to establish or update the Broadcast MBS session context including a maximum response time and the AMF has not received responses from all NG-RANs before the maximum response time elapses.

2a.	On success, the NF Service Consumer shall return a "204 No Content" response.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.5.5.2.3.1-3 shall be returned and appropriate additional error information should be returned.

* * * Next Change * * * *

##### 6.5.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [8], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 7807 [36]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json".

Multipart messages shall also be supported (see clause 6.5.2.4) using the content type "multipart/related", comprising:

-	one JSON body part with the "application/json" content type; and

-	one or more binary body parts with 3gpp vendor specific content subtypes.

The 3gpp vendor specific content subtypes defined in Table 6.5.2.2.2-1 shall be supported.

Table 6.5.2.2.2-1: 3GPP vendor specific content subtypes

| content subtype | Description |
| --- | --- |
| vnd.3gpp.ngap | Binary encoded payload, encoding NG Application Protocol (NGAP) IEs, as specified in clause 9.3 of 3GPP TS 38.413 [9] (ASN.1 encoded). |
| NOTE: Using 3GPP vendor content subtypes allows to describe the nature of the opaque payload (i.e. NGAP information) without having to rely on metadata in the JSON payload. |  |

See clause 6.5.2.4 for the binary payloads supported in the binary body part of multipart messages.

* * * Next Change * * * *

#### 6.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.5.6.1-1 specifies the data types defined for the Namf_MBSBroadcast service based interface protocol.

Table 6.5.6.1-1: Namf_MBSBroadcast specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ContextCreateReqData | 6.5.6.2.2 | Data within ContextCreate Request |
| ContextCreateRspData | 6.5.6.2.3 | Data within ContextCreate Response |
| ContextStatusNotification | 6.5.6.2.4 | Data within ContextStatusNotify Request |
| ContextUpdateReqData | 6.5.6.2.5 | Data within ContextUpdate Request |
| ContextUpdateRspData | 6.5.6.2.6 | Data within ContextUpdate Response |
| N2MbsSmInfo | 6.5.6.2.x | N2 MBS Session Management Information |
| NgapIeType | 6.5.6.3.x | NGAP Information Element Type |

Table 6.5.6.1-2 specifies data types re-used by the Namf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf service based interface.

Table 6.5.6.1-2: Namf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| MbsSessionId | 3GPP TS 29.571 [6] | MBS Session Id |
| AreaSessionId | 3GPP TS 29.571 [6] | Area Session Id |
| MbsServiceArea | 3GPP TS 29.571 [6] | MBS Service Area |
| RefToBinaryData | 3GPP TS 29.571 [6] | Reference to binary body part |
| Uri | 3GPP TS 29.571 [6] | URI |
| DurationSec | 3GPP TS 29.571 [6] | Duration in seconds |

* * * Next Change * * * *

##### 6.5.6.2.2	Type: ContextCreateReqData

Table 6.5.6.2.2-1: Definition of type ContextCreateReqData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | M | 1 | MBS Session ID |
| areaSessionId | AreaSessionId | C | 0..1 | Area Session ID This IE shall be present if this is a Location dependent broadcast MBS service. |
| mbsServiceArea | MbsServiceArea | M | 1 | MBS Service Area |
| n2MbsSmInfo | N2MbsSmInfo | M | 1 | This IE shall be present and shall contain N2 MBS Session Management related information (see clause 6.5.6.4). |
| notifyUri | Uri | M | 1 | This IE shall contain the notification URI where to be notified about the status change of the broadcast MBS session context. |
| maxResponseTime | DurationSec | O | 0..1 | Maximum response time (in seconds) to receive information about the completion of the Broadcast MBS session establishment. |

* * * Next Change * * * *

##### 6.5.6.2.3	Type: ContextCreateRspData

Table 6.5.6.2.3-1: Definition of type ContextCreateRspData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n2MbsSmInfoList | array(N2MbsSmInfo) | O | 1..10 | When present, this IE shall contain N2 MBS Session Management related information (see clause 6.5.6.4). |

* * * Next Change * * * *

##### 6.5.6.2.4	Type: ContextStatusNotification

Table 6.5.6.2.4-1: Definition of type ContextStatusNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | M | 1 | MBS Session ID |
| areaSessionId | AreaSessionId | C | 0..1 | Area Session ID This IE shall be present if this is a Location dependent broadcast MBS service. |
| n2MbsSmInfoList | array(N2MbsSmInfo) | O | 1..10 | When present, this IE shall contain N2 MBS Session Management related information (see clause 6.5.6.4). |
| operationStatus | OperationStatus | C | 0..1 | This IE shall be present and indicate the completion of the MBS session start or update operation, if the NF Service Consumer has requested to establish or update the Broadcast MBS session context and a response has been received from all NG-RANs. This IE shall be present and indicate the incompletion of the MBS session start or update operation, if the NF Service Consumer has requested to establish or update the Broadcast MBS session context within a maximum response time and the AMF has not received responses from all NG-RANs before the maximum response time elapses. |

* * * Next Change * * * *

##### 6.5.6.2.5	Type: ContextUpdateReqData

Table 6.5.6.2.5-1: Definition of type ContextUpdateReqData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsServiceArea | MbsServiceArea | O | 0..1 | MBS Service Area |
| n2MbsSmInfo | N2MbsSmInfo | O | 0..1 | When present, this IE shall contain N2 MBS Session Management related information (see clause 6.5.6.4). |
| notifyUri | Uri | O | 0..1 | When present, this IE shall contain the notification URI where to be notified about the status change of the broadcast MBS session context. |
| maxResponseTime | DurationSec | O | 0..1 | Maximum response time (in seconds) to receive information about the completion of the Broadcast MBS session update. |

* * * Next Change * * * *

##### 6.5.6.2.6	Type: ContextUpdateRspData

Table 6.5.6.2.6-1: Definition of type ContextUpdateRspData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n2MbsSmInfoList | array(N2MbsSmInfo) | O | 1..10 | When present, this IE shall contain N2 MBS Session Management related information (see clause 6.5.6.4). |

* * * Next Change * * * *

##### 6.5.6.2.x	Type: N2MbsSmInfo

Table 6.5.6.2.x-1: Definition of type N2MbsSmInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ngapIeType | NgapIeType | M | 1 | This IE shall indicate the NGAP IE type of the ngapData as specified in clause 6.5.6.3.x. |
| ngapData | RefToBinaryData | M | 1 | This IE shall contain the reference to the binary data part carrying the NGAP data. |

* * * Next Change * * * *

##### 6.5.6.3.x	Enumeration: NgapIeType

Table 6.5.6.3.x-1: Enumeration NgapIeType

| Enumeration value | Description |
| --- | --- |
| "MBS_SES_SETUP_REQ" | MBS Session Information Setup Request Transfer |
| "MBS_SES_RSP" | MBS Session Information Response Transfer |
| "MBS_SES_FAIL" | MBS Session Information Failure Transfer |
| "MBS_SES_MOD_REQ" | MBS Session Information Modify Request Transfer |

Editor's Note: The contents of the above table will be aligned with the final contents of TS 38.413.

* * * Next Change * * * *

##### 6.5.6.4.3	N2 Information

###### 6.5.6.4.3.1	Introduction

N2 Information shall encode NG Application Protocol (NGAP) IEs, as specified in clause 9.3.A of 3GPP TS 38.413 [12] (ASN.1 encoded), using the vnd.3gpp.ngap content-type.

###### 6.5.6.4.3.2	NGAP IEs

N2 Information may encode following NGAP MB-SMF related IE specified in in clause 9.3.A of 3GPP TS 38.413 [12], as summarized in Table 6.5.6.4.3-1.

Table 6.5.6.4.3-1: N2 Information content for class MBS-SM

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| MBS Session Information Setup Request Transfer | 9.3.A.w | BROADCAST SESSION SETUP REQUEST |
| MBS Session Information Response Transfer | 9.3.A.x | BROADCAST SESSION SETUP RESPONSE BROADCAST SESSION MODIFICATION RESPONSE |
| MBS Session Information Failure Transfer | 9.3.A.z | BROADCAST SESSION SETUP FAILURE BROADCAST SESSION MODIFICATION FAILURE |
| MBS Session Information Modify Request Transfer | 9.3.A.y | BROADCAST SESSION MODIFICATION REQUEST |

Editor's Note: The contents of the above table (IE names, message names, clause numbers) will be aligned with the final contents of TS 38.413.

* * * Next Change * * * *

## A.6	Namf_MBSBroadcast API

openapi: 3.0.0

info:

version: 1.0.0-alpha.1

title: Namf_MBSBroadcast

description: |

AMF MBSBroadcast Service

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

paths:

/mbs-contexts:

post:

summary: Namf_MBSBroadcast ContextCreate service Operation

tags:

- Broadcast MBS session contexts collection (Collection)

operationId: ContextCreate

requestBody:

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/ContextCreateReqData'

binaryDataN2Information:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN2Information:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

required: true

callbacks:

contextStatusNotification:

'{$request.body#/notifyUri}':

post:

requestBody:  # notification request without binary body part

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ContextStatusNotification'

multipart/related:  # notification request with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/ContextStatusNotification'

binaryDataN2Information1:

type: string

format: binary

binaryDataN2Information2:

type: string

format: binary

binaryDataN2Information3:

type: string

format: binary

binaryDataN2Information4:

type: string

format: binary

binaryDataN2Information5:

type: string

format: binary

binaryDataN2Information6:

type: string

format: binary

binaryDataN2Information7:

type: string

format: binary

binaryDataN2Information8:

type: string

format: binary

binaryDataN2Information9:

type: string

format: binary

binaryDataN2Information10:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN2Information1:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information2:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information3:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information4:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information5:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information6:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information7:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information8:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information9:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information10:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

responses:

'204':

description: successful notification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

responses:

'201':

description: MBS Broadcast context created successfully

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/namf-mbs-bc/<apiVersion>/mbs-contexts/{mbsContextRef}'

required: true

schema:

type: string

content:

application/json:  # message without binary body part(s)

schema:

$ref: '#/components/schemas/ContextCreateRspData'

multipart/related:   # message with binary body part(s)

schema:

type: object

properties: # Response parts

jsonData:

$ref: '#/components/schemas/ContextCreateRspData'

binaryDataN2Information1:

type: string

format: binary

binaryDataN2Information2:

type: string

format: binary

binaryDataN2Information3:

type: string

format: binary

binaryDataN2Information4:

type: string

format: binary

binaryDataN2Information5:

type: string

format: binary

binaryDataN2Information6:

type: string

format: binary

binaryDataN2Information7:

type: string

format: binary

binaryDataN2Information8:

type: string

format: binary

binaryDataN2Information9:

type: string

format: binary

binaryDataN2Information10:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN2Information1:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information2:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information3:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information4:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information5:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information6:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information7:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information8:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information9:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information10:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

/mbs-contexts/{mbsContextRef}/update:

post:

summary: Namf_MBSBroadcast ContextUpdate service Operation

tags:

- Individual broadcast MBS session context (Document)

operationId: ContextUpdate

parameters:

- name: mbsContextRef

in: path

description: Unique ID of the broadcast MSB session context to be updated

required: true

schema:

type: string

requestBody:

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/ContextUpdateReqData'

binaryDataN2Information:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN2Information:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

required: true

responses:

'200':

description: MBS Broadcast context updated successfully

content:

application/json:  # message without binary body part(s)

schema:

$ref: '#/components/schemas/ContextUpdateRspData'

multipart/related:   # message with binary body part(s)

schema:

type: object

properties: # Response parts

jsonData:

$ref: '#/components/schemas/ContextUpdateRspData'

binaryDataN2Information1:

type: string

format: binary

binaryDataN2Information2:

type: string

format: binary

binaryDataN2Information3:

type: string

format: binary

binaryDataN2Information4:

type: string

format: binary

binaryDataN2Information5:

type: string

format: binary

binaryDataN2Information6:

type: string

format: binary

binaryDataN2Information7:

type: string

format: binary

binaryDataN2Information8:

type: string

format: binary

binaryDataN2Information9:

type: string

format: binary

binaryDataN2Information10:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN2Information1:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information2:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information3:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information4:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information5:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information6:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information7:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information8:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information9:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2Information10:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

ContextCreateReqData:

description: Data within ContextCreate Request

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

areaSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AreaSessionId'

mbsServiceArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea'

n2MbsSmInfo:

$ref: '#/components/schemas/N2MbsSmInfo'

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

maxResponseTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- mbsSessionId

- mbsServiceArea

- ngapData

- notifyUri

ContextCreateRspData:

description: Data within ContextCreate Response

type: object

properties:

n2MbsSmInfoList:

type: array

items:

$ref: '#/components/schemas/N2MbsSmInfo'

minItems: 1

maxItems: 10

ContextUpdateReqData:

description: Data within ContextUpdate Request

type: object

properties:

mbsServiceArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea'

n2MbsSmInfo:

$ref: '#/components/schemas/N2MbsSmInfo'

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

maxResponseTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

ContextStatusNotification:

description: Data within ContextStatusNotify Request

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

areaSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AreaSessionId'

n2MbsSmInfoList:

type: array

items:

$ref: '#/components/schemas/N2MbsSmInfo'

minItems: 1

maxItems: 10

operationStatus:

$ref: '#/components/schemas/OperationStatus'

required:

- mbsSessionId

ContextUpdateRspData:

description: Data within ContextUpdate Response

type: object

properties:

n2MbsSmInfoList:

type: array

items:

$ref: '#/components/schemas/N2MbsSmInfo'

minItems: 1

maxItems: 10

N2MbsSmInfo:

description: N2 MBS Session Management information

type: object

properties:

ngapIeType:

$ref: '#/components/schemas/NgapIeType'

ngapData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

OperationStatus:

description: Status of a Broadcast MBS session start or update operation.

anyOf:

- type: string

enum:

- MBS_SESSION_START_COMPLETE

- MBS_SESSION_START_INCOMPLETE

- MBS_SESSION_UPDATE_COMPLETE

- MBS_SESSION_UPDATE_INCOMPLETE

- type: string

NgapIeType:

description: NGAP Information Element Type

anyOf:

- type: string

enum:

- MBS_SES_SETUP_REQ

- MBS_SES_RSP

- MBS_SES_FAIL

- MBS_SES_MOD_REQ

- type: string

* * * End of Changes * * * *