3GPP TSG-CT WG1 Meeting #133e-bis	C1-220343

# E-meeting, 17-21 Jauary 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.546 | CR | 0022 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of CR implementation issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some issues at implementing CT#94-e approved CRs in 24.546 17.1.0 are identified and are proposed to be corrected |  |  |  |  |  |  |  |  |
| Summary of change: |  | CR implementation corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain the TS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.5.4, C.2.1.2.2.3.1, C.2.1.4, C.2.1.4.1, C.2.1.4.2, C.2.1.4.2.1, C.2.1.4.2.2, C.2.1.4.2.3, C.2.1.4.2.4, C.2.1.4.3, C.2.1.4.3.1, C.2.1.5, C.2.1.6, C.2.1.6.1, C.2.1.6.2, C.2.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows;".

[3]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[4]	OMA OMA-TS-XDM_Core-V2_1-20120403-A: "XML Document Management (XDM) Specification".

[5]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification;".

[6]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[7]	IETF RFC 7159: "The JavaScript Object Notation (JSON) Data Interchange Format".

[8]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[9]	IETF RFC 5875: "An Extensible Markup Language (XML) Configuration Access Protocol (XCAP) Diff Event Package".

[10]	IETF RFC 6050 (November 2010): "A Session Initiation Protocol (SIP) Extension for the Identification of Services".

[11]	IETF RFC 6665 (July 2012): "SIP-Specific Event Notification".

[12]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[13]	IETF RFC 7959: "Block-Wise Transfers in the Constrained Application Protocol (CoAP) ".

[14]	IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

[15]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[16]	IETF RFC 8516: ""Too Many Requests" Response Code for the Constrained Application Protocol".

[17]	IETF RFC 8949: “Concise Binary Object Representation (CBOR)”.

[18]	IETF RFC 8610: "Concise Data Definition Language (CDDL): A Notational Convention to Express Concise Binary Object Representation (CBOR) and JSON Data Structures".

[19]	Constrained RESTful Environments (CoRE) Parameters at IANA, https://www.iana.org/assignments/core-parameters/core-parameters.xhtml

[20]	Internet draft draft-ietf-core-problem-details-01: "Problem Details For CoAP APIs".

[21]	Internet draft draft-ietf-core-new-block-14: "Constrained Application Protocol (CoAP) Block-Wise Transfer Options Supporting Robust Transmission".

* * * Next Change * * * *

#### 6.2.5.4	SCM server CoAP procedure

Upon reception of an CoAP PUT request where the CoAP URI of the request identifies Individual User Profile resource as described in Annex C.2.1.2.3.3.2, the SCM-S:

a)	shall determine the identity of the sender of the received CoAP GET request as specified in clause 6.2.1.2, and:

1)	if the identity of the sender of the received CoAP PUT request is not authorized to update requested user profile document(s), shall respond with a CoAP 4.03 (Forbidden) response to the CoAP GET request and skip rest of the steps;

b)	shall support handling an CoAP PUT request from a SCM-C according to procedures specified in IETF RFC 7252 [12]; and

## c)	shall replace the user profile documents pointed at by the CoAP URI with the "ProfileDoc" received in the request.

* * * Next Change * * * *

###### C.2.1.2.2.3.1	GET

This operation retrieves VAL user or VAL UE profile information satisfying the filter criteria.

This method shall support the URI query parameters specified in table C.2.1.2.2.3.1-1.

Table C.2.1.2.2.3.1-1: URI query parameters supported by the GET Request on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-tgt-ue | ValTargetUe | M | 1 | Identifying a VAL target UE. |

This method shall support the response data structures and response codes specified in table C.2.1.2.2.3.1-2.

Table C.2.1.2.2.3.1-2: Data structures supported by the GET Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(ProfileDoc) | M | 1..N | 2.05 Content | List of VAL user / VAL UE profile documents. This response shall include user profile information matching the query parameters provided in the request. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 shall also apply. |  |  |  |  |

* * * Next Change * * * *

### C.2.1.3	Data Model

* * * Next Change * * * *

#### C.2.1.3.1	General

Table C.2.1.3.1-1 specifies the data types defined specifically for the SU_UserProfile API service.

Table C.2.1.3.1-1: SU_UserProfile API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ProfileDoc | C.2.1.3.2.1 | Profile information associated with VAL user ID or VAL UE ID. |  |
| ProfileInfo | C.2.1.3.2.2 | Profile information including profile configurations. |  |
| ProfileConfig | C.2.1.3.2.3 | Profile configuration including configuration data. |  |
| ProfileConfigType | C.2.1.3.3.1 | Specifies type of features for which the configuration data is applicable. |  |
| ValTargetUe | C.2.1.3.2.4 | Information identifying a VAL user ID or VAL UE ID. |  |

* * * Next Change * * * *

#### C.2.1.3.2	Structured data types

* * * Next Change * * * *

##### C.2.1.3.2.1	Type: ProfileDoc

Table C.2.1.3.2.1-1: Definition of type ProfileDoc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| profileDocId | string | O | 1 | Contains the profileDocId of the complete resource URI of this user profile document according to the structure: {apiRoot}/su-up/<apiVersion>/val-services/{valServiceId}/user-profiles/{profileDocId} This attribute shall be provided by the SCM-S in CoAP responses. |  |
| profileInformation | ProfileInfo | M | 1 | Profile information associated with a VAL user or a VAL UE as specified in valTgtUe. |  |
| valTgtUe | ValTargetUe | M | 1 | Unique identifier of a VAL user or a VAL UE. |  |

* * * Next Change * * * *

##### C.2.1.3.2.2	Type: ProfileInfo

Table C2.1.3.2.2-1: Definition of type ProfileInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| profileName | string | O | 1 | Name of the user profile. |  |
| status | boolean | M | 1 | Indicates whether the user profile is enabled or disabled. |  |
| array(ProfileConfig) | ProfileConfig | O | 1..N | List of profile configurations. |  |
| isDefault | boolean | O | 1 | Indicates whether the user profile is the default profile for VAL user or not. |  |

* * * Next Change * * * *

##### C.2.1.3.2.3	Type: ProfileConfig

Table C.2.1.3.2.3-1: Definition of type ProfileConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| configType | ProfileConfigType | M | 1 | Indicates the type of the profile configuration. |  |
| configData | string | M | 1 | Actual user profile configuration data. |  |

* * * Next Change * * * *

##### C.2.1.3.2.4	Type: ValTargetUe

Table C.2.1.3.2.4-1: Definition of type ValTargetUe

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUserId | string | O | 0..1 | Unique identifier of a VAL user. |  |
| valUeId | string | O | 0..1 | Unique identifier of a VAL UE. |  |
| NOTE: Either "valUserId" or "valUeId" shall be present. |  |  |  |  |  |

* * * Next Change * * * *

#### C.2.1.3.3	Simple data types and enumerations

* * * Next Change * * * *

##### C.2.1.3.3.1	Enumeration: ProfileConfigType

Table C.2.1.3.3.1-1: Enumeration ProfileConfigType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| COMMON | Indicates VAL service specific common user profile configuration. |  |
| ON_NETWORK | Indicates VAL service specific user profile configuration for on-network features. |  |
| OFF_NETWORK | Indicates VAL service specific user profile configuration for off-network features. |  |

* * * Next Change * * * *

### C.2.1.4	Error Handling

General error responses are defined in clause C.1.3.

* * * Next Change * * * *

### C.2.1.5	CDDL Specification

* * * Next Change * * * *

#### C.2.1.5.1	Introduction

The data model described in clause C.2.1.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [17].

Clause C.2.1.5.2 uses the Concise Data Definition Language described in IETF RFC 8610 [18] and provides corresponding representation of the SU_UserProfile API data model.

* * * Next Change * * * *

#### C.2.1.5.2	CDDL document

;;; ProfileDoc

;;+ Represents user profile information associated with a VAL user ID or a VAL UE ID.

ProfileDoc = {

? profileDocId: text

profileInformation: ProfileInfo

valTgtUe: ValTargetUe

}

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

ValTargetUe = {

(

valUserId: text                 ; Unique identifier of a VAL user.

//

valUeId: text                   ; Unique identifier of a VAL UE.

)

}

;;; ProfileInfo

;;+ User profile information.

ProfileInfo = {

? profileName: text             ; Name of the profile

status: bool                    ; Indicates whether the user profile is enabled or disabled.

? profileConfigs: [+ ProfileConfig]

? isDefault: bool               ; Indicates whether the user profile is the default profile for VAL user or not.

}

;;; ProfileConfig

;;+ Profile configuration.

ProfileConfig = {

configType: ProfileConfigType

configData: text                ; Actual user profile configuration data.

}

;;; ProfileConfigType

;;+ Indicates the type of the profile configuration.

ProfileConfigType = "COMMON" / "ON_NETWORK" / "OFF_NETWORK" / text

* * * Next Change * * * *

### C.2.1.6	Media Type

The media type for a user profile document shall be "application/cbor".

Editor's Note:	Consider defining a media type for the user profile, such as "vnd.3gpp.seal-user-profile-info+cbor".

Editor's Note:	It is possible to specify other payload format for CoAP than CBOR, and the details about other payload format is FFS.

* * * End of Changes * * * *