3GPP TSG-CT Meeting #95-e	CP-220306

# Online, , 14th Mar 2022 - 16th Mar 2022	(was C4-221645)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0328 | rev | 4 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PVS Info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-03-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current definition of PvsInfo allows to have multiple occurrences of IPv4 addresses, IPv6 addresses and FQDNs. It is not clear how this can be used by a consumer (e.g. the UE). In particular, it is not clear whether all these addresses/FQDNs refer to a same single PVS server, or to multiple PVS servers; in the latter case, it is not clear how the consumer (e.g. the UE) should behave for the selection of the different serves. Should they be selected randomly or are there primary/stand-by servers? Also, the current structure does not allow the consumer to infer the relationship between each IP address occurrence and each FQDN occurrence. It is preferable to assume that the current data structure refers to a given PVS server. Then, if multiple server instances need to be communicated over different APIs, it is possible to send mutiple occurences of PvsInfo, either as array(PvsInfo), or as distinct attributes (e.g. "primaryPvsInfo", "secondaryPvsInfo"). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the name of the data type to ServerAddressingInfo, and change the names of the attributes to make them generic rather than PVS-specific. - Indicate that all addressing parameters (including all FQDNs in the list) refer to a same server. - Use an Fqdn data type for the items of the fqdnList attribute, instead of the "string" data type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PVS information sent to the consumer (e.g. the UE) is ambiguous and not clear how is should be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.76, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible corrections with impact on the following APIs: - TS29502_Nsmf_PDUSession.yaml - TS29503_Nudm_UEAU.yaml - TS29509_Nausf_UEAuthentication.yaml - TS29512_Npcf_SMPolicyControl.yaml - TS29526_Nnssaaf_AIW.yaml - TS29571_CommonData.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 4 (to CT#95e): - Add the complete list of impacted APIs in "Other comments" |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.4.4.76	Type: ServerAddressingInfo

Table 5.4.4.76 -1: Definition of type ServerAddressingInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addresses | array(Ipv4Addr) | C | 1..N | IPv4 address(es) of the server (NOTE). |
| ipv6Addresses | array(Ipv6Addr) | C | 1..N | IPv6 address(es) of the server (NOTE). |
| fqdnList | array(Fqdn) | C | 1..N | List of FQDNs (Fully Qualified Domain Names) of the server (NOTE). |
| NOTE: At least one of the addressing parameters (ipv4addresses, ipv6adresses or fqdnList) shall be included in the ServerAddressingInfo; all addressing parameters in this data type shall be understood as referring to a same sever. |  |  |  |  |

* * * Next Change * * * *

## A.2	Data related to Common Data Types

(... text not shown for clarity ...)

ServerAddressingInfo:

description: Contains addressing information (IP addresses and/or FQDNs) of a server.

type: object

anyOf:

- required:

- ipv4Addresses

- required:

- ipv6Addresses

- required:

- fqdnList

properties:

ipv4Addresses:

type: array

items:

$ref: '#/components/schemas/Ipv4Addr'

minItems: 1

ipv6Addresses:

type: array

items:

$ref: '#/components/schemas/Ipv6Addr'

minItems: 1

fqdnList:

type: array

items:

$ref: '#/components/schemas/Fqdn'

minItems: 1

(... text not shown for clarity ...)

* * * End of Changes * * * *