3GPP TSG-CT Meeting #95-e	CP-220342

# Online, 14th Mar 2022 - 16th Mar 2022											Revision of C4-220319

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0787 | rev | 2 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN impacts on UDM - PVS information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to align with SA2/SA3 agreed CR (S2-2109263, S3-214446), the Nudm_UEAuthentication Response needs to support carrying PVS Info (PVS FQDN or PVS IP address). The reason is the following. During the UE Registration with ON-SNPN, DCS may provide PVS Info to the AUSF and the AUSF forwards this to the AMF. If however DCS is hosting AUSF and UDM, the UDM will send the PVS address to AUSF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PVS info is added to Nudm_UEAuthentication Response. New 'pvsInfo' attribute of type 'PvsInfo' is added to the 'AuthenticationInfoResult' type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.2.2, 6.3.6.1, 6.3.6.2.3, A.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution proposes backward compatible changes to Nudm_UEAU API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Cover sheet is fixed Rev2: CR0328-TS29.571 in C4-221645 introduced new generic data type, 'ServerAddressingInfo', which shall be used also for the 'pvsInfo' attribute. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.4.2.2.2	Authentication Information Retrieval

Figure 5.4.2.2.2-1 shows a scenario where the NF service consumer (AUSF) retrieves authentication information for the UE from the UDM (see also 3GPP TS 33.501 [6] clause 6.1.2). The request contains the UE's identity (supi or suci), the serving network name, and may contain resynchronization info.

Figure 5.4.2.2.2-1: NF service consumer requesting authentication information

1.	The NF service consumer sends a POST request (custom method: generate-auth-data) to the resource representing the UE's security information.

2a.	The UDM responds with "200 OK" with the message body containing the authentication data information.

The AUSF shall store the authentication data information for subsequent authentication processing. If the AUSF is configured to store Kausf (e.g. based on its support of SoRProtection / UPUProtection service operations / deriving AKMA key after primary authentication), the AUSF shall preserve the Kausf and related information (e.g. SUPI) after the completion of the primary authentication. If the UDM decides that the primary authentication by an AAA server in a Credentials Holder is required, the AUSF shall perform the authentication with the AAA Server. If the Default Credentials Server (DCS) provides UDM with the information of a Provisioning Server (PVS FQDN or IP address), the UDM provides the PVS info to the AUSF.

2b.	If the operation cannot be authorized due to e.g UE does not have required subcription data, none of the CAG IDs in the CAG cell match any of the CAG IDs in the allowed CAG list, access barring or roaming restrictions, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element). If the cellCagInfo is not received, the UDM shall not assume the UE is accessing from the PLMN and shall not stop the authenthcation if the UE is allowed to access 5GS via CAG cell(s) only.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

* * * 2nd Change * * * *

### 6.3.6	Data Model

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the Nudm_UEAU service API.

Table 6.3.6.1-1: Nudm_UEAU specific Data Types

| Data type |  | Clause defined |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| AuthenticationInfoRequest |  | 6.3.6.2.2 |  | Contains Serving Network Name and Resynchronization Information |  |
| AuthenticationInfoResult |  | 6.3.6.2.3 |  | Contains an Authentication Vector (AV) |  |
| AvEapAkaPrime |  | 6.3.6.2.4 |  | Contains RAND, XRES, AUTN, CK', and IK' |  |
| Av5GHeAka |  | 6.3.6.2.5 |  | Contains RAND, XRES*, AUTN, KAUSF |  |
| ResynchronizationInfo |  | 6.3.6.2.6 |  | Contains RAND and AUTS |  |
| AuthEvent |  | 6.3.6.2.7 |  | Authentication Event |  |
| AuthenticationVector |  | 6.3.6.2.8 |  |  |  |
| RgAuthCtx |  | 6.3.6.2.9 |  | Contains the UE id (i.e. SUPI) and the authentication indication. |  |
| HssAuthenticationInfoRequest |  | 6.3.6.2.10 |  | Contains authentication method, serving network id, number of requested vectors and resynchronization information |  |
| HssAuthenticationInfoResult |  | 6.3.6.2.11 |  | Contains the authentication vectors for EPS/IMS domain |  |
| HssAuthenticationVectors |  | 6.3.6.2.12 |  |  |  |
| AvEpsAka |  | 6.3.6.2.13 |  | Contains RAND, XRES, AUTN, KASME |  |
| AvImsGbaEapAka |  | 6.3.6.2.14 |  | Contains RAND, XRES, AUTN, CK, and IK |  |
| Autn |  | 6.3.6.3.2 |  |  |  |
| Auts |  | 6.3.6.3.2 |  |  |  |
| CkPrime |  | 6.3.6.3.2 |  |  |  |
| IkPrime |  | 6.3.6.3.2 |  |  |  |
| Kausf |  | 6.3.6.3.2 |  |  |  |
| Rand |  | 6.3.6.3.2 |  |  |  |
| ServingNetworkName |  | 6.3.6.3.2 |  |  |  |
| Success |  | 6.3.6.3.2 |  |  |  |
| Xres |  | 6.3.6.3.2 |  |  |  |
| XresStar |  | 6.3.6.3.2 |  |  |  |
| AuthenticatedInd |  | 6.3.6.3.2 |  |  |  |
| ConfidentialityKey |  | 6.3.6.3.2 |  |  |  |
| IntegrityKey |  | 6.3.6.3.2 |  |  |  |
| Kasme |  | 6.3.6.3.2 |  |  |  |
| NumOfRequestedVectors |  | 6.3.6.3.2 |  |  |  |
| Autn |  | 6.3.6.3.2 |  |  |  |
| AuthType |  | 6.3.6.3.3 |  |  |  |
| AvType |  | 6.3.6.3.4 |  |  |  |
| HssAuthType |  | 6.3.6.3.5 |  |  |  |
| HssAvType |  | 6.3.6.3.6 |  |  |  |
| HssAuthTypeInUri |  | 6.3.6.3.7 |  |  |  |
| AccessNetworkId |  | 6.3.6.3.8 |  |  |  |

Table 6.3.6.1-2 specifies data types re-used by the Nudm_UEAU service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudm_UEAU service API.

Table 6.3.6.1-2: Nudm_UEAU re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [7] | Common data type used in response bodies |
| NfInstanceId | 3GPP TS 29.571 [7] | Network Function Instance Identifier |
| DateTime | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | see 3GPP TS 29.500 [4] clause 6.6 |
| Supi | 3GPP TS 29.571 [7] |  |
| CagId | 3GPP TS 29.571 [7] |  |
| ServerAddressingInfo | 3GPP TS 29.571 [7] | Server address info, which in this specification contains information of a Provisioning Server (PVS) |

* * * 3rd Change * * * *

##### 6.3.6.2.3	Type: AuthenticationInfoResult

Table 6.3.6.2.3-1: Definition of type AuthenticationInfoResult

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authType | AuthType | M | 1 | Indicates the authentication method |
| authenticationVector | AuthenticationVector | C | 0..1 | contains an authentication vector if 5G AKA or EAP-AKA's is selected |
| supi | Supi | C | 0..1 | SUPI shall be present if the request contained the SUCI within the request URI |
| akmaInd | boolean | C | 0..1 | When present, this IE shall be set as follows: - true: AKMA keys need to be generated for the UE; - false (default): AKMA keys are not needed. |
| authAaa | Boolean | C | 0..1 | When present, this IE shall be set as follows: - true: Primary authentication with AAA Server that acts as the EAP server is required, i.e. the AUSF is required to act as EAP proxy towards the AAA Server; - false (default): Primary authentication with AAA Server that acts as the EAP server is not required, i.e. the AUSF is required to act as EAP server according to the EAP method defined in authType attribute. See clause 6.3.6.3.3. |
| routingId | string | C | 0..1 | This IE shall be present if the akmaInd is set to "true". When present, this IE shall include the Routing Indicator contained in the SUCI (see 3GPP TS 23.003 [8], clause 2.2B). |
| pvsInfo | array(ServerAddressingInfo) | C | 1..N | When present, this IE shall contain the remote Provisioning Server(s) information (PVS FQDN or IP address). This is used for the case of SNPN onboarding with DCS hosting AUSF and UDM. |
| supportedFeatures | SupportedFeatures | O | 0..1 | See clause 6.3.8 |

* * * 4th Change * * * *

## A.4	Nudm_UEAU API

openapi: 3.0.0

info:

version: '1.2.0-alpha.3'

title: 'Nudm_UEAU'

description: |

UDM UE Authentication Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

## Skipped for clarity##

AuthenticationInfoResult:

type: object

required:

- authType

properties:

authType:

$ref: '#/components/schemas/AuthType'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

authenticationVector:

$ref: '#/components/schemas/AuthenticationVector'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

akmaInd:

type: boolean

default: false

authAaa:

type: boolean

default: false

routingId:

type: string

pattern: '^[0-9]{1,4}$'

pvsInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServerAddressingInfo'

minItems: 1

## Skipped for clarity##

* * * End of Changes * * * *