3GPP TSG-CT Meeting #95-e	CP-220396

# E-Meeting, 14th – 16th March 2022

(revision of CP-220380)

# Source:	China Mobile, Nokia, Nokia Shanghai Bell

# Title:	New WID on CT aspects of enhancement of RAN Slicing for NR

# Document for:	Approval

Agenda Item:	17.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	CT aspects of enhancement of RAN Slicing for NR

Acronym: ARCH_NR_Slice

Unique identifier: 	TBD

Potential target Release: Rel-17

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NR_Slice-Core | RAN2 | 911107 | Core part: Enhancement of RAN slicing for NR |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860039 | Study on enhancement of RAN slicing for NR | RAN work item |

3	Justification

RAN2 carried out a Rel-17 work item on the enhancement of RAN Slicing for NR. And "LS on Slice list and priority information for cell reselection" ( R2-2108928) from RAN2 to CT1 points out the solution agreed in RAN2#114e and #115e for cell reselection for the normative phase has the following impacts on protocols and interfaces under CT WGs' responsibilities:

- For the List of Slices with Slice Priority, UE Access Stratum (AS) expects to receive a list from NAS containing a slice priority for each of the slices contained in the list when/ before it moves to RRC_IDLE/RRC_INACTIVE and when the list and/or priorities changes while the UE is in RRC_IDLE/RRC_INACTIVE.

- Furthermore, RAN2 has been discussing a Slice Group concept and it is required to provide the UE with a mapping between slice groups and network slices.

This WID aims at carrying out the related work in CT WGs to complete the feature end to end. As the requirement to related CT work came from parent RAN2 WI "NR_Slice-Core", and SA2 carried out the corresponding work under agenda item "Rel-17 CAT B/C alignment CR(s) due to the work led by other 3GPP Working Groups", WI code "NR_Slice-Core" is suggested for this CT WI.

4	Objective

The objective of the work item is to develop the specifications according to the slice based cell reselection requirements from RAN WGs work item NR_Slice-Core. CT WGs will align to normative stage 2 updates in the technical specifications under SA2 responsibility for “RAN Slicing for NR” introduced under corresponding RAN WI code. The following areas of work are expected to be covered (non-exhaustive):

CT1

- Enhancement of the NAS protocol to support the slice based cell reselection.

- Clarification of the related interaction between the NAS and AS layers.

CT4

Potential enhancement of NSSF services to support the network slice group management.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 24.501 | Enhancement of the NAS protocol to support the slice based cell reselection. | TSG#96 (June 2022) | CT1 |
| 23.122 | Clarification of the related interaction between the NAS and AS layers. | TSG#96 (June 2022) | CT1 |
| 29.531 | Potential enhancement of NSSF services to support the network slice group management | TSG#96 (June 2022) | CT4 |

6	Work item Rapporteur(s)

Chen Xu, China Mobile, chenxu@chinamobile.com

7	Work item leadership

CT1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Mobile

ZTE

Huawei

HiSilicon

vivo

CATT

China Telecom

OPPO

Xiaomi

Ericsson

China Unicom

Nokia

Nokia Shanghai Bell