3GPP TSG-CT Meeting #95-e	CP-220401

# E-Meeting, 14th – 16th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.484 | CR | 0209 | rev | 3 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Config update to support network slicing and DN in MC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5GS |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS23.289 specifies the following network slicing requirements per PLMN "The Initial MC service UE configuration shall contain at least one network slice identity (S-NSSAI) on a per HPLMN and optionally also per VPLMN basis. … At least one S-NSSAI in the Initial MC service UE configuration shall be marked as default S-NSSAI." and per service: "Network slice identification and corresponding network slice credentials may be provided per MC service." and for DNN "The DNN shall be made available to the MC service UE either via UE (pre)configuration or via initial UE configuration on a per HPLMN and optionally also per VPLMN basis." Thus, stage-2 clearly dictates certain necessary updates of the Initial MC service UE configuration per PLMN, and also per service for slicing. In EPS, each service can be locally configured with an APN, and hence the<MC*-to-con-ref> elements have been specified. In 5GS, each service may be using a PDU session to a {DNN, S-NSSAI} combinations, which could be configured by reference as well. The parameters related to each DN can be organised in a new element. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the MCS UE initial configuration per PLMN with 1) a list of S-NSSAIs with a default indication. 2) a list of DNNs 3) a mapping of services to DNN,SNSSAIs 4) a DN-info element containing any info related to an APN/DNN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network slice and DN selection as per initial UE configuration is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.1, 7.2.2.3, 7.2.2.6, 7.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The changes over the agreed CR: 1) revert change on SNSSAI per service: moved back in the <service> element to support per PLMN configurations 2) replace “may contain” to “optionally” to have correct sentence syntax 3) added missing xsd file 4) updated justification in cover page 5) Added the missing semantics on <MCPTT-to-con-ref>, <MC-common-core-to-con-ref> and <MC-ID-to-con-ref> (omission since rel-15-not directly related to the CR) Rev.3 changes: 1) Fix in the description of the <SNSSAI> element at the very end of clause 7.2.2.1, there is a revision-on-revision in the 3rd bullet number. 2) Fixed alignment in 7.2.2.3 (XML Schema) in the definition of “VPLMNType”, the “anyExt” element description and following line should be moved to the left to align with the “service” description. |  |  |  |  |  |  |  |  |

#### 7.2.2.1	Structure

The MCS UE initial configuration document structure is specified in this clause.

The <mcptt-UE- initial-configuration> document:

1)	shall include a "domain" attribute;

2)	may include a <mcptt-UE-id> element;

3)	may include a <name> element;

4)	may include a <Default-user-profile> element;

5)	may include an <on-network> element;

6)	may include an <off-network> element; and

7)	may include any other attribute for the purposes of extensibility.

The <Default-user-profile> element shall contain:

1)	a "User-ID" attribute; and

2)	a "user-profile-index" attribute.

The <on-network> element:

1)	shall contain a <Timers> element containing:

a)	a <T100> element;

b)	a <T101> element;

c)	a <T103> element;

d)	a <T104> element;

e)	a <T132> element; and

f)	may include any other element for the purposes of extensibility;

2)	shall contain an <HPLMN> element containing:

a)	a "PLMN" attribute;

b)	a <service> element;

c)	a list of <VPLMN> elements; and

d)	optionally an <anyExt> element containing a list of <SNSSAI> elements;

3)	shall contain an <App-Server-Info> element containing:

a)	an <idms-auth-endpoint> element;

b)	an <idms-token-endpoint> element;

c)	a <http-proxy> element;

d)	a <gms> element;

e)	a <cms> element;

f)	a <kms> element; and

g)	a <tls-tunnel-auth-method> element containing:

i)	a <mutual-authentication> element;

ii)	optionally a <x509> element; and

iii)	optionally a <key> element; and

h)	may include any other element for the purposes of extensibility

4)	shall contain a <GMS-URI> element;

5)	shall contain a <group-creation-XUI> element;

6)	shall contain a <GMS-XCAP-root-URI> element;

7)	shall contain a <CMS-XCAP-root-URI> element;

8)	shall contain an <integrity-protection-enabled> element;

9)	shall contain a <confidentiality-protection-enabled> element;

10)	may contain an <anyExt> element containing:

a)	if the MCPTT service is supported, an <MCPTT-Service-Details> element, containing:

i)	one <IPv6-Required> element;

ii)	one <Server-URI> element; and

iii)	optionally an <anyExt> element with a <PDU-Session-Type> element;

b)	if the MCVideo service is supported, an <MCVideo-Service-Details> element, containing:

i)	one <IPv6-Required> element;

ii)	one <Server-URI> element; and

iii)	optionally an <anyExt> element with a <PDU-Session-Type> element;

c)	if the MCData service is supported, an <MCData-Service-Details> element containing:

i)	one <IPv6-Required> element;

ii)	one <Server-URI> element; and

iii)	optionally an <anyExt> element with a <PDU-Session-Type> element;

d)		optionally an <MCCommonCore-Service-Details> element, containing:

i)	one <IPv6-Required> element;

ii)	one <Server-URI> element; and

iii)	optionally an <anyExt> element with a <PDU-Session-Type> element;

e)	optionally an <MCIdM-Service-Details> element containing:

i)	one <IPv6-Required> element;

ii)	one <Server-URI> element; and

iii)	optionally an <anyExt> element with a <PDU-Session-Type> element;

f)	optionally a list of <DN-Info> elements; and

11)	may include any other element for the purposes of extensibility.

The <off-network> element:

1)	shall contain a <Timers> element containing:

a)	a <TFG1> element;

b)	a <TFG2> element;

c)	a <TFG3> element;

d)	a <TFG4> element;

e)	a <TFG5> element.

f)	a <TFG11> element;

g)	a <TFG12> element;

h)	a <TFG13> element;

i)	a <TFG14> element;

j)	a <TFP1> element;

k)	a <TFP2> element;

l)	a <TFP3> element;

m)	a <TFP4> element;

n)	a <TFP5> element;

o)	a <TFP6> element;

p)	a <TFP7> element;

q)	a <TFB1> element;

r)	a <TFB2> element;

s)	a <TFB3> element;

t)	a <T201> element;

u)	a <T203> element;

v)	a <T204> element;

w)	a <T205> element;

x)	a <T230> element;

y)	a <T233> element;

z)	a <TFE1> element;

za)	a <TFE2> element; and

zb)	may include any other element for the purposes of extensibility;

2)	shall contain a <Counters> element containing:

a)	a <CFP1> element;

b)	a <CFP3> element;

c)	a <CFP4> element;

d)	a <CFP6> element;

e)	a <CFG11> element.

f)	a <CFG12> element;

g)	a <C201> element;

h)	a <C204> element;

i)	a <C205> element; and

j)	may include any other element for the purposes of extensibility; and

3)	may include any other element for the purposes of extensibility.

The <VPLMN> element:

1)	shall contain a "PLMN" attribute;

2)	shall contain a <service> element; and

3)	may contain an <anyExt> element containing optionally a list of <SNSSAI> elements;

The <service> element of the <HPLMN> element and the <VPLMN> element:

1)	shall contain an <MCPTT-to-con-ref> element;

2)	shall contain an <MC-common-core-to-con-ref> element;

3)	shall contain an <MC-ID-to-con-ref> element. and

4)	may contain an <anyExt> element containing:

a)	an <MCPTT-ref-SNSSAI> element;

b)	an <MC-common-core-ref-SNSSAI> element;

c)	an <MC-ID-ref-SNSSAI> element;

d)	an <MCData-ref-SNSSAI> element; and

e)	an <MCVideo-ref-SNSSAI> element;

The <mcptt-UE-id> element:

1)	may contain a list of <Instance-ID-URN> elements; and

2)	may contain a list of <IMEI-range> elements.

The <IMEI-range> element:

1)	shall contain a <TAC> element;

2)	may contain a list of <SNR> elements; and

3)	may contain <SNR-range> element.

The <SNR-range> element:

1)	shall contain a <Low-SNR> element; and

2)	shall contain a <High-SNR> element.

The <DN-Info> element:

1)	shall contain a "DNN" attribute;

2)	may contain a <DN-AAA-Server> element;

3)	may contain a <Pap-parameters> element containing:

i)	a <user-name> element; and

ii)	a <password> element;

4)	may contain a <Chap-parameters> element containing:

i)	a <user-name> element; and

ii)	a <password> element;

5)	may contain an <anyExt> element or any other element for the purpose of extensibility.

The <SNSSAI> element:

1)	may contain a <default-indication> element;

2)	shall contain an <SST> element; and

3)	may contain an <SD> element.

***** Next change *****

#### 7.2.2.3	XML Schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:mcpttiup="urn:3gpp:mcptt:mcpttUEinitConfig:1.0"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:mcptt:mcpttUEinitConfig:1.0"

elementFormDefault="qualified" attributeFormDefault="unqualified">

<xs:import namespace="http://www.w3.org/XML/1998/namespace"

schemaLocation="http://www.w3.org/2001/xml.xsd"/>

<xs:element name="mcptt-UE-initial-configuration">

<xs:complexType>

<xs:choice minOccurs="0" maxOccurs="unbounded">

<xs:element name="mcptt-UE-id" type="mcpttiup:MCPTTUEIDType"/>

<xs:element name="name" type="mcpttiup:NameType"/>

<xs:element name="Default-user-profile" type="mcpttiup:UserProfileType"/>

<xs:element name="on-network" type="mcpttiup:On-networkType"/>

<xs:element name="off-network" type="mcpttiup:Off-networkType"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:attribute name="domain" type="xs:anyURI" use="required"/>

<xs:attribute name="XUI-URI" type="xs:anyURI"/>

<xs:attribute name="Instance-ID-URN" type="xs:anyURI"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:element>

<xs:complexType name="NameType">

<xs:simpleContent>

<xs:extension base="xs:token">

<xs:attribute ref="xml:lang"/>

<xs:attributeGroup ref="mcpttiup:IndexType"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="MCPTTUEIDType">

<xs:choice minOccurs="0" maxOccurs="unbounded">

<xs:element name="Instance-ID-URN" type="xs:anyURI"/>

<xs:element name="IMEI-range" type="mcpttiup:IMEI-rangeType"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:attributeGroup ref="mcpttiup:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="IMEI-rangeType">

<xs:sequence>

<xs:element name="TAC" type="mcpttiup:tacType"/>

<xs:choice minOccurs="0" maxOccurs="unbounded">

<xs:element name="SNR" type="mcpttiup:snrType"/>

<xs:element name="SNR-range" type="mcpttiup:SNR-rangeType"/>

</xs:choice>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attributeGroup ref="mcpttiup:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="SNR-rangeType">

<xs:sequence>

<xs:element name="Low-SNR" type="mcpttiup:snrType"/>

<xs:element name="High-SNR" type="mcpttiup:snrType"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attributeGroup ref="mcpttiup:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tac-baseType">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="8"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tacType">

<xs:simpleContent>

<xs:extension base="mcpttiup:tac-baseType">

<xs:attributeGroup ref="mcpttiup:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:simpleType name="snr-baseType">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="6"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="snrType">

<xs:simpleContent>

<xs:extension base="mcpttiup:snr-baseType">

<xs:attributeGroup ref="mcpttiup:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="UserProfileType">

<xs:attribute name="User-ID" type="xs:anyURI" use="required"/>

<xs:attribute name="user-profile-index" type="xs:unsignedByte" use="required"/>

<xs:attributeGroup ref="mcpttiup:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="VPLMNType">

<xs:sequence>

<xs:element name="service" type="mcpttiup:ServiceType"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="PLMN" type="xs:string" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="ServiceType">

<xs:sequence>

<xs:element name="MCPTT-to-con-ref" type="xs:string"/>

<xs:element name="MC-common-core-to-con-ref" type="xs:string"/>

<xs:element name="MC-ID-to-con-ref" type="xs:string"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<!-- These elements can be added under the anyExt element of the ServiceType element -->

<xs:element name="MCPTT-ref-SNSSAI" type="xs:string" minOccurs="0"/>

<xs:element name="MCData-ref-SNSSAI" type="xs:string" minOccurs="0"/>

<xs:element name="MCVideo-ref-SNSSAI" type="xs:string" minOccurs="0"/>

<xs:element name="MC-common-core-ref-SNSSAI" type="xs:string" minOccurs="0"/>

<xs:element name="MC-ID-ref-SNSSAI" type="xs:string" minOccurs="0"/>

<xs:complexType name="AuthMethodType">

<xs:sequence>

<xs:element name="mutual-authentication" type="xs:boolean"/>

<xs:element name="x509" type="xs:string" minOccurs="0"/>

<xs:element name="key" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="On-networkType">

<xs:sequence>

<xs:element name="Timers">

<xs:complexType>

<xs:sequence>

<xs:element name="T100" type="xs:unsignedByte"/>

<xs:element name="T101" type="xs:unsignedByte"/>

<xs:element name="T103" type="xs:unsignedByte"/>

<xs:element name="T104" type="xs:unsignedByte"/>

<xs:element name="T132" type="xs:unsignedByte"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="HPLMN">

<xs:complexType>

<xs:sequence>

<xs:element name="service" type="mcpttiup:ServiceType"/>

<xs:element name="VPLMN" type="mcpttiup:VPLMNType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="PLMN" type="xs:string" use="required"/>

</xs:complexType>

</xs:element>

<xs:element name="App-Server-Info">

<xs:complexType>

<xs:sequence>

<xs:element name="idms-auth-endpoint" type="xs:anyURI"/>

<xs:element name="idms-token-endpoint" type="xs:anyURI"/>

<xs:element name="http-proxy" type="xs:anyURI"/>

<xs:element name="gms" type="xs:anyURI"/>

<xs:element name="cms" type="xs:anyURI"/>

<xs:element name="kms" type="xs:anyURI"/>

<xs:element name="tls-tunnel-auth-method" type="mcpttiup:AuthMethodType"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="GMS-URI" type="xs:anyURI"/>

<xs:element name="group-creation-XUI" type="xs:anyURI"/>

<xs:element name="GMS-XCAP-root-URI" type="xs:anyURI"/>

<xs:element name="CMS-XCAP-root-URI" type="xs:anyURI"/>

<xs:element name="integrity-protection-enabled" type="xs:boolean"/>

<xs:element name="confidentiality-protection-enabled" type="xs:boolean"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attributeGroup ref="mcpttiup:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- These elements can be added under the anyExt element of the On-networkType element -->

<xs:element name="MCPTT-Service-Details" type="mcpttiup:Service-DetailsType"/>

<xs:element name="MCVideo-Service-Details" type="mcpttiup:Service-DetailsType"/>

<xs:element name="MCData-Service-Details" type="mcpttiup:Service-DetailsType"/>

<xs:element name="MCCommonCore-Service-Details" type="mcpttiup:Service-DetailsType" minOccurs="0"/>

<xs:element name="MCIdM-Service-Details" type="mcpttiup:Service-DetailsType" minOccurs="0"/>

<!-- These elements can be added under the anyExt element of a PLMN or of a service -->

<xs:element name="SNSSAI" type="mcpttiup:snssaiType" minOccurs="0" maxOccurs="unbounded"/>

<xs:complexType name="snssaiType">

<xs:sequence>

<xs:element name="default-indication" type="xs:boolean" minOccurs="0"/>

<xs:element name="SST" type="xs:unsignedByte"/>

<xs:element name="SD" type="mcpttiup:int3BytesType" minOccurs="0"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="int3BytesType">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="16777215"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="Service-DetailsType">

<xs:sequence>

<xs:element name="IPv6-Required" type="xs:boolean"/>

<xs:element name="Server-URI" type="xs:anyURI"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<!-- These elements can be added under the anyExt element of the *-Service-Details element -->

<xs:simpleType name="PDUSessionType">

<xs:restriction base="xs:string">

<xs:enumeration value="IPv4"/>

<xs:enumeration value="IPv6"/>

<xs:enumeration value="IPv4v6"/>

<xs:enumeration value="Ethernet"/>

<xs:enumeration value="Unstructured"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="Off-networkType">

<xs:sequence>

<xs:element name="Timers">

<xs:complexType>

<xs:sequence>

<xs:element name="TFG1" type="xs:unsignedShort"/>

<xs:element name="TFG2" type="xs:unsignedShort"/>

<xs:element name="TFG3" type="xs:unsignedShort"/>

<xs:element name="TFG4" type="xs:unsignedByte"/>

<xs:element name="TFG5" type="xs:unsignedByte"/>

<xs:element name="TFG11" type="xs:unsignedShort"/>

<xs:element name="TFG12" type="xs:unsignedShort"/>

<xs:element name="TFG13" type="xs:unsignedByte"/>

<xs:element name="TFG14" type="xs:unsignedByte"/>

<xs:element name="TFP1" type="xs:unsignedShort"/>

<xs:element name="TFP2" type="xs:unsignedByte"/>

<xs:element name="TFP3" type="xs:unsignedShort"/>

<xs:element name="TFP4" type="xs:unsignedShort"/>

<xs:element name="TFP5" type="xs:unsignedShort"/>

<xs:element name="TFP6" type="xs:unsignedShort"/>

<xs:element name="TFP7" type="xs:unsignedByte"/>

<xs:element name="TFB1" type="xs:unsignedShort"/>

<xs:element name="TFB2" type="xs:unsignedByte"/>

<xs:element name="TFB3" type="xs:unsignedByte"/>

<xs:element name="T201" type="xs:unsignedShort"/>

<xs:element name="T203" type="xs:unsignedByte"/>

<xs:element name="T204" type="xs:unsignedByte"/>

<xs:element name="T205" type="xs:unsignedByte"/>

<xs:element name="T230" type="xs:unsignedByte"/>

<xs:element name="T233" type="xs:unsignedByte"/>

<xs:element name="TFE1" type="xs:unsignedShort"/>

<xs:element name="TFE2" type="xs:unsignedByte"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="Counters">

<xs:complexType>

<xs:sequence>

<xs:element name="CFP1" type="xs:unsignedByte"/>

<xs:element name="CFP3" type="xs:unsignedByte"/>

<xs:element name="CFP4" type="xs:unsignedByte"/>

<xs:element name="CFP6" type="xs:unsignedByte"/>

<xs:element name="CFG11" type="xs:unsignedByte"/>

<xs:element name="CFG12" type="xs:unsignedByte"/>

<xs:element name="C201" type="xs:unsignedByte"/>

<xs:element name="C204" type="xs:unsignedByte"/>

<xs:element name="C205" type="xs:unsignedByte"/>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attributeGroup ref="mcpttiup:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:attributeGroup name="IndexType">

<xs:attribute name="index" type="xs:token"/>

</xs:attributeGroup>

<xs:complexType name="DN-InfoType">

<xs:sequence>

<xs:element name="DN-AAA-Server" type="xs:anyURI" minOccurs="0"/>

<xs:element name="Pap-parameters" minOccurs="0">

<xs:complexType>

<xs:sequence>

<xs:element name="user-name" type="xs:string"/>

<xs:element name="password" type="xs:string"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="Chap-parameters" minOccurs="0">

<xs:complexType>

<xs:sequence>

<xs:element name="user-name" type="xs:string"/>

<xs:element name="password" type="xs:string"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="anyExt" type="mcpttiup:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="DNN" type="xs:string" use="required"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

***** Next change *****

#### 7.2.2.6	Validation Constraints

If the AUID value of the document URI or node URI in the Request-URI is other than that specified in clause 7.2.2.2, then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "invalid application id used".

If the XUI value of the document URI or node URI in the Request-URI does not match the XUI of the MCPTT UE initial configuration document URI, the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "invalid XUI".

The MCS UE initial configuration document shall conform to the XML Schema described in clause 7.2.2.3.

The <mcptt-UE-initial-configuration> element is the root element of the XML document. The <mcptt-UE-initial-configuration> element can contain sub-elements.

The <mcptt-UE-initial-configuration> element may contain one <on-network> element and may contain one <off-network> element. The <mcptt-UE-initial-configuration> element shall contain at least one of either <on-network> or <off-network>.

If the <mcptt-UE-initial-configuration> element does not conform to one of the three choices above, then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "semantic error".

If the "domain" attribute does not contain a syntactically correct domain name, then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect domain name".

If the "domain" attribute contains an unknown domain name, then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "unknown domain name".

If an <Instance-ID-URN> element of the <mcptt-UE-id> element does not conform to a valid Instance ID as specified in 3GPP TS 23.003 [16], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect Instance ID URN" and contain the non-conformant <Instance-ID-URN> element.

If the <TAC> element of an <IMEI-range> element does not conform to a valid 8 digit Type Allocation Code as specified in 3GPP TS 23.003 [16], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect Type Allocation Code" and contain the non-conformant <TAC> element.

If a <SNR> element of an <IMEI--range> element does not conform to a valid 6 digit Serial Number as specified in 3GPP TS 23.003 [16], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect Serial Number" and contain the non-conformant <SNR> element.

If a <Low-SNR> element or a <High-SNR> element of a <SNR-range> element does not conform to a valid 6 digit Serial Number as specified in 3GPP TS 23.003 [16], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect Serial Number range" and contain the non-conformant <Low-SNR>  or <High-SNR> element.

If the "User-ID" attribute of the <Default-user-profile> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect User ID URI".

If the "User-ID" attribute of the <Default-user-profile> element does not contain an "XUI-URI" attribute of an MCS user profile configuration document, then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "No MCS User Profile configuration document exists for the user identified by the User ID URI".

If the "user-profile-index" attribute of the <Default-user-profile> element does not contain an "user-profile-index" attribute of a MCPTT user profile configuration document, then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "The user profile index does not identify an MCS User Profile configuration document identified of the user identified by the User ID URI ".

NOTE:	If the MCS administrator includes a <Default-user-profile> element in the MCS UE initial configuration document, at least one instance of an MCS user profile configuration document needs to first be created on the configuration management server, containing the "XUI-URI" attribute and "user-profile-index" attribute that are included in the <Default-user-profile> element.

If any of the following elements of the <Timers> element of the <on-network> element do not conform to the range of values specified below:

a)	the <T100> element contains an integer value between 0 and 255;

b)	the <T101> element contains an integer value between 0 and 255;

c)	the <T103> element contains an integer value between 0 and 255;

d)	the <T104> element contains an integer value between 0 and 255;

e)	the <T132> element contains an integer value between 0 and 255,

then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect timer value" and also contain the identity of the non-conformant timer (e.g. "T101").

If the <idms-auth-endpoint> element of the <App-Server-Info> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect identity management server URI".

If the <idms-token-endpoint> element of the <App-Server-Info> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect identity management server URI".

If the <gms> element of the <App-Server-Info> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect group management server URI".

If the <tls-tunnel-auth-method> element of the <mutual-authentication-element> of the <App-Server-Info> element is set to "true" and neither the <x509> element nor the <key> element of the <mutual-authentication-element> of the <App-Server-Info> element are present, then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "mutual authentication enabled without a X.509 certificate or pre-shared key".

If the <tls-tunnel-auth-method> element of the <mutual-authentication-element> of the <App-Server-Info> element is set to "true" and both the <x509> element and the <key> element of the <mutual-authentication-element> of the <App-Server-Info> element are present, then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "mutual authentication enabled with both a X.509 certificate and a pre-shared key".

If the <cms> element of the <App-Server-Info> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect configuration management server URI".

If the <kms> element of the <App-Server-Info> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect key management server URI".

If the <GMS-URI> element of the <on-network> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect GMS-URI".

If the <group-creation-XUI> element of the <on-network> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect group creation XUI".

If the <GMS-XCAP-root-URI> element of the <on-network> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect GMS XCAP root URI".

If the <CMS-XCAP-root-URI> element of the <on-network> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect CMS XCAP root URI".

If the <IPv6-Required> element of the <MCPTT-Service-Details> element of the <anyExt> element of the <on-network> element does not contain a value of "true" or "false", then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "semantic error".

If the <Server-URI> element of the <MCPTT-Service-Details> element of the <anyExt> element of the <on-network> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect MCPTT server URI".

If the <IPv6-Required> element of the <MCVideo-Service-Details> element of the <anyExt> element of the <on-network> element does not contain a value of "true" or "false", then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "semantic error".

If the <Server-URI> element of the <MCVideo-Service-Details> element of the <anyExt> element of the <on-network> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect MCVideo server URI".

If the <IPv6-Required> element of the <MCData-Service-Details> element of the <anyExt> element of the <on-network> element does not contain a value of "true" or "false", then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "semantic error".

If the <Server-URI> element of the <MCData-Service-Details> element of the <anyExt> element of the <on-network> element does not conform to a valid URI as specified in IETF RFC 3986 [21], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect MCData server URI".

If the "PLMN" attribute of the <HPLMN> element of the <on-network> element does not conform to the syntax of a valid PLMN code as defined in 3GPP TS 23.003 [16] then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect HPLMN value".

If the "PLMN" attribute of a <VPLMN> element of the <HPLMN> element of the <on-network> element does not conform to the syntax of a valid PLMN code as defined in 3GPP TS 23.003 [16] then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect VPLMN value" and also contain the contents of the non-conformant "PLMN" attribute.

If the "DNN" attribute of a <DN-Info> element in the <anyExt> element of the <on-network> element

does not contain a syntactically valid APN/DNN as specified in 3GPP TS 23.003 [16], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect APN/DNN value" and also contain the contents of the non-conformant "DNN" attribute.

If an <SNSSAI> element of the <anyExt> element in the <HPLMN> element, or in an <VPLMN>, of the <on-network> element does not contain a syntactically valid S-NSSAI as specified in 3GPP TS 23.003 [16], then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect S-NSSAI value" and also contain the contents of the non-conformant <SNSSAI> element.

If any of the following elements of the <Timers> element of the <off-network> element do not conform to the range of values specified below:

a)	the <TFG1> element contains an integer value between 0 and 65535;

b)	the <TFG2> element contains an integer value between 0 and 65535;

c)	the <TFG3> element contains an integer value between 0 and 65535;

d)	the <TFG4> element contains an integer value between 0 and 60;

e)	the <TFG5> element contains an integer value between 0 and 255;

f)	the <TFG11> element contains an integer value between 0 and 65535;

g)	the <TFG12> element contains an integer value between 0 and 65535;

h)	the <TFG13> element contains an integer value between 0 and 255;

i)	the <TFG14> element contains an integer value between 0 and 255;

j)	the <TFP1> element contains an integer value between 0 and 65535;

k)	the <TFP2> element contains an integer value between 0 and 60;

l)	the <TFP3> element contains an integer value between 0 and 65535;

m)	the <TFP4> element contains an integer value between 0 and 65535;

n)	the <TFP5> element contains an integer value between 0 and 600;

o)	the <TFP6> element contains an integer value between 0 and 65535;

p)	the <TFP7> element contains an integer value between 0 and 255;

q)	the <TFB1> element contains an integer value between 0 and 600;

r)	the <TFB2> element contains an integer value between 0 and 10;

s)	the <TFB3> element contains an integer value between 0 and 60;

t)	the <T201> element contains an integer value between 0 and 65535;

u)	the <T203> element contains an integer value between 0 and 255;

v)	the <T204> element contains an integer value between 0 and 255;

w)	the <T205> element contains an integer value between 0 and 255;

x)	the <T230> element contains an integer value between 0 and 255;

y)	the <T233> element contains an integer value between 0 and 255;

z)	the <TFE1> element contains an integer value between 0 and 65535;

za)	the <TFE2> element contains an integer value between 0 and 10,

then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect timer value" and also contain the identity of the non-conformant timer (e.g. "TFG1").

If any of the following elements of the <Counters> element of the <off-network> element do not conform to the range of values specified below:

a)	the <CFP1> element contains an integer value between 0 and 255;

b)	the <CFP3> element contains an integer value between 0 and 255;

c)	the <CFP4> element contains an integer value between 0 and 255;

d)	the <CFP6> element contains an integer value between 0 and 255;

e)	the <CFP11> element contains an integer value between 0 and 255;

f)	the <CFP12> element contains an integer value between 0 and 255;

g)	the <C201> element contains an integer value between 0 and 255;

h)	the <C204> element contains an integer value between 0 and 255;

i)	the <C205> element contains an integer value between 0 and 255,

then the configuration management server shall return an HTTP 409 (Conflict) response including the XCAP error element <constraint-failure>. If included, the "phrase" attribute should be set to "syntactically incorrect counter value" and also contain the identity of the non-conformant counter (e.g. "CFP1").

***** Next change *****

#### 7.2.2.7	Data Semantics

The "domain" attribute of the <mcptt-UE-initial-configuration> element contains the domain name of the mission critical organization.

The creator of the MCS UE initial configuration document may include an <mcptt-UE-id> element in the version of the MCS UE initial configuration document that is uploaded to the CMS and may also appear in the MCS UE initial configuration document when downloaded by the MCS administrator. The <mcptt-UE-id> element does not appear in the MCS UE initial configuration managed object specified in 3GPP TS 24.483 [4] that is configured to the MCS UE. If an <mcptt-UE-id> element is included then the MCS UE initial configuration document and corresponding MCS UE initial configuration management object applies only to the MCS UE(s) identified by the <mcptt-UE-id> element. If no <mcptt-UE-id> element is included then the MCS UE initial configuration document and corresponding MCS UE initial configuration management object applies to all the MCS UEs of the domain.

If one or more optional <Instance-ID-URN> elements is included in the <mcptt-UE-id> element then the MCS UE initial configuration document applies to the MCS UE with an instance ID equal to the instance ID contained in the <Instance-ID-URN> element.

The <TAC> element of the <IMEI-range> element contains the Type Allocation Code of the MCS UE.

The optional <SNR> element of the <IMEI-range> element contains the individual serial number uniquely identifying MCS UE within the Type Allocation Code contained in the <TAC> element that the MCS UE initial configuration document applies to.

If an optional <SNR-range> element is included within the <IMEI-range> element then the MCS UE initial configuration document applies to all MCS UEs within the Type Allocation Code contained in the <TAC> element with the serial number equal or greater than the serial number contained in the <Low-SNR> element and less than or equal to the serial number contained in the <High-SNR> element.

If no <SNR> element nor <SNR-range> element is included within the <IMEI-range> element then the MCS UE initial configuration document applies to all the MCS UE(s) with the Type Allocation Code contained within the <TAC> element of the <IMEI-range> element.

If no <mcptt-UE-id> element is included then the MCS UE initial configuration document applies to all MCS UEs of the mission critical organization identified in the "domain" attribute.

The <name> element of the <mcptt-UE-initial-configuration> element contains the user displayable name of the MCS UE initial configuration document and corresponds to the "Name" element of clause 8.2.3 in 3GPP TS 24.483 [4].

The "User-ID" attribute of the <Default-user-profile> element contains the XUI contained in the "XUI-URI" attribute of the MCPTT user profile that is intended to be used as default MCS user profile and corresponds to the "UserID" element of clause 8.2.6 in 3GPP TS 24.483 [4].

The "user-profile-index" attribute of the <Default-user-profile> element contains an indicator for a particular MCS user profile document when multiple MCS user profiles are defined for that MCS user and is of type "unsignedByte" and matches a value in a "user-profile-index" attribute of the MCS user profile that is intended to be used as default MCS user profile, and corresponds to the "UserProfileIndex" element of clause 8.2.7 in 3GPP TS 24.483 [4].

The optional <SNSSAI> element of the <anyExt> element in the <HPLMN> element, or in a <VPLMN> element, of the <on-network> element indicates the S-NSSAI to be used in the PLMN. The optional <default-indication> element when set to "true" indicates that the S-NSSAI is a default one. If set to "false" or not included, the S-NSSAI is not a default one.

The <MCPTT-to-con-ref>, <MC-common-core-to-con-ref> and <MC-ID-to-con-ref> elements of the <service> element in the <HPLMN> element of the <on-network> element indicate the APN/DNN to be used in the PLMN for the respective service and correspond to the "ConRef" elements of clauses 8.2.21, 8.2.24 and 8.2.27 in 3GPP TS 24.483 [4], respectively. For a <VPLMN> element the corresponding elements are specified in clauses 8.2.33, 8.2.36 and 8.2.39 in 3GPP TS 24.483 [4], respectively.

The optional <MCPTT-ref-SNSSAI>, <MCData-ref-SNSSAI>, <MCVideo-ref-SNSSAI>, <MC-common-core-ref-SNSSAI> and <MC-ID-ref-SNSSAI> elements of the <anyExt> element in the <service> element in the <HPLMN> element, or in a <VPLMN> element, of the <on-network> element indicate the S-NSSAI to be used in the PLMN for the respective service.

The <on-network> element contains MCS UE initial configuration data for on-network operation only.

The <off-network> element contains MCS UE initial configuration data for off-network operation only.

In the <on-network> element:

1)	the <Timers> element;

a)	the <T100> element contains the timer value in seconds for floor release as specified in 3GPP TS 24.380 [10] and corresponds to the "T100" element of clause 8.2.11 in 3GPP TS 24.483 [4];

b)	the <T101> element contains the timer value in seconds for floor request as specified in 3GPP TS 24.380 [10] and corresponds to the "T101" element of clause 8.2.12 in 3GPP TS 24.483 [4];

c)	the <T103> element contains the timer value in seconds for end of RTP media as specified in 3GPP TS 24.380 [10] and corresponds to the "T103" element of clause 8.2.13 in 3GPP TS 24.483 [4];

d)	the <T104> element contains the timer value in seconds for floor queue position request as specified in 3GPP TS 24.380 [10] and corresponds to the "T104" element of clause 8.2.14 in 3GPP TS 24.483 [4]; and

e)	the <T132> element contains the timer value in seconds for queued request granted MCPTT user action as specified in 3GPP TS 24.380 [10] and corresponds to the "T132" element of clause 8.2.15 in 3GPP TS 24.483 [4].

2)	the "PLMN" attribute of the <HPLMN> element contains the PLMN code of the HPLMN as defined in 3GPP TS 23.003 [16] and corresponds to the "PLMN" element of clause 8.2.18 in 3GPP TS 24.483 [4];

3)	the "PLMN" attribute of the <VPLMN> element contains the PLMN code of a VPLMN as defined in 3GPP TS 23.003 [16] and corresponds to the "PLMN element of clause 8.2.30 in 3GPP TS 24.483 [4];

4)	the <App-Server-Info> element:

a)	the <idms-auth-endpoint> element contains the URI used to contact the identity management server and corresponds to the "IDMSAuthEndpoint" element of clause 8.2.41 in 3GPP TS 24.483 [4];

b)	the <idms-token-endpoint> element contains the URI used to contact the identity management server and corresponds to the "IDMSTokenEndpoint" element of clause 8.2.41A in 3GPP TS 24.483 [4];

c)	the <http-proxy> element contains the URI used to contact the HTTP proxy and corresponds to the "HTTPProxy" element of clause 8.2.41B in 3GPP TS 24.383 [4];

d)	the <gms> element contains the URI used to contact the group management server and corresponds to the "GMS" element of clause 8.2.42 in 3GPP TS 24.483 [4];

e)	the <cms> element contains the URI used to contact the configuration management server and corresponds to the "CMS" element of clause 8.2.43 in 3GPP TS 24.483 [4];

f)	the <kms> element contains the URI used to contact the key management server and corresponds to the "KMS" element of clause 8.2.44 in 3GPP TS 24.483 [4]; and

g)	the <tls-tunnel-auth-method> element that contains the<mutual-authentication-element> that corresponds to the "Mutual" element of clause 8.2.44B in 3GPP TS 24.383 [4] and when set to "true" indicates that mutual authentication is used for the TLS tunnel authentication. The <x509> element when present contains the X.509 certificate for mutual authentication for the TLS tunnel authentication and corresponds to the "X509" element of clause 8.2.44C in 3GPP TS 24.383 [4]. The <key> element when present contains the pre-shared key for mutual authentication for the TLS tunnel authentication and corresponds to the "X509" element of clause 8.2.44D in 3GPP TS 24.383 [4].

5)	the <GMS-URI> element contains the group management service URI information to enable hiding of MCS identities, the group management service URI information contains the public service identity for performing subscription proxy function of the GMS and corresponds to the "GMSURI" element of clause 8.2.9 in 3GPP TS 24.483 [4];

6)	the <group-creation-XUI> element contains the group management server XCAP Root URI information and corresponds to the "GroupCreationXUI" element of clause 8.2.9A in 3GPP TS 24.483 [4];

7)	the <GMS-XCAP-root-URI> element contains the group management server XCAP Root URI and corresponds to the "GMSXCAPRootURI" element of clause 8.2.9B in 3GPP TS 24.483 [4];

8)	the < CMS-XCAP-root-URI> element contains the configuration management server XCAP Root URI and corresponds to the "CMSXCAPRootURI" element of clause 8.2.9C in 3GPP TS 24.483 [4];

9)	the <IPv6-Required> element of the <MCPTT-Service-Details> element of the <anyExt> element of the <on-network> element indicates whether IPv6 shall be used to access the MCPTT service.

10)	the <Server-URI> element of the < MCPTT-Service-Details> element of the <anyExt> element of the <on-network> element contains the URI used to contact the MCPTT service server;

11)	the <IPv6-Required> element of the <MCVideo-Service-Details> element of the <anyExt> element of the <on-network> element indicates whether IPv6 shall be used to access the MCVideo service.

12)	the <Server-URI> element of the <MCVideo-Service-Details> element of the <anyExt> element of the <on-network> element contains the URI used to contact the MCVideo service server;

13)	the <IPv6-Required> element of the <MCData-Service-Details> element of the <anyExt> element of the <on-network> element indicates whether IPv6 shall be used to access the MCData service.

14)	the <Server-URI> element of the <MCData-Service-Details> element of the <anyExt> element of the <on-network> element contains the URI used to contact the MCData service server;

15)	the <integrity-protection-enabled> element indicates whether integrity protection is enabled and corresponds to the "IntegrityProtection" element of clause 8.2.44E in 3GPP TS 24.383 [4];

16)	the <confidentiality-protection-enabled> element indicates whether integrity protection is enabled and corresponds to the "ConfidentialityProtection" element of clause 8.2.44F in 3GPP TS 24.383 [4];

17)	the <DN-Info> element of the <anyExt> element 	contains the information related to the DNN/APN name included in the "DNN" attribute, i.e.,:

a)	the <DN-AAA-Server> element contains the URI to be used for authentication/authorization; and

b)	may contain a PAP user name and a PAP password in the <user-name> and <password> elements of the <Pap-parameters> element and may contain a CHAP user name and a CHAP password in the <user-name> and <password> elements of the <Chap-parameters> element;

In the <off-network> element:

1)	the <Timers> element:

a)	the <TFG1> element contains the timer value in milliseconds for wait for call announcement as specified in 3GPP TS 24.379 [9] and corresponds to the "TFG1" element of clause 8.2.47 in 3GPP TS 24.483 [4];

b)	the <TFG2> element contains the timer value in milliseconds for call announcement as specified in 3GPP TS 24.379 [9] and corresponds to the "TFG2" element of clause 8.2.48 in 3GPP TS 24.483 [4];

c)	the <TFG3> element contains the timer value in milliseconds for call probe retransmission as specified in 3GPP TS 24.379 [9]; and corresponds to the "TFG3" element of clause 8.2.49 in 3GPP TS 24.483 [4]

d)	the <TFG4> element contains the timer value in seconds for waiting for the MCPTT user as specified in 3GPP TS 24.379 [9] and corresponds to the "TFG4" element of clause 8.2.50 in 3GPP TS 24.483 [4];

e)	the <TFG5> element contains the timer value in seconds for not present incoming call announcements as specified in 3GPP TS 24.379 [9] and corresponds to the "TFG5" element of clause 8.2.51 in 3GPP TS 24.483 [4];

f)	the <TFG11> element contains the timer value in milliseconds for MCPTT emergency end retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "TFG11" element of clause 8.2.52 in 3GPP TS 24.483 [4];

g)	the <TFG12> element contains the timer value in milliseconds for MCPTT imminent peril end retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "TFG12" element of clause 8.2.53 in 3GPP TS 24.483 [4];

h)	the <TFG13> element contains the timer value in seconds for timer for implicit priority downgrade (emergency) as specified in 3GPP TS 24.379 [9] and corresponds to the "TFG13" element of clause 8.2.54 in 3GPP TS 24.483 [4];

i)	the <TFG14> element contains the timer value in seconds for timer for implicit priority downgrade (imminent peril) as specified in 3GPP TS 24.379 [9] and corresponds to the "TFG14" element of clause 8.2.54A in 3GPP TS 24.483 [4];

j)	the <TFP1> element contains the timer value in milliseconds for private call request retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "TFP1" element of clause 8.2.55 in 3GPP TS 24.483 [4];

k)	the <TFP2> element contains the timer value in seconds for waiting for call response message as specified in 3GPP TS 24.379 [9] and corresponds to the "TFP2" element of clause 8.2.56 in 3GPP TS 24.483 [4];

l)	the <TFP3> element contains the timer value in milliseconds for private call release retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "TFP3" element of clause 8.2.57 in 3GPP TS 24.483 [4];

m)	the <TFP4> element contains the timer value in milliseconds for private call accept retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "TFP4" element of clause 8.2.58 in 3GPP TS 24.483 [4];

n)	the <TFP5> element contains the timer value in seconds for call release as specified in 3GPP TS 24.379 [9] and corresponds to the "TFP5" element of clause 8.2.59 in 3GPP TS 24.483 [4];

o)	the <TFP6> element contains the timer value in milliseconds for MCPTT emergency private call cancel retransmission as specified as specified in 3GPP TS 24.379 [9] and corresponds to the "TFP6" element of clause 8.2.60 in 3GPP TS 24.483 [4];

p)	the <TFP7> element contains the timer value in seconds for waiting for any message with same call identifier as specified in 3GPP TS 24.379 [9] and corresponds to the "TFP7" element of clause 8.2.61 in 3GPP TS 24.483 [4];

q)	the <TFB1> element contains the timer value in seconds for max duration as specified in 3GPP TS 24.379 [9] and corresponds to the "TFB1" element of clause 8.2.62 in 3GPP TS 24.483 [4];

r)	the <TFB2> element contains the timer value in seconds for broadcast retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "TFB2" element of clause 8.2.63 in 3GPP TS 24.483 [4];

s)	the <TFB3> element contains the timer value in seconds for waiting for the MCPTT user as specified in 3GPP TS 24.379 [9] and corresponds to the "TFB3" element of clause 8.2.64 in 3GPP TS 24.483 [4];

t)	the <T201> element contains the timer value in milliseconds for floor request as specified in 3GPP TS 24.380 [10] and corresponds to the "T201" element of clause 8.2.65 in 3GPP TS 24.483 [4];

u)	the <T203> element contains the timer value in seconds for end of RTP media as specified in 3GPP TS 24.380 [10] and corresponds to the "T203" element of clause 8.2.66 in 3GPP TS 24.483 [4];

v)	the <T204> element contains the timer value in seconds for floor queue position request as specified in 3GPP TS 24.380 [10] and corresponds to the "T204" element of clause 8.2.67 in 3GPP TS 24.483 [4];

w)	the <T205> element contains the timer value in seconds for floor granted request as specified in 3GPP TS 24.380 [10] and corresponds to the "T205" element of clause 8.2.68 in 3GPP TS 24.483 [4];

x)	the <T230> element contains the timer value in seconds for during silence as specified in 3GPP TS 24.380 [10] and corresponds to the "T230" element of clause 8.2.69 in 3GPP TS 24.483 [4];

y)	the <T233> element contains the timer value in seconds for pending user action as specified in 3GPP TS 24.380 [10] and corresponds to the "T233" element of clause 8.2.70 in 3GPP TS 24.483 [4];

z)	the <TFE1> element contains the timer value in seconds for MCPTT emergency alert as specified in 3GPP TS 24.380 [10] and corresponds to the "TFE1" element of clause 8.2.71 in 3GPP TS 24.483 [4]; and

za)	the <TFE2> element contains the timer value in seconds for MCPTT emergency alert retransmission as specified in 3GPP TS 24.380 [10] and corresponds to the "TFE2" element of clause 8.2.72 in 3GPP TS 24.483 [4]; and

2)	the <Counters> element.

a)	the <CFP1> element contains the counter value for private call request retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "CFP1" element of clause 8.2.74 in 3GPP TS 24.483 [4];

b)	the <CFP3> element contains the counter value for private call release retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "CFP3" element of clause 8.2.75 in 3GPP TS 24.483 [4];

c)	the <CFP4> element contains the counter value for private call accept retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "CFP4" element of clause 8.2.76 in 3GPP TS 24.483 [4];

d)	the <CFP6> element contains the counter value for private call accept retransmission t as specified in 3GPP TS 24.379 [9] and corresponds to the "CFP6" element of clause 8.2.77 in 3GPP TS 24.483 [4];

e)	the <CFP11> element contains the counter value for MCPTT group call emergency end retransmission ia as specified in 3GPP TS 24.379 [9] and corresponds to the "CFP11" element of clause 8.2.78 in 3GPP TS 24.483 [4];

f)	the <CFP12> element contains the counter value for MCPTT imminent peril call emergency end retransmission as specified in 3GPP TS 24.379 [9] and corresponds to the "CFP12" element of clause 8.2.79 in 3GPP TS 24.483 [4];

g)	the <C201> element contains the counter value for floor request as specified in 3GPP TS 24.380 [10] and corresponds to the "C201" element of clause 8.2.80 in 3GPP TS 24.483 [4];

h)	the <C204> element contains the counter value for floor queue position request as specified in 3GPP TS 24.380 [10] and corresponds to the "C204" element of clause 8.2.81 in 3GPP TS 24.483 [4]; and

i)	the <C205> element contains the counter valuefor floor granted request as specified in 3GPP TS 24.380 [10] and corresponds to the "C205" element of clause 8.2.82 in 3GPP TS 24.483 [4].

***** End of changes *****