3GPP TSG-CT Meeting #96	CP-221094

# Budapest, Hungary, 05th – 06th June 2022	(was C4-222144)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.562 | CR | 0103 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing Description Fields and Additional Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | GBA_5G |  |  |  |  |  | Date: |  |  | 2022-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are several deficiencies in the description of Nhss_gbaSDM and Nhss_gbaUEAU APIs, affecting their overall quality. Some aspects are non-functional and affect only the documentation aspects, such as: - Missing, or wrongly formatted, description fields in reusable schema components - Missing reference to TS 29.309 Other aspects are functional, such as: - Wrong definition of "securityFeatures" attribute in BsfInfo, which should be an array, rather than a single element - Duplicated definition of types ResynchronizationInfo and 3GAkaAv which are identical to the definitions from Nhss_imsUEAU API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing description fields - Format long description fields according to multi-line format conventions - Define "securityFeatures" in BsfInfo as an array - Reuse ResynchronizationInfo and 3GAkaAv from Nhss_imsUEAU |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiple issues in the description of Nhss_gbaSDM and Nhss_gbaUEAU APIs, affecting their overal quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.4.2.1, 6.4.6.1, 6.4.6.2.4, 6.4.6.3.2, 6.5.2.1, 6.5.6.1, 6.5.6.2.4, 6.5.6.2.5, A.5, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible corrections, with impacts on the following APIs: - TS29562_Nhss_gbaSDM.yaml - TS29562_Nhss_gbaUEAU.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1 (to CT#96): - In OpenAPI Annex A.5: removal of a spare and wrong qualifier "minItems: 1" from data type GbaSubscriberData |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 29.335: "User Data Repository Access Protocol over the Ud interface; Stage 3".

[8]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[9]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[10]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	IETF RFC 7807: "Problem Details for HTTP APIs".

[12]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[13]	3GPP TS 23.003: "Numbering, addressing and identification".

[14]	3GPP TS 33.203: "Access security for IP-based services".

[15]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[17]	IETF RFC 4740: "Diameter Session Initiation Protocol (SIP) Application".

[18]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[19]	IETF RFC 4412: "Communications Resource Priority for the Session Initiation Protocol".

[20]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces;Signalling flows and message contents".

[21]	3GPP TS 29.218: "IP Multimedia (IM) session handling; IM call model; Stage 2".

[22]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[23]	IETF RFC 8497: "Marking SIP messages to be logged".

[24]	3GPP TS 24.323: "3GPP IP Multimedia Subsystem (IMS) service level tracing Management Object (MO)".

[25]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[26]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[27]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[28]	IETF RFC 2045: "Multipurpose Internet Mail Extensions(MIME) Part One: Format of Internet Message Bodies".

[29]	ETSI ES 283 034: "Network Attachment Sub-System (NASS); e4 interface based on the DIAMETER protocol".

[30]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[31]	3GPP TS 23.078: "Customised Applications for Mobile network Enhanced Logic (CAMEL) Phase 3 - Stage 2".

[32]	3GPP TS 29.272: "Evolved Packet System; MME and SGSN Related Interfaces Based on Diameter Protocol".

[33]	3GPP TS 29.518: 5G System; Access and Mobility Management Services; Stage 3".

[34]	3GPP TR 21.900: "Technical Specification Group working methods".

[35]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[36]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[37]	3GPP TS 23.380: "IP Multimedia Subsystem (IMS); IMS Restoration Procedures".

[38]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents".

[39]	3GPP TS 29.563: "Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[40]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[41]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[aa]	3GPP TS 29.309: "Bootstrapping Server Function (GBA BSF) Services; Stage 3".

* * * Next Change * * * *

#### 6.4.2.1	General

HTTP/2, as defined in IETF RFC 7540 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nhss_gbaSubscriberDataManagement service shall comply with the OpenAPI [9] specification contained in Annex A.5.

* * * Next Change * * * *

#### 6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the structured data types defined for the Nhss_gbaSDM service API.

Table 6.4.6.1-1: Nhss_gbaSDM specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| GbaSubscriberData | 6.4.6.2.2 |  |
| Guss | 6.4.6.2.3 |  |
| BsfInfo | 6.4.6.2.4 |  |
| GbaSdmSubscription | 6.4.6.2.5 |  |
| UeId | 6.4.6.3.2 |  |

Table 6.4.6.1-2 specifies data types re-used by the Nhss_gbaSDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_gbaSDM.

Table 6.4.6.1-2: Nhss_gbaSDM re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| UssListItem | 3GPP TS 29.309 [aa] |  |
| SecFeature | 3GPP TS 29.309 [aa] |  |
| ModificationNotification | 3GPP TS 29.503 [15] |  |
| NfInstanceId | 3GPP TS 29.571 [16] |  |
| Uri | 3GPP TS 29.571 [16] |  |
| DateTime | 3GPP TS 29.571 [16] |  |
| DurationSec | 3GPP TS 29.571 [16] |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Response body of error response messages. |
| RedirectResponse | 3GPP TS 29.571 [16] | Response body of redirect response messages. |
| ImsUeId | 3GPP TS 29.562 | IMS UE ID (IMPU or IMPI) |

* * * Next Change * * * *

##### 6.4.6.2.4	Type: BsfInfo

Table 6.4.6.2.4-1: Definition of type BsfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| uiccType | UiccType | O | 0..1 | It indicates whether GBA or GBA_U is to be used; default value is GBA. |
| lifeTime | DurationSec | O | 0..1 | It indicates a user-specific key lifetime (duration in seconds). If the lifeTime element is missing the default value in the BSF is used. |
| securityFeatures | array(SecFeature) | O | 1..N | It indicates a user-specific security feature list that the UE supports. If the securityFeature element is missing, then the security features are not defined. Standard security features are defined in 3GPP TS 29.309 [aa], clause 6.1.6.3.5. |

* * * Next Change * * * *

##### 6.4.6.3.2	Simple data types

The simple data types defined in table 6.4.6.3.2-1 shall be supported.

Table 6.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| UeId |  | string |  | Identity of the UE. It can be an MSISDN, an IMSI an IMPI or an IMPU. |  |
| Msisdn |  | string |  | MS-ISDN of the UE. pattern: '^msisdn-[0-9]{5,15}$' |  |
| Imsi | string | IMSI of the UE. pattern: '^imsi-[0-9]{5,15}$' |

* * * Next Change * * * *

#### 6.5.2.1	General

HTTP/2, as defined in IETF RFC 7540 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nhss_gbaUEAuthentication service shall comply with the OpenAPI [9] specification contained in Annex A.6.

* * * Next Change * * * *

#### 6.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.5.6.1-1 specifies the structured data types defined for the Nhss_gbaUEAU service API.

Table 6.5.6.1-1: Nhss_gbaUEAU specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationInfoRequest | 6.5.6.2.2 |  |
| AuthenticationInfoResult | 6.5.6.2.3 |  |
| DigestAuthentication | 6.5.6.2.6 |  |
| Impi | 6.5.6.3.2 |  |
| AuthenticationScheme | 6.5.6.3.3 |  |
| DigestAlgorithm | 6.5.6.3.4 |  |
| DigestQop | 6.5.6.3.5 |  |

Table 6.5.6.1-2 specifies data types re-used by the Nhss_gbaUEAU service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_gbaUEAU.

Table 6.5.6.1-2: Nhss_gbaUEAU re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Autn | 3GPP TS 29.503 [15] |  |
| Auts | 3GPP TS 29.503 [15] |  |
| Rand | 3GPP TS 29.503 [15] |  |
| Xres | 3GPP TS 29.503 [15] |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Response body of error response messages. |
| RedirectResponse | 3GPP TS 29.571 [16] | Response body of redirect response messages. |
| SupportedFeatures | 3GPP TS 29.571 [16] |  |
| ResynchronizationInfo | 3GPP TS 29.562 | Clause 6.3.6.2.4 |
| 3GAkaAv | 3GPP TS 29.562 | Clause 6.3.6.2.5 |
| UeId | 3GPP TS 29.562 | Clause 6.4.6.3.2 |

* * * Next Change * * * *

##### 6.5.6.2.4	Void

* * * Next Change * * * *

##### 6.5.6.2.5	Void

* * * Next Change * * * *

## A.5	Nhss_gbaSDM API

openapi: 3.0.0

info:

version: '1.0.0-alpha.2'

title: 'Nhss_gbaSDM'

description: |

Nhss Subscriber Data Management Service for GBA.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Home Subscriber Server (HSS) Services, version 17.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-gba-sdm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

paths:

/{ueId}/subscriber-data:

get:

summary: Retrieve the subscriber data of a user

operationId: GetSubscriberData

tags:

- Subscriber Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

- oAuth2ClientCredentials:

- nhss-gba-sdm

- nhss-gba-sdm:subscriber-data:read

parameters:

- name: ueId

in: path

description: UE Identity

required: true

schema:

$ref: '#/components/schemas/UeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/GbaSubscriberData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{ueId}/subscriptions:

post:

summary: Subscribe to notifications

operationId: GbaSdmSubscribe

tags:

- GBA SDM Subscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

- oAuth2ClientCredentials:

- nhss-gba-sdm

- nhss-gba-sdm:subscriptions:create

parameters:

- name: ueId

in: path

description: UE Identity

required: true

schema:

$ref: '#/components/schemas/UeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/GbaSdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/GbaSdmSubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-gba-sdm/<apiVersion>/{ueId}/subscriptions/{subscriptionId}

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

datachangeNotification:

'{request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ModificationNotification'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{ueId}/subscriptions/{subscriptionId}:

delete:

summary: Unsubscribe from notifications

operationId: GbaSdmUnsubscribe

tags:

- GBA SDM Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

- oAuth2ClientCredentials:

- nhss-gba-sdm

- nhss-gba-sdm:subscription:modify

parameters:

- name: ueId

in: path

description: UE Identity

required: true

schema:

$ref: '#/components/schemas/UeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the subscription

operationId: GbaSdmSubsModify

tags:

- GBA SDM Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

- oAuth2ClientCredentials:

- nhss-gba-sdm

- nhss-gba-sdm:subscription:modify

parameters:

- name: ueId

in: path

description: UE Identity

required: true

schema:

$ref: '#/components/schemas/UeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful modification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-gba-sdm: Access to the Nhss GBA Subscription Data Management API

nhss-gba-sdm:subscriber-data:read: Access to read the GBA Subscriber Data resource

nhss-gba-sdm:subscriptions:create: Access to create Subscriptions resources

nhss-gba-sdm:subscription:modify: Access to update/delete a Subscription resource

schemas:

#

# COMPLEX TYPES:

#

GbaSubscriberData:

description: >

GBA subscriber data of the UE; it includes the GBA User Security Settings (GUSS) data

type: object

properties:

guss:

$ref: '#/components/schemas/Guss'

Guss:

description: >

GBA User Security Settings of the UE; it includes information about the BSF,

and a list of User Security Settings

type: object

properties:

bsfInfo:

$ref: '#/components/schemas/BsfInfo'

ussList:

type: array

items:

$ref: 'TS29309_Nbsp_GBA.yaml#/components/schemas/UssListItem'

minItems: 1

BsfInfo:

description: >

Information about the BSF; it includes an indication of whether GBA or GBA_U is to be used,

the key lifetime and a user-specific list of security features

type: object

properties:

uiccType:

$ref: '#/components/schemas/UiccType'

lifeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

securityFeatures:

type: array

items:

$ref: 'TS29309_Nbsp_GBA.yaml#/components/schemas/SecFeature'

minItems: 1

GbaSdmSubscription:

description: >

Information about a subscription created in the HSS, so the consumer (e.g. BSF) can be

notified when there are changes on the monitored data

type: object

required:

- nfInstanceId

- callbackReference

- monitoredResourceUris

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

callbackReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

expires:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

#

# SIMPLE TYPES:

#

UeId:

description: UE ID can be MSISDN, IMSI, IMPI or IMPU

anyOf:

- $ref: '#/components/schemas/Msisdn'

- $ref: '#/components/schemas/Imsi'

- $ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsUeId'

Msisdn:

description: MS-ISDN of the UE

type: string

pattern: '^msisdn-[0-9]{5,15}$'

Imsi:

description: IMSI of the UE

type: string

pattern: '^imsi-[0-9]{5,15}$'

#

# ENUMS:

#

UiccType:

description: Indicates whether GBA or GBA_U is to be used

anyOf:

- type: string

enum:

- GBA

- GBA_U

- type: string

* * * Next Change * * * *

## A.6	Nhss_gbaUEAU API

openapi: 3.0.0

info:

version: '1.0.0-alpha.2'

title: 'Nhss_gbaUEAU'

description: |

Nhss UE Authentication Service for GBA.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Home Subscriber Server (HSS) Services, version 17.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-gba-ueau/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nhss-gba-ueau

- {}

paths:

/{ueId}/security-information/generate-auth-data:

post:

summary: Generate GBA authentication data for the UE

operationId: GenerateAuthData

tags:

- Generate Auth Data (Custom Operation)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-ueau

- oAuth2ClientCredentials:

- nhss-gba-ueau

- nhss-gba-ueau:generate-auth-data:invoke

parameters:

- name: ueId

in: path

description: UE identity of the user

required: true

schema:

$ref: '#/components/schemas/UeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AuthenticationInfoRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AuthenticationInfoResult'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-gba-ueau: Access to the nhss-gba-ueau API

nhss-gba-ueau:generate-auth-data:invoke: >

Access to invoke the Generate Auth Data custom operation

schemas:

#

# COMPLEX TYPES:

#

AuthenticationInfoRequest:

description: >

Contains input data to the authentication request message (e.g. authentication

scheme, resynchronization information)

type: object

properties:

authenticationScheme:

$ref: '#/components/schemas/AuthenticationScheme'

resynchronizationInfo:

$ref: 'TS29562_Nhss_imsUEAU.yaml#/components/schemas/ResynchronizationInfo'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

AuthenticationInfoResult:

description: >

Contains authentication information returned in the authentication response message

(e.g. authentication vector, digest authentication parameters)

type: object

properties:

impi:

$ref: '#/components/schemas/Impi'

3gAkaAv:

$ref: 'TS29562_Nhss_imsUEAU.yaml#/components/schemas/3GAkaAv'

digestAuth:

$ref: '#/components/schemas/DigestAuthentication'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

DigestAuthentication:

description: Parameters used for the Digest authentication scheme

type: object

required:

- digestRealm

- digestAlgorithm

- digestQop

- ha1

properties:

digestRealm:

type: string

digestAlgorithm:

$ref: '#/components/schemas/DigestAlgorithm'

digestQop:

$ref: '#/components/schemas/DigestQop'

ha1:

type: string

#

# SIMPLE TYPES:

#

UeId:

$ref: 'TS29562_Nhss_gbaSDM.yaml#/components/schemas/UeId'

Impi:

description: IMS Private Identity of the UE

type: string

#

# ENUMS:

#

AuthenticationScheme:

description: Authentication scheme to be used in the authentication request message

anyOf:

- type: string

enum:

- DIGEST_AKAV1_MD5

- DIGEST_HTTP

- type: string

DigestAlgorithm:

description: Algorithm used for the Digest authentication scheme

anyOf:

- type: string

enum:

- MD5

- MD5_SESS

- type: string

DigestQop:

description: Quality of Protection for the Digest authentication scheme

anyOf:

- type: string

enum:

- AUTH

- AUTH_INT

- type: string

* * * End of Changes * * * *