3GPP TSG-CT WG1 Meeting #136-e	C1-224002

# E-Meeting, 12th – 20th May 2022	(was C1-223941)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.379 | CR | 0023 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCCI_CT |  |  |  |  |  | Date: |  |  | 2022-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Removal of editor’s notes. Explanations are needed for how the IWF gets access to information on groups and clients in the MCPTT system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editor’s notes are removed. Statements are added to clause 5.1 indicating the use by the IWF of the same mechanisms to retrieve user and group information as are used by the MCPTT server. NOTEs indicating that the method used by the IWF to gain information on the MCPTT user and the group homed in the MCPTT system is out of scope are deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification that does not provide a standardized way for the IWF to access needed user and group information from the MCPTT system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.1.8.1, 6.6.3.1.8.2, 6.6.3.1.8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: - Corrected the WI code from eMCCI_CT to MCCI_CT Rev 2: - Removed the changes in all but 3 clauses. - Undeleted the NOTEs - Converted the ENs to NOTEs |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

###### 6.6.3.1.8.1	Determining authorisation for initiating an MCPTT emergency alert

If the IWF performing the controlling role has received a SIP request targeted to an MCPTT group with the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true", the IWF performing the controlling role shall check the following conditions:

1)	if the user is authorized by the IWF to initiate an emergency alert; and

2)	if the IWF allows emergency alerts on the group;

then the MCPTT emergency alert request shall be considered to be an authorised request for an MCPTT emergency alert targeted to an MCPTT group. In all other cases, the MCPTT emergency alert request shall be considered to be an unauthorised request for an MCPTT emergency alert targeted to an MCPTT group.

NOTE 1:	How the IWF authorizes a user to initiate alerts and how the IWF decides to allow emergency alerts on a group is out of scope of the present document.

If the IWF performing the controlling role has received a SIP request targeted to a user with the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true", the IWF performing the controlling role shall check the following condition:

1)	if the calling user is authorized by the IWF for emergency alerts;

then the MCPTT emergency alert request shall be considered to be an authorised request for an MCPTT emergency alert targeted to a user. In all other cases, it shall be considered to be an unauthorised request for an MCPTT emergency alert targeted to a user.

NOTE 2:	How the IWF authorizes a user to initiate alerts to other users is out of scope of the present document.

NOTE 3:	How and whether the IWF obtains user profile information for MCPTT users, and whether the MCPTT system needs access to IWF user profile information is not specified in the present document.

* * * Next Change * * * *

###### 6.6.3.1.8.2	Determining authorisation for initiating an MCPTT emergency group or private call

If the IWF performing the controlling role has received a SIP request for an MCPTT group call with the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true" and:

1)	if the MCPTT user is authorized by the IWF to initiate emergency calls and if the group is configured to allow emergency calls, then the IWF performing the controlling role shall consider the MCPTT emergency group call request to be an authorised request for an MCPTT emergency group call and skip the remaining step; or

NOTE 1:	How the IWF determines whether the user is authorized to initiate an emergency group call and whether the group supports emergency calls is out of scope of the current document.

2)	if the IWF performing the controlling role does not consider the MCPTT emergency group call request to be an authorised request for an MCPTT emergency group call by step 1) above, then the IWF performing the controlling role shall consider the MCPTT emergency group call request to be an unauthorised request for an MCPTT emergency group call.

If the IWF performing the controlling role has received a SIP request for an MCPTT private call with the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true", the IWF performing the controlling role determines whether the user is authorized to initiate an emergency private call.

NOTE 2:	How the IWF determines whether the user is authorized to initiate an emergency private call is out of scope of the current document.

NOTE 3:	How and whether the IWF obtains user profile information for MCPTT users, and whether the MCPTT system needs access to IWF user profile information is not specified in the present document.

* * * Next Change * * * *

###### 6.6.3.1.8.5	Determining authorisation for initiating an MCPTT imminent peril call

If the IWF performing the controlling role has received a SIP request with the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true"; and

1)	if the MCPTT user is authorized by the IWF performing the controlling role to initiate an imminent peril call; and

2)	if the IWF allows the group to support imminent peril calls;

NOTE 1:	How the IWF authorizes the user to initiate imminent peril calls and how the IWF determines whether to allow imminent peril calls on a group is out of scope of the present document.

then the MCPTT imminent peril call request shall be considered to be an authorised request for an MCPTT imminent peril call. In all other cases, it shall be considered to be an unauthorised request for an MCPTT imminent peril call.

NOTE 2:	How and whether the IWF obtains user profile information for MCPTT users, and whether the MCPTT system needs access to IWF user profile information is not specified in the present document.

* * * End of Changes * * * *