3GPP TSG-CT Meeting #96	CP-221229

# Budapest, Hungary, 05th – 06th June 2022                                                   (was C1-224273)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0324 | rev | 2 | Current version: | 17.6.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Group area configuration procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17 |  |  |  |  |  | Date: |  |  | 2022-05-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | C1-223691 specifies configuration parameters related to areas where group members may affiliate and where they are mandated to de-affiliate. This corresponds to a stage 1 requirement since rel-14. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add text how the UE behaves when the conditions in the new configuration parameters in the group document are met. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 2 functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6, 8.2.6.1 (new), 8.2.6.2 (new), 8.2.6.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: changed the name from <permitted-geographic-area> to <mandatory-geographic-area> to be aligned with 24.481 proposed names. Rev 2: Added the condition in 8.2.6.3 that the client is within the area specified in the <permitted-geographic-area> element in order to be allowed to affiliate. Aligned the conditions for affiliation and deaffiliation Added Airbus as cosigning company |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

### 8.2.6	Rules based affiliation status change procedure

##### 8.2.6.1	General

The MCData client can based on configuration decide to affiliate or de-affiliate to a group.

#### 8.2.6.2	User profile defined rules

User profile based affiliation rules are controlled by the elements <RulesForAffiliation> or <RulesForDeaffiliation> of the MCData user profile document identified by the MCData ID of the MCData user (see the MCData user profile document specified in 3GPP TS 24.484 [50]). The rules can be composed of location criteria (including heading and speed) or functional alias based criteria. A rule is fulfilled if  any of the location criteria and any of the functional alias based criteria are met. These rules are evaluated whenever a change of location occurs and whenever a functional alias is activated or deactivated. If any defined rule is fulfilled, the MCData client shall initiate the affiliation status change procedure as specified in clause 8.2.2.

NOTE:	Hysteresis can be applied to location changes to avoid too frequent affiliation changes. In addition, the definition of area entry and exit criteria can be specified to provide a buffer space to minimize ping-ponging into and out of an area.

##### 8.2.6.3	Group configuration defined rules

If the <permitted-geographic-area> element of the <list-service> element of an MCS group document is present and the MCData client is within the area specified in the <permitted-geographic-area> element, the MCData client is allowed to affiliate to the group.

If the <mandatory-geographic-area> element of the <list-service> element of an MCS group document is present and the MCData client is not within the area specified in the <mandatory-geographic-area> element the MCData client shall de-affiliate from the group.

* * * End of Changes * * * *