3GPP TSG-CT Meeting #96	CP-221244

# Budapest, Hungary, 06th – 07th June 2022									(Revision of C3-223742)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.591 | CR | 0080 | rev | 3 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defining FQDN information for EAS deployment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2022-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The FQDNs for EAS Deployment in CT3 are defined as "arrays of FQDNs". In CT3#121e meeting, it was discussed the possibility to allow expressions that match against the received FQDN. Other alternatives based on string matching rules were discussed and the discussion was postponed. With the current data type definition, FQDNs would have to be provisioned individually. The current solution may make sense when the list of FQDNs is small. In order to allow a future proof solution and thus optimize the current solution other alternatives may be considered. This CR proposes to use a common data type to be defined in TS 29.571, FqdnPatternMatchingRule that introduces the FQDN patterns that are allowed. The same approach should be followed in all the APIs that need to provide an extensive list of strings (e.g. FQDNs) that can be optimized with a matching operator condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the attribute fqdns to a new attribute, fqdnPatternList, and change the array of FQDNs to an array of FqdnPatternMatchingRule defined in TS 29.571 to identify the matching conditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned specifications and suboptimal system design. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.1; 5.2.6.2.5; A.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 29.571 CR#0362 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is a backwards compatible correction in the OpenAPI file of the Nnef_EASDeployment API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision to Plenary: Update required property in the OpenAPI file to “fqdnPatternList” |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.2.6.1	General

This clause specifies the application data model supported by the API.

Table 5.2.6.1-1 specifies the data types defined for the Nnef_EASDeployment service based interface protocol.

Table 5.2.6.1-1: Nnef_EASDeployment specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EasDeployInfoNotif | 5.2.6.2.3 | Represents notifications on EAS Deployment Information changes event(s) that occurred for an Individual EAS Deployment Event Subscription resource. |  |
| EasDeploySubData | 5.2.6.2.2 | Represents EAS Deployment Information changes event(s) subscription data. |  |
| EasDeployInfoData | 5.2.6.2.5 | Represents the EAS Deployment Information to be reported. |  |
| EasDepNotification | 5.2.6.2.4 | Represents the EAS Deployment Notifcation. |  |
| EasEvent | 5.2.6.3.3 | represents the EAS event. |  |

Table 5.2.6.1-2 specifies data types re-used by the Nnef_EASDeployment service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_EASDeployment service based interface.

Table 5.2.6.1-2: Nnef_EASDeployment re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DnaiInformation | 3GPP TS 29.522 [15] | Represents the DNAI inormation including the DNS server identifier (consisting of IP address and port) and/or IP address(s) of the EAS in the local DN for the DNAI. |  |
| DnnSnssaiInformation | 3GPP TS 29.522 [15] | Identifies a combination of (DNN, S-NSSAI). |  |
| FqdnPatternMatchingRule | 3GPP TS 29.571 [16] | Represents the FQDN pattern matching rule. |  |
| GroupId | 3GPP TS 29.571 [16] | Contains a Group identifier. |  |
| Uri | 3GPP TS 29.571 [16] | Contains a URI. |  |

# * * * * Next change * * * *

##### 5.2.6.2.5	Type: EasDeployInfoData

Table 5.2.6.2.5-1: Definition of type EasDeployInfoData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appId | string | O | 0..1 | Identifies the application for which the EAS Deployment Information corresponds to. |  |
| dnaiInfos | map(DnaiInformation) | O | 1..N | list of DNS server identifier (consisting of IP address and port) and/or IP address(s) of the EAS in the local DN for each DNAI. The key of map is the DNAI. |  |
| dnn | Dnn | O | 0..1 | DNN for the EAS Deployment Information. |  |
| fqdnPatternList | array(FqdnPatternMatchingRule) | M | 1..N | Supported FQDN pattern(s) for application(s) deployed in the Local part of the DN where each FQDN pattern is described by a FQDN Pattern Matching Rule. |  |
| interGroupId | GroupId | O | 0..1 | Internal Group ID for the EAS Deployment Information. |  |
| snssai | Snssai | O | 0..1 | S-NSSAI for the EAS Deployment Information. |  |

# * * * * Next change * * * *

# A.3	Nnef_EASDeployment API

openapi: 3.0.0

info:

title: Nnef_EASDeployment

version: 1.0.0-alpha.1

description: |

NEF EAS Deployment service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.591 V17.5.0; 5G System; Network Exposure Function Southbound Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.591/

servers:

- url: '{apiRoot}/nnef-eas-deployment/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-eas-deployment

paths:

/subscriptions:

post:

summary: subscribe to notifications

operationId: CreateIndividualSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDeploySubData'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/EasDeploySubData'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nnef-eas-deployment/<apiVersion>/subscriptions/{subscriptionId}.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

notifUri:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDeployInfoNotif'

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: retrieve subscription

operationId: GetIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subscriptionId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/EasDeploySubData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: unsubscribe from notifications

operationId: DeleteIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subscriptionId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-eas-deployment: Access to the Nnef_EASDeployment API

schemas:

EasDeploySubData:

description: Represents an Individual EAS Deployment Event Subscription resource.

type: object

properties:

appId:

type: string

dnnSnssaiInfos:

type: array

items:

$ref: 'TS29522_AMInfluence.yaml#/components/schemas/DnnSnssaiInformation'

minItems: 1

description: Each of the element identifies a (DNN, S-NSSAI) combination.

eventId:

$ref: '#/components/schemas/EasEvent'

eventsNotifs:

type: array

items:

$ref: 'TS29522_EASDeployment.yaml#/components/schemas/EasDeployInfo'

minItems: 1

description: >

Represents the EAS Deployment Information changes event(s) to be reported.

Shall only be present if the "immRep" attribute is included and sets to true,

and the current status of EAS Deployment Information is available.

immRep:

type: boolean

description: >

Indication of immediate reporting. Set to true: requires the immediate reporting of the

current status of EAS Deployment Information, if available. Set to false (default): EAS

Deployment Information event report occurs when the event is met.

interGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

notifId:

type: string

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

required:

- eventId

- notifId

- notifUri

EasDeployInfoNotif:

description: >

Represents notifications on EAS Deployment Information changes event(s) that occurred for an

Individual EAS Deployment Event Subscription resource.

type: object

properties:

easDepNotifs:

type: array

items:

$ref: '#/components/schemas/EasDepNotification'

minItems: 1

notifId:

type: string

required:

- easDepNotifs

- notifId

EasDepNotification:

description: Represents the EAS Deployment Notifcation.

type: object

properties:

easDepInfo:

$ref: '#/components/schemas/EasDeployInfoData'

eventId:

$ref: '#/components/schemas/EasEvent'

required:

- easDepInfo

- eventId

EasDeployInfoData:

description: Represents the EAS Deployment Information to be reported.

type: object

properties:

appId:

type: string

dnaiInfos:

type: object

additionalProperties:

$ref: 'TS29522_EASDeployment.yaml#/components/schemas/DnaiInformation'

minProperties: 1

description: >

list of DNS server identifier (consisting of IP address and port) and/or IP address(s)

of the EAS in the local DN for each DNAI. The key of map is the DNAI.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

fqdnPatternList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/FqdnPatternMatchingRule'

minItems: 1

internalGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- fqdnPatternList

# Simple data types and Enumerations

EasEvent:

anyOf:

- type: string

enum:

- EAS_INFO_CHG

- type: string

description: >

Possible values are

- EAS_INFO_CHG: Indicates that the EAS Deployment Information is changed.

# * * * * End of changes * * * *