3GPP TSG-CT Meeting #98e	CP-223234

# Online, 12th - 14th, December, 2022										(Revision of C3-225588)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0261 | rev | 2 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for CAPIF_API_Invoker_Management_API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF-CT |  |  |  |  |  | Date: |  |  | 2022-12-2 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The APIList data type defined in the OpenAPI file is incorrect and cannot be used. The P column of "serviceAPIDescriptions" attribute in APIList data type needs to be updated to avoid introducing the NBC change in the OpenAPI file. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the definition of APIList data type in the OpenAPI file and update the P column of "serviceAPIDescriptions" attribute. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data type defined is not workable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.4.2.4, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI file for CAPIF_API_Invoker_Management_API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of C3-225588: - Add the description field for APIList data type in the OpenAPI file. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

8.4.4.2.4	Type: APIList

Table 8.4.4.2.4-1: Definition of type APIList

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceAPIDescriptions | array(ServiceAPIDescription) | O | 1..N | Definition of the service API. |  |

# *** Next Change ***

A.5	CAPIF_API_Invoker_Management_API

openapi: 3.0.0

info:

title: CAPIF_API_Invoker_Management_API

description: |

API for API invoker management.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0"

externalDocs:

description: 3GPP TS 29.222 V16.3.0 Common API Framework for 3GPP Northbound APIs

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-invoker-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222

paths:

/onboardedInvokers:

post:

description: Creates a new individual API Invoker profile.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

description: Notify the API Invoker about the onboarding completion

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingNotification'

responses:

'204':

description: No Content (successful onboarding notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: API invoker on-boarded successfully

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/api-invoker-management/v1/onboardedInvokers/{onboardingId}'

required: true

schema:

type: string

'202':

description: The CAPIF core has accepted the Onboarding request and is processing it.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/onboardedInvokers/{onboardingId}:

delete:

description: Deletes an individual API Invoker.

parameters:

- name: onboardingId

in: path

description: String identifying an individual on-boarded API invoker resource

required: true

schema:

type: string

responses:

'204':

description: The individual API Invoker matching onboardingId was offboarded.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an individual API invoker details.

parameters:

- name: onboardingId

in: path

description: String identifying an individual on-boarded API invoker resource

required: true

schema:

type: string

requestBody:

description: representation of the API invoker details to be updated in CAPIF core function

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

description: Notify the API Invoker about the API invoker update completion

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingNotification'

responses:

'204':

description: No Content (successful API invoker update notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'200':

description: API invoker details updated successfully

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

'202':

description: The CAPIF core has accepted the API invoker update details request and is processing it.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

OnboardingInformation:

type: object

properties:

apiInvokerPublicKey:

type: string

description: The API Invoker’s public key

apiInvokerCertificate:

type: string

description: The API Invoker’s generic client certificate, provided by the CAPIF core function.

onboardingSecret:

type: string

description: The API Invoker’s onboarding secret, provided by the CAPIF core function.

required:

- apiInvokerPublicKey

APIList:

type: object

description: Represents a list of APIs.

properties:

serviceAPIDescriptions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: The list of service APIs that the API Invoker is allowed to invoke.

APIInvokerEnrolmentDetails:

type: object

properties:

apiInvokerId:

type: string

description: API invoker ID assigned by the CAPIF core function to the API invoker while on-boarding the API invoker. Shall not be present in the HTTP POST request from the API invoker to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses.

readOnly: true

onboardingInformation:

$ref: '#/components/schemas/OnboardingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: Generic information related to the API invoker such as details of the device or the application.

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- onboardingInformation

- notificationDestination

description: Information about the API Invoker that requested to onboard

OnboardingNotification:

type: object

properties:

result:

type: boolean

description: Set to "true" indicate successful on-boarding. Otherwise set to "false"

resourceLocation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiInvokerEnrolmentDetails:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

apiList:

$ref: '#/components/schemas/APIList'

required:

- result

# *** End of Changes ***