3GPP TSG-CT Meeting #98e	CP-223236

# Online, 12th - 14th, December, 2022										(Revision of C3-225594)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0281 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for data type of CAPIF services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF-CT |  |  |  |  |  | Date: |  |  | 2022-12-2 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The "aefId" attribute in SecurityNotification data type is inconsistent with the OpenAPI file. There are two options for this issue: - Option 1: Change the presence of the attribute from "M" to "O" in the data type. - Option 2: Remove the "aefId" attribute from the data type and the OpenAPI file since there is no stage 2 requirement defined in clause 8.23.2.3 of TS 23.222. Propose to use option 1 as a way forward because this attribute was introduced from Rel-15 and the proposal will not impact the OpenAPI file. - The cardinality of "allowedInvocationTimeRangeList" attribute in ApiInvokerPolicy data type is inconsistent with the OpenAPI file. - The "uri" attribute in Log data type is inconsistent with the OpenAPI file. Propose to change the presence of the attribute from "M" to "O" in the data type because this attribute was introduced from Rel-15 and the proposal will not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the presence of the "aefId" attribute in SecurityNotification data type and "uri" attribute in Log data type. Update the cardinality of "allowedInvocationTimeRangeList" attribute in ApiInvokerPolicy data type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.2.5, 8.6.4.2.3, 8.7.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of C3-225594: - Update the CR number from 5281 to 0281. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

8.5.4.2.5	Type: SecurityNotification

Table 8.5.4.2.5-1: Definition of type SecurityNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerId | string | M | 1 | String identifying the API invoker assigned by the CAPIF core function |  |
| aefId | string | O | 0..1 | String identifying the AEF. |  |
| apiIds | array(string) | M | 1..N | Identifier of the service API |  |
| cause | Cause | M | 1 | The cause for revoking the API invoker authorization to the service API |  |

# *** Next Change ***

8.6.4.2.3	Type: ApiInvokerPolicy

Table 8.6.4.2.3-1: Definition of type ApiInvokerPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerId | string | M | 1 | API invoker ID assigned by the CAPIF core function |  |
| allowedTotalInvocations | integer | O | 0..1 | Total number of invocations allowed on the service API by the API invoker. |  |
| allowedInvocationsPerSecond | integer | O | 0..1 | Invocations per second allowed on the service API by the API invoker. |  |
| allowedInvocationTimeRangeList | array(TimeRangeList) | O | 0..N | The time ranges during which the invocations are allowed on the service API by the API invoker. |  |

# *** Next Change ***

8.7.4.2.3	Type: Log

Table 8.7.4.2.3-1: Definition of type Log

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | String identifying the API invoked. |  |
| apiName | string | M | 1 | Name of the API which was invoked, it is set as {apiName} part of the URI structure as defined in subclause 4.4 of 3GPP TS 29.501 [18]. |  |
| apiVersion | string | M | 1 | Version of the API which was invoked |  |
| resourceName | String | M | 1 | Name of the specific resource invoked |  |
| uri | Uri | O | 0..1 | Full URI of the API resource as defined in subclause 4.4 of 3GPP TS 29.501 [18]. |  |
| protocol | Protocol | M | 1 | Protocol invoked. |  |
| operation | Operation | C | 0..1 | Operation that was invoked on the API, only applicable for HTTP protocol. |  |
| result | string | M | 1 | For HTTP protocol, it contains HTTP status code of the invocation |  |
| invocationTime | DateTime | O | 0..1 | Date on which it was invoked |  |
| invocationLatency | DurationMs | O | 0..1 | Latency for the API invocation. |  |
| inputParameters | ANY TYPE (NOTE) | O | 0..1 | List of input parameters |  |
| OutputParameters | ANY TYPE (NOTE) | O | 0..1 | List of output parameters |  |
| srcInterface | InterfaceDescription | O | 0..1 | Interface description of the API invoker. |  |
| destInterface | InterfaceDescription | O | 0..1 | Interface description of the API invoked. |  |
| fwdInterface | string | O | 0..1 | It includes the node identifier (as defined in IETF RFC 7239 [20] of all forwarding entities between the API invoker and the AEF, concatenated with comma and space, e.g. 192.0.2.43:80, unknown:_OBFport, 203.0.113.60 |  |
| NOTE: Any basic data type defined in OpenAPI 3.0.0 Specification [3] may be used. |  |  |  |  |  |

# *** End of Changes ***