3GPP TSG-CT Meeting #98e	CP-223237

# Online, 12th - 14th, December, 2022										(Revision of C3-225772)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.576 | CR | 0037 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The time stamp of data and analytics notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-12-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In S2-2208989, SA2 agrees that the time stamp is optional provided for requested data or analytics in Nmfaf_3caDataManagement_Notify service operation. And if the MFAF has received the notifications from another source (e.g. NWDAF) without a timestamp, then the MFAF adds itself a timestamp based on the time it received the notification. In current specification, the attributes reused NnwdafEventsSubscriptionNotification and DataNotification data types, while NnwdafEventsSubscriptionNotification shall be provided the time stamp with the condition of ADRF is deployed. Hence, the timestamp is not a mandatory condition for NmfafDataAnaNotification, then the "timeStamp" attribute shall be provied in NmfafDataAnaNotification data type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification for MFAF behaviour when receiving the notifications from another source without a timestamp. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cause wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact openAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of C3-225772: - Remove the change over change and the redundant "timeStamp" attribute in Table 5.2.6.2.4-1. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.2.6.2.4	Type: NmfafDataAnaNotification

Table 5.2.6.2.4-1: Definition of type NmfafDataAnaNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anaNotifications | array(NnwdafEventsSubscriptionNotification) | C | 1..N | List of analytics subscription notifications. (NOTE 1, NOTE x) |  |
| dataNotif | DataNotification | C | 0..1 | Data subscription notification. (NOTE 1, NOTE x) |  |
| NOTE 1: Exactly one of these attributes shall be provided. NOTE x: If the MFAF has received the notifications from another source without a timestamp, then the MFAF adds itself a timestamp based on the time it received the notification in "timeStamp" attribute contained in DataNotification data type or "timeStampGen" attribute contained in NnwdafEventsSubscriptionNotification data type. |  |  |  |  |  |

# *** End of Changes ***